Глава 2 
 Термины и Сокращения

Эта глава содержит и определяет термины и сокращения, используемые в этой спецификацию.

| Access.bus | Шина Access.bus разработана Access.bus Industry Group, основана на технологии Phillips I2 C и программной модели DEC . Изменение 2.2 определяет шину для операции со скоростью 100 КБ, но технология имеет запас(headroom), что позволит повысить до 400 КБ. |
| --- | --- |
| ACK | Подтверждение. Пакет Рукопожатия, указывающий положительное подтверждение. |
| Активное Устройство (Active Device) | Устройство, которое включено и не в приостановленном состоянии. |
| ADB | Смотри Apple Desktop Bus.Служба |
| APM | Управление энергоресурсом (Advanced Power Management). APM - спецификация для управления, остановкой и возобновлением операций с сохранением мощности на хост-системе. |
| Apple Desktop Bus | Последовательная шина, используемая персональными компьютерами, произведенными Apple Computer, Inc. |
| Асинхронные Данные (Asynchronous Data) | Data transferred at irregular intervals with relaxed latency requirements. |
| Асинхронная АС (Asynchronous RA) | The incoming data rate, Fsi, and the outgoing data rate, Fso, of the RA process are independent (i.e., no shared master clock). |
| Асинхронные Asynchronous SRC | The incoming sample rate, Fsi, and outgoing sample rate, Fso, of the SRC process are independent (i.e., no shared master clock). |
| Аудио устройство (Audio Device) | A device that sources or sinks sampled analog data. |
| AWG# | The measurement of a wire’s cross section as defined by the American Wire Gauge standard. |
| Babble | Unexpected bus activity that persists beyond a specified point in a frame. |
| Пропускная способность (Bandwidth) | The amount of data transmitted per unit of time, typically bits per second (bps) or bytes per second (Bps). |
| Big Endian | A method of storing data that places the most significant byte of multiple byte values at a lower storage addresses. For example, a word stored in big endian format places the least significant byte at the higher address and the most significant byte at the lower address. Смотри Little Endian. |
| Бит (Bit) | Единица информации, используемая цифровыми компьютерами. Представляет самый маленький фрагмент адресуемой памяти внутри компьютера. Бит предоставляет выбор между двумя возможностями и обычно представляется логическими единицей (1) или нулем (0). |
| Вставка бит (Bit Stuffing) | Вставка из “0” битов в поток данных, чтобы вызвать электрический переход на проводах данных, позволяя PLL оставыться блокированный. |
| Bps | Скорость передачи, выраженная в байтах за секунду. |
| Буфер (Buffer) | Storage used to compensate for a difference in data rates or time of occurrence of events, when transmitting data from one device to another. |
| Передача типа Bulk (Bulk Transfer) | Nonperiodic, large burst y communication typically used for a transfer that can use any available bandwidth and also be delayed until bandwidth is available. |
| Перенумерация шины (Bus Enumeration) | Обнаружение и идентификация устройств Универсальной Последовательной Шины. |
| Байт (Byte) | Элемент данных, размер которого восемь бит. |
| Возможности (Capabilities) | Those attributes of a Universal Serial Bus device that are administerable by the host. |
| Характеристики (Characteristics) | Those qualities of a Universal Serial Bus device that are unchangeable; for example, the device class is a device characteristic. |
| CHI | Concentration Highway Interface. CHI - полнодуплексный последовательный интерфейс с мультиплексированием сигнала, предназначенный для передачи голоса (звука) в цифровом виде по четырехпроводной линии. В настоящее время спецификация поддерживает скорости передачи данных до 4.096 МБ. |
| Клиент (Client) | Software resident on the host that interacts with host software to arrange data transfer between a function and the host. The client is often the data provider and consumer for transferred data. |
| COM порт (COM Port) | Communications port. On personal computers, an eight-bit asynchronous serial port is typically used. |
| Configuring Software | The host software responsible for configuring a Universal Serial Bus device. This may be a system configurator or software specific to the device. |
| Control Pipe | Тоже самое что и поток структурированных сообщений (message pipe). |
| Control Transfer | One of four Universal Serial Bus Transfer Types. Control transfers support configuration/command/status type communications between client and function. |
| CRC | Смотри Cyclic Redundancy Check. |
| CTI | Computer Telephony Integration. |
| Циклический Контроль по Избыточности (Cyclic Redundancy Check) | Проверка, выполняемая над данными, чтобы увидеть, произошла ли ошибка в передаче, чтении, или записи данные. Результат CRC обычно сохраняется или передается с проверенными данными. Сохраненный или переданный результат сравнивается с CRC, расчитаным для переданных данных, чтобы определить, произошла ли ошибка. |
| Default Address | An address defined by the Universal Serial Bus Specification and used by a Universal Serial Bus device when it is first powered or reset. The default address is 00h. |
| Создаваемый по умолчанию канал (Default Pipe) | Канал сообщения, созданный программным обеспечением системы Универсальной Последовательной Шины для прохождения управляющих данных и информации состояния между хостом и Конечной точкой 0 устройства Универсальной Последовательной Шины. |
| Устройство (Device) | A logical or physical entity that performs a function. The actual entity described depends on the context of the reference. At the lowest level, device may refer to a single hardware component, as in a memory device. At a higher level, it may refer to a collection of hardware components that perform a particular function, such as a Universal Serial Bus interface device. At an even higher level, device may refer to the function performed by an entity attached to the Universal Serial Bus; for example, a data/FAX modem device. Devices may be physical, electrical, addressable, and logical. When used as a non-specific reference, a Universal Serial Bus device is either a hub or a function. |
| Device Address | The address of a device on the Universal Serial Bus. The Device Address is the Default Address when the Universal Serial Bus device is first powered or reset. Hubs and functions are assigned a unique Device Address by Universal Serial Bus software. |
| Device Endpoint | A uniquely identifiable portion of a Universal Serial Bus device that is the source or sink of information in a communication flow between the host and device. |
| Device Resources | Resources provided by Universal Serial Bus devices, such as buffer space and endpoints. Смотри Host Resources and Universal Serial Bus Resources. |
| Device Software | Software that is responsible for using a Universal Serial Bus device. This software may or may not also be responsible for configuring the device for use. |
| DMI | An acronym for Desktop Management Interface. A method for managing host system components developed by the Desktop Management Task Force. |
| Downstream | The direction of data flow from the host or away from the host. A downstream port is the port on a hub electrically farthest from the host that generates downstream data traffic from the hub. Downstream ports receive upstream data traffic. |
| Драйвер (Driver) | When referring to hardware, an I/O pad that drives an external load. When referring to software, a program responsible for interfacing to a hardware device; that is, a device driver. |
| DWORD | Двойное слово. Элемент данных, размер которого 2 слова, 4 байта, или 32 бита. |
| Dynamic Insertion and Removal | The ability to attach and remove devices while the host is in operation. |
| E2PROM | Смотри EEPROM. |
| СППЗУ (EEPROM) | Электрически Стираемое Программируемое Постоянное Запоминающее Устройство. Энергонезависимая переперезаписываемая технология хранения в памяти. |
| Конечный пользователь (End User) | Пользователь хоста. |
| Конечная точка (Endpoint) | Смотри Device Endpoint. |
| Endpoint Address | The combination of a Device Address and an Endpoint Number on a Universal Serial Bus device. |
| Endpoint Number | A unique pipe endpoint on a Universal Serial Bus device. |
| EOF1 | End of frame timing point #1. Used by the hub to monitor and disconnect bus activity persisting near or past the end of a frame. |
| EOF2 | End of frame timing point #2. Used by hubs to detect bus activity near the end of frame. |
| EOP | Конец пакета. |
| Fs | Смотри Sample Rate. |
| False EOP | A spurious, usually noise induced, event that is interpreted by a packet receiver as an end of packet. |
| FireWire | Стандарт шины реализованный Apple Computer’s, тоже что и IEEE P1394. |
| Frame | The time from the start of one SOF token to the start of the subsequent SOF token; consists of a series of transactions. |
| Frame Pattern | A sequence of frames that exhibit a repeating pattern in the number of samples transmitted per frame. For a 44.1 kHz audio transfer, the frame pattern could be nine frames containing 44 samples followed by one frame containing 45 samples. |
| Полнодуплексная (Full-duplex) | Компьютерная передача данных, позволяющая передовать в обоих направлениях одновременно. |
| Функция (Function) | Устройство USB Шины, которое обеспечивает возможности хоста. Например, ISDN соединение, цифровой микрофон, или колонки. |
| GeoPort | Последовательная шина, разработанная Apple Computer Inc. В настоящее время спецификация GeoPort поддерживает скорости передачи данных до 2 МБ и обеспечивает соединение точка точка в радиусе 4 футов. |
| Пакет Квитирования (Handshake Packet) | A packet that acknowledges or rejects a specific condition. For examples, see ACK and NACK. |
| Хост (Host) | Главная компьютерная система, где установлен хост контроллер USB Шины. В его входит аппаратная платформа (Центральный Процессор, шина, и т.д.) и операционная система. |
| Хост-контроллер (Host Controller) | Интерфейс USB Шины хоста. |
| Драйвер Хост-контроллера (Host Controller Driver) | The Universal Serial Bus software layer that abstracts the host controller hardware. Host Controller Driver provides an SPI for interaction with a host controller. Host Controller Driver hides the specifics of the host controller hardware implementation. |
| Host Resources | Resources provided by the host, such as buffer space and interrupts. Смотри Device Resources and Universal Serial Bus Resources. |
| Концентратор (Hub) | Устройство USB Шины, которое обеспечивает дополнительные соединения с USB Шиной. |
| Hub Tier | The level of connect within a USB network topology given as the number of hubs that that the data has to flow through. |
| I2C | Acronym for the Inter-Integrated Circuits serial interface. The I2C interface was invented by Philips Semiconductors. |
| IEEE P1394 | A high performance serial bus. The P1394 is targeted at hard disk and video peripherals, which may require bus bandwidth in excess of 100 Mb/s. The bus protocol supports both isochronous and asynchronous transfers over the same set of four signal wires. |
| Industry Standard Architecture | The 8 and/or 16 bit expansion bus for IBM AT or XT compatible computers. |
| Integrated Services Data Network | An internationally accepted standard for voice, data, and signaling using public, switched telephone networks. All transmissions are digital from end-to-end. Includes a standard for out-of-band signaling and delivers significantly higher bandwidth than POTS. |
| Interrupt Request | A hardware signal that allows a device to request attention from a host. The host typically invokes an interrupt service routine to handle the condition which caused the request. |
| Передача Прерывания (Interrupt Transfer) | Один из четырех Типов Передачи Универсальной Последовательной Шины. Характеристики Передачи Прерывания - мало данных, не периодические, низко частотные, ограниченное время ожидания, связь инициализируется устройством, обычно используется, чтобы сообщить хосту о необходимости обслуживания устройства. |
| IRQ | Смотри Interrupt Request. |
| ISA | Смотри Industry Standard Architecture. |
| ISDN | Смотри Integrated Services Data Network. |
| Isochronous Data | A stream of data whose timing is implied by its delivery rate. |
| Isochronous Device | An entity with isochronous endpoints, as defined in the USB specification, that sources or sinks sampled analog streams or synchronous data streams. |
| Isochronous Sink Endpoint | An endpoint that is capable of consuming an isochronous data stream. |
| Isochronous Source Endpoint | An endpoint that is capable of producing an isochronous data stream. |
| Изохронная Передача (Isochronous Transfer) | Один из четырех Типов Передачи Универсальной Последовательной Шины. Isochronous transfers are used when working with isochronous data. Isochronous transfers provide periodic, continuous communication between host and device. |
| Флуктуация (Jitter) | A tendency toward lack of synchronization caused by mechanical or electrical changes. More specifically, the phase shift of digital pulses over a transmission medium. |
| kbs | Скорость передачи, выраженная в килобитах за секунду. |
| kBs | Скорость передачи, выраженная в килобайтах за секунду |
| Line Printer Port | A port used to access a printer. On most personal computers, an eight-bit parallel interface is typically used. |
| Little Endian | Method of storing data that places the least significant byte of multiple byte values at lower storage addresses. For example, a word stored in little endian format places the least significant byte at the lower address and the most significant byte at the next address. Смотри Big Endian. |
| LOA | Loss of bus activity characterized by a start of packet without a corresponding end of packet. |
| LPT Порт (LPT Port) | Смотри Line Printer Port. |
| LSB | Самый младший Бит. |
| Mbs | Скорость Передачи, выраженная в мегабитах за секунду. |
| MBs | Скорость Передачи, выраженная в мегабайтах за секунду. |
| Канал Сообщений (Message Pipe) | A pipe that transfers data using a request/data/status paradigm. The data has an imposed structure which allows requests to be reliably identified and communicated. |
| Micro Channel Architecture | A 32 bit expansion bus used on some IBM PS/2 compatible computers. |
| Модем (Modem) | An acronym for Modulator/Demodulator. Component that converts signals between analog and digital. Typically used to send digital information from a computer over a telephone network which is usually analog. |
| MSB | Старший Бит. |
| NACK | Отрицательное Подтверждение. Квитирование указывающее отрицательное подтверждение. |
| Без Возврата к Нулю, с Инвертированием (Non Return to Zero Invert) | A method of encoding serial data in which ones and zeroes are represented by opposite and alternating high and low voltages where there is no return to zero (reference) voltage between encoded bits. Eliminates the need for clock pulses. |
| NRZI | Смотри Non Return to Zero Invert. |
| Объект (Object) | Host software or data structure representing a Universal Serial Bus entity. |
| Пакет (Packet) | A bundle of data organized in a group for transmission. Packets typically contain three elements: control information (e.g., source, destination, and length), the data to be transferred, and error detection and correction bits. |
| Packet Buffer | The logical buffer used by a Universal Serial Bus device for sending or receiving a single packet. This determines the maximum packet size the device can send or receive. |
| Пакет ID (Packet ID) | Поле в пакете Универсальной Последовательной Шины, которое указывает тип пакета, и логический вывод формата пакета и типа обнаружения ошибок, примененные к пакету. |
| PBX | Смотри Private Branch eXchange. |
| PCI | Смотри Peripheral Component Interconnect. |
| PCMCIA | Смотри Personal Computer Memory Card Industry Association. |
| Peripheral Component Interconnect | 32- или 64-разрядная, независимая от процессора расширенная шина, используемая на персональных компьютерах. |
| Personal Computer Memory Card International Association | Организация, которая стандартизирует и поддерживает технологию PC Card. |
| Фаза (Phase) | Маркер, данные, или пакет квитирования; транзакция имеет три фазы. |
| Физическое устройство (Physical Device) | Устройство, которое физичеси реализовано; например колонки, микрофоны и проигрыватели CD. |
| PID | Смотри Packet ID. |
| Канал (Pipe) | Логическая абстракция, представляющая соединение между оконечной точкой на устройстве и программном обеспечении на хосте. Канал имеет несколько атрибутов; например, через канал можно передать данные как поток в канале (Stream Pipe) или как канал сообщений (Message Pipe). |
| Plain Old Telephone Service | Basic service supplying standard single line telephones, telephone lines, and access to public switched networks. |
| Plug and Play | Технология для конфигурирования устройств ввода - вывода, предназначена чтобы использовать не-конфликтующие ресурсы в хосте. Ресурсы, управляемые PnP включают адресные интервалы ввода - вывода, адресные интервалы памяти, вектора прерываний, и каналы ПДП(DMA). |
| PnP | Смотри Plug and Play. |
| Polling | Asking multiple devices, one at a time, if they have any data to transmit. |
| POR | Смотри Power On Reset. |
| Порт (Port) | Место доступа к или из системы или схемы. Для USB Шины, место куда присоединенно USB устройство. |
| POTS | Смотри Plain Old Telephone Service. |
| Power On Reset | Restoring a storage device, register, or memory to a predetermined state when power is applied. |
| PLL | Phase Locked Loop. A circuit that acts as a phase detector to keep an oscillator in phase with an incoming frequency. |
| Private Branch eXchange | A privately owned telephone switching system which is not regulated as part of the public telephone network. |
| Programmable Data Rate | Either a fixed data rate (single frequency endpoints), a limited number of data rates (32 kHz, 44.1 kHz, 48 kHz, É), or a continuously programmable data rate. The exact programming capabilities of an endpoint must be reported in the appropriate class-specific endpoint descriptors. |
| Протокол (Protocol) | Специфический набор правил, процедур, или соглашений в отношении формата и синхронизации передачи данных между двумя устройствами. |
| RA | Смотри Rate Adaptation. |
| Адаптация Скорости (Rate Adaptation) | The process by which an incoming data stream, sampled at Fsi is converted to an outgoing data stream, sampled at Fso with a certain loss of quality, determined by the rate adaptation algorithm. Error control mechanisms are required for the process. Fsi and Fso can be different and asynchronous. Fsi is the input data rate of the RA; Fso is the output data rate of the RA. |
| Запрос (Request) | A request made to a Universal Serial Bus device contained within the data portion of a SETUP packet. |
| Retire | The action of completing service for a transfer and notifying the appropriate software client of the completion. |
| Корневой концентратор Root Hub | A Universal Serial Bus hub directly attached to the host controller. This hub is attached to the host; tier 0. |
| Корневой порт Root Port | Upstream порт на концентраторе. |
| Sample | Самый маленький модуль данных, с которым оперирует конечная точка; собственность(property) конечной точки. |
| Sample Rate (Fs) | Число выборок(samples) за секунду, выраженное в Герцах. |
| Sample Rate Conversion | A dedicated implementation of the RA process for use on sampled analog data streams. The error control mechanism is replaced by interpolating techniques. |
| SCSI | Смотри Small Computer Systems Interface. |
| Сервис (Service) | Процедура, обеспеченная SPI. |
| Service Interval | The period between consecutive requests to a Universal Serial Bus endpoint to send or receive data. |
| Service Jitter | The deviation of service delivery from its scheduled delivery time. |
| Service Rate | The number of services to a given endpoint per unit time. |
| Small Computer Systems Interface | Локальная шина ввода - вывода, которая позволяет присоединять периферийне устройства к хосту, используя общие аппаратные средства системы и программное обеспечение. |
| SOF | Start of Frame - Начало Кадра. SOF - первая транзакция в каждом кадре. SOF позволяет конечным точкам обозначить начало кадра и синхронизировать внутренние часы конечной точки хосту. |
| SPI | Смотри System Programming Interface. |
| SRC | Смотри Sample Rate Conversion. |
| Стадия (Stage) | Одна часть последовательности, составляющей передачу управления; то есть, стадия установки, стадия данных, и стадия состояния. |
| Поток в канале (Stream Pipe) | Канал, который передает данные как поток выборок не придерживаясь определенной структуры USB. |
| Synchronization Type | A classification that characterizes an isochronous endpointÕs capability to connect to other isochronous endpoints. |
| Synchronous RA | The incoming data rate, Fsi, and the outgoing data rate, Fso, of the RA process are derived from the same master clock. There is a fixed relation between Fsi and Fso. |
| Synchronous SRC | The incoming sample rate, Fsi, and outgoing sample rate, Fso, of the SRC process are derived from the same master clock. There is a fixed relation between Fsi and Fso. |
| System Programming Interface | Определенный интерфейс услуг, обеспечиваемый программным обеспечением системы. |
| TDM | Смотри Time Division Multiplexing. |
| Согласовательные сопротивления (Termination) | Пассивные компоненты, присоединяемые к концам кабеля, чтобы предотвратить отражение сигнала или отображения помех на экране. |
| Мультиплексирование с временным уплотнением (Time Division Multiplexing) | A method of transmitting multiple signals (data, voice, and/or video) simultaneously over one communications medium by interleaving a piece of each signal one after another. |
| Time-out | The detection of a lack of bus activity for some predetermined interval. |
| Token Generator | Смотри Initiator. |
| Маркерный пакет (Token Packet) | A type of packet that identifies what transaction is to be performed on the bus. |
| Транзакция (Transaction) | The delivery of service to an endpoint; consists of a token packet, optional data packet, and optional handshake packet. Specific packets are allowed/required based on the transaction type. |
| Передача (Transfer) | One or more bus transactions to move information between a software client and its function. |
| Тип передачи (Transfer Type) | Determines the characteristics of the data flow between a software client and its function. Four Transfer types are defined: control, interrupt, bulk, and isochronous. |
| Turnaround Time | The time a device needs to wait to begin transmitting a packet after a packet has been received to prevent collisions on Universal Serial Bus. This time is based on the length and propagation delay characteristics of the cable and the location of the transmitting device in relation to other devices on Universal Serial Bus. |
| Universal Serial Bus | Набор устройств USB, программного и аппаратного обеспечения, который позволяет соединять возможности, имеющихся функциий хоста. |
| Universal Serial Bus Device | Includes hubs and functions. Смотри device. |
| Universal Serial Bus Interface | The hardware interface between the Universal Serial Bus cable and a Universal Serial Bus device. This includes the protocol engine required for all Universal Serial Bus devices to be able to receive and send packets. |
| Universal Serial Bus Resources | Resources provided by Universal Serial Bus, such as bandwidth and power. Смотри Device Resources and Host Resources. |
| Universal Serial Bus Software | Основное программное обеспечение хоста, ответственное за управление взаимодействиями между хостом и присоединенными к Универсальной Последовательной Шине устройствами. |
| USB | Смотри Universal Serial Bus. |
| USBD | Смотри Universal Serial Bus Driver. |
| Universal Serial Bus Driver | The host resident software entity responsible for providing common services to clients that are manipulating one or more functions on one or more Host Controllers. |
| Upstream | The direction of data flow towards the host. An upstream port is the port on a device electrically closest to the host that generates upstream data traffic from the hub. Upstream ports receive downstream data traffic. |
| Virtual Device | A device that is represented by a software interface layer; e.g., a hard disk with its associated device driver and client software that makes it able to reproduce an audio .WAV file. |
| WFEOF2 | Wait for EOF2 point. One of the four hub repeater states. |
| WFEOP | Wait for end of packet. One of the four hub repeater states. |
| WFSOF | Wait for start of frame. One of the four hub repeater states. |
| WFSOP | Wait for start of packet. One of the four possible hub repeater states. |
| Word | Элемент данных, размер которого составляет два байта или 16 бит. |