Цель работы: Ознакомиться с сетевой утилитой Ethereal, изучить его настройки и принципы конфигурирования.

Программа Ethereal представляет собой анализатор сетевого трафика. Ethereal предлагает все выгоды средства командной строки, такого как Tcpdump, а также ряд дополнительных преимуществ. Он обладает удобным пользовательским графическим интерфейсом, поэтому не нужно изучать все опции командной строки. Кроме того, он предлагает значительно больше аналитических и статистических возможностей. К числу достоинств Ethereal относятся:

- Более ясный формат вывода. По сравнению с необработанными наборами перехваченных пакетов в Tcpdump, выдачу Ethereal значительно легче читать и понимать.

- Поддержка значительно большего числа форматов протоколов. Ethereal может интерпретировать более 300 различных сетевых протоколов, что охватывает почти все когда-либо изобретенные виды сетей.

- Поддержка большего числа физических форматов сетей. Сюда входят новые протоколы, такие как IP поверх ATM и FDDI.

- Возможность интерактивно просматривать и сортировать перехваченные сетевые данные.

- Возможность сохранения выдачи в виде обычного текста или в формате PostScript.

- Наличие режима фильтрации вывода с широкими возможностями, включая выделение цветом некоторых пакетов. Имеется графический интерфейс создания фильтра, облегчающий данный процесс.

- Возможность следить за потоком TCP и просматривать его содержимое в текстовом виде. Это может быть очень полезно, когда требуется читать межсерверные сообщения, чтобы отслеживать проблемы электронной почты или Web. Данная возможность позволяет оперативно следить за общением между взаимодействующими узлами.

- Возможность работать с рядом программ и библиотек перехвата. Ethereal, помимо libpcap, работает также со специализированным оборудованием. В число поддерживаемых программ входят Sniffer и Sniffer Pro от Network Associates; LANalyser от Novell; некоторые устройства от Cisco, Lucent и Toshiba; некоторые беспроводные устройства анализа, такие как NetStumbler и Kismet Wireless. Ethereal работает как встраиваемый модуль для многих из этих программ и устройств.

- Возможность сохранять сеансы в нескольких форматах. Это полезно, если вы хотите проводить дополнительный анализ с помощью других средств, таких как libcap (по умолчанию), Sun Snoop, Microsoft Network Monitor и Sniffer от Network Associates.

- Поддержка терминального режима командной строки, предназначенного для тех, кому не по душе графический интерфейс, хотя подавляющая часть полезных свойств Ethereal проистекает из его графического инструментария.

Ethereal настолько полезен в качестве сетевого средства, что он был оценен на web-сайте Insecure.org, посвященном безопасности, как занимающий вторую позицию по популярности среди доступных средств сетевой безопасности. Помимо собственно безопасности, Ethereal допускает множество применений; на самом деле его можно использовать и как универсальное средство анализа сети.

Настройка фильтра может осуществляться командами, аналогичными TCPDump в левом верхнем окне, как показано на рисунке 1:

Рис.1 Настройка фильтра.

Ftp-пакеты, перехваченные с помощью Ethereal:

Листинг 1. пакет-запрос протокола ftp:

No.     Time        Source                Destination           Protocol Info

37 12.112659   192.168.254.254       172.16.0.4            FTP      Response: 220 Welcome to the eagle-server FTP service.

Frame 37 (100 bytes on wire, 100 bytes captured)

Ethernet II, Src: Cisco_cb:46:d4 (00:14:f2:cb:46:d4), Dst: DellEsgP_4e:e5:8f (00:0b:db:4e:e5:8f)

Destination: DellEsgP_4e:e5:8f (00:0b:db:4e:e5:8f)

Source: Cisco_cb:46:d4 (00:14:f2:cb:46:d4)

Type: IP (0x0800)

Internet Protocol, Src: 192.168.254.254 (192.168.254.254), Dst: 172.16.0.4 (172.16.0.4)

Version: 4

Header length: 20 bytes

Differentiated Services Field: 0x00 (DSCP 0x00: Default; ECN: 0x00)

Total Length: 86

Identification: 0x652b (25899)

Flags: 0x04 (Don't Fragment)

Fragment offset: 0

Time to live: 62

Protocol: TCP (0x06)

Header checksum: 0x6bbb [correct]

Source: 192.168.254.254 (192.168.254.254)

Destination: 172.16.0.4 (172.16.0.4)

Transmission Control Protocol, Src Port: ftp (21), Dst Port: 1066 (1066), Seq: 1, Ack: 1, Len: 46

File Transfer Protocol (FTP)

220 Welcome to the eagle-server FTP service.\r\n

Response code: Service ready for new user (220)

Response arg: Welcome to the eagle-server FTP service.

Листинг 2. Пакет-ответ протокола ftp:

No.     Time        Source                Destination           Protocol Info

40 12.180490   172.16.0.4            192.168.254.254       FTP      Request: USER anonymous

Frame 40 (70 bytes on wire, 70 bytes captured)

Ethernet II, Src: DellEsgP_4e:e5:8f (00:0b:db:4e:e5:8f), Dst: Cisco_cb:46:d4 (00:14:f2:cb:46:d4)

Destination: Cisco_cb:46:d4 (00:14:f2:cb:46:d4)

Source: DellEsgP_4e:e5:8f (00:0b:db:4e:e5:8f)

Type: IP (0x0800)

Internet Protocol, Src: 172.16.0.4 (172.16.0.4), Dst: 192.168.254.254 (192.168.254.254)

Version: 4

Header length: 20 bytes

Differentiated Services Field: 0x00 (DSCP 0x00: Default; ECN: 0x00)

Total Length: 56

Identification: 0x058f (1423)

Flags: 0x04 (Don't Fragment)

Fragment offset: 0

Time to live: 128

Protocol: TCP (0x06)

Header checksum: 0x0000 [incorrect, should be 0x8975]

Source: 172.16.0.4 (172.16.0.4)

Destination: 192.168.254.254 (192.168.254.254)

Transmission Control Protocol, Src Port: 1066 (1066), Dst Port: ftp (21), Seq: 1, Ack: 47, Len: 16

File Transfer Protocol (FTP)

USER anonymous\r\n

Request command: USER

Request arg: anonymous

Листинг 3. Пакет данных протокола ftp:

No.     Time        Source                Destination           Protocol Info

139 16.726189   192.168.254.254       172.16.0.4            FTP-DATA FTP Data: 256 bytes

Frame 139 (310 bytes on wire, 310 bytes captured)

Arrival Time: Nov 23, 1996 10:52:01.537374000

Time delta from previous packet: 0.050264000 seconds

Time since reference or first frame: 16.726189000 seconds

Frame Number: 139

Packet Length: 310 bytes

Capture Length: 310 bytes

Protocols in frame: eth:ip:tcp:ftp-data

Coloring Rule Name: TCP

Coloring Rule String: tcp

Ethernet II, Src: Cisco_cb:46:d4 (00:14:f2:cb:46:d4), Dst: DellEsgP_4e:e5:8f (00:0b:db:4e:e5:8f)

Destination: DellEsgP_4e:e5:8f (00:0b:db:4e:e5:8f)

Address: DellEsgP_4e:e5:8f (00:0b:db:4e:e5:8f)

.... ...0 .... .... .... .... = Multicast: This is a UNICAST frame

.... ..0. .... .... .... .... = Locally Administrated Address: This is a FACTORY DEFAULT address

Source: Cisco_cb:46:d4 (00:14:f2:cb:46:d4)

Address: Cisco_cb:46:d4 (00:14:f2:cb:46:d4)

.... ...0 .... .... .... .... = Multicast: This is a UNICAST frame

.... ..0. .... .... .... .... = Locally Administrated Address: This is a FACTORY DEFAULT address

Type: IP (0x0800)

Internet Protocol, Src: 192.168.254.254 (192.168.254.254), Dst: 172.16.0.4 (172.16.0.4)

Version: 4

Header length: 20 bytes

Differentiated Services Field: 0x00 (DSCP 0x00: Default; ECN: 0x00)

Total Length: 296

Identification: 0xf041 (61505)

Flags: 0x04 (Don't Fragment)

0... = Reserved bit: Not set

.1.. = Don't fragment: Set

..0. = More fragments: Not set

Fragment offset: 0

Time to live: 62

Protocol: TCP (0x06)

Header checksum: 0xdfd2 [correct]

Good: True

Bad : False

Source: 192.168.254.254 (192.168.254.254)

Destination: 172.16.0.4 (172.16.0.4)

Transmission Control Protocol, Src Port: 49226 (49226), Dst Port: 1072 (1072), Seq: 1, Ack: 1, Len: 256

Source port: 49226 (49226)

Destination port: 1072 (1072)

Sequence number: 1    (relative sequence number)

Next sequence number: 257    (relative sequence number)

Acknowledgement number: 1    (relative ack number)

Header length: 20 bytes

Flags: 0x0018 (PSH, ACK)

0... .... = Congestion Window Reduced (CWR): Not set

.0.. .... = ECN-Echo: Not set

..0. .... = Urgent: Not set

...1 .... = Acknowledgment: Set

.... 1... = Push: Set

.... .0.. = Reset: Not set

.... ..0. = Syn: Not set

.... ...0 = Fin: Not set

Window size: 5840

Checksum: 0x8174 [correct]

FTP Data

FTP Data: drwxr-xr-x   13 0        0            4096 Mar 30  2007 eagle1\r\ndrwxr-xr-x    2 0        0            4096 Jan 12  2007 eagle2\r\ndrwxr-xr-x    2 0        0            4096 Jan 12  2007 eagle3\r\ndrwxr-xr-x    2 0        0