DTD 5

ARTICLE IN PRESS

Computer Communications xx (2005) 1–7

www.elsevier.com/locate/comcom

Abstract

End-site routing support for IPv6 multihoming1*

Marcelo Bagnulo*, Alberto Garcıґa-Martıґnez, Juan Rodrıґguez, Arturo Azcorra

Universidad Carlos III de Madrid, Av. Universidad, 30. Leganeґs, Madrid, Spain

Multihoming is currently widely used to provide fault tolerance and trafﬁc engineering capabilities. It is expected that, as

telecommunication costs decrease, its adoption will become more and more prevalent. Current multihoming support is not designed to scale

up to the expected number of multihomed sites, so alternative solutions are required, especially for IPv6. In order to preserve interdomain

routing scalability, the new multihoming solution has to be compatible with Provider Aggregatable addressing. However, such addressing

scheme imposes the conﬁguration of multiple preﬁxes in multihomed sites, which in turn causes several operational difﬁculties within those

sites that may even result in communication failures when all the ISPs are working properly. In this paper we propose the adoption of Source

Address Dependent routing within the multihomed site to overcome the identiﬁed difﬁculties.

q 2005 Published by Elsevier B.V.

Keywords: Multihomed site; CIDR; Internet Service Providers

1. Introduction

Since the communications of a wide range of organiz-

ations rely on the Internet, access links are a critical resource

to them. As a result, sites are improving the fault tolerance

and QoS capabilities of their Internet access through multi-

homing, i.e. the achievement of global connectivity through

several connections supplied by different Internet Service

Providers (ISPs). However, the extended usage of the

currently available IPv4 multi-homing solution is jeopardiz-

ing the future of the Internet since it has become a major

contributor to the post-CIDR growth in the number of global

routing table entries [1]. Therefore, in IPv6, the usage of

Provider Aggregatable (PA) addressing is recommended for

all sites, included multihomed ones, in order to preserve

inter domain routing system scalability. While such

addressing architecture reduces the amount of routing

table entries in the Default Free Zone	of the Internet, its

adoption presents a fair number of challenges for the

*

This work has been partly supported by the European Union under the

E-Next Project FP6-506869 and by OPTINET6 project TIC-2003-09042-

C03-01.

* Corresponding author.

E-mail addresses:	marcelo@it.uc3m.es (M. Bagnulo), alberto@it.

uc3m.es (A. Garcıґa-Martıґnez), jrh@it.uc3m.es (J. Rodrıґguez), azcorra@

it.uc3m.es (A. Azcorra).

0140-3664/$ - see front matter q 2005 Published by Elsevier B.V.

doi:10.1016/j.comcom.2005.08.010

end-sites, especially for those who multihome. Essentially,

when a PA addressing is adopted, a multihomed site will

have to conﬁgure multiple addresses, one per ISP, in every

node of the site, in order to be reachable through all its

providers. Such conﬁguration pose quite a number of

challenges for its adoption, since current hosts are not

prepared to deal with multiple addresses per interfaces as it

is required. In this paper, we will present how	Source

Address Dependent (SAD) routing can be adopted to deal

with some of the difﬁculties imposed in this conﬁguration.

The rest of this paper is structured as follows: First we

will present the rationale for adopting SAD routing within

multihomed sites. Then, we will detail the different

conﬁgurations of SAD routing that may be required in

different sites, including some trials performed and next we

will present the capabilities of the resulting conﬁguration.

Finally we will present the conclusions of this work.

2. Rationale

2.1. Current IPv4 multihoming technique and capabilities

As mentioned above, a site is multi-homed when it

obtains Internet connectivity through two or more service

providers. Through multi-homing an end-site improves the

fault tolerance of its connection to the global network and it

can also perform	Trafﬁc Engineering (TE)	techniques to

DTD 5

2

PS

Internet

Default Free Zone

ARTICLE IN PRESS

M. Bagnulo et al. / Computer Communications xx (2005) 1–7

PS	PX (BGP)

Internet

PY (BGP

ISP X

PS

SiteS

Prefix: PS

ISP Y

PS

ISP X

Prefix: PX

Site S

PXsite:H1

PYsite:H1

ISP Y

Prefix: PY

Fig. 1. BGP route injection for the provision of multihoming in IPv4

networks.

select the path used to reach the different networks

connected to the Internet.

In IPv4, the most widely deployed multi-homing solution

is based in the announcement of the site preﬁx through all its

providers. In this conﬁguration, the site S obtains a Provider

Independent (PI)	preﬁx allocation directly from the

Regional Internet Registry. Then, the site announces this

preﬁx to its providers using BGP [2]. Then the multihomed

site providers announce the preﬁx to its providers and so on,

so that eventually the route is announced in the Default Free

Zone (Fig. 1).

This mechanism provides fault tolerance capabilities,

including preserving established connections throughout

an outage. In addition, the following TE tools are

available to the multihomed site: The multihomed site

can deﬁne which one of the available exit paths will be

used to carry outgoing trafﬁc to a given destination, by

proper conﬁguration of the	LOCAL_PREFERENCE	attri-

bute of BGP [3]. For incoming trafﬁc, the multihomed site

can inﬂuence the ISP through which it prefers to receive

trafﬁc by using the	AS prepending	technique, which

consists in artiﬁcially making the route through one of the

providers less attractive to external hosts by adding AS in

the	AS_PATH	attribute of BGP	[3]. (It should be noted

that in this case, the ultimate decision of which ISP will

be used to forward packets to the site belongs to the

external site that is actually forwarding the trafﬁc).

While the IPv4 multihomed solution presented provides

fairly good features regarding to fault tolerance and TE, it

presents very limited scalability properties with respect to

the interdomain routing system. Because of the usage of PI

addressing by the multihomed sites, each multi-homed site

using this solution contributes with routes to the Default

Free Zone routing table, imposing additional stress to

already oversized routing tables. Additionally, the increase

of the number of ASs contributing to the BGP routing table

resulting from PI addressing may affect to the time required

for ﬁnding a valid path after a failure, since this time

depends on the length of the longest path between the site

originating the preﬁx and the site computing the path [4].

Finally, as the number of the ASs grows, the frequency of

additions and withdraws of routes also increases, leading to

a more unstable system. For this reason, more scalable

Fig. 2. Provider aggregation of end-site preﬁxes.

multi-homing solutions are being explored for IPv6 [5], in

particular solutions that are compatible with the usage of PA

addressing in multihomed sites, as it will be presented next.

2.2. Provider aggregation and multi-homing

In order to reduce the routing table size, the usage of PA

addressing is required. This means that sites obtain preﬁxes

which are part of their provider’s allocation, so that its

provider only announce the complete aggregate to their

providers, and they do not announce preﬁxes belonging to

other ISP aggregates, as presented in Fig. 2.

When provider aggregation of end-site preﬁxes is used,

each end-site host interfaces obtains one IP address from

each allocation, in order to be reachable through all the

providers and beneﬁt from multi-homing capabilities since

ISPs will only forward trafﬁc addressed to their own

aggregates.

This conﬁguration presents several concerns as it will be

presented next.

†	Ingress ﬁltering	[6]	is a widely used technique for

preventing the usage of spoofed addresses. However, in

the described conﬁguration, its usage presents additional

difﬁculties for the source address selection mechanism

and intra-site routing systems, since the exit path and

source address of the packet must be coherent with the

path, in order to bypass ingress ﬁltering mechanisms.

†	Difﬁculties in establishing communication in case of

failure. When	Link1	or	Link3	becomes unavailable,

addresses containing the PXsite preﬁx are unreachable

from the Internet.

†	Established connections will not be preserved in case of

outage. If	Link1	or	Link3	fails, already established

connections that use addresses containing PXsite preﬁx

will fail, since packets addressed to the PXsite aggregate

will be dropped because there is no route available for

this destination. Note that an alternative path exists, but

the routing system is not aware of it.

The presented difﬁculties show that additional mechan-

isms are needed in order allow the usage of PA addresses

while still provide incumbent multi-homing solution

DTD 5

ARTICLE IN PRESS

M. Bagnulo et al. / Computer Communications xx (2005) 1–7

3

equivalent beneﬁts. In this paper, we will explore the

possibility of using Source Address Dependent routing as a

tool to help to overcome the identiﬁed difﬁculties.

3. Source address dependent (SAD) routing

Source Address Dependent (SAD) routing essentially

means that routers maintain as many routing tables as source

ISPA

SAD

Non-SAD

ISPB

ISPC

ISPD

address preﬁxes involved, and the packet is routed

according to the routing table corresponding to the source

address preﬁx that best matches the source address

contained in the packet (Fig. 3). The idea of using SAD

routing for site multihoming is presented in [7].

SAD routing can be used to provide ingress ﬁltering

compatibility when routing packets ﬂowing from the

multihomed site to the Internet. In this case, the source

address of the exiting packets has been determined by the

host that initiated the communication (the host in the

multihomed site or the external host through the selection of

the destination address of the initial packet) and then the

routing system will forward the packet to the appropriate

exit router in order to guarantee ingress ﬁltering compat-

ibility. The source address selection determines the ISP to

be used for routing packets. Because of address ﬁltering, the

source address determines the forward path from the

multihomed homed to the rest of the Internet. Additionally,

as the source address used in the initial packets will become

the destination address of the reply packets, this selection

determines the ISP to be used in the reverse path.

Since source address selection implies ISP selection, the

adoption of SAD routing will also affect the mechanisms to

be used in multihomed sites to deﬁne TE. In particular, it

will shift TE capabilities from the routing system to the

hosts themselves.

We will next evaluate the adoption of SAD routing in

two typical multihomed conﬁgurations: sites running BGP

but without redistributing the BGP information into IGP,

and sites running an IGP to select the exit path. There is

an additional possible conﬁguration using static routes in

Fig. 4. SAD routing domain.

the multihomed site. However, this last conﬁguration is

fairly simple and several commercial routers already

support it, so we will not provide a full description of it.

Nevertheless, it should be noted that when SAD routing is

used, it is possible to obtain fault tolerance and TE

capabilities in sites with static routing since those features

are now supported by the hosts themselves and not by the

routing system.

In order to enable SAD routing in a multihomed site,

SAD routing support is not required in all the routers within

the site, but SAD routing has to be adopted creating a

connected SAD routing domain that contains all the exit

routers—see Fig. 4—as presented in [7].

Note that it is not necessary for the generic routing

domain to be connected, i.e. it can be formed by a set of

disconnected domains, all connected to the SAD routing

domain.

3.1. Sites running BGP without redistribution of BGP

information into IGP

Current IPv4 multihomed sites usually run BGP with

their providers. Through BGP, they obtain reachability

information from each of their ISPs. However, because

operational issues, some sites do not redistribute the

information obtained through BGP into the IGP [3]. So, in

order to be able to properly select the intra site path towards

an external destination, they include all the routers that are

required to properly select the exit path in the IBGP mesh,

including not only site exit routers, but also other internal

routers that have access to multiple exit routers. This means

ISP X

Prefix: PX

Router X

ISP Y

Prefix: PY

Router Y

that the IBGP cloud is wrapping the non-BGP aware routing

domain, as presented in Fig. 5.

It should be noted that only the IBGP mesh must be

connected, and that the non-BGP aware may be formed by

multiple disconnected domains, only connected by the

IBGP domain. It is clear that only the routers included in the

Multihomed

site

Source pref. PX

0 /0 –Router X

Internalroutes: …

Source pref. PY

0 /0 –Router Y

Internalroutes: …

Other source pref.

Internal routes: …

IBGP mesh need to implement SAD routing in order to

properly select the site exit path. So, since all these routers

are running BGP, we can use BGP capabilities to provide

SAD routing support.

In order to implement SAD routing, each exit router

that is running EBGP has to attach a color tag to the

Fig. 3. SAD routing.

routes received from the ISP, so that it is possible to

DTD 5

4

ISPA

ISPB

ARTICLE IN PRESS

M. Bagnulo et al. / Computer Communications xx (2005) 1–7

Com_i	to all the routes received from	ISPi, when

ISPC

announcing them through IBGP.

IBGP

full mesh

No BGP

Fig. 5. IBGP full mesh.

ISPD

The resulting behavior is that each router within the

IBGP mesh will have separate routing tables containing the

information learned through each ISP. Packets containing a

source address with the preﬁx of the	ISPi will be routed

using the corresponding routing table. An example is shown

in Fig. 6.

3.2. Sites using IGP

In this scenario, the multihomed site is using an IGP to

identify the routes learned through each different ISP.

Additionally, once the routing information is colored, it is

necessary to map each of the colors to a source address

preﬁx. Once that both color and its correspondent preﬁx

information is available, it is possible to construct SAD

routing tables that contain routing information per source

preﬁx.

SAD routing can be implemented in this scenario using

the BGP Communities	[8]	attribute to color the routing

information. So, we assume a multihoming scenario where a

multihomed site has n ISPs, each one of them has assigned

Pref_i to the multihomed site, with iZ1,...,n. So, in order to

adopt SAD routing it is required that:

First, a private community value is assigned to each

different ISP. Therefore,	Com_i	value is assigned to the

routes obtained from ISPi, being iZ1,..,n

Second, n routing tables are created in each of the routers

involved, so that each router has one routing table per preﬁx

in the site (i.e. per ISP). Additionally each router is

conﬁgured to route packets containing a source address

matching Pref_i using the routing table i.

Third, BGP processing rules are conﬁgured in each

router, so that routes containing a community attribute value

equal to Com_i only affect routing table i.

Finally, each exit router that is peering with an external

router in ISPi is conﬁgured to attach the community value

inform about both internal and external destinations. The

IGP learns about external destinations in one of the

following three ways:

– Manually conﬁgured routes are imported into the

IGP

– BGP redistribution into the IGP

– IGP exchange with the providers

As in the previous case, the whole multihomed site

routing system is not required to support SAD routing but

only a connected domain that has to contain all the exit

routers. However, while BGP provides mechanisms to tag

routing information so that the same protocol instance can

be used to propagate information with different scopes, as

presented in the previous section, current IGPs do not

provide such capability.

So, in order to provide SAD routing support, different

instances of the routing protocol run in parallel, each one of

them associated with a source address preﬁx. In this way,

different instances of the IGP will update different routing

tables within the routers. The main difﬁculty with this

approach is how to differentiate messages corresponding to

the different instances of the IGP. Normally, different

instances of the IGP run in different interfaces, so that each

instance only receives its own messages. But in this case we

want to run multiple instances of the IGP in the same

Fig. 6. SAD routing deployment using BGP communities.

DTD 5

ARTICLE IN PRESS

M. Bagnulo et al. / Computer Communications xx (2005) 1–7

5

interfaces, so we need a way to separate messages according

to the instance of the IGP they belong to.

OSPF for IPv6 [9]	explicitly supports running multiple

instances in the same link and packets belonging to different

instances are identiﬁed using the Instance_ID ﬁeld in

the OSPF header.

Another possibility is to send IGP messages using global

addresses as source addresses. Usually, IGP messages are

sent using link local addresses. But, since each router can be

conﬁgured with multiple IP addresses, one per preﬁx, the

router includes different source addresses in the messages

corresponding to different instances of the IGP. This ships-

in-the-night strategy would allow each IGP instance to

believe that they are running alone in the link

3.3. Experimenting with SAD routing

We will next analyze the deployability of the approach

by evaluating the available support for SAD routing in

current implementations. In order to asses the deployment

effort required to adopt the proposed solution we have built

a testbed with widely available commercial routers and

perform some trials in the framework of the Optinet6

research project. The testbed evaluated the capabilities to

support SAD routing of Cisco 2500 routers, Cisco 7500

routers and Juniper M10 routers.

All the tested routers support static SAD routing, i.e.

routing based on the source address of the packets according

to statically deﬁned routes. However, the implementation of

the SAD routing support differs considerably between them.

Cisco IOS supports static SAD routing through manually

deﬁned rules that affect the processing of packets, called

route-maps. In order to enable SAD routing, route-maps

corresponding to each source address dependent route have

to be deﬁned. On the other hand, Juniper routers support

multiple routing tables, so that it is possible to create as

many routing tables as source address preﬁxes are involved,

and then deﬁne the required rules so that the router will

forward packets according to the routing table associated

with the preﬁx contained in the source address. In the case of

static SAD routing, the multiple routing tables are

conﬁgured manually with the desired static routes.

Regarding dynamic SAD routing, the support provided

by Cisco routers is very limited. Because SAD routing is

supported as a manually deﬁned route-map, and because

route-map deﬁnition is mainly a manual process performed

by the router operator, Cisco routers cannot update the

routing information (i.e. route-maps) involved in the SAD

routing. This means, that neither the BGP nor the IGP case

are supported by this router vendor.

Because Juniper routers support multiple parallel routing

tables, the support for dynamic SAD routing is provided

more naturally. In the case of BGP, it is needed that different

routing tables are updated depending on the values of

the community attribute contained in the BGP route. While

this seems pretty straightforward, it is not currently supported

by Juniper routers because of the existent constraint that

imposes that a given instance of a routing protocol can only

update a single routing table, making not viable that the BGP

instance can update different routing tables based on the

value of the community attribute. Such limitation does not

apply for the IGP case, since the considered approach

proposes the usage of multiple instances of the IGP running

simultaneously, one per source preﬁx involved, and that each

instance of the IGP updates its corresponding routing table.

This conﬁguration is currently supported in Juniper routers

for OSPFv2 and also for BGP. It should be noted that this

approach can be used as a temporary solution for the BGP

case i.e. running multiple instances of BGP in parallel while

the community based approach is not available.

4. Resulting capabilities

4.1. Fault tolerance capabilities

As we have presented earlier, the main motivation for

multihoming is an improvement of the fault tolerance

capabilities of the Internet access. The enhanced fault

tolerance features must allow the site to communicate in the

event of an outage affecting one of the available Internet

accesses. In this section we will see that the availability of

the SAD routing in the multihomed site enables the

provision of the required fault tolerance capabilities.

Since the basic assumption behind adopting SAD routing

for multihoming support is that the source address is

determined by the initiating host, and that each source address

preﬁx determines an exit ISP, fault tolerance capabilities will

be provided by the hosts themselves. As described in the Host

Centric Approach [7], such mechanisms are based on the trial

and error procedure. Considering that each source address

available in a host is bound to an exit path, the host can

try different exit paths by changing the source address. The

main difference between the fault tolerance provision of the

different approaches for SAD routing presented in this paper is

how fast the host can learn that a destination address is

unreachable through the selected path.

When external routes are static, the intra site routing

system has no external reachability information, so the

packet will be forwarded outside the site and only when it

reaches routers that have richer knowledge about the

topology, it will be possible to determine whether the

requested destination is reachable through the selected path.

In the worst case, the initiating host will timeout and will

retry with a different path.

When the multihomed site runs BGP or an IGP with its

providers, reachability information is available closer

to the host, i.e. in the site’s routers, so in some cases

the unreachability feedback will be provided faster than in

the general case, where unreachability information is

learned through timeouts. So, the host will attempt to use

one of its source addresses to reach a certain destination.

DTD 5

6

ARTICLE IN PRESS

M. Bagnulo et al. / Computer Communications xx (2005) 1–7

The packet will be routed through the generic routing

domain to the SAD routing domain. Once there, the routers

will determine whether the selected destination is reachable

with the selected source address. This means that a route to

the selected destination exists in the routing table associated

with the selected source address preﬁx. The possible

resulting behaviors are:

If the selected destination is reachable through the

selected source address, then the packet is forwarded

towards the site exit router that leads to the ISP

corresponding to the source address preﬁx selected.

If the selected destination is not reachable through the

selected source address, but it is reachable through an

alternative source address, then the packet is discarded and

an	ICMP Destination Unreachable	with	Code 5	which

means Source Address Failed Ingress Policy	[10] is sent

back to the host. The information about the proper source

address preﬁx can be included in this message, for instance

in the source address of the ICMP message. The host will

then retry using the suggested source address.

If the selected destination is unreachable, the packet is

discarded and an ICMP destination unreachable is sent back

to the host. In this case, the host may retry if an alternative

destination address is available.

4.2. Trafﬁc engineering (TE) capabilities

As a consequence of using multiple preﬁxes in multi-

homed sites in conjunction with SAD routing, the party

selecting the address of the multihomed host to be used

during the communication is the party that determines the

ISP to be used for the packets involved in this communi-

cation. So, TE mechanisms will have to inﬂuence such

selection. It must be noted, that addresses used in a

communication are determined by the party initiating the

communication, so in this environment, policy mechanisms

will not affect incoming and outgoing trafﬁc separately as in

the IPv4 case, but they will affect packets belonging to

externally initiated communication and packets belonging

to internally initiated communications differently. This is

the ﬁrst difference with the IPv4 case.

4.2.1. TE for externally initiated communications

When a host outside the multihomed hosts attempts to

initiate a communication with a host within the multihomed

site, it ﬁrst obtains the set of destination addresses, and then

it selects one according to the Default Address Selection

procedure [11]. It seems then that the only point where the

multihomed site can express TE considerations is through

the DNS server replies. The DNS server can be conﬁgured

in order to modify the order of the addresses returned to

express some form of TE constraint.

This mechanism can work ﬁne to provide some form of

load balancing and load sharing. The DNS server can be

conﬁgured so that x% of the queries are replied with an

address with preﬁx of ISPA ﬁrst and the rest of the times

(100Kx)% are replied with an address with preﬁx of ISPB

ﬁrst. In addition SRV [12] records can be used to provide

enhanced capabilities by those applications that support

them. When the host receives the list of addresses, it will

process them according to RFC3484. If none of the rules

described works, the list is unchanged and the ﬁrst address

received is tried ﬁrst. Note that the list may be changed by

the address selection algorithm because of the host policies.

4.2.2. TE for internally initiated communications

For internally initiated communications, the exit ISP is

determined by the source address included in the initiating

packet. This means that the source address selection

mechanism	[11]	will determine the exit ISP. RFC 3484

deﬁnes a policy table that can be conﬁgured in order to

express TE considerations. The policy table allows a ﬁne-

grained policy deﬁnition where a source address can be

matched with a destination address/preﬁx, allowing most of

the required policy conﬁgurations.

5. Conclusions

In this paper we have presented the case for the adoption

of SAD routing in multihomed environments. The scal-

ability limitations of the current multihoming solution based

on the usage of Provider Independent addressing have been

largely acknowledged by the Internet community, and only

a new multihoming solution compatible with PA addressing

will preserve IPv6 inter domain routing system scalability.

However, the adoption of PA addressing in multihomed

environments implies that multihomed sites need to

internally conﬁgure as many preﬁxes as providers they

multihome to, causing several difﬁculties, such as incom-

patibilities with ingress ﬁltering, incapability to preserve

established connections through outages and so on and so

forth. This is basically due to the fact that when multiple PA

preﬁxes are present in the multihomed site, the source

address selection process determines the ISP to be used in

the communication. This is so because in order to preserve

ingress ﬁltering compatibility, the packet has to be

forwarded through the ISP that is compatible with the

selected source address. Current destination address based

routing does not take into account the source address of the

packet, making it unsuitable to provide ingress ﬁltering

compatibility that is source address related. SAD routing is

then the natural option to overcome the difﬁculties caused

by ingress ﬁltering. Moreover, once that SAD routing is

available on the multihomed site, it is possible to obtain

additional beneﬁts such as fault tolerance and trafﬁc

engineering capabilities with a reduced complexity. SAD

routing is not a new technology and it is available in some

limited form in current router implementation, which

facilitates its adoption and deployment. However, SAD

routing currently is a special feature whose applicability was

limited to very speciﬁc scenarios.

DTD 5

ARTICLE IN PRESS

M. Bagnulo et al. / Computer Communications xx (2005) 1–7

7

References

[1] G. Huston, Commentary on inter-domain routing in the internet, RFC

3221, 2001.

[2] Y. Rekhter, T. Li, A border gateway protocol 4 (BGP-4), RFC 1771,

1995.

[3] I. Van Beijnum, BGP, Oreilly, 2002.

[4] C. Labovitz, R. Wattenhofer, S. Venkatachary, A. Ahuja, The

impact of internet policy and topology on delayed routing

convergence Proceedings of INFOCOM 2001, Anchorage, Alaska,

2000.

[5] J. Abley, B. Black, V. Gill, Goals for IPv6 site-multihoming

architectures, RFC 3582, 2003.

[6] P. Ferguson, D. Senie, Network ingress ﬁltering: defeating denial of

service attacks which employ IP source address spooﬁng, RFC 2827,

2000.

[7] C. Huitema, R. Draves, M. Bagnulo, Host-Centric IPv6 Multihoming,

Internet-Draft (Work in proress).

[8] R. Chandra, P. Traina, T. Li, BGP communities attribute, RFC 1997,

1996.

[9] R. Coltun, D. Ferguson, J. Moy, OSPF for IPv6, RFC 2740, 1999.

[10] A. Conta, S. Deering, Internet control message protocol (ICMPv6) for

the internet protocol version 6 (IPv6) speciﬁcation, RFC 2463, 1998.

[11] R. Draves, Default address selection for internet protocol version 6

(IPv6), RFC 3484, 2003.

[12] A. Gulbrandsen, P. Vixie, L. Esibov, A DNS RR for specifying the

location of services (DNS SRV), RFC 2782, 2003.