Computer Networks 40 (2002) 131–148

www.elsevier.com/locate/comnet

Engineering a multiservice IP backbone to support tight SLAs

Clarence Filsﬁlsa,*, John Evansb

a	Cisco Systems, Brussels, Belgium

b	Cisco Systems Ltd., 10 New Square Park, Bedfont Lakes, Feltham, Middlesex, UK W14 8HA

Abstract

This paper describes technologies that enable IP service providers to oﬀer tighter service level agreements for IP

performance, in order to create competitive advantage and better serve their customers. The SLA parameters that need

to be tightened are deﬁned and then the technologies that should be considered are described, together with the decision

criteria on where each technology shouldbe used. This paper is basedupon current best practise andincludes results

from both lab testing and deployment experience. The speciﬁc technologies discussed are diﬀerentiated services, fast

IGP convergence, and traﬃc engineering. Consideration is given to how these technologies should be deployed and

operated.

2002 Elsevier Science B.V. All rights reserved.

IDT: Diﬀerentiatedservices; MPLS; Traﬃc engineering; IGP convergence

1. Introduction

When using public transport, the traveller may

beneﬁt from contractual commitments from the

transport service provider, for example that 95%

of journeys will arrive within 5 min of the sched-

uledtime. The commitments may include other

parameters or metrics such as number of stops en

route andany meals included. The more compet-

itive the market for the particular service, the more

comprehensive andthe tighter the commitments or

service level agreements that are oﬀered. In the

same way, the increase of competition between IP

service providers (SPs) together with the height-

enedimportance of IP to business operations has

* Corresponding author.

E-mail addresses:	cﬁlsﬁl@cisco.com,	cf@cisco.com (C.

Filsﬁls), joevans@cisco.com (J. Evans).

ledto an increaseddemandandconsequent supply

of IP services with tighter service level agreements

(SLAs) for IP performance.

The IP technical community has developed a set

of technologies that enable IP networks to be en-

gineeredto support tight SLA commitments:

•	Diﬀerentiated services (Diﬀserv).	The Diﬀserv

architecture allows diﬀerentiated delay, jitter

andloss commitments to be supportedon the

same IP backbone for diﬀerent types or classes

of service.

•	Faster IGP convergence.	New developments in

Interior Gateway routing protocols (IGPs) al-

low for faster convergence upon link or node

failure, hence enabling higher service availabil-

ity to be oﬀered.

•	MPLS traﬃc engineering.	MPLS traﬃc engi-

neering (Diﬀserv-aware or not) introduces con-

straint-basedrouting andadmission control to

1389-1286/02/$ - see front matter	2002 Elsevier Science B.V. All rights reserved.

PII: S 1 3 8 9 - 1 2 8 6 ( 0 2 ) 0 0 2 7 1 - 2

132

C. Filsﬁls, J. Evans / Computer Networks 40 (2002) 131–148

IP backbones. This allows optimum use to be

made of the installed backbone bandwidth ca-

pacity, or conversely allows the same level of

service to be oﬀeredfor less capacity. It can also

be usedto ensure that the amount of low-jitter

traﬃc per link does not exceed a speciﬁed max-

imum.

•	MPLS traﬃc engineering fast reroute.	MPLS

traﬃc engineering fast reroute is an IP protec-

tion technique that enables connectivity to be

restoredaroundlink andnode failures in a

few tens of milliseconds.

For an SP IP service, the SLA commitments are

basedon delay, jitter, packet loss rate, throughput

andavailability. This paper focuses on deﬁning

these SLA parameters, anddescribing why and

how the above technologies shouldbe usedin

order for these SLA parameters to be tightened.

Consideration is also given to how networks using

these technologies shouldbe operated. There are

other factors that will aﬀect a service provider’s

ability to oﬀer tight SLAs in addition to the

technologies described, but which are not covered

in this paper; these include: network security and

BGP convergence.

In focussing on service provider IP backbone

networks, it is notedthat the mechanisms em-

ployed at the edge of the network to deliver tight

SLAs may be diﬀerent from those used in the core.

In the backbone, where traﬃc is aggregated, SLA

requirements for a traﬃc class can be translated

into the appropriate bandwidth requirements, and

the problem of SLA assurance can eﬀectively be

reduced to that of bandwidth provisioning. At the

network edge, other considerations, such as seri-

alisation delay, become signiﬁcant. The mecha-

nisms employedat the edge of the network are not

considered further in this paper.

2. Customer requirements—what an SLA should

commit to

The following metrics are highlightedas the

most important for specifying the quality of an IP

backbone service [15], For each metric, typical

values are given for diﬀerent types or classes of

service.

2.1. Network one-way delay

In IP backbone terms, network one-way delay

characterizes the time diﬀerence between the re-

ception of an IP packet at an ingress point of

presence (POP) andits transmission at an egress

POP. Network one-way delay is made up of four

components:

•	Propagation delay.	Propagation delay is con-

strainedby the speedof light in a medium and

for optical ﬁbre is around5 ms per 1000 km.

Propagation delay can vary as network topol-

ogy changes, when a link fails, for example, or

when an underlying network (e.g. SDH/SO-

NET) reroutes its circuit paths.

•	Switching delay. Switching or processing delay

is the time diﬀerence between receiving a packet

on an incoming router interface andthe enqueu-

ing of the packet in the scheduler of its out-

bound interface. Switching delays on today’s

high-performance routers are negligible, typi-

cally in the order of 10–20 ls per packet.

•	Scheduling delay. Scheduling (or queuing) delay

is deﬁned as the time diﬀerence between the en-

queuing of a packet on the outboundinterface

scheduler, and the start of clocking the packet

onto the outboundlink. This is a function of

the scheduling algorithm used and of the sched-

uler queue utilization, which is in turn a func-

tion of the queue capacity andthe oﬀered

traﬃc loadandproﬁle. This eﬀect is analysed

in more detail later in this paper.

•	Serialisation delay. Serialisation delay is the time

taken to clock a packet onto a link andis depen-

dent upon the link speed and the packet size. Se-

rialisation delay is considered negligible at link

speeds above STM-1/OC3, such as backbone

links: a 1500 byte packet is clockedat STM-1/

OC3 rate (155 Mbps) in 80	ls, at STM-16/

OC48 rate (2.5 Gbps) in 5	ls andat STM-64/

OC192 rate (10 Gbps) in 1.25 ls.

The goal commonly usedin designing networks

to support voice over IP (VoIP) is a delay budget

C. Filsﬁls, J. Evans / Computer Networks 40 (2002) 131–148

133

of 150 ms from mouth to ear.1A design should

apportion this budget to the various components

of network delay (propagation delay through the

backbone, scheduling delay due to congestion, and

the access link serialisation delay) and service

delay (due to VoIP gateway codec and de-jitter

buﬀer). Propagation delay is often budgeted by

using the widest diameter in the network, which

for example, in a national network in the US would

give a worst case (coast-to-coast) of 6000 km or 30

ms of one-way propagation delay.

Interactive data applications often require a

roundtrip time (RTT) of less than 250 ms to allow

for a smooth interaction between the human and

the application server.

SPs today typically commit on monthly average

one-way delay over all their POP-to-POP pairs.

2.2. Network delay-jitter

Network delay-jitter characterizes the variation

of network delay; it is generally computed as the

variation of the delay for two consecutive packets.

Jitter is causedby the variation in the components

of delay previously described:

•	Propagation delay. Propagation delay can vary

as network topology changes, when a link fails,

for example, or when the topology of a lower

layer network (e.g. SDH/SONET) changes,

causing a sudden peak of jitter. Current IP

backbone experience wouldsuggest that these

occurrences are more common than is generally

believed[6].

•	Switching delay.	Switching delay can vary as

some pacakets might require more process-

ing than others. This eﬀect is becoming less of

a consideration as packet switching is imple-

mentedusing hardware pipelines whose switch-

ing delay characteristics are deterministic.

•	Scheduling delay. Variation in scheduling delay

is causedas scheduling queues oscillate between

empty andfull.

1	ITU standard G.114 states that 150 ms of end-to-end one-

way delay does not cause a perceivable degradation in voice

quality for most use of telephony.

Jitters buﬀers (also known as play-out buﬀers)

are usedto remove delay variation by turning

variable network delays into constant delays at the

destination end systems. Consequently, in net-

works that are engineeredto support low-delay

services such as VoIP it is important that they are

also engineeredfor low jitter. IP backbones that

are engineeredto support high-quality VoIP ser-

vices typically budget for 5–10 ms of jitter in the

backbone; assuming 10 backbone hops, this gives

a jitter budget per hop of 500–1000 ls. Schedulers

that implement a priority queuing mechanism such

as the ‘‘modiﬁed’’2Deﬁcit roundrobin (mDRR)

scheduler [25] implemented on high-performance

routers have jitter characteristics 5–10 times better

than this. Data applications do not generally re-

quire speciﬁc constraints on jitter.

2.3. Loss

Loss characterizes the packet drops that occur

between the ingress link of the ingress POP andthe

egress link of the egress POP. It is observedthat

US backbone service providers usually oﬀer an

average monthly loss rate of less than 1%. Back-

bones engineeredfor high-quality VoIP services

typically plan for a loss rate of less than 0.25%.

The same target range is also usedfor high-quality

data services; TCP throughput has been shown to

decrease as an inverse of the square root of the

probability of packet loss [19].

2.4. Bandwidth and throughput

IP services are commonly soldwith a deﬁned

bandwidth, where the bandwidth reﬂects the ac-

cess link capacity provisionedfor the service. De-

ﬁned bandwidth may not be the same as achieved

throughput, however. Throughput characterises

the available user bandwidth between an ingress

POP andegress POP. The requirement for this

SLA parameter is obvious for point-to-point ser-

vices such as virtual wires, as being deﬁned by the

Pseudo Wire Emulation Edge to Edge (PWE3)

2	The DRR algorithm usedon Cisco routers has been

modiﬁed by Cisco to add support for a strict priority queue.

134

C. Filsﬁls, J. Evans / Computer Networks 40 (2002) 131–148

Real-time video is commonly impacted by re-

ordering as the end-user applications either do not

have the time to reorder the received frames or

simply do not support this functionality; in both

cases, reordering means a higher packet loss. Most

TCP implementations interpret the receipt of three

consecutive similar acknowledgements as an in-

dication of packet loss and hence retransmit the

next packet and slow down their rate; reordering

therefore has a signiﬁcant impact on TCP through-

put [18]. Whilst the reordering magnitude would

needto be very signiﬁcant in order to aﬀect a VoIP

ﬂow whose inter-packet gap is a multiple of 10 ms,

the reordering might cause signiﬁcant jitter due to

Fig. 1. TCP throughput as a function of packet loss andRTT

with MSS ј 1460 bytes.

Working Group [22] within the IETF. For multi-

point-to-multipoint services, such as VPNs, the

SLA deﬁnition will need to exclude cases where the

loss of throughput is due to customer-based ag-

gregation. For example, where 10 sites each with

STM-1/OC3 access links are all sending traﬃc at

full rate to a single site with only an STM-4/OC12

access link [9].

For TCP/IP traﬃc, achievedthroughput is lar-

gely dependent upon the probability of packet loss

andthe achievedRTT. Consequently, contracted

bandwidth may not relate to achieved throughput.

The graph in Fig. 1 uses the relationship deﬁned by

Mathis [19] to show how TCP throughput varies

as a function of packet loss andRTT.

2.5. Per-ﬂow sequence preservation

Per-ﬂow sequence preservation is not yet a

common component of IP service SLA commit-

ments even though it is acceptedbest practise in IP

network design. Due to the impact that packet

reordering can have on some applications, how-

ever, it is starting to be oﬀeredby SPs for high-

quality data and video services. To prevent out of

order packet delivery, it is important that any load

balancing is achievedat a per-ﬂow level rather

than at a per packet level, andscheduling algo-

rithms assure that packets from the same ﬂow are

servicedin order andfrom the same queue.

variation in the propagation delay (for example,

US east coast to west coast via a north or the south

route).

2.6. Availability

Availability for IP services is generally deﬁned

in one of two ways; in terms of either network

availability or service availability. Network avail-

ability is deﬁned as the fraction of time that net-

work connectivity is available between a speciﬁed

ingress point anda speciﬁedegress point. Service

availability is deﬁned as the fraction of time the

service is available between a speciﬁedingress point

anda speciﬁedegress point within the bounds of

the deﬁned SLAs. There can be overlap between

the deﬁnition of network or service availability

andthe deﬁnition of other SLA parameters, for

example, 100% network availability implies 0%

packet loss.

2.7. Classes of service

Amongst the QoS enabledIP services oﬀeredby

service providers today there is common support

for classes of service designed to meet the needs of

three traﬃc types: real-time traﬃc, business data

traﬃc andbest-eﬀort data traﬃc. Typical SLA

commitments andservice characteristics for these

classes are as follows:

•	Real-time. This class targets applications such as

VoIP and video. It is deﬁned in terms of low loss

(less than 0.25%), low delay, and low jitter (typ-

C. Filsﬁls, J. Evans / Computer Networks 40 (2002) 131–148

135

ically 5 ms within the backbone) andhas a spec-

iﬁed bandwidth and availability. Attainable

throughput is derived from bandwidth and loss.

The class may support a commitment for per-

ﬂow sequence preservation.

•	Business data. This class represents business crit-

ical interactive applications such as SNA, SAP

R/3, Telnet, andpossibly intranet Web applica-

tions to selectedURLs. It is deﬁnedin terms of

delay (RTT should be less than 250 ms), and loss

(less than 1% loss rate is typical, with targets of

less than 0.1% also available), with a speciﬁed

bandwidth and availability. Attainable through-

put is derivedfrom loss andRTT. Jitter is not

important for this service class andis not de-

ﬁned. The class may support a commitment for

per-ﬂow sequence preservation.

•	Best-eﬀort data.	This class represents all other

traﬃc that has not been classiﬁedas	real-time

or	business. It is deﬁned in terms of a loss rate

with a speciﬁed bandwidth and availability. At-

tainable throughput is derived from loss. Delay

andjitter are not important for this service and

are not deﬁned. The class may support a com-

mitment for per-ﬂow sequence preservation.

2.8. Design objectives

In designing an IP backbone network with

support for diﬀerent classes of traﬃc, service pro-

viders have three key design objectives:

•	Committing to the diﬀerent per class SLA re-

quirements.

•	Making optimal use of available bandwidth.

•	Keeping the design as simple as possible.

In the remainder of this paper, we describe the

diﬀerent technologies that are required to satisfy

these requirements.

3. The diﬀerentiated services architecture

3.1. IP backbone diﬀserv overview

The Diﬀserv architecture [3] is the preferred

technology for large-scale IP QoS deployments

today, such as service provider backbone networks.

Diﬀserv achieves scalability through performing

complex QoS functions such as classiﬁcation,

marking, and conditioning operations at the edges

of the network. Traﬃc is classiﬁedandthen

markedusing the Diﬀserv code point (DSCP) [21]

into a limitednumber of traﬃc aggregates or

classes. Within the core of the network, scheduling

andqueuing control mechanisms are appliedto the

traﬃc classes basedupon the DS ﬁeldmarking; all

traﬃc conditioning and dropping is handled intel-

ligently at the network layer using IP Diﬀserv

quality of service mechanisms. Diﬀserv is not pre-

scriptive in deﬁning the scheduling and queuing

control algorithms that shouldbe implementedat

each hop, but rather, uses a level of abstraction in

deﬁning the externally observable forwarding be-

haviours, termedper-hop behaviours (PHBs), that

can be appliedto traﬃc at each hop. Currently,

three PHBs are deﬁned:

•	The expedited forwarding (EF) PHB. The EF

PHB [8,16] is usedto support traﬃc with low

loss, low delay, low jitter, assured bandwidth re-

quirements, such as VoIP.

•	The assured forwarding (AF) PHB.	The AF

PHB [14] is usedto support data traﬃc with as-

sured bandwidth requirements.

•	The default PHB. This PHB [21] represents the

default forwarding behaviour. Packets, which

are not identiﬁed as belonging to another class,

belong to this aggregate.

A typical Diﬀserv backbone implementation,

that couldbe usedto support the three classes of

service previously deﬁned, is described as follows:

•	Real-time. The real-time traﬃc class is treated

with an EF per-hop behaviour. This is typically

implementedusing a strict priority queue within

a scheduling mechanism,3which services the

strict priority queue with priority above all other

queues. Such a queuing mechanism assures that

the real-time class is isolatedfrom the impact of

the other classes.

3	Such as the modiﬁed Deﬁcit Round Robin scheduler used

on high-performance Cisco routers.

136

C. Filsﬁls, J. Evans / Computer Networks 40 (2002) 131–148

The forwarding resources for the class are al-

locatedto ensure that with the expectedload

there will be no congestion within the class,

such that it receives gooddeterministic service

with low loss, low delay and low jitter.

•	Business data.	Business data class traﬃc is

treatedwith an AF per-hop behaviour. This is

typically allocatedto a queue within a weighted-

fair-queuing-like scheduling mechanism. Such a

queuing mechanism assures that the class re-

ceives a bandwidth allocation relative to the

best-eﬀort data class and that it can use all

available forwarding resources after the VoIP

andbest-eﬀort data class queues have been ser-

viced.

The forwarding resources for the class are allo-

catedto ensure that for the majority of the time,

with the expectedloadthere is no congestion

within the class such that it receives goodde-

terministic service with low loss.

The assumption is made that the majority of the

business data class traﬃc is TCP/IP [20], and

hence the Random Early Detection (RED) [13]

congestion control mechanism is usedwithin the

class queue rather than tail drop to ensure that

TCP throughput within the class is maximised

when congestion occurs. If a service is deﬁned

with in-andout-of-contract capabilities (similar

to the concept of discard eligibility [DE] set/unset

within frame-relay), WeightedRED (WRED) is

usedto have two diﬀerent RED proﬁles4per

queue: an aggressive proﬁle for out-of-contract

traﬃc, anda gentler proﬁle for in-contract traﬃc

to make sure that in case of class congestion, out-

of-contract packets are dropped ﬁrst.5

4	The characteristics of RED are deﬁned by a minimum

queue threshold(minth), maximum queue threshold(maxth),

andprobability of discardat maxth(maxp). WRED allows

multiple redproﬁles to be supportedin the same queue with

separately deﬁned minth, maxthandmaxpper proﬁle. This

results in diﬀerent drop characteristics (and consequently

probability of drop) per proﬁle.

5	This is achievedby choosing a maximum thresholdfor out-

of-contract traﬃc smaller or equal to the minimum threshold

for in-contract traﬃc.

•	Best-eﬀort data. The best-eﬀort class traﬃc uses

an AF per-hop behaviour. This wouldtypically

be allocatedto a queue within a weightedfair-

queuing scheduling mechanism. Such a queuing

mechanism assures that the class receives a

bandwidth allocation relative to the business

data class and that it will be able to use all avail-

able forwarding resources after the VoIP

andbusiness data class queues have been ser-

viced.

As with the business data class, the assump-

tion is made that the majority of the best-eﬀort

data class traﬃc is TCP/IP, therefore the RED

congestion control mechanism is usedwithin

the class queue rather than tail drop to ensure

that TCP goodput within the class is maximised

when congestion occurs.

This paper does not consider the classiﬁcation,

conditioning and marking functions performed at

the edges of the network, but assumes that such

mechanisms are used. An edge design may oﬀer

more classes of service; for example, two or three

diﬀerent classes for business data traﬃc to either

isolate applications and/or ease the budgeting of

the bandwidth cost between diﬀerent customer

departments. In this case, the capability to map

several edge classes into a single aggregate back-

bone class is requiredon the SP’s edge router.

Several edge business data classes may be mapped

into the backbone business data class or two dis-

tinct edge VoIP and video classes may be mapped

into the aggregate backbone real-time class. This

mapping can be realizedin two ways:

•	A backbone class can match several DSCPs.

For example, if at the edge of the network, a

DSCP value of 40 represents VoIP at the edge

anda value 32 represents Video, the backbone

aggregate real-time class wouldmatch both

DSCP 40 andDSCP 32.

•	When MPLS is usedin the backbone, the edge

SP router can set the 3 bit MPLS experimental

(EXP) ﬁeldas a function of the receivedDSCP.

For example, if an EXP value of 5 is usedfor

the aggregate real-time backbone class, the SP’s

edge router will impose MPLS labels headers

with an EXP value of 5 for packets received

C. Filsﬁls, J. Evans / Computer Networks 40 (2002) 131–148

137

with DSCP 40, representing the edge VoIP

class, or with DSCP 32, representing the edge

Video class.

3.2. Diﬀserv backbone design for low loss and low

jitter

Designing an IP backbone network for low loss,

low delay and low jitter can be relatively simple:

one simply needs to overprovision the bandwidth

comparedto the actual load[4,6,7]. Ref. [6] shows

that for a best-eﬀort IP backbone, worst case jitter

was measuredat less than 1 ms for probes sent at

1Mbps during a seven-day period between the east

andwest coast POPs of a Tier-1 Internet Service

Provider (ISP). During this period, the loss rate

was zero. These measurements demonstrate the

excellent performance that can be achievedin an

IP backbone when designed with high-speed links

(from STM-4/OC12 to STM-64/OC192) andwith

conservative capacity provisioning rules where

links are upgraded when utilisation reaches 40–

50%, aiming to ensure that there is at least twice as

much capacity as actual load.

Such a simple design rule allows tight SLAs to

be achievedfor delay, jitter andloss. Unfortu-

nately, however, this does not satisfy our initial

requirements as this represents an expensive op-

tion for the SP. If, for example, between two POPs

there is 150 Mbps of VoIP traﬃc and1.5 Gbps of

best-eﬀort data traﬃc, then by the above rule,

twice the sum of the VoIP andbest-eﬀort traﬃc

wouldbe neededto assure low jitter andloss of the

VoIP class. In this case, the sum of the VoIP and

data traﬃc is 1.65 Gbps and hence 3.3 Gbps of

capacity needbe provisioned. In practice, this

wouldtypically be supportedusing two STM-16/

OC48 links, resulting in 5 Gbps of bandwidth

being deployed to support 1.65 Gbps of aggregate

loadwith 150 Mbps of low delay, low jitter, low

loss traﬃc.

Diﬀserv provides a solution to this problem, in

that it allows per class virtual backbones to be

built on a single physical backbone. This gives SPs

the ﬂexibility to have diﬀerent under- or over-

booking ratios (the ratio of oﬀeredloadto avail-

able capacity) for each service class. Using the

previous example, this couldallow the VoIP class

capacity to be over provisionedby a factor of 46

hence ensuring that the class receives goodservice

(with low delay, low jitter and low loss), whilst the

data class capacity could be over provisioned by a

factor of 1.2 (a realistic ﬁgure still giving good

service). This wouldresult in 2.1 Gbps of band-

width required in total, or rounded up to a single

STM-16/OC48 link, which represents a potential

saving of 1	STM-16/OC48 link over the non-

Diﬀserv case. To explain how this calculation was

derived, the VoIP class traﬃc is assumed to be

servicedfrom a strict priority queue andthus will

eﬀectively have access to all bandwidth on the

physical link; for an STM-16/OC48 link this would

result in eﬀective over provisioning by a factor of

2.5 Gbps/150 Mbps ј  16. The best-eﬀort class

traﬃc being servicedfrom a weighted-fair queue

would, however, have access to all available

bandwidth on the physical link once the VoIP class

traﬃc has been serviced; for an STM-16/OC48 link

this would result in eﬀective bandwidth over pro-

visioning by a factor of [(2.5 Gbps–150 Mbps)/1.5

Gbps] ј  1:6.

This example is intended to highlight two key

points: that backbone Diﬀserv deployment is con-

ceptually simple andthat the concepts involved

have been proven by experience with deployed

best-eﬀort IP networks. It is simple in that it allows

diﬀerent service levels to be supported merely by

using diﬀerent under- or over-provisioning ratios

per class; the higher the available capacity com-

paredto the oﬀeredload, the tighter the SLA

(lower delay, jitter, and loss rates) that can be

supported.7It is proven in that empirical evidence

6	In practise, an overprovision factor of 4 is used, based

upon the simple assumption that single-link or node failure

conditions can result in a doubling of the per link load.

Consequently, using as overprovision factor of 2 2	the

maximum expectedloadin non-failure conditions aims to

ensure that even in failure conditions, there is twice as much

capacity as traﬃc loadandhence low delay, jitter andloss

service is maintained.

7	Assuming two classes, A andB, such that A’s overprovi-

sioning ratio is higher than B. A tighter delay SLA for A can be

expressedeither by a smaller delay boundfor A than B but with

the same availability, or by the same delay bound but with an

higher availability for A than for B, or a combination of both

schemes.

138

C. Filsﬁls, J. Evans / Computer Networks 40 (2002) 131–148

from high-speedbest-eﬀort (single class of service)

SP IP backbones indicates that extremely good

jitter andloss targets can be achievedwith simple

rules of over provisioning.

3.3. Diﬀserv backbone deployment considerations

Consideration obviously needs to be given to

whether the cost of deploying Diﬀserv outweighs

the beneﬁts it provides. There is no generic answer

to this question, andthe beneﬁts that will be

gained will vary deployment by deployment. In the

example above, if the Diﬀserv deployment cost

exceeds the cost of an additional STM-16/OC48

link (andthe router ports which terminate it)

then there is clearly no sense in deploying Diﬀserv.

We consider some of the most signiﬁcant factors

that impact the economic viability of deploying

Diﬀserv:

•	Economic beneﬁt. The maximum potential eco-

nomic beneﬁt stands to be gained from Diﬀserv

deployments where the traﬃc requiring the high-

est SLA targets represents a minor proportion of

the overall capacity. As the previous example

demonstrates, the absence of Diﬀserv leads the

designer to provision capacity equal to the ag-

gregate loadacross all classes multipliedby the

over dimensioning ratio of the tightest-SLA

class. This can be extremely expensive when

the tightest-SLA class represents a low propor-

tion of the aggregate traﬃc. Conversely, when

all classes require the same level of service, and

hence the same overbooking ratio, there is no

beneﬁt to be gainedfrom Diﬀserv.

•	Impact on router performance.	If Diﬀserv EF/

AF forwarding behaviours have an impact on

router forwarding performance, the less aggre-

gate throughput the router can support with

Diﬀserv enabled, and consequently, the higher

the per port cost of the network deployment.

Today’s high-performance routers typically im-

plement the EF/AF forwarding behaviours in

ASICs, ensuring that there is no forwarding

penalty associatedwith the support of the Diﬀ-

serv functionality

•	Simplicity of deployment. Backbone Diﬀserv de-

ployments generally require relatively minor and

simple changes to existing router conﬁgurations.

A typical Diﬀserv design consists in deﬁning

three queues: an EF class queue for real-time

traﬃc such as VoIP, andtwo AF class queues,

one for the business data class and the other

for the best-eﬀort class. The real-time class traf-

ﬁc is servicedwith a priority queuing treatment,

andthe expectedloadfor the class is below 25%

(using an overprovision factor of 4) of the avail-

able link capacity in most designs. The band-

width remaining once the real-time class has

been servicedis allocatedwith 90% to the busi-

ness data class and 10% to the best-eﬀort class.

An example router conﬁguration, which could

be used to implement the described Diﬀserv de-

sign, is shown in Fig. 2; additional lines of con-

ﬁguration requiredto implement the Diﬀserv

policy are shown in boldtype in the ﬁgure.

As can be seen from Fig. 2, only 12 addi-

tional lines of conﬁguration are requiredto

implement the Diﬀserv policy for a single inter-

face. Only a single additional line of conﬁgura-

tion is required for each additional interface

where this Diﬀserv template will be conﬁgured.

Typically, in backbone Diﬀserv deployments

these conﬁgurations are appliedonce, andthen

remain static andare never changed.

Furthermore, migrating a backbone to Diﬀ-

serv can be achievedseamlessly: the backbone

conﬁguration can be undertaken independently

of the conﬁguration requiredat the network

edge to ensure that traﬃc is appropriately con-

ditioned and marked on ingress to the network.

The beneﬁts of Diﬀserv, however, will not be

realiseduntil both edge andbackbone compo-

nents are complete.

•	NMS/OSS requirements.	Backbone network

management systems (NMS) andoperational

support systems (OSS) typically needenhancing

to support Diﬀserv deployments:

The NMS system needs to be enhanced to re-

trieve bytes/packets transmittedanddropped

per class rather than per interface. In response

to the Diﬀserv deployments that have oc-

curred during the 24 months preceding the

publication of this paper, NMS applications

now typically provide support for such statis-

tics.

C. Filsﬁls, J. Evans / Computer Networks 40 (2002) 131–148

Fig. 2. Example backbone router Diﬀserv conﬁguration.

139

The deployment of an active SLA probing

system is highly advisable [15] in order to be

able to monitor (andreport) delay andjitter.

Some router vendors implement software

agents in their routers8that sendandreceive

probes with user-deﬁnable DSCP and proto-

col identities (e.g. FTP, HTTP, DNS). Lever-

aging the installedbase of routers in each

POP allows rapiddeployment of an SLA ac-

tive monitoring system without any major rol-

lout of new network equipment.

•	Capacity planning. In terms of operational pro-

cess, the capacity planning of a Diﬀserv back-

bone is similar to a single-class best-eﬀort IP

network: loadstatistics are collectedon a per-

class of service basis andwhen loadthresholds

are reached, an addition of network bandwidth

is triggered. The accuracy of this capacity plan-

ning can be tunedbasedupon the active SLA

probing results which allows correlation be-

tween per class loadandSLA parameter reports

of delay and jitter.

8	For example, service assurance agent (SAA) functionality

in Cisco IOS.

•	Training. Diﬀserv is a new technology and

therefore it is inevitable that training of design

andoperational staﬀ will be requiredto support

a backbone Diﬀserv deployment.

3.4. Diﬀserv backbone performance characteristics

We conclude the discussion on Diﬀserv by pre-

senting the results of router basedtesting, which

illustrate both the tight-latency, jitter, andloss

capabilities of today’s router technology and the

potential beneﬁts andcharacteristics that can be

achieved with backbone Diﬀserv deployment de-

scribedabove.

The testing was undertaken using a Cisco 12416

router. This router has a distributed architecture

that supports the EF andAF per-hop behaviours

implementedin ASICs on each line cardusing a

deﬁcit round robin (DRR) [25] scheduling algo-

rithm, which has been modiﬁed by Cisco to add

support for a strict priority queue for EF class

traﬃc.

The Diﬀserv conﬁguration shown in Fig. 2 was

usedfor all tests. The packet size usedfor real-time

traﬃc during testing was 200 bytes, whilst the

business data and best-eﬀort traﬃc followed an

140

C. Filsﬁls, J. Evans / Computer Networks 40 (2002) 131–148

Internet-mix packet size distribution.9The router

under test has three ingress STM-16/OC-48 ports

receiving traﬃc from a traﬃc generator. The rou-

ter aggregates this traﬃc andforwards it onto the

single-hop link under test which is an STM-16/OC-

48 Packet over SDH/SONET (POS) link.

Three characteristics of the router EF andAF

implementation, which are key to the successful

deployment of the Diﬀserv design described

above, form the basis of the tests andresults pre-

sented:

•	Latency of EF class. The ﬁrst test measuredthe

one-way delay of the real-time class in the pres-

ence of interface congestion. A worst-case delay

success criterion of 500	ls was set in order to

ensure that the 500	ls delay-jitter target was

never exceeded.10

•	Latency of AF class. The secondtest measured

the delay of the business data class traﬃc under

increasing loadwithin that class.

•	Accuracy of AF bandwidth allocation. The ﬁnal

test measured the bandwidth allocation accu-

racy of the AF class traﬃc (business data and

best-eﬀort), for diﬀerent relative bandwidth al-

locations. Successful Diﬀserv deployment de-

pends upon being able to manage the relative

under- and over-booking ratios between the

classes, which is in turn dependent upon the ac-

curacy of the scheduler implementation in terms

of AF bandwidth allocation.

3.4.1. Latency for EF traﬃc

The ﬁrst results demonstrate the low delay that

can be achievedusing an EF compliant priority

queue scheduler. Fig. 3 charts the recorded per-

centile distribution for the real-time class (EF)

traﬃc delay through a single-hop link running at

STM-16/OC48 andbeing congestedwith 30% of

VoIP, 45% of business traﬃc and150% of best-

eﬀort traﬃc.

9	Fifty eight percentage of the packets are small packets (40

bytes), 33% of the packets are medium sized packets (552 bytes)

and9% of the packets are large packets (1500 bytes).

10	The delay-jitter can never exceed the diﬀerence between

the best-case andworst-case measuredone-way delay.

Fig. 3. Latency for EF class.

Fig. 3 clearly illustrates the low-jitter service

provided by the priority queuing mechanisms to

the VoIP (EF) traﬃc. The percentile 100 delay of

the VoIP packets is 125 ls. This result was inde-

pendent of the load of the other classes even under

225% congestion of the outboundport. These

results are signiﬁcantly better than the target of

500 ls.

3.4.2. Latency for business class

This test demonstrates that very good maxi-

mum delay can be achieved for a well-provisioned

AF class queue, such as the business data class

queue in our case. In this test, the link under test

was loaded with bursty traﬃc in the PQ up to 30%

of the STM-16/OC48 link rate, and150% of best-

eﬀort class traﬃc. The business data class load was

then variedfrom 0% to 200% (of the conﬁgured

business class capacity) andthe maximum delay of

the business class traﬃc was measured. Fig. 4 il-

lustrates the test results.

With business class loadless than business class

capacity, no packet loss is experiencedfor the class

andthe latency remains extremely low at	160 ls

until	85% business class load. As the business

loadincreases to 100% the latency increases to	1

ms at 100% load. Above 100% business class load

packet loss occurs andthe average delay mea-

suredincreases to	100 ms at 220% load, which

in the case of our testing was the expectedresult

C. Filsﬁls, J. Evans / Computer Networks 40 (2002) 131–148

141

Fig. 4. Latency for an AF class.

Fig. 5. Bandwidth accuracy for AF queues.

determined by the RED conﬁguration that we

used.11

3.4.3. Bandwidth allocation between AF queues

The ﬁnal test demonstrates the accuracy of the

bandwidth allocation that can be achieved be-

tween diﬀerent AF classes. In this test, the link

under test was loaded with bursty traﬃc in the PQ

up to 30% of the STM-16/OC48 link rate, and

variable equal rates of business andbest-eﬀort

class traﬃc giving an aggregate interface loadof

greater than 200% of the STM-16/OC48 link rate.

The ratio of the conﬁguredbandwidth allocations

between the business andbest-eﬀort classes was

then varied, and the actual bandwidth allocation

ratio was measuredandcomparedto the expected

result. Fig. 5 graphs the diﬀerence between the

measuredbusiness bandwidth allocation andthe

11	The worst-case latency through a RED-controlledqueue

is relatedto the maxth. In our design, this is tuned to the pipesize

(RTT* bandwidth) with an estimated RTT of 100 ms.

expectedbandwidth allocation, as a percentage of

the STM-16/OC48 link rate.

Fig. 5 shows diﬀerent test cases, which corre-

spond to diﬀerent relative bandwidth allocations

between the business andbest-eﬀort classes (50/50,

66/34, 75/25 etc.). In all tests, the bandwidth allo-

cation accuracy is well within 1% of the expected

result.

4. Fast IGP convergence

Link or node failures in an IP backbone cause

packet losses until the network has reconverged

aroundthe failedlink or node. These packet losses

directly impact the availability that can be oﬀered

for SLAs across all classes. To assess the signiﬁ-

cance of this one can compute the amount of

downtime corresponding to diﬀerent availability

commitments: the often-quoted99.999% or ‘‘ﬁve-

nines’’-target ﬁgure for network availability po-

tentially equates to less than 1 s of downtime per

day.

Another way to illustrate the signiﬁcance of

downtime due to a link or node failure is to con-

sider the impact on applications and end-users.

For VoIP calls, the end-users will perceive a glitch

in the call as soon as a few samples are lost. For

example, with one sample every 20 ms, a loss in

connectivity of 100–150 ms will be perceptible to

the human ear. If the connectivity loss is for 1–2 s

the call can be dropped. Consequently, networks

supporting high-quality VoIP services are being

engineeredfor network convergence upon link/

node failure of 1 s or less. The time taken for an IP

network to reconverge is dependent upon the size

of the network,12the Interior Gateway Routing

protocol (IGP) usedandits speciﬁc conﬁguration.

For high-SLA availability targets to be oﬀered, it

is important that the routing protocol is tunedfor

rapidconvergence. A key component of tuning

IGP convergence is the tuning of the timers, which

determine how frequently the main routing pro-

tocol events can occur. Historically, this has re-

sultedin a trade-oﬀ between rapidconvergence

12	In terms of number of links andnodes in the network and

number of routes carriedin the IGP.

142

C. Filsﬁls, J. Evans / Computer Networks 40 (2002) 131–148

andincreasedrouting protocol stability: short

timers resultedin rapidconvergence but with more

potential for instability, where longer timers re-

sultedin increasedstability but slower conver-

gence. The pragmatic result of this trade-oﬀ was

that routing protocol timers were generally set

conservatively andIP network convergence was

typically a few tens of seconds.

Experience gainedfrom large-scale service pro-

vider deployment, however, indicated that such

IGP implementations were very stable andhence

that more emphasis couldbe placedon faster

convergence. Further, recent developments to IS–

IS andOSPF link state IP IGPs have focussedon

combining the best of both worlds leading to sig-

niﬁcant reductions in the convergence that can be

achievedwhilst still maintaining stability. Whereas

previously IGP timers were static andlong, now

with the introduction of dynamic timers they can

adapt their responsiveness depending upon the

stability of the network. This allows IGPs to be

tunedsuch that when the network is stable, their

timers will be short andthey will react within a few

milliseconds to any network topology changes. In

times of network instability (e.g. causedby a ﬂap-

ping link), however, the IGP timers will increase in

order to throttle the rate of response to network

events. This scheme ensures fast convergence when

the network is stable andmoderate routing proto-

col overhead(e.g. CPU cycles consumed) when the

network is unstable.

In addition to the advancements in the tuning

of routing protocol timers, a number of other de-

velopments have improvedthe IGP convergence

that can be achieved:

•	Partial routing calculation and incremental SPF

[23,24]. IGP implementations have been en-

hancedto react more optimally to various topol-

ogy changes. For example, with the introduction

of partial routing calculations for IS–IS and

OSPF, if only an IP leaf subnetwork changes

without a topological change to the shortest

path tree, then the router does not need to re-

compute the shortest-path tree, it just recom-

putes its routing table basedupon the existing

tree. On the other hand, when only a part of

the graph has changed, incremental SPF opti-

mises the calculation by only recomputing the

part that has changedinsteadof the whole

graph. Such optimisation leads to faster rout-

ing computations, hence to faster convergence.

Early empirical data from large-scale network

deployments suggests that the average IGP rout-

ing table computation time can be reduced by

90% due to such optimisations.

•	Design practice. Best practise for service pro-

vider IGP design today aims to reduce the num-

ber of routes that are carriedin the IGP; all

Internet andcustomer routes are carriedby

the Border Gateway protocol (BGP); in some

cases even the interior link preﬁxes are carried

in BGP. This practise signiﬁcantly reduces

IGP routing table computation times andhence

results in faster IGP convergence. Obviously,

the routing table for a network of 100 routers

and1000 IP preﬁxes can be computedfaster

than one with 100 routers and10,000 preﬁxes.

•	Rapid layer 2 failure detection. Most service pro-

vider backbones today are built using Packet

Over SDH/SONET (POS) links where signal-

ling inherent in SDH/SONET is designed to de-

tect link or node failures in less than 10 ms. This

rapid failure detection can considerably speed

up convergence, when comparedto other media

such as Ethernet, which rely on hellos at the IP

layer to detect failure.

The combination of these optimisations has re-

sultedin a reduction of IGP convergence times

from several 10 s of seconds, to 1–2 s being prag-

matically achievable today. This has been con-

ﬁrmedin testing in a 1000 router network with IP

4000 preﬁxes in the IGP (work in progress). Fur-

ther, as we learn from additional deployment

experience, and with additional tuning and en-

hancements, sub-secondIGP convergence may

become a realistic possibility.

5. MPLS traﬃc engineering and diﬀserv-aware

traﬃc engineering

5.1. MPLS traﬃc engineering

In conventional service provider IP networks

routing protocols such as OSPF andIS–IS for-

C. Filsﬁls, J. Evans / Computer Networks 40 (2002) 131–148

143

wardIP packets on the shortest cost path to the

destination IP address of each IP packet. The

computation of the shortest cost path is based

upon a simple additive metric, where each link has

an appliedmetric, andthe cost for a path is the

sum of the link metrics in the path. Availability of

network resources, such as bandwidth, is not taken

into account and, consequently, traﬃc can aggre-

gate on the shortest path, potentially causing links

on the shortest path to be congestedwhile links on

alternative paths are under-utilised.

This property of conventional IP routing pro-

tocols, of traﬃc aggregation on the shortest path,

can cause sub-optimal use of network resources,

andcan consequently impact the SLAs that can be

oﬀered(or require more network capacity than is

optimally required).

MPLS traﬃc engineering (TE) [2] uses the im-

plicit MPLS characteristic of separation between

the data plane (also known as the forwarding

plane) andcontrol plane to allow routing decisions

to be made on criteria other than the destination

IP address in the IP header, such as available link

bandwidth. MPLS TE eﬀectively provides an ex-

plicit routing capability at Layer 3, allowing paths

to be usedother than the shortest cost path to a

destination, thereby avoiding traﬃc aggregation

on the shortest path andproviding more optimal

use of available bandwidth.

MPLS TE uses the following mechanisms:

•	Information on available network resources,

including a pool of available bandwidth main-

tainedper link, are ﬂoodedby means of exten-

sions to link-state basedIP routing protocols

such as IS–IS [26] andOSPF [17].

•	A constraint-basedrouting (CBR) algorithm is

usedto compute the traﬃc path basedupon

a ﬁt between the available network resources

(advertised via IS–IS or OSPF) and the re-

sources required, i.e. a requested amount of

bandwidth.

•	The Resource ReSerVation Protocol (RSVP)

[5], with enhancements for MPLS TE [1], is used

to signal andmaintain an explicit route (termed

a ‘‘traﬃc engineeredtunnel’’), from head-end to

tail-end, in the form of an MPLS Label Switched

Path (LSP). This LSP follows the path deter-

minedby the constrain-basedrouting algorithm.

In signalling the tunnel, admission control is

performedat every hop.

•	Traﬃc routedonto these LSPs or tunnels will

then follow the traﬃc engineeredexplicit route

to the destination, rather than the conventional

IGP shortest path.

The following conditions can all be drivers

for the deployment of MPLS traﬃc engineering

[28]:

•	Network asymmetry. Asymmetrical network to-

pologies can often leadto traﬃc being aggre-

gatedon the shortest path whilst other viable

paths are under-utilised. Network designers will

often try to ensure that networks are symmetri-

cal such that where parallel paths exist, they are

of equal cost andhence the loadcan be bal-

ancedacross them using conventional IGPs.

Ensuring network symmetry, however, is not al-

ways possible due to economic or topological

constraints. Traﬃc engineering oﬀers obvious

beneﬁts in these cases.

•	Unexpected demand.	In the presence of unex-

pected traﬃc demand (e.g. due to some new

popular content), there may not be enough ca-

pacity on the shortest path (or paths) to satisfy

the demand. There may be capacity available on

non-shortest paths, however, andhence traﬃc

engineering can provide beneﬁt.

•	Long bandwidth lead-times.	There may be in-

stances when new traﬃc demands are expected

andnew capacity is requiredto satisfy the de-

mand, but is not available in suitable timescales.

In these cases, traﬃc engineering can be usedto

make use of available bandwidth on non-short-

est path links.

The use of TE gives the service provider ﬂexi-

bility in how to manage their backbone bandwidth

in order to achieve their SLAs. The more eﬀec-

tive use of bandwidth potentially allows higher

service availability targets to be oﬀeredwith the

existing backbone bandwidth. Alternatively, it

oﬀers the potential of achieving the existing ser-

vice availability targets with less backbone band-

width.

144

5.2. Diﬀserv and TE

C. Filsﬁls, J. Evans / Computer Networks 40 (2002) 131–148

MPLS TE andDiﬀserv can be deployedcon-

currently in an IP backbone, with TE determining

the path that traﬃc takes on aggregate basedupon

aggregate bandwidth constraints, and Diﬀserv

being usedon each link for diﬀerential scheduling

of packets on a per class basis. Whilst TE and

Diﬀserv are orthogonal technologies they can be

usedin concert for combinedbeneﬁt: TE allows

distribution of traﬃc on non-shortest paths for

more eﬃcient use of available bandwidth, whilst

Diﬀserv allows over/under-provisioning ratios to

be determined on a per class basis.

MPLS TE, however, computes tunnel paths for

aggregates across all traﬃc classes andtraﬃc from

diﬀerent classes may use the same TE tunnels.

MPLS TE is aware of only a single aggregate

global pool of available bandwidth per link and is

unaware of what speciﬁc link bandwidth resources

are allocatedto which queues, andhence to which

class.

Consequently, MPLS TE has been extended

with Diﬀserv-aware traﬃc engineering (DS-TE)

[10], which introduces the concept of an additional

andmore restrictive pool of available bandwidth

on every link. This more restrictive bandwidth

pool is termedthe sub-pool, while the regular TE

bandwidth is called the global pool (the sub-pool is

a portion of the global pool). The sub-pool may be

usedfor constraint-basedrouting andadmission

control of tunnels for ‘‘guaranteed’’ or EF class

traﬃc andthe global pool usedfor regular (non-

‘‘guaranteed’’) or AF class traﬃc.

In supporting DS-TE, extensions have been

added to IS–IS and OSPF [11] to advertise the

available sub-pool bandwidth per link as well as

the available global-pool bandwidth. In addition,

the TE constraint-basedrouting algorithms have

been enhancedfor DS-TE in order to take into

account the constraint of available sub-pool

bandwidth in computing the path of sub-pool

tunnels. RSVP has also been extended [11] to in-

dicate if it is signalling a sub-pool or global-pool

tunnel.

It is understood that setting an upper bound on

the EF class (e.g. VoIP) eﬀective utilization per

link allows a way to restrict the eﬀects of delay and

Fig. 6. DS-TE deployment example.

jitter due to accumulated burst [4,7]. DS-TE can be

usedto assure that this upper boundis not ex-

ceeded. For example, consider the network in Fig.

6 DS-TE couldbe usedto ensure that traﬃc is

routedover the network so that, on every link,

there is never more than 25% (or any assigned

percentage) of the link capacity for EF class traﬃc,

whilst there can be up to 100% of the link capacity

for EF andAF class traﬃc in total. Each link in

Fig. 6 is 100 Mbps. R1 is sending an aggregate of

40 Mbps of traﬃc to R8, andR2 is also sending an

aggregate of 40 Mbps of traﬃc to R8. An IGP and

TE metric value of one is appliedto each link.

In this case, both the IGP andnon-Diﬀserv

aware TE wouldpick the same route. The IGP

wouldpick the top route рR1=R2 ! R3 ! R4 !

R5 ! R8Ю because it is the shortest path (metric of

4), whilst TE wouldpick the same path because it

is the shortest path that has suﬃcient bandwidth

available (metric of 4, 100 Mbps bandwidth avail-

able, 80 Mbps required). The decision to route

both traﬃc aggregates via the top path may not

seem appropriate if we examine the composition of

the aggregate traﬃc ﬂows.

If each of the aggregate ﬂows is comprisedof 5

Mbps of VoIP traﬃc and35 Mbps of business

data traﬃc, then in this case the total VoIP traﬃc

loadon the top links will be 10 Mbps, which is

within our EF class boundof 25%. If, however,

each traﬃc aggregate is comprisedof 20 Mbps of

VoIP and20 Mbps of business data traﬃc then

such routing wouldaggregate 40 Mbps of VoIP

traﬃc on the R3 ! R4 ! R5 links, thereby ex-

ceeding our EF class bound of 25%. DT-TE can be

usedto overcome this problem: each link is con-

ﬁguredwith an available global pool bandwidth

of 100 Mbps, andan available sub-pool bandwidth

C. Filsﬁls, J. Evans / Computer Networks 40 (2002) 131–148

145

of 25 Mbps (i.e. 25% of 100 Mbps). A global-pool

tunnel of 20 Mbps is then conﬁguredfrom R1 to

R8 for business data traﬃc, and a sub-pool tunnel

of 20 Mbps for VoIP traﬃc. Similarly, from R2 to

R8 a global-pool tunnel of 20 Mbps is conﬁgured

for business data traﬃc, and a sub-pool tunnel of

20 Mbps for VoIP class traﬃc. The DS-TE con-

straint basedrouting algorithm wouldthen route

the sub-pool tunnels to ensure that the 25 Mbps

boundis not exceededon any link, andof the

tunnels from R1 andR2 to R8, one sub-pool

tunnel wouldbe routedvia the top path	рR1=

R2 ! R3 ! R4 ! R5 ! R8Ю	andthe other via

the bottom path	рR1=R2 ! R6 ! R7 !	R5 !

R8Ю.13In this particular case, there wouldbe en-

ough available bandwidth for both global pool

tunnels to be routedvia the top path	рR1=R2 !

R3 ! R4 ! R5 ! R8Ю, which has the shortest

metric.

DS-TE enables service providers to perform

separate route computation andadmission control

for diﬀerent classes of traﬃc. This enables the dis-

tribution of EF andAF class loadover all available

EF andAF class capacity making optimal use of

available capacity. It also provides a tool for con-

straining the EF class utilization per link to a

speciﬁedmaximum thus providing a mechanism to

help bound the delay and jitter. In order to provide

these beneﬁts, however, the conﬁgured bandwidth

for the sub-pool andglobal pool must represent

queuing resources, which are only available for

traﬃc-engineeredtraﬃc, andhence non-traﬃc en-

gineeredtraﬃc shouldbe queuedseparately on

each link. By combining DS-TE with Diﬀserv

queuing mechanisms on each link, the service

provider can oﬀer tight-SLAs for EF class traﬃc

(such as VoIP) with admission control without

large over-provisioning of capacity.

6. MPLS traﬃc engineering fast reroute

In Section 4, we highlightedthat link or node

failures in an IP backbone can signiﬁcantly impact

13	A propagation-delay constraint can also be speciﬁed for

the sub-pool tunnels to ensure that the chosen path exhibits a

propagation delay smaller or equal to the speciﬁed value [12].

the availability that can be oﬀeredfor SLAs across

all classes. Whilst sub-secondconvergence for IP

routing protocols is a realistic prospect, it is ex-

pectedthat IGP convergence will not be able to

match the capabilities of SDH/SONET networks,

which use the capabilities of Multiplexer Section

Protection (MSP) andAutomatic Protection

Switching (APS) respectively to recover around

failures in tens of milliseconds. This is because the

functions are performed in fundamentally diﬀerent

ways: IGP convergence is basedon a distributed

computation14whereas SDH/SONET restoration

is basedupon local detection andpre-computed

local protection aroundthe failure.

MPLS traﬃc engineering fast reroute (FRR)

extends the concepts of local failure detection and

protection to MPLS TE in order to provide very

rapidrecovery aroundfailures (e.g. a few tens of

milliseconds) prior to any distributed convergence/

reoptimisation. Without FRR, under failure con-

ditions, the head-end of a TE tunnel determines

a new route for the tunnel LSP. Recovery at the

head-end provides for the optimal use of resources,

however, due to messaging delays, the head-end

cannot recover as fast as is possible by making a

repair at the point of failure.

MPLS TE Fast Reroute adds additional capa-

bilities to MPLS TE in that it provides local pro-

tection of tunnel LSPs in the presence of link

failure. This enables all traﬃc carriedby tunnel

LSPs that traverse a failedlink to be rerouted

aroundthe failure. The reroute decision is com-

pletely controlledlocally by the router interfacing

the failedlink.

MPLS TE FRR uses the following mechanisms:

•	Hierarchical LSPs. FRR uses the concept of hi-

erarchical LSPs; the protectedtunnel LSPs are

switchedrapidly into backup tunnel LSP at

the point of failure (also known as the point of

local repair). The backup tunnel-LSP provides

an explicit route aroundthe failedlink or node.

To support FRR, extensions have been added to

14	The local node next to the failure distributes the

information andthen it is up to all the other nodes to each

compute their routing table to route aroundthe failure.

146

C. Filsﬁls, J. Evans / Computer Networks 40 (2002) 131–148

RSVP [27] to indicate which tunnels are fast

reroutable andto carry additional label infor-

mation requiredfor FRR. For maximum pro-

tection, the path for the backup tunnel LSP

can be determined to use disjoint resources (such

as optical channels, ﬁbres or ducts) from the

protectednode or link.

•	Rapid failure detection.	If POS links are used,

SDH/SONET signalling allows link or node

failures to be detected by directly connected

nodes in less than 10 ms. Where POS links are

not used, RSVP hellos can be used between ad-

jacent nodes for failure detection.

•	Rapid local protection.	Rapidlocal protection

aroundthe failure is possible because the switch-

ing entries for the backup tunnel-LSP are pre-

computed. Once a failure is detected, the local

node has only to copy the pre-computed backup

tunnel-LSP switching entry into its switching

table for recovery aroundthe failure to be

achieved.

•	Path reoptimisation. The head-end of the tunnel

is also notiﬁedof the link failure through the

IGP or through RSVP; the head-end may then

attempt to establish a new, andpossibly more

optimal, LSP that bypasses the failure, using

make-before-break signalling with RSVP. This

is a signiﬁcant advantage compared to SDH/

SONET protection.

The local nature of FRR allows very rapid

protection andrestoration aroundfailures in IP

backbone networks. For SDH/ SONET links, de-

tecting the failure of a link is typically done in less

than 10 ms, andwith FRR, many hundreds of

protectedtunnel-LSPs can be switchedaroundthe

failure in less than 50 ms. This is equivalent to the

level of protection provided by MSP and APS and

in SDH andSONET networks respectively.

FRR is designed for backbone deployment

where the number of network components is typ-

ically relatively low, but where the failure of those

components can have severe impacts on services

andSLAs. The determination of optimal routing

for FRR backup tunnels in diﬀerent failure sce-

narios is, however, a complex problem andneeds

to take into account factors including the available

bandwidth on potential backup paths, tunnel inter

relationships andinterdependencies on the lower

layer network topologies. This subject is currently

the focus of further research anddevelopment ef-

forts.

For the service provider, FRR provides the

capability to signiﬁcantly improve the perception

of service quality for IP telephony users. For IP

telephony, if IP connectivity is lost for a few

hundreds of milliseconds, the users will perceive

a glitch in their call. By deploying FRR to pro-

tect key network resources, service providers can

ensure that link failures are imperceptible to IP

telephony users andcan hence oﬀer the highest

availability of service for VoIP class traﬃc. Fur-

ther, enabling this protection technique at the IP

layer allows for better statistical multiplexing,

better reoptimisation upon active protection and

cost advantage due to the consolidated architec-

ture, when comparedto providing this capability

at lower layers, using SDH/SONET capabilities

for example.

7. Conclusion

In the context of ever more competitive service

provider oﬀerings and ever more demanding re-

quirements from their customers, this paper has

analysedwhich SLA parameters are signiﬁcant for

IP service performance (delay, jitter, loss band-

width/throughput, per-ﬂow sequence preservation

andavailability) andhas listedthe targets usually

set for these parameters for the typical backbone

aggregatedclasses of service.

We have reviewedthe technologies that can be

usedto tighten these SLAs andhence serve as

foundation for a multiservice IP backbone: Diﬀ-

serv, fast IGP convergence, traﬃc engineering,

Diﬀserv-aware traﬃc engineering andMPLS TE

FRR.

The Diﬀserv analysis showedthat there should

be an important economical beneﬁt in leveraging

this technology, which enables under- or over-

booking to be performedon a per class rather than

aggregate basis. This per class under-/over-book-

ing capability enables tight-SLAs to be oﬀeredfor

some classes of traﬃc without aggregate over-

provisioning of capacity, leading to savings in

C. Filsﬁls, J. Evans / Computer Networks 40 (2002) 131–148

147

terms of required bandwidth. It was noted that the

typical Diﬀserv deployment strategy targeted by

current designs is relatively simple and only re-

quires a one-time router conﬁguration. The results

of router-basedtesting ﬁnally illustrates the tight-

SLA capabilities of today’s high-performance rou-

ters andconﬁrmedthat the previously deﬁned

SLA targets couldeasily be met. For these reasons,

a multiservice IP backbone design should leverage

Diﬀserv technology.

Recent developments in IGP implementations

have resultedin signiﬁcant improvements in IP

backbone IGP convergence with convergence

times 1–2 s being pragmatically achievable today.

This reduction in convergence times allows higher

availability targets to be oﬀeredfor SLAs across

all service classes. Consequently, fast IGP con-

vergence is also recommended as a foundation of

multiservice IP backbone network designs

MPLS traﬃc engineering gives the service pro-

vider the capability to use available backbone

bandwidth more eﬀectively. Either it allows higher

service availability targets to be oﬀeredwith the

existing backbone bandwidth or, alternatively, it

oﬀers the potential of achieving the existing service

availability targets with less backbone bandwidth.

Notwithstanding these beneﬁts, it is noted that not

all networks will beneﬁt from the deployment of

TE

Diﬀserv-aware MPLS traﬃc engineering ex-

tends the base capabilities of TE to allow route

computation andadmission control to be per-

formedseparately for diﬀerent classes of service.

By combining DS-TE with Diﬀserv mechanisms

on each link, the service provider can oﬀer tight-

SLAs for EF class traﬃc (such as VoIP) with ad-

mission control without large over-provisioning of

capacity.

Finally, for networks seeking sub-100 ms con-

vergence, MPLS TE fast reroute provides the ca-

pability for protection aroundfailures at the IP

Layer. By deploying FRR to protect key network

resources, service providers can ensure that link

failures are imperceptible to IP telephony users

andcan hence oﬀer the highest availability of

service for VoIP class traﬃc.

In summary, the foundation for a multiservice

tight-SLA IP network consists in a Diﬀserv and

Fast-IGP design. This design is complemented by

MPLS technologies such as traﬃc engineering,

Diﬀserv-aware traﬃc engineering andfast reroute

depending on the speciﬁc context and require-

ments of the considered network.

Acknowledgements

The authors wouldlike to thank Olivier Bona-

venture, Thomas Telkamp andBruce Thompson

for their valuable feedback and constructive

comments.

References

[1] D.O. Awduche et al., RSVP-TE: extensions to RSVP for

LSP tunnels, RFC3209, December 2001.

[2] D. Awduche et al., Requirements for traﬃc engineering

over MPLS, RFC 2702, September 1999.

[3] S. Blake et al., RFC2475, An Architecture for Diﬀerenti-

atedService, December 1998.

[4] T. Bonald, A. Proutiere, J. Roberts, Statistical guarantees

for streaming ﬂows using expedited forwarding, INFO-

COM 2001.

[5] R. Braden et al., RFC2205, Resource Reservation Protocol

(RSVP) Version 1 Functional Speciﬁcation, September

1997.

[6] S. Casner, C. Alaettinoglu, C.-C. Kuan, A ﬁne-grained

view of high-performance networking, Packet Design,

NANOG 22, May 20–22, 2001.

[7] A. Charny, J.-Y. Le Boudec, Delay bounds in a network

with aggregate scheduling, in: First International Work-

shop on Quality of Future Internet Services, Berlin,

Germany, 2000.

[8] B. Davie (Ed.) et al., An expedited forwarding PHB,

Internet draft, April 2001 Charny et al., An expedited

forwarding PHB, Internet Draft.

[9] N.G. Duﬃeldet al., A ﬂexible model for resource

management in virtual private networks, SIGCOMM’99.

[10] Le Faucheur et al., Requirements for support of Diﬀ-Serv-

aware MPLS Traﬃc Engineering, Internet Draft.

[11] Le Faucheur et al., Protocol extensions for support of

Diﬀ-Serv-aware	MPLS	Traﬃc	Engineering,	Internet

Draft.

[12] Le Faucheur et al., Use of IGP Metric as a secondTE

Metric, Internet Draft.

[13] S. Floyd, V. Jacobson, Random early detection gateways

for congestion avoidance, IEEE/ACM Transactions on

Networking 1 (4) (1993) 397–413.

[14] J. Heinanen et al., RFC2597, AssuredForwarding PHB

Group, June 1999.

[15] http://www.ietf.org/html.charters/ippm-charter.html.

148

C. Filsﬁls, J. Evans / Computer Networks 40 (2002) 131–148

[16] V. Jacobson et al., RFC2598, An expedited forwarding

PHB, June 1999.

[17] D. Katz et al., Traﬃc engineering extensions to OSPF,

Internet Draft.

[18] M. Laor, L. Gendel, Eﬀect of packet reordering in a

backbone link on applications throughput, submittedto

IEEE Network Magazine.

[19] M. Mathis, The macroscopic behavior of the TCP

Congestion Avoidance Algorithm, Computer Communi-

cation Review, July 1997.

[20] S. McCreary, K. Claﬀy, Trends in wide area IP traﬃc

patterns––a view from Ames Internet Exchange, in: Pro-

ceedings of 13th ITC Specialist Seminar on Internet Traﬃc

Measurement andModelling, Monterey, CA, 18–20 Sep-

tember, 2000.

[21] K. Nichols et al., RFC2474, Deﬁnition of the Diﬀerenti-

atedServices Field(DS Field) in the IPv4 andIPv6

Headers, December 1998.

[22] http://www.ietf.org/html.charters/pwe3-charter.html.

[23] J.M. McQuillan, I. Richer, E.C. Rosen BBN Report 3803,

ARPANET Routing Algorithm Improvements, First Semi-

annual Technical Report, April 1978.

[24] J.M. McQuillan, I. Richer, E.C. Rosen, The New Routing

Algorithm for the ARPANET, IEEE Transactions on

Communications, May 1980.

[25] M. Shreedhar, G. Varghese, Eﬃcient fair queuing using

deﬁcit round robin, SIGCOMM 1995.

[26] H. Smit, T. Li, ISIS extensions for traﬃc engineering,

Internet Draft.

[27] G. Swallow, R. Goguen, RSVP Label allocation for

backup tunnels, Internet Draft.

[28] T. Telkamp, Hot Interconnect, Standford 2001.

Clarence Filsﬁls	has an Engineering

Degree in Computer Sciences from the

Institute Monteﬁore of the University

of Liege, Belgium anda Business De-

gree from the Solvay Business School,

Brussels Belgium. He joinedCisco in

1996 andas a DistinguishedEngineer;

he focuses on IP Core Routing and

Capacity Management (IP QoS/Traﬃc

Engineering) designs.

John Evans	receiveda B.Eng. (Hons)

degree in Electronic Engineering from

the University of Manchester Institute

of Science andTechnology (UMIST),

UK in 1991 andan M.Sc. in Com-

munications Engineering from UMIST

in 1996. He joinedCisco Systems in

1998 andas a Consulting Engineer; he

focuses on IP network design and de-

velopment with special interests in core

routing andtraﬃc management, in-

cluding IP quality of service and traﬃc

engineering. Prior to joining Cisco, he

worked on the design and development of large-scale networks

for the ﬁnancial community andfor the military.