Computer Communications 26 (2003) 603–610

www.elsevier.com/locate/comcom

Lambda GLSP setup with QoS requirements in optical Internet

SuKyoung Leea,*, David Grifﬁtha, Joo-Seok Songb

aNational Institute of Standards and Technology, 100 Bureau Drive, Stop 8920, Gaithersburg, MD 20899, USA

bDepartment of Computer Science, Yonsei University, Seoul, South Korea

Received 17 July 2001; revised 27 May 2002; accepted 18 June 2002

Abstract

GMPLS is currently being speciﬁed as the control plane for next generation optical networks. While one merit of GMPLS is that network

operators are able to control the trafﬁc of Internet Protocol (IP) and thus offer performance superior to that of traditional Internet Gateway

Protocols (IGP), it has been a challenge to develop an efﬁcient Trafﬁc Engineering (TE) technique to satisfy the different trafﬁc requirements

of end-users in GMPLS domain. Several TE techniques have been thought of by creating a set of Generalized Label Switched Paths (GLSPs)

in the optical network.

In this paper we present a technique for establishing GLSP in optical Internet, that supports Quality of Service (QoS) considering the trafﬁc

ﬂows with delay QoS requirements. This approach accounts for the processing delay at the network layer that may be encountered in optical

nodes that terminate lightpaths and perform some types of trafﬁc grooming. Additionally, we extend the QoS routing framework by deﬁning

additional extensions that can be used in conjunction with Shared Risk Group (SRG) identiﬁers to reserve backup paths that do not share

network resources with the working paths that they are protecting. We provide some simulation results to demonstrate the effectiveness of

our routing algorithms.

q 2002 Elsevier Science B.V. All rights reserved.

Keywords: GLSP; Optical Internet; TE; Backup path

1. Introduction

Recent years have seen a growing need to support

applications that require a high Quality of Service (QoS) on

networks that use the Internet Protocol (IP), voice trafﬁc

being a classic example. The response to this need has been

the development of a range of tools, such as Integrated

Services (IntServ) and Differentiated Services (DiffServ), to

support various levels of QoS. In addition, it is now possible

to implement Trafﬁc Engineering (TE) in IP networks using

tools such as Multi-Protocol Label Switching (MPLS) [1].

Using TE extensions to OSPF in combination with MPLS, it

is possible to route connections across the network in order

to optimize network utilization, avoid islands of stranded

bandwidth, and balance load. The rapid growth in data

trafﬁc currently being experienced is fueled by the Internet,

posing new challenges for transport network providers. To

meet the bandwidth demand of the Internet, it is natural to

use optical technologies with Wavelength Division Multi-

plexing (WDM) to offer the capability of building very large

* Corresponding author.

E-mail address: sukyoung@antd.nist.gov (S.K. Lee).

wide area networks with throughput of the order of terabits

per second.

Recently, with the advent of new trafﬁc engineering

protocols like MPLS, there has been considerable activity

in several standards groups to integrate MPLS and WDM

networking technologies into a uniﬁed structure for

optical Internet	[2]. This extended version of MPLS

that is so-called GMPLS will allow many carriers to

deploy optical Internets where lambdas (wavelengths) are

treated as very low-level point-to-point links for the

transmission of packets between high performance

routers. In this optical Internet, optical switches translate

label assignments into corresponding wavelength assign-

ments and setup GLSP using local control interfaces to

switch devices. Subsequent to GLSP setup, no explicit

label lookup/processing operations are performed by

Optical Labeling Switched Routers (O-LSRs).

One of the beneﬁts of GMPLS is that it supports TE

by allowing the node at the network ingress to specify

the route that a GLSP will take by using Explicit

Routing (ER). An explicit route is speciﬁed by the

ingress as a sequence of hops that must be used to reach

the egress, which is different from the hop-by-hop

0140-3664/03/$ - see front matter q 2002 Elsevier Science B.V. All rights reserved.

PII: S 0 1 4 0 - 3 6 6 4 ( 0 2 ) 0 0 1 8 4 - 6

604

S.K. Lee et al. / Computer Communications 26 (2003) 603–610

routing, that is usually associated with packet-switch

capable networks. ER features can also be used to

facilitate QoS support for multiple class of services in

GMPLS networks (e.g. DiffServ). There have been

several efforts directed towards using QoS information

to make routing decisions, but their focus is different

from ours. A method for supporting QoS using the

IETF’s Open Shortest Path First (OSPF) routing protocol

can be found in	Ref. [4]. This approach advertises

available bandwidth and delay metrics in OSPF Link

State Advertisements (LSAs), but does not use the delay

metric in computing paths. Some centralized routing

schemes have been proposed, notably that in	Ref. [4],

which seek to ﬁnd the transiting path whose delay is less

than a prescribed limit and whose bandwidth is greater

than a speciﬁed lower bound. That approach uses

Dijkstra’s algorithm to ﬁnd a suitable path but does not

speciﬁcally account for delays at layer 3 or for relative

throughput, as our algorithm does.

Routing to support QoS requirements could be also

applied to optical networks [5]. QoS routing selects a path

ofﬂine for each ﬂow or connection to satisfy diverse

performance requirements and optimize resource usage.

Even though network resource is still available, uneven

distribution of trafﬁc prevents from meeting QoS con-

straints, that is, leads to worse delay performance in terms of

delay QoS. In this case, TE arranges trafﬁc ﬂows so that

congestion caused by uneven network utilization can be

avoided.

In this paper, we propose a mechanism to provide better

delay performance and improve network utilization in the

optical network. We do this by using a linear programming

approach that seeks to minimize the total path delay,

including delay introduced by packet processing at layers

above optical. While other researchers, speciﬁcally [6], have

introduced linear programs that account for the bottleneck

effect at layer 3, their approach seeks to maximize total

network throughput, while ours is focused on meeting QoS

requirements.

The optimization routine that we are proposing will

take path delay, including queueing delay at layer 3, into

account. In optical Internet using lambda labeling,

processing at layer 3 would constitute a signiﬁcant

bottleneck whose impact we wish to minimize. In

transparent optical networks where it is possible to

create lightpath connections between every pair of edge

routers, this is not an issue. But in large networks with

thousands of devices at the edge, creating a virtual

topology that is a full mesh is impractical. Thus, we can

expect that intermediate nodes with layer 3 functionality

will exist in a lambda labeling network (most likely at

the edges of optical subnetworks in the core). Thus it

would be useful to account for the impact of layer 3

processing when the ingress computes the explicit route.

The layer 3 processing overhead at intermediate nodes is

accounted for in the proposed path set-up algorithm.

The rest of paper is structured as follows. In Section 2 we

introduce the background. We describe our algorithm in

Section 3 where an integer linear program (ILP) based

solution is used. In Section 4 we present simulation results,

and ﬁnally in Section 5 conclusion is drawn.

2. Background

Core optical network consists of the Optical Cross-

Connects (OXCs) and ﬁber links interconnecting OXCs.

Each OXC can route an input wavelength to an output

wavelength. These OXCs, in most of the cases, can preclude

the electronic processing, and thus a optical connection can

be established between edge nodes. In lambda labeling

network, GMPLS control plane is attached onto OXC

devices and treats them as GMPLS nodes, hereafter called

as O-LSR nodes, allowing GMPLS signaling to compute

and set up Generalized Label Switched Paths (GLSPs)

between O-LSR nodes in a similar way as MPLS LSPs.

O-LSRs can be viewed as a combination of a router and

an OXC. The router component is responsible for all the

layer 3 functions such as addressing, routing, and global

topology discovery. It is also responsible for optimizing

network performance, which can be carried out via TE with

QoS support, management of optical resources (i.e.

wavelength assignment in coordination with the optical

channel sublayer), and restoration. That is, the most

important advantage of GMPLS turns out to be its ability

to do TE of IP networks although it was originally devised

with other goals such as faster forwarding of IP datagrams.

Even if GMPLS is very useful as protocols for establishing

trafﬁc engineered paths through optical Internet, many

researchers have recently begun working on the mechan-

isms of computing these paths [7]. Therefore, the objective

of this paper is to formulate the TE problems in

mathematical form to motivate more work towards the

consideration of multiple QoS classes. As can be seen in

Fig. 1. Trafﬁc engineering with QoS requirements.

S.K. Lee et al. / Computer Communications 26 (2003) 603–610

605

Fig. 1, the wavelengths can be assigned to the GLSPs in

accordance with QoS class [5]. GMPLS would be applicable

to DiffServ based networks as the growing trend is to use

DiffServ in the edge O-LSRs and lightpaths in the core. In

this paper, ingress O-LSRs determine the GLSPs that IP

trafﬁc ﬂows will be routed through, in terms of the QoS

requirements such as the maximum acceptable delay.

Additionally, this path computation based on delay

requirement can be also applied to establishing a backup

path with reservations every time a primary path is

established across the network. Seeing the objective of TE

in terms of priorities, although we consider only a single

level of priority for delay QoS in this paper, the proposed

approach naturally can extend to multiple priority levels. As

a consequence, the performance of the QoS service classes

is oblivious to the presence of the delay-sensitive trafﬁc.

Reliable network operation is another important aspect

of TE. Failure recovery scenarios must be designed to

ensure continuity of service following network impair-

ments. Therefore, the operational capability must exist to

reroute trafﬁc through the remnant capacity when failures

occur. While this rerouting procedure should make more

effective use of the residual post-failure capacity, it

should establish a backup GLSP to still satisfy the QoS

requirement even after the trafﬁc is recovered onto the

backup path.

As a signaling protocol to support the QoS, there are two

methods in GMPLS. One uses TE extensions to the

Resource Reservation Protocol (RSVP-TE)	[8,9]	and the

other one utilizes extensions to the Label Distribution

Protocol (LDP) called Constraint-based Routing LDP (CR-

LDP)	[8,10]. IETF have been developing both signaling

protocols. Either RSVP-TE or CR-LDP can be used

That is, Interior Gateway Protocol (IGP), such as OSPF

with extensions for optical and TE attributes will allow

nodes to exchange information about optical network

topology,	resource	availability,	and	even	policy

information.

Let	G ј (N,L) describe the given network with node

set	N ј n1; n2; …; nlNland link set	L ј l1; l2; …; llLl;

where	lNl	and lLl	denote the cardinalities of the node

set and the link set, respectively. A link will be referred

to by the ordered node pair it connects, (n1,	n2). Let

D ј {Dij} be the distance matrix from node i	to node	j

that means a propagation delay from node i to node jрi –

jЮ:	Let	K ј k1; k2; …; klKlbe the set of trafﬁc demands

belonging to a speciﬁc service class between a pair of

edge O-LSRs. Typically there are several QoS service

classes, each having characteristic speciﬁcations, such as

delay. Here, we assume that the service class belongs

to delay-sensitive class, since the algorithm focuses on

delay QoS. Each trafﬁc demand k [ K is deﬁned by the

ordered triple (Ik,	Ek,	1k), where	Ikis the ingress OXC,

Ekis the egress OXC, and 1kis the maximum edge-to-

edge delay that is allowed for request k. For trafﬁc ﬂow,

We deﬁne	lIkEk	to be the average ﬂow associated with

the	kth trafﬁc demand belonging to the delay-sensitive

class requesting CR-GLSP set-up. These demands are

assumed to arrive one at a time. Like	lIkEk ; lIEij	denotes

the trafﬁc from ingress	I	to egress	E	that ﬂows over an

intermediate virtual link (i,j ).

The unknown variables that need to be determined based

on optimizing a certain objective function and satisfying a

set of constraints are the following. These binary valued

variables indicate a set of virtual connectivities:

(

allowing a GLSP to be explicitly speciﬁed across the

optical core (Fig. 1) since both signaling protocols were also

recently extended to support GMPLS.

vijј

1 if the virtual topology has a direct fiber link рi; jЮ;

0 otherwise

р1Ю

3. GLSP set-up algorithm

The main problem of the proposed algorithm is to

establish a GLSP that meets the delay requirement (We call

this GLSP as CR-GLSP: Constraint-based Routed GLSP).

The algorithm routes the GLSPs over the topology, and

assigns wavelengths optimally to the various lightpaths.

This assignment problem very likely belongs to the class of

NP-hard problems	[11], since its sub-problem, the static

lightpath establishment has been shown to be NP-hard in

Ref. [12]. Therefore, it is righteous to formulate the problem

as an ILP.

We consider a connected network with N OXCs and L

directed links such as ﬁbers. Each ﬁber carries a certain

number of wavelengths. A subset of these nodes is

assumed to be edge O-LSRs between which lightpaths

can be set up. We are given all the information on the

virtual topology and status through a link state protocol.

where	i; j ј 1; 2; …;lNl;	i – j:	If the TE function per-

forms CR-GLSP setup online when a new CR-GLSP

request arrives after a virtual topology is determined

initially, these variables would assume the role of

constants via virtual topology matrix. On the other

hand, in case that the virtual topology is being formally

modeled over the physical topology ofﬂine, the TE tool

should compute the variables to overlay the virtual

GLSPs. Thus, the online TE tool could be used to solve,

in real time, problems such as the QoS based routing and

rerouting of GLSPs.

When the network manager gets a request for a new

CR-GLSP with a certain QoS requirement, it should

determine whether this CR-GLSP can be routed to meet

the QoS. The decision variables assume a binary value

and denote which virtual link the GLSP select in the

network. In mathematical form,

(1if the CR-GLSP has a lightpath on link рi; jЮ

eijј	р2Ю

0	otherwise

606

S.K. Lee et al. / Computer Communications 26 (2003) 603–610

where	i; j ј 1; 2; …;lNl	and	i – j:	That is,	eijindicates

whether the CR-GLSP is routed over the virtual link

from node	i	to node j	as in	Ref. [13].

xiindicates the layer 3 routing capability of the node as

follows:

(

if there is no virtual link connecting them. Additionally,

we can incorporate the layer 3 port throughput into

constraint (7). When the trafﬁc ﬂowing through a link is

going forward an intermediate O-LSR, the trafﬁc demand

should not be larger than the sum of the maximum

throughput supported by IP router port:

xiј

0	if node i has no layer 3 processing

1	otherwise

р3Ю

lij# WijC{1 ю рa 2 1ЮQj};

р10Ю

The set of variables has to satisfy the following constraints.

X	X

where	a # 1 denotes the maximum layer 3 port

throughput, expressed as a fraction of the optical layer

j

vij# Ti;

i

vij# Rjfor all i; j:

р4Ю

throughput.

A packet traversing the CR-GLSP experiences an end-to-

where Tiand Rjare the number of transmitters and receivers,

respectively, at node i and j (i, j [ N ).

X

lijј	vijlIEijюeijlIkEk	for all i; j:	р5Ю

I;E

8

end delay ofPi;jeijlIijkEkDijюdnode;where dnode	denotes the

total waiting time at all nodes over the CR-GLSP. The

dominant delay in light to moderately loaded high bandwidth

optical networks is known as the propagation delay, which is

uniquely determined by the length of the optical path over the

physical WDM network. However, since wavelengths and

XlIijkEk2XlIjikEk	ј

j	j

lij# WijC;

>>< lIkEk ;	i ј Ik

2lIkEk ;	i ј Ek

>>:

0;	otherwise:

р6Ю

р7Ю

ﬁbers are ﬁnite, a lightpath is not always established between

each source and destination IP routers. Moreover, if trafﬁc

grooming is available in some nodes, it is not always

necessary to deploy a direct lightpath between two nodes to

carry trafﬁc between them. Instead, the trafﬁc demand

between two nodes may be carried by multiple lightpaths

where C is the capacity of each wavelength on a ﬁber and

Wijdenotes the number of wavelengths per link in the virtual

topology between the nodes i and j for all i and j.

lIijkEkј eijlIkEk :	р8Ю

among some intermediate nodes without direct lightpath

deployment between the two (end) nodes. Consequently, IP

packets are forwarded to the destination by multi-hop IP

routing. By this routing, layer 3 processing may still be a

bottleneck of a network. Thus,	dnode	is deﬁned to be the

eij# vij:

р9Ю

waiting and processing time at O-LSRs, since these nodes are

the likely bottleneck points on any CR-GLSP. The delay

Constraint (4) ensures that we can set up only single

lightpath per port at each node. Constraints (5) and (6)

are related to the trafﬁc ﬂow on virtual link for all	i	and

j. When the new trafﬁc demand is routed, the deﬁnition

of the total trafﬁc over link (i,	j ),	lijis expressed in

constraint (5). Constraint (6) assures that the ﬂow of

trafﬁc is conserved at each node. Although splitting is

used for load balancing purposes by routing demands

over multiple GLSPs at the ingress point, it is not

permissible for the routing algorithm to always split

trafﬁc in an arbitrary manner since the trafﬁc being

routed may be inherently unsplittable. In this paper, the

trafﬁc ﬂow lIkEk	will not bifurcate either at any point in

the network from the fact that the proposed TE algorithm

is trying to route a CR-GLSP request explicitly to meet

the QoS. Thus, constraint (6) is brought in so the trafﬁc

ﬂowing into an OXC to be tantamount to that ﬂowing

out of the OXC for any OXC other than the ingress and

egress O-LSRs. In terms of the constraints, the link

capacity should not be exceeded with the addition of the

new CR-GLSP along its path through the network that is

presented in constraint (7). Constraint (8) speciﬁes that if

the link (i,	j ) does not exist on the CR-GLSP, no trafﬁc

can be routed on that link. Finally, constraint (9) keeps

the new CR-GLSP from being set up between two nodes

increases with the number of hops and the expected

processing delay at the intermediate nodes, since there can

be an arbitrary number of intermediate O-LSRs on the GLSP.

we deﬁne	lIEm	to be the aggregate input rate to the mth

рm ј 1; 2; …; MЮ	O-LSR.	Under	the	independence

assumption on interarrivals we can model the delay

experienced at layer 3 using an	M/M/1 queueing model.

Fig. 2	depicts the operation of such an optical Internet.

Given that	mIEm	be the service rate in each intermediate

node, we can get the average layer 3 processing delay

Fig. 2. Delay with layer 3 processing.

seen by the trafﬁc ﬂow as

S.K. Lee et al. / Computer Communications 26 (2003) 603–610

Step 2: if there is any GLSP to minimize Eq. (13)

then go to Step 4;

607

t ј

X

mј1

| r | IE m |
| --- | --- |

р1 2 rIEmЮlIEm;

р11Ю

else if the request is negotiable

then relax 1kand go to Step 2;

else go to Step 3;

where	rIEmјlIEm=mIEm	is the utilization. Using the local

connectivity and processing rate variables, the above

Eq. (11) becomes

Step 3: Block this GLSP request and go to Step 5;

Step	4: Provision resources for the GLSP and update

network state;

t ј

X

i;j

eijQj

mj2 рP‘l‘jюlIkEk Ю:

р12Ю

Step 5: End;

To satisfy the requirements of diverse routing, rerouting

and restoration as well as TE, ER is necessary for

Thus, for the kth trafﬁc ﬂow belonging to DS class, the

objective function for delay can be written as

X

f рeijЮ ј р	eijlIkEk DijЮ ю t;	р13Ю

i;j

In Eq. (13), the second term	t will be zero if there is no

layer 3 processing at any point on the GLSP. The

objective function, Eq. (13), should be minimized in

order to support the delay QoS requirement of the	kth

ﬂow belonging to the delay-sensitive class. If the

minimum value of Eq. (13) does not satisfy the delay

QoS requirement, the values computed from the mini-

mization would not be applied to the variable	eij.

On top of the constraints in Eqs. (4) – (9), an additional

two constraints should be deﬁned, that are related to QoS

and layer 3 processing rate. One constraint for tolerable QoS

can be deﬁned as

constructing lightpaths. The route on which a new lightpath

is to be established is speciﬁed by an object contained in the

GLSP setup message. This route is typically be chosen by

the ingress O-LSR, but it could be determined by a higher

level network management system. The route may be

speciﬁed either as a series of routers/OXCs, or in terms of

the speciﬁc links used. Therefore, the above mechanism

performs the calculation of primary and restoration light-

path routes on-line as the individual requests arrive. These

lightpaths could be computed all at once by doing an ofﬂine

calculation that accounts for all the pending requests.

Because the network loading varies over time, the

consideration of the optimal route selection could require

the reconﬁguration of established lightpath routes, as

described in Ref. [3]. Although frequent lightpath reroutings

may not be acceptable, a limited number of lightpath

reroutings could improve the network state, supporting the

0 # f рeijЮ # 1k:

р14Ю

requested QoS of the future trafﬁc while maintaining the

QoS of the trafﬁc that the network is already supporting. In

The other one for layer 3 processing rate can be expressed as

mj$ vijXlIEijю eijQjlIkEk	for all I; E:	р15Ю

i

It is generally known that the multicommodity network ﬂow

problem with integer constraints is NP-hard [11]. Thus the k

disjoint route problem which is NP-hard in Ref. [11] can be

dealt with the same as that the k distinct egress node pairs

ﬁnd k mutually link-disjoint routes.

Whenever a new trafﬁc ﬂow belonging to delay-sensitive

class requests explicitly routed GLSP, the virtual lightpath

will be conﬁgured and the network state will be updated.

Unless it is possible to support the GLSP with the desired

QoS, the request can be blocked or renegotiated and

attempted again. Alternatively, the network manager can

preempt the minimum number of lower priority ﬂows that

will allow the GLSP to be set up. The procedure for the kth

request, which does not account for preemption, proceeds as

follows:

Step	0: Obtain the parameters associated with the	kth

setup request

Step 1: if a transmitter or a receiver is not available at Ik

or Ek

then go to Step 3;

the initial conﬁguration stage where there is no conﬁgured

virtual topology, the appropriate virtual lightpath could be

found by repeating the above procedure. This procedure is

applied to the trafﬁc that has the most stringent delay limit

among all initial trafﬁc demands of delay-sensitive service

class at all egress O-LSRs being done so to the trafﬁc with

the next highest QoS requirement, in turn. Like setting up

usual GLSP, the virtual topology would be conﬁgured for

the trafﬁc demand of delay-sensitive service class at each

egress OXC by minimizing the objective function of delay.

By ﬁnding the values of all the elements of the set {lIEij}; we

can obtain a full set of routing assignments for all the trafﬁc

in the optical network.

Failure of LSPs due to link failures is detected via

GMPLS signaling protocol (e.g. CR-LDP or RSVP-TE)

information by the edge routers. They can request a

rerouting of the LSPs after the link-state database has

been updated by routing protocols or by other means. (An

alternative, not studied in the paper, is to setup a disjoint

path backup GLSP so that failures can be accommodated by

changing the FEC to GLSP mapping at the ingress routers.)

Let	S	be the set of Shared Risk Link Groups (SRLGs)

associated with elements of the network. A single link or

node may belong to more than one SRLG. Deﬁne	Sijto

be the set of SRLG identiﬁers associated with the link

608

S.K. Lee et al. / Computer Communications 26 (2003) 603–610

(i,	j ) and deﬁne	Snto the set of all Shared Risk Group

(SRG) identiﬁers associated with node	n. Then the full

set of SRG identiﬁers for the network is just the union of

all these sets: S ј рSn[NSnЮSрS‘[LS‘Ю: In order to set

up a M:N protection group with QoS constraints that

generally routes M backup paths that are node disjoint

from N primary paths, we can proceed in the following

way. Let	K ј {ki}Kiј1	be the set of requests for working

paths in the protection group. For the ﬁrst working path

k1, we can use the algorithm that we have described

above since there is no SRLG/SRG constraint. The

SRLG/SRG identiﬁers associated with the working path

are accumulated in the set	S(k1). In order to compute a

backup path for this working path, which will have the

same source and destination nodes	Ik1andEk1	and the

same delay and bandwidth QoS requirements	1k1	and

Bk1, we can do several things. One possibility is to

execute the Shortest Path First (SPF) algorithm described

in Ref. [4], with some modiﬁcations. Like Eq. (11), let

tnјXrIEn

Fig. 3. Network model.

results of the proposed algorithm and compare it to the well-

known SPF algorithm using a set of simulations. We have

run our algorithm several times using GMPLS Lightwave

Agile Switching Simulator (GLASS) which was developed

by NIST to evaluate control algorithms in integrated

MPLS/optical networks	[14], and calculated average

blocking probability and average end-to-end delay. That

is, the performance is analyzed in terms of the number of

I;E

р1 2 rIEnЮlIEn

rejected CR-GLSP requests and delay.

be the average layer 3 processing delay at node n due to

all the existing trafﬁc ﬂows that are passing through the

node. Whenever a ﬂow is added, modiﬁed, or deleted,

{tn}n[N	should be updated. Recall that we deﬁned	Dijto

be the propagation delay on the link from Node	i	to

Node	j. Deﬁne	Bijto be the available bandwidth on the

link from	i	to j.

Step	1: If	Bij, Bk1, set Dijј 1.

Step	2: If	Sij[ S(k1), set	Dijј 1.

Step 3: (Backup path initialization.) Set P ј {Ik}; m ј Ik:

In the ﬁrst place, we describe a simulation environment

used to validate our algorithm in providing service

requested by delay-sensitive class. The simulation tests

were carried out on a model of the network shown in

Fig. 3(a), which has also been used in	Ref. [15]. In this

topology model, a maximum of four wavelengths are

available for use in each link and the capacity is 10 Gbps.

The propagation delay Dijis assumed to be 0.05 ms for all

links (i, j ), and the maximum tolerable delay limit of every

GLSP request from delay-sensitive trafﬁc is 0.5 ms. For this

topology, we consider in this paper two network models:

1

1

Let f рPЮ ј tIk	be the total delay on the backup path.

one model where the values of the xjare assumed to be 1 for

1

Step 4: Determine the node npwith the lowest delay cost

with respect to node m, which satisﬁes

minn[N;n	PЅDm;nюtn:

Step 5: Add npto P. f рPЮ ј f рPЮ ю Dm;npю tnp:

Step 6: If f рPЮ . 1k1;no backup path satisfying the desired

QoS requirements can be found.

If npј Ek1, the algorithm has executed successfully.

Step 7: Set m ј npand go to Step 4.

If it is not possible to ﬁnd a backup path with the desired

QoS characteristics, the algorithm could be run again with

relaxed values for the maximum delay or minimum

bandwidth, although this could result in a degradation of

performance in the event of a failure on the primary path, even

if the ﬂow is successfully switched over to the backup path.

4. Performance evaluation from simulation

Some simulation experiments have been conducted in

order to study the performance of the proposed CR-GLSP

routing algorithm. In this section, we present the simulation

all the edge nodes and the other one where the network layer

processing overhead is placed on the nodes 5, 8, 9, 12 and

14. Every GLSP request arrives at all edge nodes, according

to a Poisson process with mean arrival rate, lrof 200, 1000,

1500, and 2000 in each experiment, respectively. Like in

Ref. [16], the arrival rates of trafﬁc for the different source

and destination pair were chosen from a uniformly

distributed random variable in the interval [1,	lIE]. The

value of	lIEwas selected as 2.5 Gbps in this simulation

experiment. 10% of total trafﬁc ﬂows belongs to delay-

sensitive service class. We assume that the time between

GLSP set-up and teardown is exponentially distributed with

a mean of 1 min. When a GLSP set-up request arrives at an

ingress node, the destination is chosen randomly among all

the edge nodes except the ingress with the set-up request.

All the simulations were run till 5000 GLSP set-up requests

were generated and each simulation was repeated 10 times

to get the average blocking rate and the average delay

measured during the simulation.

The average blocking performance versus the GLSP set-

up requests is shown in Fig. 4 where graph (a) and (b) is the

performance of overall trafﬁc and that of delay sensitive

S.K. Lee et al. / Computer Communications 26 (2003) 603–610

Fig. 4. Blocking probability for model 1.

609

trafﬁc, respectively. Fig. 5 also shows the average blocking

performance from the other set of simulations where the

location of intermediate O-LSRs is changed, i.e. the layer 3

processing is at nodes 5, 8, 9, 12 and 14. The average

blocking rate increases at the increasing of the GLSP set-up

requests. Moreover, the gain due to the consideration of

network layer processing is evident from these graphs.

Therefore, we ﬁnd that the proposed CR-GLSP algorithm

performed better with respect to the blocking rate, which

means that the CR-GLSP algorithm utilizes network

resources more efﬁciently. This improvement is realized

due to the fact that although CR-GLSP algorithm would not

be able to ﬁnd the available bandwidth over the path with

the minimum delay, it tries to utilize the path with the next

minimum delay keeping the tolerable delay.

Without performance degradation in blocking rate, we

obtain the better performance in terms of average end-to-

end delay. The average delay associated with both

algorithms is presented in	Table 1	measured during the

overall simulation time for the two topology models where

model 1 performs layer 3 processing at all the edge nodes

and the model 2 does so at only the 5 nodes listed above.

From	Table 1, it can be observed that the average delay

increases at the increasing of the GLSP set-up requests since

the more GLSPs are requested to be established, the more

the trafﬁc ﬂows into the network. Despite the increasing of

the average delay, the average delay of the CR-GLSP

algorithm is still smaller than that of SPF algorithm. This is

due to the fact that the CR-GLSP algorithm incorporates

network layer processing delay into routing decision. It

resulted from this incorporation that the CR-GLSP algor-

ithm chose different path from that in SPF algorithm

between source node 2 and destination node 9. As can be

seen in Fig. 3(b), the SPF algorithm routed the GLSP over

the route 2 ! 1 ! 4 ! 9 while the CR-GLSP algorithm

selected the route 2 ! 3 ! 7 ! 9.

Therefore, it is known that our formulation in CR-GLSP

is applicable to ER of backup GLSP as well as working

GLSP in large IP over WDM networks with QoS

constraints.

Fig. 5. Blocking probability for model 2.

610

Table 1

S.K. Lee et al. / Computer Communications 26 (2003) 603–610

QoS. At that time, it is worth noting multiple service

Delay performance (Unit: ms: 1/lr; GLSP requests)

categories whose level of restoration is different.

1/lr

Model 1

200

1000

1500

2000

Model 2

200

1000

1500

2000

5. Conclusion

CR-GLSP algorithm

0.266

0.292

0.321

0.343

0.215

0.228

0.257

0.275

SPF algorithm

0.424

0.477

0.499

0.521

0.371

0.417

0.421

0.429

References

[1] E. Rosen, A. Viswanathan, R. Callon, Multiprotocol Label Switching

Architecture, RFC 3031, January, 2001.

[2] A. Banerjee, J. Drakes, J.P. Lang, B. Turner, K. Kompella, Y.

Rekhter, Generalized multiprotocol label switching: an overview of

routing and management enhancements, IEEE Commun. Mag.

January (2001).

[3] G. Apostolopoulos, D. Williams, S. Kamat, R. Guerin, A. Orda, T.

Przygienda, QoS Routing Mechanisms and OSPF Extensions, RFC

2676, August, 1999.

[4] Z. Wang, J. Crowcroft, “Quality-of-Service Routing for Supporting

Multimedia Applications”, IEEE JSAC 14 (7) (1996).

[5] N. Golmie, T.D. Ndousse, D. Su, A differentiated optical services

model for WDM networks, IEEE Commun. Mag. 38 (2) (2000).

[6] H. Harai, F. Kubota, H. Nakazato, Design of reconﬁgurable lightpaths

The success of next generation IP over WDM networking

depends on the ability to offer and support QoS to

customers. It is clear that network layer processing cannot

be neglected for designing TE algorithms as well as for this.

In this paper, we formulated an optimization problem for

constraint-based routing to support the QoS requested by

delay-sensitive trafﬁc in optical Internet. The CR-GLSP

algorithm described in this paper is based on the idea that

trafﬁc ﬂows encounter the delay resulting from layer 3

processing at subnetwork edge nodes. The algorithm uses

the current state of the network to determine the delay

associated with each possible path, and then chooses the

path with the minimum total delay. The simulation results

showed that the proposed algorithm improves the delay

performance as well as the probability of successful GLSP

establishment in comparison to SPF algorithm, which do not

take the network layer processing into account. In the

simulation test, GLASS tool was used as a simulation tool

that was developed by NIST for optical Internet. The

presented algorithm can also be used for establishing

backup path with delay constraint and we described how

to apply to protection.

The future study is network survivability which is also

treated as an important issue recently. We are designing a

several efﬁcient restoration algorithms that ensure its

survivability in optical Internet while we sustain the delay

in IP over WDM networks, IEICE Trans. Commun. E83-B (10)

(2000).

[7] D. Awduche, J. Malcolm, J. Agogbua, M. O’Dell, J. McManus,

Requirements for Trafﬁc Engineering over MPLS, RFC 2702,

September, 1999.

[8] Data Connection Computer Technology, MPLS Trafﬁc Engineering:

A Choice of Signaling Protocols,	http://www.dataconnection.com/

download/crldprsvp.pdf.

[9] P. Ashwood, A. Banerjee, L. Berger, G. Bernstein, J. Drake, Y. Fan,

K. Kompella, J.P. Lang, F. Liaw, E. Mannie, P. Pan, B. Rajagopalan,

Y. Rekhter, D. Saha, V. Sharma, G. Swallow, Z.B. Tang, Generalized

MPLS Signaling-RSVP-TE Extensions, Work on Progress, draft-ietf-

mpls-generalized-rsvp-te-04.txt, July, 2001.

[10] P. Ashwood, A. Banerjee, L. Berger, G. Bernstein, J. Drake, Y. Fan,

D. Fedyk, K. Kompella, E. Mannie, J.P. Lang, B. Rajagopalan, Y.

Rekhter, V. Sharma, G. Swallow, Z.B. Tang, Generalized MPLS

Signaling-CR-LDP Extensions, Work on Progress, draft-ietf-mpls-

generalized-cr-ldp-04.txt, July, 2001.

[11] M. Garey, D. Johnson, Computers and Intractability, W.H. Freeman,

New York, 1979.

[12] I. Chlamtac, A. Ganz, G. Karmi, Lightpath communications: an

approach to high bandwidth optical WANs, IEEE Trans. Commun. 40

(7) (1992).

[13] R. Ramaswami, K.N. Sivarajan, Optical Networks: a Practical

Perspective, Morgan Kaufmann, Los altos, CA, 1998.

[14] NIST, http://www.antd.nist.gov/proj/asia/glass/.

[15] K. Kar, M. Kodialam, T.V. Lakshman, Minimum interference routing

of bandwidth guaranteed tunnels with MPLS trafﬁc engineering

applications, IEEE JSAC 18 (12) (2000).

[16] I. Widjaja, A.I. Elwalid, Study of GMPLS Lightpath Setup over

Lambda-Router Networks, ICC’02, May, 2002.