Computer Communications 25 (2002) 1699–1710

www.elsevier.com/locate/comcom

Accurate bandwidth measurement in xDSL service networks

Liang Cheng*, Ivan Marsic

Department of Electrical and Computer Engineering and the CAIP Center, Rutgers, The State University of New Jersey, Piscataway, NJ 08854-8088, USA

Received 14 May 2001; revised 6 February 2002; accepted 27 February 2002

Abstract

In xDSL service networks, accurate bandwidth measurement is essential for network management and trafﬁc engineering, such as isolating

line faults and verifying guaranteed quality of service speciﬁcations, where ISP and xDSL service providers may be involved. Existing

Internet bandwidth measurement techniques are not suitable because link asymmetry and ATM trafﬁc shaping in xDSL deployments impact

the accuracy of their measurements. In this paper, we present a novel stepwise algorithm for bandwidth measurement that performs

accurately in xDSL service networks. The stepwise algorithm requires a smooth trafﬁc generator since otherwise the accuracy would be

affected by the ATM trafﬁc shaping. A multi-packet technique is used to avoid explicit packet ﬁltering. The scheme is lightweight in terms of

implementation complexity and probe trafﬁc overhead. The accuracy of the algorithm is demonstrated by laboratory and ﬁeld experiments.

The results show that the measurement errors in both upstream and downstream cases are within the range of 5% in a commercial ADSL

service network tested under the following ATM network conﬁgurations: CBR, rt-VBR, and UBR service categories. q 2002 Elsevier

Science B.V. All rights reserved.

Keywords: xDSL networks; Bandwidth measurement; Asymmetric networks; ATM trafﬁc shaping

1. Introduction

The growth in deployment of Internet applications

requiring high bandwidth has created a need for faster

local loops for users connecting to the Internet through a

dial-in Internet Service Provider (ISP). In most cases, a

serial modem solution, which offers a maximum bandwidth

of up to 56 Kbps, is not sufﬁcient. Therefore, various

technologies, such as Digital Subscriber Line (DSL) and

cable modems, have emerged to achieve high-speed

local-loop connectivity to the Internet. Local phone

companies in the United States have been installing

commercial xDSL lines since 1998. xDSL refers to

different variations of DSL, such as Asymmetric DSL

(ADSL), Symmetric DSL (SDSL), High bit-rate DSL

(HDSL), Rate Adaptive DSL (RADSL), and Very High-

speed DSL (VDSL)	[22]. xDSL has several advantages

compared to other access technologies [9]: (i) its initial

cost can be predicted and spent only when a customer

requests the service, (ii) it requires no change to the

central ofﬁce switch software, (iii) it can interface with

a number of different premises arrangements, (iv) a

* Corresponding author. Tel.: ю 1-732-445-0660; fax: ю 1-732-445-4775.

E-mail addresses: chengl@caip.rutgers.edu (L. Cheng),

marsic@caip.rutgers.edu (I. Marsic).

DSL line can simultaneously carry both data and voice

signals with the data part of the line continuously

connected. In addition, xDSL can be used for residential

users, Small Ofﬁce, Home Ofﬁce (SOHO) users, and

large organizations because of its wide range of service

rates. For example, ADSL enables receiving data at

rates up to 6.1 Mbps (of a theoretical 8.448 Mbps).

Typical ADSL connection services provide from

512 Kbps to 1.544 Mbps downstream (from service

provider to customer, or from Central Ofﬁce to Customer

Premises) and about 64 – 128 Kbps upstream. The down-

stream speeds of VDSL range from 13 to 55 Mbps

depending on distance, while its upstream speeds start at

1.5 Mbps and go up to about 26 Mbps [9].

The xDSL deployment architecture in the ADSL case is

abstracted in Fig. 1. The customer’s premises equipment is

an ADSL Termination Unit—Remote (ATU-R) or an ADSL

modem. An ADSL Termination Unit—Central Ofﬁce

(ATU-C) is located at a Central Ofﬁce end of the phone

company that provides the xDSL service. The ATU-Cs are

grouped into a DSL Access Module (DSLAM) unit that

terminates the multiplexed trafﬁc into an ATM switch. In

some cases, an IP router can be used instead of the ATM

switch depending on budget constraints. However, the

performance aspects of ATM networks and services justify

the use of ATM switches	[9]. In the ﬁeld experiments

0140-3664/02/$ - see front matter q 2002 Elsevier Science B.V. All rights reserved.

PII: S 0 1 4 0 - 3 6 6 4 ( 0 2 ) 0 0 1 0 4 - 4

1700

L. Cheng, I. Marsic / Computer Communications 25 (2002) 1699–1710

Fig. 1. xDSL deployment architecture (ADSL case).

reported here, an ATM switch is used. The ATU-C

terminates multiple subscriber loops and connects the

ATU-R to the Internet through a Gateway Router. The

deployment and maintenance of the line between the ATU-

R and the Gateway Router is the responsibility of the phone

company and there is a need to measure the actual physical

line speed of the ADSL link for reasons explained below.

1.1. Motivation for accurate bandwidth measurement

In addition to general beneﬁts that can be gained by

measuring network link bandwidth, such as application

adaptation to network conditions	[17], there are speciﬁc

motivations for accurate bandwidth measurement in xDSL

service networks. In this paper, we study the link bandwidth

measurement, which is the smallest bandwidth (bottleneck-

link bandwidth) along the xDSL data path in the xDSL

service network. (The xDSL service network, or ‘xDSL

network,’ refers to the customer and the ADSL service

provider parts in Fig. 1 and does not include the ISP part.) It

can also be the case that applications themselves might be

interested in available bandwidth, given that several

applications are simultaneously using the same pipe,

however, this is not studied here.

Commercial xDSL networks combine the beneﬁts of the

Discrete Multi-Tone (DMT) or Carrierless Amplitude Phase

(CAP) modulation and ATM technologies, which result in

bandwidth and service ﬂexibility. For example, in ADSL

service networks upstream and downstream bit rates can be

chosen freely and quasi-continuously up to the maximum

physical limits. Based on the maximum possible bit rate

(with a predetermined margin), the service management

system of an xDSL network can set the bit rate to the level

determined by the customer’s service proﬁle, thus max-

imizing noise margin and/or minimizing transmit power. In

addition, xDSL networks can support a random mix of

services with different bit rates and trafﬁc requirements,

which are introduced on a per-customer basis.

Accurate bandwidth measurement in xDSL service

networks is essential for service provisioning. For example,

customers experiencing slow Internet-connection speeds

may want to know what the exact service parameters are. In

this case, a bandwidth measurement tool can help the

customers and the service providers (both the ISP and the

xDSL providers) to verify service quality. Accurate

bandwidth measurement thus contributes to better customer

service. For example, it can help customers identify which

xDSL category is the most appropriate for them. They can

determine whether ADSL is sufﬁcient or whether SDSL is

needed.

Accurate bandwidth measurement is important for net-

work management in xDSL networks, such as isolating line

faults and performing trafﬁc engineering. How xDSL

networks should be tested, repaired, and managed is in

fact an outstanding issue in the provisioning of xDSL

services	[9]. From the network administrator’s point of

view, trafﬁc loads can be balanced knowing the bandwidths

in xDSL networks. Lastly, accurate bandwidth measure-

ment is helpful for xDSL-network design. Once a bottleneck

link is identiﬁed, extra capacity can be added to enhance the

network performance.

1.2. Related work

A number of techniques exist for bandwidth measure-

ment. Most of these can be categorized into two groups [16].

One group is a variant of pathchar [7,11,12] based on the

one-packet model. The disadvantage of this group is the

heavy overhead in bandwidth consumption. The other is a

variant of packet-pair	[3,4,17,20,21] based on the packet-

pair model. This group imposes lower overhead compared

to the ﬁrst group.

In	Ref. [16], a multi-packet model is presented to unify

the one-packet and the packet-pair model and a packet-

tailgating technique	tailgater	is proposed. However, as

pointed out in Ref. [15], similar to pathchar, the tailgater

measures individually the bandwidth of each link along a

path, which can be time-consuming and unnecessary for

applications that only want to know the bottleneck

bandwidth such as the xDSL case studied here.

Our focus is on xDSL networks and the common

disadvantage of the existing techniques is that their

accuracy is not satisfactory for xDSL service provisioning

since their error range is greater than 5%, and sometimes

10%. This is mainly due to ATM trafﬁc shaping in xDSL

service networks as explained in Section 2.2. For example,

L. Cheng, I. Marsic / Computer Communications 25 (2002) 1699–1710

1701

Ref. [15]	reports that	nettimer	has a measurement error

greater than 10% of the nominal ADSL link bandwidth.

In addition to bandwidth measurement, the related

throughput measurement techniques have been used in

applications and protocols, such as Network Weather

Service (NWS) [23]	and TCP. However, these techniques

are likewise not accurate and not suitable for bandwidth

measurement in xDSL networks. For example, during each

measurement in NWS [23], the round-trip time of a single-

word packet in a TCP connection is measured and the

resulting value is divided by two to yield an approximation

of the latency or start-up overhead associated with the

communication. Obviously, the approximation is not correct

in ADSL’s asymmetric case. The approximation is rough

without considering the ﬂow and congestion control by

TCP. There is also ongoing research	[6]	addressing

problems in bandwidth measurement caused by the ATM

network model and link asymmetry. However, no public

information is available as of now.

In theory, the one- or two-packet techniques and the

tailgating technique can get good estimation of xDSL link

bandwidth. However, the existing tools do not regulate the

probe packets so that ATM trafﬁc shaper will regulate the

probe packets for them by packet delay or packet drop. One

possible solution is to explicitly regulate the probe trafﬁc,

but then it is difﬁcult to specify the source rate. That is why

we propose a stepwise scheme: in the ﬁrst step we get a

rough estimate of the bandwidth and regulate the subsequent

probe trafﬁc according to the estimate. We also use multi-

packet technique to avoid explicit packet ﬁltering.

In our research, we designed an accurate bandwidth

measurement	scheme	for	xDSL	service	network,

implemented it as a Java-based tool, and tested it in both

laboratory and commercial setup experiments. The accuracy

is achieved by implementing a novel stepwise bandwidth

measurement algorithm with an original trafﬁc generator for

smooth trafﬁc ﬂow and a multi-packet bandwidth measure-

ment technique. The experiments reported in this paper

demonstrate the accuracy of our measurement scheme.

The primary concern of this paper is the measurement

accuracy which is essential for xDSL service provisioning.

Other concerns could be imposed by protocol or layer

constraints (it may be required to use certain protocol, such

as TCP, User Datagram Protocol (UDP), ICMP, etc.) or by

packet type constraints (it may be required to use payload

packets for measurement, as opposed to probe packets

which add overhead trafﬁc). We are not considering those

concerns here, but we are using transport layer ﬂows. The

advantage of using transport layer ﬂows, e.g. a UDP packet

ﬂow, is that they can penetrate Network Address Translation

(NAT) gateways, which are more and more used in xDSL

deployments.

The paper is organized as follows. Section 2 discusses

ATM trafﬁc shaping and its effect on the accuracy of

bandwidth measurement. Section 3 introduces a multi-

packet bandwidth measurement technique and its appli-

cations to ADSL. Section 4 presents a stepwise algorithm

for accurate bandwidth measurement using a trafﬁc

generator with stable and accurate performance. Section 5

presents the evaluation results and Section 6 concludes the

paper.

2. ATM trafﬁc shaping

xDSL deployments are typically implemented over ATM

to provide different service qualities. For example, the

ATU-Cs in ADSL networks are grouped into a DSLAM unit

that terminates the multiplexed trafﬁc into an ATM switch

(Fig. 1). Based on the synchronization speed of the ADSL

modem, the ATM switch sets the service class for the virtual

circuit as the class to which a particular customer has been

mapped.

There are ﬁve service categories/classes in ATM net-

works speciﬁed by the ATM Forum	[1]: Unspeciﬁed Bit

Rate (UBR), Available Bit Rate (ABR), Constant Bit Rate

(CBR), Real-Time Variable Bit Rate (rt-VBR), and Non-

Real-Time Variable Bit Rate (nrt-VBR) services. UBR

service does not have any guarantee with regard to the

quality of service (QoS) such as delay, loss or bandwidth.

No trafﬁc management is performed in the ATM layer for

the UBR service class, so the applications using UBR

service should be able to handle the ﬂuctuations in these

parameters by using error-correction and ﬂow control

techniques. ABR service guarantees only the cell-loss rate

(CLR), which is achieved by exchanging resource manage-

ment cells between the source and sink ATM nodes. CBR

service class is suitable for applications having strict QoS

requirements in delay and loss because it guarantees that a

static amount of bandwidth is always available to an

accepted connection. It is characterized by Peak Cell Rate

(PCR) and Cell Delay Variation Tolerance (CDVT). In

rt-VBR and nrt-VBR service classes, PCR, CDVT,

Sustained Cell Rate (SCR), and Maximum Burst Size

(MBS) are used to specify the trafﬁc parameters so that

they can utilize network resources efﬁciently by multi-

plexing several connections with bursty trafﬁc.

2.1. Trafﬁc shaping and buffer management

The goal of ATM trafﬁc shaping is to regulate the trafﬁc

ﬂow as per the parameters describing the negotiated QoS to

achieve better network efﬁciency. The basic idea of ATM

trafﬁc shaping is buffering the packets/cells and controlling,

mostly delaying, their entry into the network, thereby

ensuring a more constant ﬂow of trafﬁc in the network.

There are two important design factors in ATM trafﬁc

shaper: buffer management and scheduling algorithms. As

discussed in Section 3.2 below, packet loss is a signiﬁcant

factor affecting the accuracy of bandwidth measurement in

xDSL networks. Here we discuss the buffer management

since it directly affects the packet loss.

1702

L. Cheng, I. Marsic / Computer Communications 25 (2002) 1699–1710

Fig. 2. Trafﬁc shaping illustration diagram.

In	general,	different	service	categories	are

implemented through different queues in an ATM

switch. Buffering method deﬁnes the amount of buffer

space available to a given queue and speciﬁes how the

space is shared among the different queues. In the

Complete Partitioning	scheme, each queue gets a ﬁxed

amount of the buffer space while in the	Complete

Sharing	scheme, all buffer space is fully shared among

all the queues. As a compromise between the two

schemes, the	Sharing with Minimum Allocation	scheme

reserves a minimum buffer space for each queue while

the rest of the buffer is completely shared among the

queues	[19]. Commercial ATM implementations favor

buffer sharing schemes because of lower memory

requirements, e.g. Cisco LightStream 1010 with total

buffer capacity of only 64K cells	[18].

Fig. 2 and the following discussion adapted from Ref.

[19]	illustrates that the amount of the buffer space in the

ATM trafﬁc shaper is a major factor in determining if and

when cell loss occurs. If we denote the input trafﬁc rate rvas

positive and the output trafﬁc rate roas negative values in

Fig. 2, the sum of the averages of the input and output trafﬁc

rates, r, which is denoted in Eq. (1), will determine the cell/

packet loss

2.2. Effects of ATM trafﬁc shaping on bandwidth

measurement

The network model for bandwidth measurement is

shown in Fig. 3. The ADSL service rates are independent

of the ATM service categories chosen. The ADSL service

rates are chosen by using different line cards on the ATU-C

unit. We assume that the ADSL link is always the bottleneck

link since ATU-R, e.g. ADSL modems, will accommodate

ATM transport with variable rates and compensation for

ATM overhead [2]. The bearer channel of ADSL interface

can be programmed to carry bits in any multiple of 32 Kbps.

For example, when ADSL is used for downstream ATM cell

transport, service rates of 1.760, 3.488, 5.216, and

6.944 Mbps are the options. The odd-looking speeds are a

result of the desire to preserve compatibility with existing

AAL1 and circuit deﬁnitions already entrenched in ATM

speciﬁcation. ADSL service rates that are not simple

multiples of 32 Kbps can be supported by carrying ‘extra’

bits in the shared overhead area of ADSL frame	[9].

Therefore, the measured bandwidth in a general case is the

nominal value of the ADSL service rate since the ADSL link

is the bottleneck.

However, ADSL link bandwidth can sometimes be larger

than the ATM link bandwidth because of the following

r ј roю rvрt1=t0Ю

р1Ю

reasons: (i) the characteristics of a copper telephone line

vary over time, even from daylight to night time, due to

The variables t1and t2deﬁne the ON and OFF periods

of the ON/OFF VBR trafﬁc ﬂow, and	t0ј t1ю t2(Fig.

2). If	r	is positive, it means that the trafﬁc shaper

receives data at a greater rate than it can deliver, and

thus the cell/packet loss will eventually occur. If it is

zero or negative, a zero loss can be achieved by

buffering the cells during the bursty period	t1, and the

amount of data that must be buffered is	t1(rvю ro). In

this case some cells will be delayed.

temperature, other trafﬁc in the same cable bundle, and

external RF interferences	[10]; and, (ii) the matching

difference between the rate of the bearer channel of ADSL

interface and the rate of the ATM switch. In these cases, the

ATM link will be the bottleneck link. However, because

both the ADSL service rate and ATM bit rate are supposedly

conﬁgured to be the same, the measured bandwidth still

represents the nominal value of the ADSL service rate.

In the cases of downstream trafﬁc or upstream trafﬁc with

Fig. 3. Network model for bandwidth measurement.

L. Cheng, I. Marsic / Computer Communications 25 (2002) 1699–1710

1703

Fig. 4. Packet-pair technique for bandwidth measurement (adapted from

Ref. [15]).

ATM bit rate being lower than the ADSL service rate, the

ATM trafﬁc shaping mechanism may introduce inaccuracies

in bandwidth measurement. Bandwidth measurement tech-

niques, such as pathchar in Ref. [11], send a large number of

probe packets without explicit trafﬁc regulation, which

hundred for each measurement, while the number of packets

used by other currently available bandwidth measurement

tools [16] is of the order of a thousand. Thus our scheme is

lightweight in terms of implementation complexity and

probe trafﬁc overhead. We ﬁrst describe the packet-pair

technique and then present the multi-packet technique.

3.1. Packet-pair technique

The packet-pair technique for bandwidth measurement is

based on FIFO queuing network model. Various forms of

the packet-pair technique are studied by Bolot	[3], Carter

and Crovella	[4], Paxson [20], and Lai and Baker [17]. Its

essential idea is using inter-packet time to estimate the

characteristics of the bottleneck link. If two packets, e.g.

Internet Control Message Protocol (ICMP) probe packets,

travel together so that they are queued as a pair at the

bottleneck link with no packet intervening between them,

then their inter-packet spacing is proportional to the

processing time required for the bottleneck link to transmit

the second packet of the pair (Fig. 4). Therefore the link

bandwidth of the bottleneck link, b, can be computed as:

subjects them the ATM trafﬁc shaping. The shaper either

delays the packets or overﬂows and causes some probe

b ј P=D

р2Ю

packets to be dropped. The phenomenon is similar to the

scenario where for some service classes, such as the CBR

where P is the packet size. Note that the inter-packet spacing

at the source must be sufﬁciently small so that:

service, an increase in the data rate beyond the provisioned

bandwidth may result in heavy packet loss caused by the

ATM trafﬁc shaping. The effect on the bandwidth

measurement is distorting the results since the accuracy of

those techniques depends on timely delivery of the probe

t22 t1# P=b ј t42 t3

and

D ј t42 t3

р3Ю

р4Ю

trafﬁc.

To minimize the effect of ATM trafﬁc shaping, we

designed a stepwise bandwidth measurement algorithm. In

the ﬁrst step, a few packets (with the total size less than

10 KB) are used to get the trial bandwidth of the xDSL link.

No packet loss is expected because the buffer in ATM

switches is generally big enough to tolerate a small burst.

The discussion in Ref. [15]	about the measurement agility

implies that such small amount of data is suitable to get a

rough estimate of the nominal bandwidth. Then in the

following steps a relatively large number of packets are used

for results convergence and consistence. The packets are

generated from a smooth trafﬁc generator so that most of

them are not dropped by the ATM trafﬁc shaper.

3. Multi-packet bandwidth measurement

Our bandwidth measurement scheme uses a multiple

packet technique instead of two packets in the packet-pair

technique in order to achieve accuracy of the measurement,

without deploying explicit packet-ﬁltering techniques [16].

Our experience shows that the number of probe packets

required to achieve accurate measurement is on the order of

We generalized this technique to a multi-packet technique

and applied it to achieve accurate bandwidth measurement

in xDSL networks, including ADSL networks.

3.2. Multi-packet technique

Since packet-pair methodology depends on the correct

inter-packet time, two factors could diminish the accuracy

of measurement results	[16]. One is called	time com-

pression, where the ﬁrst packet of the packet pair gets

delayed because of some other cross trafﬁc along the

measurement path and the time interval of the packet pair is

decreased. Thus the result will be larger than the actual

bandwidth. The other is called	time extension, where the

second packet of the packet-pair gets delayed and the time

interval is increased. Thus the result will be smaller than the

actual value. The packet loss in the packet-pair falls in the

latter case. Packet-pair ﬁltering algorithms	[16]	can be

applied to handle these cases so that inaccurate packet-pair

time intervals are ﬁltered out and the measurement results

become more accurate.

In xDSL networks, customers and administrators use our

tool as a network testing or monitoring tool. The data path

from the end-user’s computer to the bandwidth measure-

ment server is an ATM-circuit-like path. It is safe to assume

1704

L. Cheng, I. Marsic / Computer Communications 25 (2002) 1699–1710

packets are simultaneously sent, no packet will be dropped

by the ATM trafﬁc shaping. The ﬁrst step in our stepwise

algorithm conforms to Theorem 1 by using a very small

number of packets. The algorithm is presented in Section 4

below.

The condition of sending the packets at the same time can

be relaxed if the time interval between consecutive packets

is not greater than the time required to transmit a packet

over the bottleneck link. This is illustrated in Fig. 4, where

the inter-packet spacing after the bottleneck link does not

Fig. 5. Upstream multi-packet bandwidth measurement.

that there is no cross trafﬁc, meaning that time compression

is not a problem in our case. However, time extension may

occur due to ATM trafﬁc shaping in xDSL service and due

to possible packet loss. Hence, we apply a	maximum

operator to handle the inaccuracy caused by time extension.

In order to reduce the effect of	time extension, we

propose to use multiple packets instead of two packets in

packet-pair to conduct the bandwidth measurement. The

multi-packet technique does not need to deploy explicit

packet ﬁltering in order to achieve accurate measurement

because of its intrinsic ﬁltering due to the averaging of

results over the multiple packets. Theorem 1 provides the

foundation for the multi-packet technique.

Theorem 1.	Multi-packet bandwidth measurement.	Let

bmin# biр;i; 1 # i # mЮ; where m is the number of links in

the path and biis bandwidth of the i-th link between the node

i 2 1 and the node i. Then if multiple, N ю 1, packets of the

same size рs0ј s1ј · · · ј sNј sЮ are sent at the same time

рt0ј t1ј · · · ј tNЮ	and there is no packet drop and no

cross-trafﬁc along the path, they will arrive with a difference

in time equal to the sum of all packet sizes divided by the

smallest bandwidth along the path, i.e. tNрmЮ	2 t0рmЮј

рs Ј NЮ=bmin; or bminј рs Ј NЮ=рtNрmЮ	2 t0рmЮЮ; where tj(‘)is

the time when packet j fully arrives at node ‘.

Using results from	Ref. [16], we prove Theorem 1 in

Appendix A.

3.3. Discussion

Theorem 1 assumes that there is no packet drop, which

means that N should be small enough to ensure that when N

Fig. 6. Downstream multi-packet bandwidth measurement.

depend on the inter-packet spacing before the bottleneck

link, as long as the latter is smaller. This is also stated in Eq.

(3). In the second and subsequent steps of the stepwise

algorithm, a smooth trafﬁc generator sends the packets with

the inter-packet intervals such that the rate is slightly higher

than the bottleneck link bandwidth estimated in the ﬁrst

step. Therefore, Theorem 1 still holds valid.

3.4. Application to asymmetric DSL

The asymmetric nature of the ADSL network makes it

necessary to have different measurement methodologies for

upstream and downstream cases. The design is based upon

the assumption that the upstream bandwidth is lower than

the downstream bandwidth.

3.4.1. Upstream methodology

A ﬁxed number, N ю 1, of UDP packets of uniform size,

P bytes, are sent from the customer’s computer (client) at a

rate slightly higher than the nominal bottleneck bandwidth

of the ADSL network. Probe trafﬁc with a slightly higher

rate (about 10%) is necessary to saturate the pipe. A server

process on the Bandwidth Measurement Server (Fig. 1),

echoes back the packets as they arrive at the server end. Fig.

5 shows this scheme. The time difference, T, is measured

between the arrival of the ﬁrst packet and the last packet at

the client end. The upstream bottleneck link bandwidth, b1,

is computed as:

b1ј рN Ј P Ј 8Ю=T ЅKbps	р5Ю

3.4.2. Downstream methodology

A trafﬁc generator at the Bandwidth Measurement Server

generates a downstream trafﬁc. A receiving process at the

client measures the arrival time of the packets. Because of

the nature of ADSL, such that the upstream bandwidth is

smaller than the downstream bandwidth, the client does not

echo the probe packets back. Instead, the client computes

the downstream bandwidth using the same Eq. (5) as for the

upstream case. Fig. 6 shows the downstream scheme. The

downstream method can be used for the upstream case, but

the results should then be sent from the Bandwidth

Measurement Server back to the client for display.

L. Cheng, I. Marsic / Computer Communications 25 (2002) 1699–1710

1705

Table 1

Pseudo-code for the simple trafﬁc generator

//Initialize:

//maxNumOfPackets is the maximum number of packets

allowed to be sent

//packetSize is the size of packet in bits, rate is the

expected trafﬁc rate

delay ј packetSize/rate;

for (k ј 0; k , maxNumOfPackets; kю ю)

{

sleep(delay);

sendPacket();

}

4. Stepwise bandwidth measurement algorithm with

smooth trafﬁc generator

4.1. Stepwise bandwidth measurement algorithm

Our stepwise algorithm consists of at least two steps. It

can be used for both downstream and upstream measure-

ments described in Section 3.4. The generated trafﬁc should

be smooth rather than bursty to avoid excessive ATM trafﬁc

shaping. We run the stepwise algorithm several times and

report the maximum measured value as the link bandwidth.

During the ﬁrst step, a small number of packets, e.g. 10,

Table 2

Pseudo-code for the new trafﬁc generator

//Initialize:

//maxNumOfPackets is the maximum number of packets

allowed to be sent

//packetSize is the size of packet in bits, rate is the

expected trafﬁc rate

//pps: the number of packets to be sent per second

pps ј rate/packetSize;

fraction ј 0;

stopSleep ј getTimeMillis(); //get the current time in

ms

startSleep ј stopSleep;

i ј 0; //number of packets have been sent

do

{

sleep(1); //sleep for 1 ms

stopSleep ј getTimeMillis();

packetCount ј (int) (fraction ю pps * (stopSleep 2

startSleep)/1000);

are sent sequentially back-to-back from the client to the

server. This small number of packets will not likely be

affected by ATM trafﬁc shaping, as most ATM networks are

capable of handling such small bursts. The computed result

is used in the second step as the trial bandwidth of the xDSL

link.

The subsequent step(s) assumes that the accurate xDSL

bandwidth is close to the trial result obtained from the ﬁrst

step. A larger number of packets than that in the ﬁrst step are

used in the measurement to ensure that the results are

convergent and in the end consistent. Had it not been for the

ﬁrst step that approximately determined the bandwidth, the

sustained higher-than-provisioned trafﬁc would result in a

considerable loss or queuing delay caused by ATM trafﬁc

shaping, and this would deteriorate the results, as explained

in Section 2.2 earlier.

4.2. Smooth trafﬁc generator

As discussed in Sections 2.2 and 3.3 above, the accuracy

of the bandwidth measurement depends on whether the

trafﬁc generator can generate a smooth trafﬁc at a rate

slightly higher than or equal to the nominal bottleneck

bandwidth of the xDSL network, thus it is crucial to

implement an accurate and stable trafﬁc generator with

smooth trafﬁc ﬂow.

A straightforward method of generating packets at a

known rate employs the algorithm shown in Table 1.

While the above pseudo-code seems to be correct in

theory, it does not yield the expected behavior. Owing

to the scheduling mechanisms of operating systems and

coarse granularity of timers, the packets are not

necessarily sent at the required rate with uniform

spacing. Consider the case when the call to	sleep()

returns and the process is scheduled out. Instead of

sending a packet immediately, the packet is sent only

when the OS reschedules the process. This results in

data rate oscillations or bursts in trafﬁc generating,

which is shown in Section 4.3 later.

To overcome the randomness caused by the scheduling

mechanisms of operating systems and coarse granularity of

timers, we devised an alternative way of generating smooth

trafﬁc at the desired rate. The pseudo-code for the improved

algorithm is shown in Table 2. The algorithm achieves the

stable and smooth trafﬁc ﬂow by adjusting the trafﬁc nearly

fraction ј (fraction ю pps

*

(stopSleep 2

every millisecond. It is similar to the leaky bucket algorithm;

startSleep)/1000) 2	packetCount;

startSleep ј stopSleep;

upLimit ј i ю packetCount;

if(upLimit . maxNumOfPackets)

upLimit ј maxNumOfPackets; //reached the maximum

number of packets allowed to be sent

for (; i , upLimit; iю ю)

sendPacket();

} while(i , maxNumOfPackets);

see, e.g. Section 13.3.4 in Ref. [13]. Intuitively, for every

millisecond the trafﬁc generator accumulates tokens into

packetCount, which is a real number, e.g. 1.3. The

algorithm checks the	packetCount	every millisecond

and the integer part represents the number of packets to be

sent (if greater than zero). The remainder, called	frac-

tion, e.g. 0.3, is then used in the next millisecond’s

accumulation. The performance of the new trafﬁc generator

is shown in Section 4.3.

1706

L. Cheng, I. Marsic / Computer Communications 25 (2002) 1699–1710

Fig. 7. Performance comparison for trafﬁc generators. The parameters used in simulation are: a ј 1; b ј 10:

The packet size is computed as:

P ј MAC header ю IP header ю UDP header ю payload

р6Ю

The default payload size is 1400 bytes for packets to ﬁt the

Ethernet frame size. The payload is a ZIP-compressed ﬁle to

avoid the effects of the compression on the V90 analog

modem.

4.3. Performance study of the trafﬁc generator

We simulated in Matlab both the simple trafﬁc generator

caused by the scheduling mechanisms of operating

systems.

3. The randomness caused by the coarse granularity of

timers is multiplicative because it is accumulated by each

tick of the timer, and it is limited to a certain range. For

example, in case a thread is set to sleep for	tsleep

milliseconds, the actual time the thread will sleep, tdelay,

can be expressed as:

tdelayјtsleepюRtimer	р8Ю

where

(Table 1) and our original trafﬁc generator (Table 2) and

compared their performance in terms of stability and

RtimerјSgnрRnЮMinрAbsрRoundрtsleepRnЮЮ; bЮ

р9Ю

accuracy. The simulation models can be expressed as

follows:

1. Physical network interface of the data source is 10/

100 Mbps Ethernet, which is popular in xDSL

implementation with PPPoE (Point-to-Point Protocol

over Ethernet).

2. The randomness caused by the scheduling mechanisms

of operating systems is additive and limited to a certain

range. For example, since the minimum time used to send

a packet of 1400 bytes is	tminmilliseconds	рtminј

1400 Ј 8 Ј 1000=107in case of the 10 Mbps Ethernet),

the actual time used to send a packet	tsend	can be

expressed as:

where Sgn(·) is the signum function, Abs(·) is the

absolute value function, Min(·) is the minimum value

function, and b(b . 0) is the upper limit on the random

value caused by the coarse granularity of timers (in ms).

The simulation results are shown in Fig. 7. The data rate

from the new trafﬁc generator is stable and smooth.

Compared to the oscillations and deviations in trafﬁc rate

generated by the simple trafﬁc generator, the new generator

has much better performance than the simple one. The new

trafﬁc generator is an essential part of our accurate

bandwidth measurement scheme.

tsendјRoundрtminю RosЮ

р7Ю

5. Evaluation

where	Rosј aRn;	Rnis a uniformly distributed real

random number on [0,1), Round(·) is an operator that

rounds a number towards the nearest integer, and a is the

upper limit (in ms) of the additive random component

We have implemented the above bandwidth measure-

ment scheme as a Java-applet based tool and deployed it in a

commercial setup [5]. We ﬁrst evaluated the tool using a

serial link in a laboratory testbed network in which the

L. Cheng, I. Marsic / Computer Communications 25 (2002) 1699–1710

Fig. 8. Testbed topology.

bandwidth could be altered manually. Next, we evaluated

the tool over commercial ADSL service networks.

5.1. Laboratory experiments

Fig. 8 shows the topology of the testbed. The client and

the server are in different subnets that are connected by a

serial link by introducing a High-Speed Serial Interface

(HSSI) between the two gateways. The serial link is the

bottleneck link. The endpoints for the tests were located

across the two ends of the HSSI. There is no cross trafﬁc.

We compared our measurement tool with other popular

tools, such as pathchar [7,11], clink [8], and nettimer [14].

The experiments were performed on an Intel Pentium II

266 MHz machine with Red Hat Linux 7.2 installed. For a

certain bandwidth setting of the HSSI, the test was repeated

100 times for the Rutgers tool which generates measurement

results in seconds, and 10 times for both pathchar and clink

which generate results in minutes, as illustrated in Table 3.

The test was also repeated 10 times for the nettimer tool for

several parameter and HSSI bandwidth settings. The results

1707

were generated within seconds or minutes, depending on the

nettimer parameter settings.

5.1.1. Rutgers tool vs. pathchar and clink

Table 3	shows the experimental results for the case of

10 Mbps link. Both pathchar and clink use ICMP packets in

estimating the link characteristics. The Rutgers tool is

implemented as a Java applet which uses UDP packets and it

is thus platform-independent and very easy to use. Unlike

pathchar	and	clink, no root or super-user privileges are

required to use our tool.

Fig. 9	compares the performance of our bandwidth

measurement tool to that of pathchar and clink on a range of

bottleneck link bandwidths. The ﬁgure shows that our tool

compares favorably with the other tools in terms of

measurement accuracy. In addition, it is much faster to

run, consumes much less bandwidth, and it is platform

independent. Both	pathchar	and	clink	show signiﬁcant

deterioration of measurement accuracy starting at about

10 Mbps. We did not perform measurements for higher

bandwidths, but it is reported in Ref. [16] that the accuracy

becomes worse with increasing bandwidth.

Fig. 9. Comparison of tools for bandwidth measurement. The HSSI clock is

only able to accurately set the link speeds up to 4000 Kbps. Consequently

there is a gap from 4000 to 10 000 Kbps, which is the link speed for a

10 Mbps Ethernet LAN.

5.1.2. Rutgers tool vs. nettimer

Nettimer version 2.3.8 was used in the tests. It was run in

the mode that actively probes the bottleneck link bandwidth

by sending a given number of TCP FIN packets to ports on

the destination host. These packets cause TCP RESET

packets to be sent back to the measurement host so that

nettimer captures these packets and makes calculations.

Table 4	illustrates the measurement results. For each

HSSI bandwidth setting, the nettimer was tested 10 times

with each of these three parameter settings: (1) all

parameters of nettimer	were set as default, (2) the probe

packet size active_probing_packet_size was set

as 1500 bytes, and (3) the probe packet size	active_

probing_packet_size	was set as 1500 bytes, the

minimum number of samples,	active_probing_

min_samples, and the maximum number of probe

packets sent,	active_probing_max_packets_

sent, were set as 5000. Setting the value of probe packet

1708

Table 3

L. Cheng, I. Marsic / Computer Communications 25 (2002) 1699–1710

Laboratory experiment results in case of a 10 Mbps link

Measurement tool	Version

Mean value (Mbps)	Standard deviation	Number of probe packets	Mean time per measurement

Rutgers tool

1.0

9.95

197 Kbps

450a

0.09 min

Pathchar [7]

Clink [8]

Alpha; April 21, 1997	8.34

1.0; August 14, 1998	9.26

336 Kbps

1.10 Mbps

h 2880b

h 1488

7.50 min

1.33 minc

a	The number of probe packets can be conﬁgured. In all experiments reported here, 450 packets have been transmitted for each measurement. This number is

statistically signiﬁcant enough to offset the effect of the system timer inaccuracy.

bhis the number of hops between the two endpoints.

c	This time is variable and reaches 8.50 min for smaller HSSI bottleneck link bandwidths.

Table 4

Measurement results using nettimer in active probing mode

Measurement tool

Rutgers tool

Nettimer [14]

Nominal bandwidth

Parameter setting #1

Parameter setting #2

Parameter setting #3

56 Kbps

55.0 Kbps

57.1 Kbps

54.5 Kbps

54.9 Kbps

125 Kbps

124.0 Kbps

133.3 Kbps

122.5 Kbps

133.3 Kbps

500 Kbps

501.2 Kbps

400 Kbps

600 Kbps

600 Kbps

2 Mbps

2.01 Mbps

1.48 Mbpsa

10 Mbps

9.95 Mbps

9.65 Mbpsa

a	The data is reproduced from Ref. [15] since we were unable to obtain reasonable results at high-speed rates in our laboratory. In our correspondence with

the author, he indicated that problem probably arises due to the limitation of the timer resolution in our conﬁguration. If that is the case, such sensitivity to timer

resolution is not desirable for general xDSL user community.

size to 1500 bytes is suggested in nettimer manual [14] for

better performance when measuring high bandwidth links

on slow hosts. The minimum number of samples and the

maximum number of probe packets send were set to 5000

since it was observed that the large values ensure the

convergence of the measurement results. The results

demonstrate that our tool has generated more accurate

measurement results in all cases that we were able to

measure. For the high-speed rates we use the results

reported in Ref. [15], where error varies from 3.5 to 7.0%

in the 10 Mbps case, but the results for our tool are still more

accurate (error of 0.5%) as shown in Table 4.

5.2. Field experiments

The ﬁeld experiments were performed in the deployment

abstracted as in	Fig. 1. The accuracy of our bandwidth

measurement tool is evaluated in the following ATM

service categories in an ADSL service network:

†	Constant Bit Rate (CBR) service category

†	Real-Time Variable Bit Rate (rt-VBR) service category

†	Unspeciﬁed Bit Rate (UBR) service category

For each category, the upstream and the downstream link

bandwidths are measured for several nominal ADSL service

rates: 90 and 680 Kbps in the upstream case, and 640 Kbps,

1.7 and 7.1 Mbps in the downstream case. Each measure-

ment was repeated 100 times and the mean values and

standard deviations were computed. Figs. 10 and 11 show

that our tool generates accurate results. The standard

deviations are omitted in the graphs but they follow similar

trend as in Fig. 9, never exceeding 5%.

The possible reasons for not achieving 100% accuracy

could be some of those discussed in Ref. [16]. Of those, the

most feasible ones seem to be unreachable nominal values

and the timing resolution of the operating system.1

To verify the necessity of stepwise algorithm, we also

performed ﬁeld measurements with the single-step algor-

ithm. The experiment explicitly sets the trafﬁc generator’s

sending rate in the client, lets it send sustained trafﬁc at that

rate, and then records the result at the measurement server.

The results show that the measurement inaccuracy of the

single-step algorithm ranges up to 25%. Since the stepwise

algorithm prevents packet loss caused by ATM trafﬁc

shaping in xDSL service networks, it increases the accuracy

of the bandwidth measurement.

6. Conclusion

Telecom operators use xDSL service networks to provide

high-speed digital services by taking advantage of their

copper infrastructure via overlay and without interfering

with the traditional analog telephone service, such as Plain

Old Telephone Service (POTS). Because xDSL is intro-

duced on a per-user basis and has full bandwidth and service

ﬂexibility, it is essential for both customers and xDSL-

service providers to have tools to accurately measure the

xDSL link bandwidth.

We presented a novel stepwise bandwidth measurement

1	In addition to the timer used in trafﬁc generation, there are also times

recorded at the beginning and end of each measurement cycle.

L. Cheng, I. Marsic / Computer Communications 25 (2002) 1699–1710

Fig. 10. Measurement accuracy in ﬁeld experiments of ADSL downstream link.

1709

scheme for accurately measuring link bandwidth in xDSL

service networks, considering the impact of ATM trafﬁc

shaping and link asymmetry. It uses a multi-packet

technique and a smooth trafﬁc generator. The scheme

solves the inaccuracy problem caused by ATM trafﬁc

shaping encountered by bandwidth measurement techniques

that do not regulate their trafﬁc. Given that the ATM shaper

rate is greater than or equal to the ADSL service rate, our

tool measures the actual ADSL link bandwidth. The

deployment of the multi-packet technique does not require

packet ﬁltering to achieve accurate measurements. It is

lightweight in terms of implementation complexity and

probe trafﬁc overhead. Our new trafﬁc generator helps to

achieve the measurement accuracy of the multi-packet

technique.

Evaluation experiments demonstrate that our bandwidth

Fig. 11. Measurement accuracy in ﬁeld experiments of ADSL upstream

link.

measurement tool offers accurate bandwidth measurement

in xDSL service networks. The measurement errors in the

ﬁeld experiments in both upstream and downstream cases

are within the range of 5%, tested in an ADSL network

under the following ATM network conﬁgurations: CBR, rt-

VBR, and UBR service categories. We even plan to further

improve the accuracy of our tool by using the arrival time of

other packets instead of just the ﬁrst and the last packet in

the multi-packet technique. Our future work also includes

more extensive evaluations and embedding this scheme in

network-aware adaptive applications.

Acknowledgements

The authors are grateful to Senthilkumar Rajasekharan

for helping perform the measurements and collecting data in

the ﬁeld experiments. The authors would like to thank

anonymous reviewers and to Dr Edward J. Devinney for

their valuable comments. This research is supported by

grants from Verizon, Inc. (formerly Bell Atlantic), Cisco

Systems, Inc., and NSF KDI IIS-98-72995, and by the

Rutgers Center for Advanced Information Processing

(CAIP).

Appendix A. Multi-packet bandwidth measurement

Proof. We perform induction on N, the number of packet

sent. For N ј 1;

tNрmЮ2t0рmЮјt1рmЮ2t0рmЮ

1710

According to Ref. [16],

tNрmЮ 2 t0рmЮјs=bmin

that is:

tNрmЮ 2 t0рmЮј рs Ј NЮ=bmin

L. Cheng, I. Marsic / Computer Communications 25 (2002) 1699–1710

puter Communications, Cambridge, MA (1999) 241 – 250. August/

September.

[8] A.B. Downey, Clink: a tool for estimating Internet link characteristics,

Wellesley College, Online at:	http://rocky.wellesley.edu/downey/

clink/.

[9] W. Goralski, ADSL and DSL Technologies, McGraw-Hill, New

York, 1998.

for N ј 1: This proves the N ј 1 case.

For N . 1, assume the equation holds for N ј n case, i.e.

tnрmЮ2t0рmЮј рs Ј nЮ=bmin

then for N ј n ю 1 case,

tnю1рmЮ 2 t0рmЮј рtnю1рmЮ 2 tnрmЮЮ ю рtnрmЮ 2 t0рmЮЮ

ј рtnю1рmЮ 2 tnрmЮ Ю ю рs Ј nЮ=bmin

According to Ref. [16], since there is no cross trafﬁc,

tnю1рmЮ 2 tnрmЮјt1рmЮ	2 t0рmЮјs=bmin

therefore,

tnю1рmЮ 2 t0рmЮј рs Ј рn ю 1ЮЮ=bmin

This proves the N ј n ю 1 case.

References

[1] ATM Forum, Online at: http://www.atmforum.com/.

[2] DSL Forum, Online at: http://www.adsl.com/.

[3] J.C. Bolot, End-to-end packet delay and loss behavior in the Internet,

Proceedings of the ACM SIGCOMM ’93 Conference on Appli-

cations, Technologies, Architectures, and Protocols for Computer

Communications (1993). pp. 289 – 298 San Francisco, CA, USA.

[4] R.L. Carter, M.E. Crovella, Measuring bottleneck link speed in

packet-switched networks, Technical Report TR-96-006, Department

of Computer Science, Boston University, March 1996, Online at:

http://www.cs.bu.edu/faculty/crovella/papers.html.

[5] L. Cheng, I. Marsic, Java-based bandwidth measurement tool for

digital subscriber line networks, Proceedings of the Ninth Mediterra-

nean Conference on Control and Automation, Dubrovnik, Croatia,

June (2001).

[6] A.B. Downey, From pathchar to netchar: keeping an eye on your ISP,

Presentation at Cisco Systems Inc., Online at: http://rocky.wellesley.

edu/downey/clink/.

[7] A.B. Downey, Using pathchar to estimate Internet link characteristics,

Proceedings of the ACM SIGCOMM ’99 Conference on

Applications, Technologies, Architectures, and Protocols for Com-

[10] D. Greggains, ADSL and high bandwidth over copper lines,

International Journal of Network Management 7 (5) (1997) 277 – 287.

[11] V. Jacobson, Pathchar, 1997, Online at: ftp://ftp.ee.lbl.gov/pathchar.

[12] V. Jacobson, Pathchar—A tool to infer characteristics of Internet

paths, Presented at the Mathematical Science Research Institute, April

21, 1997, Online at: ftp://ftp.ee.lbl.gov/pathchar.

[13] S. Keshav, An Engineering Approach to Computer Networking: ATM

Networks, the Internet, and the Telephone Network, Addison-Wesley,

Reading, MA, 1997.

[14] K. Lai, Nettimer manual, Online at: http://gunpowder.stanford.edu/

~laik/projects/nettimer/#Documentation, last visited and downloaded

on January 15, 2002.

[15] K. Lai, M. Baker, Nettimer: a tool for measuring bottleneck link

bandwidth, Proceedings of the third USENIX Symposium on Internet

Technologies and Systems, San Francisco, CA, March (2001) pp.

122 – 133.

[16] K. Lai, M. Baker, Measuring link bandwidths using a deterministic

model of packet delay, Proceedings of the ACM SIGCOMM 2000

Conference on Applications, Technologies, Architectures, and Proto-

cols for Computer Communications, Stockholm, Sweden, August

(2000) pp. 283 – 294.

[17] K. Lai, M. Baker, Measuring bandwidth, Proceedings of the

Conference on Computer Communications (IEEE INFOCOM ’99),

New York, NY, USA, March (1999) 235 – 245.

[18] A. Lin, LightStream 1010 Switch Architecture and Trafﬁc Manage-

ment, White Paper, Cisco Systems Inc., July 2000, Online at: http://

www.cisco.com/warp/public/cc/so/neso/vvda/atm/lsatm_wp.htm.

[19] T. Lizambri, F. Duran, S. Wakid, Priority scheduling and buffer

management for ATM trafﬁc shaping, Proceedings of Seventh IEEE

Workshop on Future Trends of Distributed Computing Systems

(FTDCS ’99), Cape Town, South Africa December (1999) 36 – 43.

Online at:	http://w3.antd.nist.gov/Publications/Hsnt/lizambri_1299.

html.

[20] V. Paxson, Measurements and Analysis of End-to-End Internet

Dynamics, PhD Thesis, University of California, Berkeley, April,

1997.

[21] V. Paxson, End-to-end Internet packet dynamics, Proceedings of the

ACM SIGCOMM ’97 Conference on Applications, Technologies,

Architectures, and Protocols for Computer Communications (1997)

pp. 139 – 152 Cannes, France.

[22] The DSL Forum, Online at: http://www.adsl.com.

[23] R. Wolski, Dynamically forecasting network performance to support

dynamic scheduling using the network weather service, Proceedings

of the Sixth IEEE Symposium on High-Performance Distributed

Computing, August (1997). pp. 316 – 325 Portland, OR, USA.