Analysis of MPLS Signaling Protocols and Traffic Dissemination in OSPF

and MPLS

Sang-Chul KimO and Jong-Moon Chung

Department of Computer Science, Kookmin University

sckim7@kookmin.ac.kr

School of Electrical Engineering, Yonsei University

jmc@yonsei.ac.kr

Abstract

This paper analyzes MPLS signaling protocols for traffic engineering, shows the capability of

providing traffic engineering in MPLS compared to the conventional routing protocol, and explains

the MPLS LSR operations based on the basic LSR functionality of classification, queue, and

scheduling. CR-LDP, RSVP and RSVP-TE are summarized and analyzed based on how to set up

LSP for TE with help of protocol messages. In addition, the comparisons of CR-LDP, RSVP, and

RSVP-TE are conducted based on the aspects of LSP reliability and LSP adaptability. A

performance metric such as throughput is adopted in order to measure the capability of MPLS

traffic engineering based on computer-based simulation.

1. Introduction

The explosive growth of the Internet over past a few

years has made the IP protocol suite the most

predominant networking technology. Furthermore,

the convergence of voice and data communications

over a single network infrastructure is expected to

happen over IP-based networks. Traditional IP

networks offer little predictability of service, which is

unacceptable for applications such as telephony, as

well as for emerging and future real-time applications.

One of the primary goals of traffic engineering is to

enable networks to offer predictable performance.

As recent history tells us, the upper limit of

transmittable bandwidth doubles and sometimes

quadruples every nine to twelve months. Already

transmission of tens of tera bits-per-second over a

single optical fiber is possible and matching data

transferring topologies as well as improved system

reliability are currently needed. Based on the above

facts, two major candidates that are in competition to

become the dominant future network protocol and

network architecture are differential services (DS)

and multiprotocol label switching (MPLS) [1]. In the

competition of DS and MPLS, MPLS has been

emerging as the protocol of the future for the

following reasons. Realizing the features provided by

MPLS makes it an easy choice. First, it is a true

“multiprotocol architecture” utilizing a simple label

switching mechanism, which is where its versatility

in application exists, e.g., MPLS over ATM, frame

relay	(FR), etc. Second, through	utilizing

classification, queue, and scheduling (CQS) traffic-

engineering topologies MPLS is capable of providing

controllable quality of service (QoS) features [2].

Third, MPLS provides a solution to scalability and

enables significant flexibility in routing. Fourth, the

connection oriented architecture and QoS reliability

features easily enable high quality end-to-end service

features that are necessary in applications such as

virtual private networks (VPN) [3]. These benefits of

MPLS networking are made possible through traffic

engineering. Currently, the constraint-based routing

label distribution protocol (CR-LDP) and the resource

reservation protocol (RSVP) are the signaling

algorithms used for traffic engineering. In this paper,

we compare the signaling procedures of the CR-LDP

and RSVP algorithms and discuss the appropriateness

of the applications in MPLS traffic engineering

networks. Applying MPLS is truly a protocol

architecture matter where the software over the

routers/gateways and switches/bridges need to be

reconfigured to include label edge router (LER) and

label switching router (LSR) functionalities. This

means that the existing network architecture can be

utilized as MPLS architecture without digging out

cables and replacing whole new devices. Although

systems that highly depend on hardware functionality

will unfortunately have to be replaced to include

MPLS architecture. Conventional IP networks reflect

Proceedings of the First Asia International Conference on Modelling & Simulation (AMS'07)

0-7695-2845-7/07 $20.00  © 2007

the unpredictable and undifferentiated packet loss and

jitter characteristics of traditional best-effort routers.

Queuing introduces latency and the potential for

packet loss if a queue overflows. In order to provide a

solution to this, this research plan investigates the

requirements of MPLS networking for predictable

differentiated loss, latency, and jitter characteristics to

traffic classes of applications. Also, this paper

provides the construction methods of restoration to

MPLS router for the network facility failures in

operating high speed. This paper is organized as

follows: Section 2 introduces the operation of MPLS

LSR. Sections 3 and 4 explain a detailed explanation

of the CR-LDP and RSVP-TE signaling respectively.

Section 5 compares MPLS signaling protocols based

on LSP reliability and LSP adaptability. Section 6

yields the numerical experiments and results in detail.

Finally, Section 7 summarizes our work and

concludes this paper.

2. MPLS LSR OPERATION

The LSR that conducts the differential services is

required to conduct a three-step procedure to enable

traffic engineering. These three basic steps are

classification, queue, and scheduling (CQS). As label

attached packets arrive at the input ports, the input

label is used to identify the forwarding equivalent

class (FEC) and the corresponding output label. The

output label will replace the input label of the packet.

Then, based on the output label and FEC, the packet

will be sent to the corresponding output queue where

the scheduling multiplexer will decide on the output

order, timing, and the output port for the packet to be

sent out. The setup of the LSR is done by the

signaling protocols (CR-LDP, RSVP-TE). The

functional diagram of a LSR is provided in Fig. 1 [4].

FIG.1. FUNCTIONAL DIAGRAM OF THE LSR CLASSIFICATION, QUEUE,

AND SCHEDULING (CQS) OPERATION.

3. BASIC CR-LDP SIGNALING

CR-LDP standards attempt to enable the LDP

protocol to work over an explicit route, transporting

various traffic parameters for resource reservation as

well as the options for CR-LSP robustness features

[5]. Both LDP and CR-LDP are hard state protocols,

where signaling messages are transmitted once

without any refreshing-information requirements. The

transport mechanism for peer discovery is UDP,

while TCP is used for session, advertisement,

notification, and LDP messages. To setup an explicit

route, a LABEL REQUEST message containing a list

of nodes along the constraint-based route to be

traversed is sent. The signaling message will be sent

to the destination following the selected path, and if

the requested path is able to satisfy the requirements,

labels are allocated and distributed by means of

LABEL MAPPING messages starting with the

destination and propagating in the reverse direction

back to the source. Assuming that resources are

available, the LSP setup is completed after a single

round-trip of the signaling message. CR-LDP is

capable of establishing both strict and loose path

setups with setup and holding priority, path

preemption, and path re-optimization. The procedure

for reporting failures in CR-LDP is based on ingress

and egress router’s TCP layer transport operations.

CR-LDP enables multiprotocol operations by using

an opaque FEC, which allows core LSRs to be

indifferent with respect to the type of traffic being

transported across the network. The opaque FECs are

also used for security purposes as well, not enabling

the LSRs to know the transport data services identity.

4. BASIC RSVP SIGNALING

Based on RFC 2205 [6], the RSVP signaling protocol

standard published by the IETF is intended for soft

state resource reservation focusing on enterprise

networks to support integrated services [7]. RSVP

inherently is a soft state protocol that uses PATH and

RESV commands to establish a LSP. In RSVP, based

on the destination IP address and protocol ID, packets

are transferred based on raw IP datagram routing. The

ingress LSR uses a PATH message to inform every

router along the selected LSP to acknowledge that

this is a desired LSP to be established. Following this,

the receiving LSR will use the RESV message with

traffic and QoS parameters traversing upstream to

reserve the resources on each node along the desired

LSP. The node along the LSP will install the

reservation for the related state by creating an entry

on the label-forwarding table. At every node along

the path, the PATH and RESV messages are used

periodically to refresh the path and reservation states.

Problems in resource reservation can result based on

the RSVP soft state mechanism and the merging

points along the selected LSP. Overall, there is no

Proceedings of the First Asia International Conference on Modelling & Simulation (AMS'07)

0-7695-2845-7/07 $20.00  © 2007

guarantee that the resources will be reserved based on

the end-to-end request.

RSVP-TE has been made and proposed to support

ER-LSP as well as provide additional features to

RSVP [8]. Since the RSVP protocol was proposed to

support MPLS LSP setups, a considerable amount of

modifications and extension have been made to the

original protocol to cope up with the traffic

engineering requirements. The major modifications

and extensions fall into the areas of adding traffic

engineering capabilities and resolving scalability

problems. The revised RSVP protocol has been

proposed to support both strict and loose explicit

routed LSPs (ER-LSP). For the loose segment in the

ER-LSP, the hop-by-hop routing can be employed to

determine where to send the PATH message. Thus,

RSVP also supports hop-by-hop downstream-on

demand ordered mode.

5.	COMPARISON OF	SIGNALING 	PROTOCOL

TOPOLOGIES

In this section, the signaling protocols of MPLS

traffic engineering are compared. The signaling

protocols in comparison are the CR-LDP, original

RSVP, and the RSVP-TE. The features of the three

signaling protocols are organized in Table 1 [4].

Table 1. A COMPARISON OF CR-LDP, RSVP, AND RSVP-TE

CR-LDP was created to enable LSP setup for reliable

end-to-end differentiated services in MPLS networks.

Compared to this, RSVP was established to support

soft state resource reservation of integrated services

over IP networks. RSVP was created before CR-LDP

with originally a different intension of where it would

be used. Therefore, it is not surprising that RSVP is

not suitable for traffic engineering in MPLS networks.

The RSVP-TE contains several specifications to

support differentiated services with RSVP for MPLS

traffic engineering networks, although some of the

key components of the architecture are the same. For

example, the original protocol base of using the

internetworking protocol (IP) is the same, also the

hop-by-hop soft state refreshing algorithms are

basically the same (although somewhat modified), as

well as the reverse upstream LSP setup topology

remains the same. Several features of CR-LDP, that

were not a part of RSVP, are now possible by the

RSVP-TE.

As in terms of scalability, CR-LDP is a hard state

protocol, and due to this, it inherently possess better

scaling properties in terms of the volume of signaling

traffic in the network as the number of CR-LSPs

increase. One of the significant drawbacks of RSVP

is its scalability when there are a large number of

paths passing through a node. This is due to the soft

state characteristics of RSVP and RSVP-TE, which

require periodical refreshing of the state for each path.

5-1. LSP RELIABILITY

In RSVP and RSVP-TE signaling for traffic

engineering, the failure notification process contains

several problems. Relying on raw IP creates possible

problems that RSVP may not be able to quickly

inform the edge routers that the connectivity between

them has failed. RSVP-TE does have explicit tear

down messages, but due to relying on raw IP

transporting they are not sent reliably enough. As a

result, the edge LSRs may not start to re-route traffic

until the expiration of the timeout interval. Based on

the recommendations of RFC 2205 [6], 30 seconds of

a refresh interval and 90 seconds of a cleanup timeout

interval have been proposed. These values are

significantly too large for backbone network

operations. If the timing intervals were reduced, the

traffic load due to the refresh operations would create

more scalability problems.

Additionally, to handle loop detection, RSVP-TE

uses the RECORD REROUTE OBJECT, which

provides route information of a certain LSP for route

diagnostic purpose. In order to solve scalability

problems due to the soft state characteristics, RSVP-

TE allows aggregation of the refresh messages to

reduce the total number of transmissions. To reduce

the processing load of these refresh messages on a

node, a MESSAGE ID is introduced with the

intentions of letting the receiving node quickly

identify a state change. However, the use of the ID

needs very careful management of the ID numbers

and messages by the nodes to avoid many possible

errors, such as a mismatch or a duplicate, which

imposes other overheads. The RSVP-TE standards

Proceedings of the First Asia International Conference on Modelling & Simulation (AMS'07)

0-7695-2845-7/07 $20.00  © 2007

also suggest the nodes to completely suppress the

refreshes. In RSVP-TE, the LSRs are proposed to use

the HELLO protocol to detect the loss of neighboring

routers or link states. On the other hand, the soft state

design does provide some robustness to the signaling

system mechanism. By occasional rechecks, failures

in neighboring routers or link states can be detected

early.

In comparison to this, the TCP end-to-end connection

oriented controlling mechanism of the CR-LDP relies

on the ingress and egress LSRs to manage the LSP.

Based on the fact that the CR-LDP is a hard state

protocol, scalability is not an issue to consider. If a

link is to fail, the TCP process will detect this and the

ingress LSR will determine the procedures to take. In

this case, the LSP options of being strict, loose, or

pinned will define the options to take.

5-2. LSP ADAPTABILITY

In RSVP, the shared explicit (SE) reservation style is

used to set up alternative paths through “make-

before-break” procedures. This requires a session to

be established before leaving the previously used path.

The newly selected LSP will have a different tunnel

ID compared to the original one. In RSVP-TE, the

protocol does have explicit tear down messages,

although if this were to fail under high traffic

pressure, the old LSP will be left to timeout (without

being refreshed to stay alive) and will eventually be

terminated. This possible scenario could result in

serious problems for the network. First, the timeout

period is much too long for backbone networks to be

waiting for path termination, which results in a

contain the pinning option as an additional feature. In

CR-LDP, path re-optimization is conducted by the

ingress LSR, which is the most proper method to

stably control the rerouting. The process is governed

by the ingress LSR where end-to-end checking of the

sequence of operation commands is protected by the

TCP layer mechanism.

6. EXPERIMENT AND OBSERVATION

(a) Network Topology for OSPF

(b) Network Topology for MPLS

FIG.2. NETWORK TOPOLOGIES FOR MPLS AND OSPF.

For simple example to show the TE property of

MPLS, Open Shortest Path First (OSPF) may be

compared with MPLS.	OSPF routing protocol can

provide traffic load balance when multiple paths have

equal costs to the destinations.	However, if the

multiple paths have different costs to the destinations,

OSPF chooses a shortest path first. Therefore, traffic

is not evenly distributed to the multiple paths and it

significant waste of bandwidth. Second, the 	may increase network traffic load. MPLS-TE can be

remaining LSP may cause looping problems or other 	used in this case. MPLS uses signaling protocols to

confusions to the LSRs, which is most undesirable. 	disseminate the traffic to multiple paths and to do

For the case of path preemption, RSVP uses setup 	QoS and DS by installing LSP among the multiple

and holding priorities to determine if a new path can 	paths. Therefore, the traffic load at each path can be

preempt an existing path. Transport mechanism of 	divided based on the LSP’s traffic dissemination

RSVP, which is on raw IP, may cause problems again	parameters.

for this feature support. Because preemption is often	Fig. 2 shows the network topologies that were used to

required when the network is running short of 	simulate the MPLS TE in OPNET. Figs. 2(a) and (b)

resources, the RSVP signaling messages may get lost 	contain two sources, two destinations, and two

in this case. Then the path preemption feature would	traffics (data flows) where the traffic requires 10

not be executed at all. Compared to this, CR-LDP 	Mbps (bits/sec) from a source to a destination.

relies on TCP, which shields the signaling protocol	Between a source and a destination, Figs. 2(a) and (b)

by continuously checking errors as well as the 	include several LSRs where LSRs in Fig. 2(a) have

sequence of the data sessions executed. The rerouting	been composed of OSPF and LSRs in Fig. 2(b) have

capability in RSVP may be used to re-optimize the 	been composed of with MPLS signaling protocols

path, which is executed by all participating nodes 	such as CR-LDP and RSVP-TE. In order to perform

exchanging local traffic information to reselect the 	the traffic engineering, the topology has been

new path. The standards for RSVP do not have the 	constructed as follows. Ingress LER has two input

pinning option included, although the RSVP-TE does 	traffics from two sources and one output traffic to

Proceedings of the First Asia International Conference on Modelling & Simulation (AMS'07)

0-7695-2845-7/07 $20.00  © 2007

LER1. One data flow follows a route (let’s say route

A) from LER1 LSR2, LSR3 to LSR7. The other data

flow follows a route (let’s say route B) from LSR1,

LSR4, LSR5, LSR6, to LSR7. It is assumed that there

is no other background traffic in LSRs. Since all

LSRs in Fig. 2(a) are configured with OSPF, the data

flow of 20 Mbps from Ingress_LER follows the route

A since the	route A provides the shortest path

compared the route B following LSR1, LSR4, LSR5,

LSR6, and LSR7. However, all LSRs in Fig. 2(b) are

configured with MPLS signaling protocols such as

CR-LDP and RSVP-TE, the data flow of 20Mbps is

separately divided into two data flows of 10Mbps and

route A and route B service each data flow 10Mbps

separately. Two LSPs are setup in the route A and the

route B. Figs 3 (a) and (b) show the graphs of

simulation results. In both figures, x axis shows the

simulation time in second unit and y axis shows the

throughput between LSR1 and LSR2 and the

throughput between LSR1 and LSR4. Based on the

results, it can be expected that in the case of OSPF,

the link between LSR1 and LSR2 causes the problem

of hot-spot; however, the link between LSR1 and

LSR4 is idle during the simulation time since it is not

used. In the case of MPLS, the link between LSR1

and LSR2 has no more problems of hot-spot and the

link between LSR1 and LSR4 is moderately used

compared to its throughput.

7. CONCLUSION

This paper explains the MPLS LSR operations based

on the basic LSR functionality of classification,

queue, and scheduling. In addition, MPLS signaling

protocols such as CR-LDP, RSVP and RSVP-TE are

summarized and analyzed based on how to set up

LSP for TE with help of the protocol messages. CR-

LDP is a hard-state protocol and capable of

establishing both strict and loose path setups with

setup and holding priority, path preemption, and path

re-optimization. RSVP inherently is a soft state

protocol that uses PATH and RESV commands to

establish a LSP. RSVP-TE	has been proposed to

support ER-LSP as well as provide additional

features to RSVP and contains several specifications

to support differentiated services with RSVP for

MPLS traffic engineering networks. Based on

comparison of signaling protocols, it can be found

that RSVP has drawback in its scalability when there

are a large number of paths passing through a node

due to the periodical refreshing of the state for each

path. In the simulation, when MPLS signaling

protocols were implemented in a MPLS network of

TE, the traffic in hot spot can be reduced and the

traffic is moderately distributed into several LSPs,

which is not able to achieve in the conventional

routing protocol.

(a) Throughput of links configured with OSPF

(b) Throughput of links configured with MPLS

FIG.3. THROUGHPUT COMPARISON FOR MPLS AND OSPF.

8. ACKNOWLEDGMENT

This research was supported by the Seoul R&BD

program of Korea.

9. REFERENCES

[1] E. Rosen, A. Viswanathan, and R. Callon, ‘‘Multiprotocol Label

Switching Architecture,’’ RFC 3031, Jan. 2001.

[2] J. L. Marzo, E. Calle, C. Scoglio, and T. Anjali, “QoS on-line routing

and MPLS multilevel protection: a survey,” IEEE Commun. Mag.,

vol. 41, pp. 126-132, Oct. 2003.

[3] J.-M. Chung, E. Marroun, H. Sandhu, and S.-C. Kim “VoIP over MPL

S Networking Requirements,” Proceedings of IEEE International

Conference on Networking 2001 (IEEE ICN’01), Colmar, France,

July 11-13, 2001.

[4] J.-M. Chung, “Analysis of MPLS Traffic Engineering,” Proceedings

of the IEEE Midwest Symposium on Circuits and Systems 2000

Conference (IEEE MWSCAS’00), East Lansing, MI, USA, Aug. 8-

11, 2000.

[5] B. Jamoussi et al, “Constraint-Based LSP Setup using LDP,” IETF

RFC 3212, Jan. 2002.

[6] L. Zhang et al, “Resource ReSerVation Protocol (RSVP),” IETF RFC

2205, Sep. 1997.

[7] J. Wroclawski, “The Use of RSVP with IETF Integrated Services,”

IETF RFC 2210, Sep. 1997.

[8] D. Awduche et al, “RSVP-TE: Extensions to RSVP for LSP Tunnels,”

IETF RFC 3209, Dec. 2001.

Proceedings of the First Asia International Conference on Modelling & Simulation (AMS'07)

0-7695-2845-7/07 $20.00  © 2007