Computer Standards & Interfaces 27 (2005) 117 – 132

www.elsevier.com/locate/csi

Extension of Multiprotocol Label Switching for long-range

dependent traffic: QoS routing and performance in IP networks

Tibor Gyiresa, H. Joseph Wenb,*

aSchool of Information Technology, College of Applied Science and Technology, Illinois State University, Normal, IL 61790-5150, USA

b Department of Accounting and Management Information Systems, Donald L. Harrison College of Business,

Southeast Missouri State University, Mail Stop 5815, Cape Girardeau, MO 63701, USA

Received 21 November 2003; accepted 28 May 2004

Available online 26 June 2004

Abstract

This paper presents an extension to the Multiprotocol Label Switching (MPLS) traffic engineering in IP networks with long-

range dependent traffic. The extension provides the ability to diverge traffic flows away from the shortest path calculated by the

traditional IP routing protocols into a less congested area of the network. When the traffic burstiness of a packet flow exceeds a

predefined threshold, the extension calculates the cost of the traffic distribution and the effectiveness of Label Switching

Routers (LSPs) to minimize the number of discarded packets. The simulation results demonstrate that the extension

significantly improves the overall network performance in link utilization, port processor utilization, message delay, number of

dropped packets, and buffer usage level.

D 2004 Elsevier B.V. All rights reserved.

Keywords: Telecommunications; Simulation modeling; Traffic burstiness; MPLS; QoS; RSVP

1. Introduction

The purpose of a communications network is to

support the sharing of the communications links.

When too many packets are present in (a part of)

the subnet, performance degrades. This situation is

called congestion. When the number of packets sent

into the subnet by the hosts is within the network’s

carrying capacity, the packets are all delivered (except

for a few that are damaged or lost due to transmission

errors), and the number delivered is proportional to

* Corresponding author. Tel.: +1-573-651-2121; fax: +1-309-

438-5113.

E-mail address: hjwen@ilstu.edu (H.J. Wen).

0920-5489/$ - see front matter D 2004 Elsevier B.V. All rights reserved.

doi:10.1016/j.csi.2004.05.004

the number sent. However, as traffic increases too far,

the routers are no longer able to forward the packets,

and they begin losing them. This tends to make

matters worse. At very high traffic, performance

collapses completely, and almost no packets are

delivered. Congestion can be caused by several fac-

tors. The most dangerous cause of congestion is the

burstiness of the network traffic. Recent results make

evident that high-speed network traffic is burstier and

its variability cannot be predicted as assumed previ-

ously. It has been shown that network traffic has

similar statistical properties on many time scales.

Traffic that is bursty on many or all time scales can

be described statistically using the notion of long-

range dependency	[1,10,20,31]. Long-range depen-

118

T. Gyires, H.J. Wen / Computer Standards & Interfaces 27 (2005) 117–132

dent traffic has observable bursts on all time scales

resulting in packet losses, extremely long response

times and overrun communications link capacities.

The traditional way to control congestion is that the

router detecting the congestion sends a packet to the

traffic source or sources, announcing the problem.

Obviously, these extra packets increase the load when

the subnet is already congested. Feedback-based con-

gestion control systems are not responsive enough to

the varying capacities of transmission links and net-

work delay. The larger the end-to-end delay, the

longer it takes to inform the sending nodes that the

network has become congested. It was shown in Ref.

[5] that the duration of the congestion at the congested

routers is directly related to the bandwidth-delay

product.

In all traditional feedback schemes, the expectation

is that knowledge of congestion will cause the hosts to

take appropriate action to reduce the congestion. But

due to the inherent network delay, the hosts will react

too sluggishly to be of any real use. End-to-end

feedback mechanisms introduce an unacceptable de-

lay. The situation is getting even worse when the

traffic is generated at high-speed in long bursts and

the traffic characteristics cannot be predicted by

traditional statistical methods [16,17].

A good traffic engineering solution that guarantees

the availability of network resources for mission-

critical, interactive and delay-sensitive applications

is Quality of Service (QoS) [2,8]. Its features, such

as queuing, RSVP and multicast services also provide

the control that is needed to handle different traffic in

different ways. Traffic engineering implies the use of

mechanisms to avoid congestion by allocating net-

work resources optimally, rather than continually

increasing network capacities. It is accomplished by

mapping traffic flows to the physical network topol-

ogy along predetermined paths. Traffic engineering

provides the ability to diverge traffic flows away from

the shortest path calculated by the traditional IP

routing protocols into a less congested area of the

network avoiding congestions caused by, e.g., long-

range dependent traffic.

Multiprotocol Label Switching (MPLS) has

emerged as a technology that can provide connection

oriented traffic engineering and QoS. Many future

core networks will use MPLS, including converged

data and voice networks. Traffic engineering is

achieved by forwarding large volumes of voice and

data between Label-Switched Routers (LSRs) together

with bandwidth reservation for traffic flows with

various Quality of Service requirements. MPLS uses

a label switching technique to forward data. A fixed-

format label is inserted in front of each data packet on

entry into the MPLS network. At each hop, the packet

is routed based on the value of the incoming label and

sent out on an outgoing interface with a new label

value. The path that data traverses through a network

is defined by the transition in label values, as the label

is swapped at each LSR. Such a path is called a Label

Switched Path (LSP). In order to distribute the labels

along an LSP, a signaling protocol is required, such as

the Resource Reservation Protocol (RSVP) [4,9].

The objective of our paper is to present an exten-

sion to MPLS in networks with long-range dependent

traffic. We describe a network architecture for mini-

mizing the packet loss due to congestions caused by

long-range dependent traffic. When the traffic bursti-

ness of an LSP increases at an LSR, the router

distributes the traffic flow over n new LSPs leading

to the destination(s) of the original traffic. The selec-

tion of the new LSPs is based on cost estimates of the

flow distribution and routers’ effectiveness in reduc-

ing the number of dropped packets in previous flow

distributions. We define an architecture for the com-

ponents of our MPLS network and the flow distribu-

tion protocol. Using a discrete event simulation

model, we demonstrate how our extension improves

the MPLS technology.

The second section introduces QoS, MPLS and

RSVP. The third section gives an overview of long-

range dependent traffic. The fourth section presents

our Extended MPLS framework and the proposed

flow distribution protocol followed by the conclusion.

2. Overview of QoS, MPLS, and RSVP

2.1. Quality of service (QoS)

A recent survey by infonctics [26] listed QoS as the

second leading concern of IT managers, behind ‘‘se-

curity’’ in its importance in their network design

decisions. QoS is the ability of a network to differen-

tiate between different types of traffic and prioritize

accordingly. It is the cornerstone of any convergence

T. Gyires, H.J. Wen / Computer Standards & Interfaces 27 (2005) 117–132

119

strategy. Voice, video, and data display very different

traffic patterns in the network. Voice and video are

very delay-dependent and have very predictable pat-

terns, whereas data is very bursty and is less delay-

sensitive. If all three types of traffic occur on a

network, the data traffic usually interferes with voice

and video and causes it to be unintelligible.	Fig. 1

shows bandwidth requirement and delay-sensitivity of

different types of business applications.

Networked applications are evolving far faster than

the infrastructure that supports them. All this is

creating a problem-poor application performance in

the enterprise. Many enterprises today are faced with

an aged-old dilemma that seems to creep into every

network: Throw more bandwidth at the problem, or

manage the existing bandwidth more effectively. QoS

can effectively improve the usage of existing band-

width	[21,22]. It concerns itself with four different

categories of services-bandwidth, latency, jitter, and

loss. The first service category, bandwidth, concerns

itself with how the network manages the entire stream

of data packets flowing through it, particularly in

times of network congestion. The second service

category is latency, the end-to-end delay of a flow.

Numerous applications, including voice and video,

have a specific end-to-end delay budget. If a packet is

delayed beyond the allocated budget, the data

becomes stale or is no longer relevant. The third

category addresses the need to control jitter, the

variations in latency between packets. The final

category concerns the need to manage packet loss.

As a consequence of congestion, packet loss has two

purposes. First, reducing the number of packets

competing for output link can relieve the level of

congestion. Second, when sending hosts notice that

some packets are being discarded, they usually reduce

the volume of traffic they are injecting into the

network. While this is an effective approach for

managing congestion, many applications can only

tolerate a small amount of packet loss. The proposed

MPLS extension is mainly to minimize packet loss

and to control jitter.

Fig. 1. QoS requirements of various computer applications.

120

T. Gyires, H.J. Wen / Computer Standards & Interfaces 27 (2005) 117–132

2.2. Multiprotocol Label Switching (MPLS)

MPLS has emerged as an enabling technology for

new public networks due to its support for traffic

engineering. It is capable to forward large volumes of

voice and data between Label-Switched Routers

(LSRs). Because MPLS can simultaneously support

multi-layer services (Layers 2 and 3), it has gained

support from many standards organizations, equip-

ment providers and service providers. Telecommuni-

cations carriers can integrate multiple single-service

networks onto an integrated MPLS core. There is

already a large MPLS market because the technology

provides the foundation of many IP Virtual Private

Network (VPN) services. The next generation of

MPLS development will improve traffic engineering,

offer new services and integrate existing service plat-

forms. Each development will require a new infra-

structure because existing equipment will not be

powerful enough to support the new services. Vendors

have started to develop extensions to MPLS that are

focused on particular applications. Our proposed

extension to MPLS is unique in its traffic engineering

category. The addition of the new features largely

depends on the resource and QoS requirements of the

implementation in the routers and switches in an

MPLS network.

MPLS does not replace IP routing, but will work

together with existing routing technologies to provide

very high-speed data forwarding between LSRs. A

fixed-format label is inserted in front of each data

packet on entry into the MPLS network. At each hop

across the network, the packet is routed based on the

value of the incoming label and sent out on an

outgoing interface with a new label value. The path

that data traverses through a network is defined by the

transition in label values, as the label is swapped at

each LSR. Since the mapping between labels is

constant at each LSR, the path is determined by the

initial label value. Such a path is called a Label

Switched Path (LSP). The basic operation of an

MPLS network is depicted in	Fig. 2	similar to the

one in Ref. [6].

At the entry to the network, called ingress, each

packet is examined to determine which LSP it should

use based on the destination address, the quality of

service requirements, and the current state of the

network. The group of similar packets forwarded in

the same way is known as a Forwarding Equivalence

Class (FEC). One or more FEC may be mapped to a

single LSP. The diagram above shows two data flows

from host A: one to host B and one to host C. Two

LSPs are depicted. LSR1 is the ingress point to the

network to transmit data from and to host A. When

LSR1 receives packets from A, it determines the FEC

for each packet, determines the LSP to use, and inserts

a label to the packet. LSR1 then forwards the packet

on the appropriate outgoing interface defined for the

LSP. LSR2 examines the labeled packet received on

the incoming interface and refers to a lookup table to

decide the outgoing interface to send out the packet.

As shown in Fig. 2, each packet with label 15 will be

sent out on the interface to LSR3 with label 36. Label

23 will be swapped with label 19 and the packets will

be sent to LSR4. LSR3 and LSR4 are exit routers,

called egress LSRs from the MPLS network. The

egress LSRs remove the labels from the packets and

forward them using traditional IP routing.

Fig. 2. LSPs and LSRs in an MPLS network.

T. Gyires, H.J. Wen / Computer Standards & Interfaces 27 (2005) 117–132

121

LSPs are setup using the lookup tables at each LSR

that map the pairs (incoming interface, label) to the

pairs (outgoing interface, label) in the table entries.

The process that updates and creates the table entries

is called label distribution. The label distribution

protocol that establishes LSPs between LSRs can play

an important role in implementing Traffic Engineering

functions. The class of service and quality of service

required for traffic flows can also be included in the

Traffic Engineering process. There are currently two

label distribution protocols that provide support for

Traffic Engineering: RSVP and Constrain-based rout-

ed Label Distribution Protocol (CR-LDP). The main

differences between the two protocols are the reliabil-

ity of the transport protocol and direction of the

resource reservation, i.e., the reservation is done in

the forward or reverse direction. Each protocol has its

supporters and opponents, and the protocols are still

being developed by the Internet Engineering Task

Force (IETF). We chose RSVP to demonstrate our

Traffic Engineering protocol for reasons discussed

below.

2.3. Overview of RSVP

Originally, RSVP was developed as the signaling

protocol of the Integrated Services (IntServ) frame-

work to provide QoS support in IP based networks

[27,29]. The framework defines a flow as a stream of

related datagrams between hosts that is created by a

single application or user activity and requires the

same QoS. The physical path of the flow across a

network is determined by conventional destination-

based routing. A flow is a simplex mode of packet

transmission. Quality of Service is implemented in a

router and a host for a particular data flow by a

module called traffic control. It includes the Packet

Classifier, Admission Control, Policy Control, Packet

Scheduler, and the Estimator [27,29].

RSVP reserves resources in routers along the

traffic flow as requested by an application. RSVP

can be considered the configuration protocol for

IntServ. The sending host sends a PATH message

to the destination(s) of the traffic flow. It carries the

flow specification of the offered load: the identity of

the application, the traffic characterization (band-

width, burstiness, average and peak transmission

rate), and data for traffic classification. When the

PATH message arrives at the destination(s), it

responds by sending RESV messages identifying

the requested flow specification: the receiving appli-

cation, the type of services (latency bound, mini-

mum bandwidth guarantee, etc.) and the amount of

resources requested by the destination(s). RESV

messages return to the source along the reverse path

of the PATH messages. As RESV messages arrive at

the routers, the devices decide whether there are

sufficient resources available for the traffic flow. If

there are enough resources and there is no conflict

between the request and local policies established

for the destination, the request is admitted. Other-

wise, the request is rejected and an error message is

sent to the network devices along the path notifying

other RSVP-aware devices that the request was not

admitted.

RSVP has become a proposed standard and is

currently widely implemented in IP networking equip-

ment. However, RSVP has not been widely used in

service provider networks because of the lack of its

scalability and the overhead required supporting large

number of host-to-host flows. A number of extensions

were added to the original RSVP specification to

reduce the effect of the deficiencies above. The

designers of MPLS chose to further extend RSVP

into a signaling protocol to create LSPs that could be

automatically routed away from network congestions.

The main reasons that the designers of MPLS chose to

extend RSVP rather than design a new signaling

protocol to support traffic engineering requirements

are the following [30]:

(1) RSVP was designed for resource reservation

across a set of multicast or unicast Internet traffic

paths. Reservation is an important component of

traffic engineering that has made RSVP a good

candidate to implement it.

(2) RSVP allows carrying opaque objects in its

messages processed by the various modules in a

router. This feature makes it easier to add new

RSVP objects that can be used to create and

maintain distributed state information other than

resource reservation. The traffic engineering

extensions can easily be developed to extend

RSVP to support explicit routing and label

distribution that are essential traffic engineering

requirements.

122

T. Gyires, H.J. Wen / Computer Standards & Interfaces 27 (2005) 117–132

(3) The extended RSVP is backward compatible with

traditional RSVP implementations. It can differ-

entiate between LSP signaling and traditional

RSVP reservations by examining the objects

contained in the signaling messages.

3. Long-range dependent traffic

For more details of long-range dependency in

time series and the associated statistical tests, see

Refs. [19,23,31]. We follow the definitions of these

papers.

3.1. Definitions

Let X=(Xt: t = 0, 1, 2, . . .) be a covariance stationary

stochastic process. Such a process has a constant mean

l = E[Xt], finite variance	r2= E[(Xtl)2], and an

autocorrelation function	r(k) = E[(Xtl)(Xt+ kl)]/

E[(Xtl)2] (k = 0, 1, 2, . . .) that depends only on k. It

is assumed that X has an autocorrelation function of the

form:

H ! 1, the extent of both self-similarity and long-

range dependence increases. It can also be shown that

for self-similar processes with long-range dependency

H>0.5 [31].

There are various network models for characteriz-

ing the bursty nature of network traffic. Most of the

models focus on the Hurst parameter only and do not

consider the time scale from which the degree of long-

range dependency, the burstiness of the traffic, begins

to appear. As it has been demonstrated in Ref. [28],

the Hurst parameter alone is not sufficient to com-

pletely characterize traffic burstiness. The Hurst pa-

rameter only shows how fast or slowly the burstiness

increases or decreases but does not capture the range

of time scales over which the long-range dependency

nature of the network traffic becomes visible. The

paper describes that the time scales can be captured by

a parameter called Fractal Onset Time Scale (FOTS).

It shows the beginning of the time scale from which

the bursty nature of the traffic becomes evident. As

the FOTS becomes a smaller and smaller scale, the

traffic burstiness increases. Therefore, the traffic

burstiness can be characterized not just by the Hurst

parameter alone but by three parameters, namely the

rрkЮfakb;	k ј 1; 2; 3; . . .

р1Ю

average arrival rate of the network traffic, the Hurst

parameter, and the FOTS. After the authors of the

where 0 < b < 1 and	a	is a positive constant. Let

paper	[28], we call them the Three Fundamental

X(m)=(X(k)

(m):k = 1, 2, 3, . . ., m = 1, 2, 3, . . .) repre-

Parameters (TFPs). The paper illustrates the use of

sent a new time series obtained by averaging the

original series	X	over non-overlapping blocks of

size m. For each m = 1, 2, 3,. . .,	X(m)	is specified by

Xk(m) = 1/m(Xkmm + 1+. . . + Xkm), (k z 1). Let	r(m)

denote the autocorrelation function of the aggregat-

ed time series	X(m).

The process X is called exactly self-similar with self-

similarity parameter H = 1	b/2 if the corresponding

aggregated processes X(m)	have the same correlation

structure as X, i.e., r(m)(k) = r(k), for all m = l, 2, . . .

(k = l, 2, 3, . . .).

A covariance stationary process X is called asymp-

totically self-similar with self-similarity parameter

H = 1	b/2 if for all	k	large enough	r(m)(k) ! r(k),

m = 1, 2, 3, . . ., 0.5 V H V 1.

A stationary process is called long-range depen-

dent if the sum of the utocorrelation values ap-

proaches infinity: Akr(k) ! l.

The Hurst parameter	H	represents the speed of

decay of a process’ autocorrelation function. As

the TFPs in various Fractal Point Process models that

have been implemented in the OPNET network mod-

eling tool in the form of Raw Packet Generators. For

technical reasons, we chose the M/Pareto model to

characterize traffic burstiness. We implemented the

model and the TFPs in the COMNET modeling tool

based on captured bursty traffic traces. See the details

in Appendix A.

4. MPLS extension

Our framework is similar to the one in	Ref. [15]

applied for MPLS network. It extends MPLS by

adding new functionalities to the framework. The

new functionalities further augment the functions of

the LSR’s Estimator. We assume that the Traffic

Control database of an LSR has the following knowl-

edge of flows terminating at or crossing the router:

flow identifier, required resources, priority, and the

T. Gyires, H.J. Wen / Computer Standards & Interfaces 27 (2005) 117–132

123

maximum number of flows that can be established on

outgoing links.

Reassignment cost: Assuming a packet flow Ti. In

the subsequent paragraphs, we will explain that	Tiis

going to be distributed over some number of new

LSPs if the traffic burstiness exceeds a threshold

value. Let us denote the distributed portion of the

packet flow Tion path j by Tij. Each router that is in a

distribution path may have to make some resource and

routing reassignments. Aij(R) denotes the cost of such

a reassignment at LSR R for a distributed flow Tij. It

includes the costs for assigning buffers to the flow

being distributed, rerouting lower priority flows, etc.

The LSR’s Estimator estimates this cost based on

predefined entries in a cost table created by the

network service provider.

Discarded packets: Each Estimator can estimate

the packet loss in the new, distributed flow.	Pij(R)

denotes the estimated number of packets discarded at

LSR R in the distributed flow Tij.

E-parameter: The	E-parameter for LSR	R,

denoted by	Eij(R) is a measure of its efficiency in

rerouting a distributed portion Tijof the packet flow

Tion path	j. Small (close to zero) values of the	E-

parameter indicate high quality and efficiency, and

high values indicate that router	R is unreliable and

unstable in rerouting of distributed traffic. The Esti-

mator recalculates a router’s	E-parameter after each

flow distribution using the	E	values from previous

flow distributions. After each flow distribution, it is

determined how realistic an LSR’s cost estimate has

been with respect to the actual cost. By testing a

statistical hypothesis, it can be determined if it has

been unrealistic. If the corresponding hypothesis	H0

is rejected, no request messages will be sent to this

router in the future for diverting distributed flow. If a

router’s cost estimate has been realistic, i.e., the

corresponding hypothesis	H0is accepted, the	E-

parameter is recalculated. The closer the estimated

cost is to the actual cost, the smaller is the	E-

parameter. A local network administrator provides

initial values of	E. Subsequent values of the	E-

parameter are calculated using the statistical sam-

pling method given in Ref. [12]. The motivation for

using the	E-parameter is to enable the traffic distri-

bution process to learn from past performances and

use this knowledge to select the most efficient

distribution paths.

Distribution cost: The cost associated with a dis-

tributed flow Tijof flow Tiis the sum of reassignment

costs, the estimated discarded packets, and the	E-

parameters along a path	j. The cost of a distributed

flow	Tij(R,P) between two remote LSRs R and P is

denoted by C(Tij(R,P)) and defined recursively by:

CрTijрR; PЮЮ ј ЅCрTijрQ; PЮ ю EiрQЮ ю AijрQЮ

ю PijрQЮ;

where R and Q denote adjacent LSRs along the path j.

4.1. Flow distribution protocol

IntServ recommends the use of the Token Bucket

scheme to characterize the degree of burstiness. A

Token Bucket is defined by two parameters: a token

rate	T	that specifies the sustainable data rate of the

traffic flow, and the bucket size B that specifies the

amount by which the data rate can exceed T for short

periods of time. However, measurements of real traffic

indicate that burstiness is present on a wide range of

time scales that can be described statistically using the

notion of long-range dependency. Long-range depen-

dent traffic has observable bursts on all time scales.

As we discussed it in the previous section, an appro-

priate way of measuring traffic burstiness is based on

the estimate of the TFPs of the network traffic.

Therefore, characterizing burstiness based on the

TFPs is more appropriate than the Token Bucket

scheme that is based on measurements of only short

periods of time. We are going to apply the statistical

method developed in Ref. [11] to estimate the FOTS

of bursty traffic flows.

In our flow distribution protocol, the specification

of a new packet flow and subsequent control packets

carry the anticipated traffic’s TFPs. It is calculated

from previous connections and applications’ traffic

patterns at the sending host. It can also be estimated

and updated by intermediate LSRs. Different LSRs

can accommodate the same bursty traffic differently.

For instance, an LSR can reserve more input buffers

for the incoming packet flow; another router can

assign more processing power for handling the flow,

etc. We also assume that the specification of a new

packet flow contains the average traffic volume V in

bits per second that the router can handle based on

empirical observations. (Note that this measurement

124

T. Gyires, H.J. Wen / Computer Standards & Interfaces 27 (2005) 117–132

is not necessarily the forwarding capability of the

router).

When the TFPs of the flow specification of a new

request or the traffic burstiness of an existing flow

exceed certain thresholds, the router divides the traffic

into n new LSPs. Instead of using a single path, the

LSR will divide the traffic flow over n number of new

paths, leading to the destination(s) of the original

traffic. (For simplicity, we assume unicast instead of

multicast sessions. The protocol below can easily be

expanded for multicast as well.) The n new LSPs will

reunite at the destination using some existing techni-

ques, such as packet sequence numbering. The flow

distribution is implemented in the following steps.

Step 1. Assume that LSR X detected that the traffic

burstiness of a flow or the TFPs of a new request

exceed certain thresholds.	X	estimates the volume

requirement of the flow based and sends a PATH

message and the flow specification to the destination

LSR	Y	requesting resources for the flow. The PATH

message also request LSR Y to provide a label for the

new LSPs.

Step 2. LSR Y sends an RESV packet back to X on

all paths towards	X.	The RESV packet can follow

existing LSPs or any paths determined by standard IP

routing. The RESV packet contains the label that the

downstream LSR communicates to its upstream

neighbor. The RESV packets are reproduced at each

subsequent LSR along the paths to	X. The RESV

packet is sent upstream towards LSR	X. Each LSR

that receives a RESV packet with a label will use the

received label for outgoing traffic along the new LSP.

Step 3. Each RESV stores all visited LSRs along

the path in the packet’s data field.

Step 4. Each RESV carries the flow specification

requesting resources for flow distribution at each

LSR. An LSR’s Estimator estimates the resource

requirement of the flow. If there are enough resources,

the request is tentatively admitted and reservation is

made for a portion or for all of the requested resour-

ces. If the request is rejected, reject messages are sent

along the path, notifying MPLS-aware routers of the

failure. (Note that the reservation does not guarantee

that packets will not be lost because it is based on an

estimate only.) The reservation is valid only for a

limited	t	time interval. If there is no confirmation

arriving for the reservation in time t, the resources can

be reallocated for other purposes.

Step 5. Each RESV collects estimates of the

reassignment costs, discarded packets, the E-parame-

ter, and the reserved traffic volume from the Estimator

at each visited LSR.

Step 6. Upon receiving all cost estimates from all

paths, LSR	X	selects the	n	LSPs with the least

distribution costs that collectively can carry the vol-

ume of the original flow, and distributes the new or

existing traffic along the new	Tijflows, j = 1,2,. . .n.

(See the calculation below).

Step 7. After flow distribution, each Estimator

compares the actual cost and the estimated cost and

recalculates its own E-parameter for subsequent flow

distribution	[12]. It can be proven similarly to Ref.

[12] that the newly selected LSPs are optimal in terms

of the distribution cost and overall packet losses.

When the traffic burstiness falls below a certain

threshold value, the distributed traffic flows are col-

lapsed into the original single flow. If the flow’s

resource requirement exceeds the LSR’s resources

before the selection of the ‘‘least-cost’’ LSPs is

completed, then	X	makes the decision immediately

based on data from previous flow distributions.

5. Simulation method

A network system is a set of network elements

[18], such as routers, switches, links, users, and

applications working together to achieve some tasks.

The state of a network system is the set of relevant

variables and parameters that describe the system at a

certain time that comprise the scope of this study.

Instead of building a physical model of a network, we

build a mathematical model representing the behavior

and the logical and quantitative relations between

network elements. By changing the relations between

network elements, we can analyze the model without

constructing the network physically, assuming that the

model behaves similarly to the real system, i.e., it is a

valid model. For instance, we can calculate the

utilization of a link analytically, using the formula

U = D/T, where	D	is the amount of data sent at a

certain time and T is the capacity of the link in bits per

second. This is a very simple model that is very rare in

real world problems. Unfortunately, the majority of

real world problems are too complex to answer ques-

tions using simple mathematical equations. However,

T. Gyires, H.J. Wen / Computer Standards & Interfaces 27 (2005) 117–132

125

in highly complex cases, simulation technique is more

appropriate. Simulation models can be classified in

many ways. The most common classifications are as

follows:

5	Static and dynamic simulation models: A static

model characterizes a system independently of

time. A dynamic model represents a system that

changes over time.

5	Stochastic and deterministic models: If a model

represents a system that includes random elements,

it is called a stochastic model. Otherwise it is

deterministic. Queuing systems, the underlying

systems in network models, contain random

components, such as arrival time of packets in a

queue, service time of packet queues, output of a

switch port, etc.

5	Discrete and continuous models: A continuous

model represents a system with state variables

changing continuously over time. Examples are

differential equations that define the relationships

for the extent of change of some state variables

according to the change of time. A discrete model

characterizes a system where the state variables

change instantaneously at discrete points in time.

At these discrete points in time some event or

events may occur, changing the state of the

system. For instance, the arrival of a packet at a

router at a certain time is an event that changes the

state of the port buffer in the router.

In our study, we assume dynamic, stochastic, and

discrete network models. We refer to these models as

discrete-event simulation models.

Due to the complex nature of computer communi-

cations, network models tend to be complex as well.

The development of special computer programs for a

certain simulation problem is a possibility, but it may

be very time consuming and inefficient. Recently, the

application of simulation and modeling packages has

become more customary, saving coding time and

allowing the modeler to concentrate on the modeling

problem in hand instead of the programming details.

In a world of more and more data, computers,

storage systems, and networks, the design and man-

agement of systems are becoming an increasingly

challenging task. As networks become faster, larger,

and more complex, traditional static calculations are

Fig. 3. The COMNET model of an IP network.

126

T. Gyires, H.J. Wen / Computer Standards & Interfaces 27 (2005) 117–132

no longer reasonable approaches for validating the

implementation of a new network design and multi-

million dollar investments in new network technolo-

gies. Complex static calculations and spreadsheets,

bottleneck analysis and/or queuing analysis are not

appropriate tools any more due to the stochastic nature

of network traffic and the complexity of the overall

system.

Organizations depend more and more on new

network technologies and network applications to

support their critical business needs. As a result, poor

network performance may have serious impacts on the

successful operation of their businesses. In order to

evaluate the various alternative solutions for a certain

design goal, network designers increasingly rely on

methods that help them evaluate several design pro-

posals before the final decision is made and the actual

systems is built. A widely accepted method is perfor-

mance prediction through simulation. A simulation

model can be used by a network designer to analyze

design alternatives and study the behavior of a new

system or the modifications to an existing system

without physically building it. A simulation model

can also represent the network topology and tasks

performed in a network in order to obtain statistical

results about the network’s performance.

Simulation of large networks with many network

elements can result in a large model that is difficult to

analyze due to the large amount of statistics generated

during simulation. Therefore, it is recommended to

model only those parts of the network which are

significant regarding the statistics we are going to

Fig. 5. Application bursty source in the simulation.

obtain from the simulation. It is crucial to incorporate

only those details that are significant for the objectives

of the simulation.

Depending on the objectives, the same network

might need different simulation models. For instance,

if the modeler wants to determine the overhead of a

new service of a protocol on the communication links,

the model’s links need to represent only the traffic

generated by the new service. In another case, when

the modeler wants to analyze the response time of an

Fig. 4. The command repertoire in the simulation.

T. Gyires, H.J. Wen / Computer Standards & Interfaces 27 (2005) 117–132

127

Table 1

Number of packets dropped and input buffer usage

results when looking at an entire network. Modeling

the details of the routers’ internal operations in the

Node	# of packets blocked

Max. buffer use in bytes

order of magnitude of nanoseconds does not contrib-

Without traffic

With traffic	Without traffic

With traffic

ute significantly to the end-to-end delay analysis in

distribution

LSP2	127

LSP3	0

LSP4	0

LSP5	0

distribution

0

0

0

0

distribution

4,992,000

0

0

0

distribution

2,202,849

2,202,849

2,202,849

4,405,698

the higher order of magnitude of microseconds or

seconds. The additional accuracy gained from higher

model granularity is far outweighed by the model’s

complexity and the time and effort required by the

inclusion of the routers’ details.

Simplification can also be made by applying sta-

application under maximum offered traffic load, the

model can ignore the traffic corresponding to the new

service of the protocol analyzed in the previous model.

Another important question is the granularity of the

model, i.e., the level of details at which a network

element is modeled. For instance, we need to decide

whether we want to model the internal architecture of

a router or we want to model an entire packet switched

network. In the former case, we need to specify the

internal components of a router, the number and speed

of processors, types of buses, number of ports, amount

of port buffers, and the interactions between the

router’s components. However, if the objective is to

analyze the application level end-to-end response time

in the entire packet switched network, we would

specify the types of applications and protocols, the

topology of the network and link capacities, rather

then the internal details of the routers. Although the

low level operations of the routers affect the overall

end-to-end response time, modeling the detailed oper-

ations do not significantly contribute to the simulation

tistical functions. For instance, modeling cell errors in

an ATM network does not have to be explicitly

modeled by a communication link by changing a bit

in the cell’s header, generating a wrong CRC at the

receiver. Rather, a statistical function can be used to

decide when a cell has been damaged or lost. The

details of a cell do not have to be specified in order to

model cell errors. These discussions demonstrate that

our goal of network simulation in this study is to

reproduce the functionality of a network pertinent to a

certain analysis, not to emulate it.

6. Simulation results

For the sake of simplicity and for the illustration of

our model, we implemented a slightly modified ver-

sion of the distribution cost calculation in the simula-

tion. Let V denote the average traffic volume in bits

per second that is going to be distributed. Let

C1V C2V ,. . . V Ckdenote the cost estimates along k

Fig. 6. Utilization of link AB (a) without and (b) with traffic distribution.

128

Table 2

T. Gyires, H.J. Wen / Computer Standards & Interfaces 27 (2005) 117–132

Table 3

Average and maximum utilization of Link1

Average and maximum message delay between LSR1 and LSR5

Link1

Average

utilization (%)

Maximum

utilization (%)

Message delay between

LSR1 and LSR5

Average

delay (msec)

Maximum

delay (msec)

Without traffic distribution	0.2600

60.0

Without traffic distribution

46.83

1517

With traffic distribution

0.0848

17.8

With traffic distribution

10.815

261.8

LSPs. LSR X will choose the first n LSPs, n V k, for

which	V = V/C1 + V/C2 + . . . + V/Cn, and 1/C1 + . . . +

1/Cn = 1.

In our simulation, we want to make sure that the

traffic be distributed among the LSRs routers pro-

portionally to the available buffer sizes. Let the cost

estimate	Ci= 1/Ri, where	Riis the available buffer

size in bytes at LSRi. Let us arrange the available

buffer sizes	R1z R2z ,. . . z Rkand compute the

sum	R1+ R2+ . . . + Rk= R. The simulation model

distributes the flow proportionally to the available

buffer size:

V ј VR1=R ю VR2=R ю . . . ю VRn=R;	where

R1=R ю . . . ю Rn=R ј 1:

The motivation of LSR	X’s decision is that the

higher an LSR’s cost estimate the lesser amount of

traffic will be distributed via that LSR.

We implemented the flow distribution protocol in

the discrete event simulation system COMNET	[7]

based on real traffic traces captured in a large private

network with bursty traffic. With slight modifications,

similar models have been used in various papers and

contexts to illustrate the affect of burstiness on differ-

ent network protocols, such as DiffServ and IntServ,

etc. [13 – 15,24,32]. In this paper, we assume that the

selection algorithm above has already identified the

paths with the least cost estimates. In order to dem-

onstrate our distribution algorithm, we constructed the

following simplified model in COMNET.

In	Fig. 3, LSR1 through LSR5 denote the routers

of the MPLS network connected by OC-3 links:

Link1, Link2, etc. LSR1 is the origin; LSR5 is the

destination of one or more Label Switched Path

(LSP). There are three objects, so-called application

sources, attached to the LSRs: ‘‘Bursty Source,’’

‘‘Compute Available Buffer,’’ and ‘‘Forward’’. Appli-

cation sources implement the behavior of an applica-

tion by creating simple scripting language commands

in the node where the application sources are attached

to. For instance, the node LSR1 implements the

commands in	Fig. 4. Commands are taken from a

Library of commands specific to a node and custom-

ized according to the application source using these

commands.

The application source ‘‘Bursty Source’’ attached

to LSP1 represents an MPLS path and implements the

Path Distribution Protocol. Bursty Source sends mes-

Fig. 7. Message delay (a) without and (b) with traffic distribution.

T. Gyires, H.J. Wen / Computer Standards & Interfaces 27 (2005) 117–132

Fig. 8. Port processor utilization (a) without and (b) with traffic distribution.

129

sages to the destination with varying TFPs. If the

traffic burstiness is less than a threshold, LSR1 will

choose a path via LSR2. Otherwise, it will distribute

the traffic among the adjacent routers identified by the

selection algorithm shown in Fig. 5.

The ‘‘Compute Available Buffer’’ takes samples of

the current size of buffer capacities of the LSPs in

every second and transmits it to LSP1. The sizes of

the available buffers are needed for the path distribu-

tion protocol. We can assume that routers can deter-

mine the buffers available for a certain traffic flow.

The ‘‘Forward’’ application source implements the

forwarding function of the LSPs.

We ran the simulation for 8000 s with and without

traffic distribution and measured the number of pack-

ets dropped at the routers. The simulation statistics

show (in Table 1) that 127 packets have been dropped

without traffic distribution and no packets have been

dropped with traffic distribution due to the lack of

buffer space.

For Link1, as shown in	Fig. 6	and	Table 2, the

average utilization was 0.26% and 0.09% without and

with the traffic distribution respectively. The maxi-

mum utilization for Link1 could reach 60% when the

traffic distribution method was not implemented.

Fig. 7 is the results for the corresponding message

delay between LSP1 and LSP5. The message delay

gets longer if no traffic distribution is implemented.

Longer message delay (response time) can critically

affect users’ productivity. Table 3	demonstrates that

Fig. 9. Input buffer level for LSR2 (a) without and (b) with traffic distribution.

130

T. Gyires, H.J. Wen / Computer Standards & Interfaces 27 (2005) 117–132

the average message delay between the source and

destination LSPs is significantly less with traffic

distribution.

The traffic distribution method also results in

significant reduction of the port processor’s utilization

as it is shown in	Fig. 8. The port utilization is very

close to 100% without flow distribution. Very high

processor utilization may drastically decrease the

performance of the LSRs.

As a consequence of high port processor utiliza-

tion, the buffer usage levels of the LSPs are also

higher without traffic distribution. The result of high

buffer usage level is in longer waiting times in packet

queues and increasing number of dropped packets.

Fig. 9	shows the buffer level of LSR2 with and

without flow distribution. Due to space limitations,

we do not include all the details, such as the message

interarrival time, traffic burstiness threshold, buffer

sizes, the full command sequence, etc. The details are

available from the authors upon request.

In summary, the simulation results demonstrate

that the flow distribution improves the overall net-

work performance in link utilization, port processor

utilization, message delay, number of dropped pack-

ets, and buffer usage level.

7. Conclusion

The paper presented an extension of MPLS traffic

engineering to reduce the number of discarded packets

during congestions caused by bursty, long-range de-

pendent traffic. When the traffic burstiness of a packet

flow exceeds a certain threshold at an LSR, the router

selects n new LSPs to distribute the original long-

range dependent traffic over these LSPs. The selection

is based on cost estimates of the traffic distribution

and the effectiveness of the LSPs in reducing the

number of discarded packets in previous traffic dis-

network resources. In our proposed extension version,

an LSP can still be established for long-range depen-

dent traffic that would not be possible in the current

version of MPLS. The main limitation of this study,

however, is the inadequate capabilities of the current

network routers. The routers have to implement the

functions needed to make decisions on various net-

work parameters, such as burstiness threshold, divid-

ing the amount of traffic flow into sub flows, etc. The

algorithm can be included in standards only if the

router/switch vendors agree on a widely accepted

standard and can provide enough processing power

in the routers and switches.

Appendix A

Results in	Refs. [1,24]	have proven that the M/

Pareto model is appropriate for modeling long-range

dependent traffic flow characterized by long bursts.

Originally, the model was introduced in Ref. [20] and

applied in the analysis of ATM buffer levels. The M/

Pareto model was also used to predict the queuing

performance of Ethernet, VBR video, and IP packet

streams in a single server queue [24,25]. We apply the

M/Pareto model not just for a single queue, but also

for predicting the performance of an interconnected

system of links, switches and routers affecting the

individual network elements’ performance. We make

use of some of the calculations presented in	Refs.

[24,25].

The M/Pareto model is a Poisson process of over-

lapping bursts with arrival rate k. A burst generates

packets with arrival rate r. Each burst, from the time of

its interval, will continue for a Pareto-distributed time

period. The use of Pareto distribution results in gen-

erating extremely long burts that characterize long-

range dependent traffic. The probability that a Pareto-

distributed random variable X exceeds threshold x is:

tributions. We implemented the flow distribution

protocol in a discrete event simulation model and

demonstrated that our proposed MPLS extension can

minimize the packet loss caused by long-range de-

pendent traffic. The current version of MPLS applies

PрX	> xЮ ј

8

<

:

x

d

1;

c

;	xzd

otherwise

9

=

;

р1Ю

label distribution protocols, such as RSVP that

reserves the network resources along an LSP in

advance. An LSP may not be established in case of

long-range dependent traffic that requires excessive

1 < c < 2; d > 0:

The mean of X, the mean duration of a burst a = dc/

(c	1) and its variance is infinite [26]. Assuming a t

T. Gyires, H.J. Wen / Computer Standards & Interfaces 27 (2005) 117–132

131

time interval, the mean number of packets M in the

time interval t is:

mean of the Pareto distribution is implemented in

Comnet as:

M ј ktrdc=рc	1Ю;	and

k ј M рc	1Юtrdc

р2Ю

р3Ю

M ј dc=рc	1Ю

and

The M/Pareto model is described in Refs. [20,24]

as asymptotically self-similar and it is shown that for

the Hurst parameter the following equation holds:

d ј M *рc	1Ю=c

The relation (1) allows us to model bursty traffic

based on real traffic traces by performing the follow-

H ј р3	cЮ=2

р4Ю

ing steps:

7.1 . Implementation of the M/Pareto model in Comnet

Comnet models a transaction by a message source

and a destination, the size of the message, and the

communication devices, and links along the path. The

rate at which messages are sent is specified by an

interarrival time distribution. The M/Pareto model’s

Poisson distribution represents the number of mes-

sage arrivals in a certain time interval. In Comnet,

this information is expressed as the time interval

between successive arrivals, hence, we use the Ex-

ponential distribution as the traditional way to specify

interarrival time. Using the Exponential distribution

will result in an arrival pattern characterized by the

Poisson distribution. The interarrival time in the

model is 1 s.

The length of a message is specified by the Pareto

distribution defined by two parameters in Comnet: the

location and the shape. The location parameter corre-

sponds to the packet arrival rate	d	in a burst. The

shape parameter corresponds to the c parameter of the

M/Pareto model calculated in the relation (1) as

c ј 3	2H

In the M/Pareto model, each burst will continue

for a Pareto-distributed time period. In our test bed,

we cannot measure the number of packets in a burst,

only the number of packets in a message. Hence, we

assume that a burst is equivalent to the number of

bytes in a message sent or received in a second.

Because the time period of each burst is proportional

to the length of the message, we further assume that

the length of the messages is also Pareto-distributed.

So we derive the packet arrival rate d in a burst not

from the mean number of packets in a burst but from

the mean length of messages denoted by	M. The

(a) Collect traffic traces using a network analyzer.

(b) Compute the mean arrival rate, the Hurst

parameter	H	and the location parameter that is

equivalent to the FOTS in the Fractal Point

Process models in	Ref. [28]. We used the Benoit

package with the traffic trace as input to compute

these parameters [3].

(c) Use the Exponential and Pareto distributions in

the COMNET modeling tool with the parameters

calculated above to specify the interarrival time

and length of messages.

рdЮ	Generate traffic according to the modified M/

Pareto model and measure network performance

parameters.

References

[1]	R. Addie, M. Zukerman, T. Neame, Broadband traffic mod-

eling: simple solutions to hard problems, IEEE Communica-

tions Magazine (1998) 88 – 95.

[2]	M. Balakrishnan, R. Venkateswaran, QoS and differentiated

services in a multiservice network environment, Bell Labs

Technical Journal (1998 Oct – Dec) 222 – 239.

[3]	Benoit 1.1, Trusoft Int’l,	http://www.trusoft-international.

com.

[4]	Y. Bernet, The complementary roles of RSVP and differenti-

ated services in the full-service QoS network, IEEE Commu-

nications Magazine (2000 February) 154 – 162.

[5]	J. Bolot, A. Shankar, Dynamical behavior of rate based flow

control systems, Computer Communication Review 20

(1990, Apr.) 35 – 49 (ACM SIGCOMM).

[6] P. Brittain, A. Farrel, MPLS Traffic Engineering: A Choice of

Signaling Protocols, Analysis of the Similarities and differ-

ences between the two primary MPLS label distribution pro-

tocols: RSVP and CR-LDP, Data Connection Limited 2000.

[7] ‘‘COMNET III Application Notes: Modeling ATM Networks

with COMNET III’’ CACI Products Company, 3333 North

Torrey Pines Ct., La Jolla, California 92037, 1998.

[8] A. Durresi, S. Kota, M. Goyal, R. Jain, V. Bharani, Achieving

132

T. Gyires, H.J. Wen / Computer Standards & Interfaces 27 (2005) 117–132

QoS for TCP traffic in satellite Networks with differentiated	to modeling broadband traffic streams, Proceedings of ICON

services, Space Communications 17 (2001) 125 – 136.	’99, Brisbane, Queensland, Australia, 28 September – 1 Octo-

[9] G. Eichler, H. Hussmann, G. Mamais, I. Venieris, C. Salsano,

S. Salsano, Implementing integrated and differentiated serv-

ices for the internet with ATM networks: a practical ap-

proach, IEEE Communications Magazine (2002 January)

132 – 141.

[10]	B.V. Ghita, S.M. Furnell, B.M. Lines, D. Le-Foll, E.C.

Ifeachor, Network quality of service monitoring for IP tele-

phony, Internet Research 11 (2001) 26 – 34.

[11]	T. Gyires, Methodology for modeling the impact of traffic

burstiness on high-speed networks, Proceedings of the 1999

IEEE Systems, Man, and Cybernetics Conference, October

12 – 15 Tokyo, Japan, 1999, pp. 980 – 985.

[12]	T. Gyires, B. Muthuswamy, A bidirectional search approach

for restoring circuits in communication networks, The Journal

of Computer Information Systems (1997 Winter) 85 – 93.

[13]	T. Gyires, Active agents algorithm for differentiated services

in networks with bursty traffic, Proceedings of the Commu-

nication Systems, Networks and Digital Signal Processing

(CSNDSP’ 2002), Third International Symposium, Stafford,

UK, July 15 – 17, Co-Sponsored by: IEEE, IEE, EURASIP,

BCS, 2002, pp. 74 – 77.

[14]	T. Gyires, Simulation of the harmful consequences of self-

similar network traffic, The Journal of Computer Information

Systems (2002 Summer) 94 – 111.

[15]	T. Gyires, Software Agents Architecture for Controlling

Long-range Dependent Network Traffic, accepted for publi-

cation in the Special Issue of Mathematical & Computer

Modelling, Elsevier Science.

[16]	P. Herrmann, H. Krumm, O. Drogehorn, W. Geisselhardt,

Framework and tool support for formal verification of

high-speed transfer protocol designs, Telecommunications

Systems 20 (2002) 291 – 310.

[17]	M. Khan, Network management: don’t let unruly internet

apps bring your network to its knees, Communications News

37 (2000) 30 – 32.

[18]	M. Law, W.D. Kelton, Simulation Modeling and Analysis,

third edition, McGraw-Hill Higher Education, 2000.

[19]	W. Leland, M. Taqqu, W. Willinger, D. Wilson, On the self-

similar nature of Ethernet traffic, computer communications

review 23, Proc. of the ACM/SIGCOMM’93, San Francisco,

1993 (September), pp. 183 – 193.

[20]	N. Likhanov, B. Tsybakov, N. Georganas, Analysis of an

ATM Buffer with Self-Similar (‘‘Fractal’’) Input Traffic,

Proceedings of the IEEE INFOCOM Conference, 1995,

pp. 985 – 992.

[21] S. Mishima, L. Moy-Yee, G. Yee-Madera, E. Yousefi, Broad-

band packet switch processor, Space Communications 18

(2002) 91 – 95.

[22]	M. Nadeau, S. Chalifour, QoS in the Relentless Pursuit of

Customers, Telephony 5 (2000 June) 234 – 239.

[23] E. Neame, Investigation of traffic models for high speed data

networks, Proceedings of ATNAC ’95, 1995, pp. 145 – 151.

[24] T. Neame, M. Zukerman, Application of the M/Pareto process

ber, 1999, pp. 53 – 58.

[25]	T. Neame, M. Zukerman, R. Addie, A practical approach for

multi-media traffic modeling, Proceedings of Broadband

Communications ’99, Hong Kong, 10 – 12, November,

1999, pp. 73 – 82.

[26]	B.D. Reimers, In depth quality of service: Carriers expand

QoS options, InternetWeek (2001 June 25) 37 – 38.

[27]	Request for Comments: 2205, L. Zhang, S. Berson, S.

Herzog, S. Jamin, Resource ReSerVation Protocol (RSVP),

September 1997.

[28]	B. Ryu, S. Lowen, Fractal traffic models for internet simula-

tion, HRL laboratories and mclean hospital, Proceedings of

IEEE Int’l Symposium on Computer Communications, 2000,

pp. 287 – 291.

[29]	C. Semeria, RSVP Signaling Extensions for MPLS Traffic

Engineering, White Paper, Juniper Networks, 2000.

[30]	C. Semeria, RSVP Signaling Extensions for MPLS Traffic

Engineering, White Paper, Juniper Networks, 2000.

[31]	M. Taqqu, V. Teverovsky, W. Willinger, Estimators for long-

range dependence: an empirical study, Fractals 3 (1995)

785 – 788.

[32]	W. Weiss, QoS with differentiated services, Bell Labs Tech-

nical Journal (1998 Oct – Dec) 48 – 62.

Tibor Gyires is a Professor in the School

of Information Technology at Illinois State

University, Normal, Illinois. He received

his PhD degree in Mathematics at the

Kossuth Lajos University, Hungary. Dr.

Gyires’ research areas include distributed

artificial intelligence, search algorithms,

high-speed networks, simulation modeling

of networks, and network capacity planning

and performance prediction. He has pub-

lished several papers in referred journals,

proceedings, and books. His hobbies are skiing, sailing, and playing

basketball.

H. Joseph Wen is an Associate Professor

of MIS and chairperson of the Department

of Accounting and Management Informa-

tion Systems at Donald L. Harrison College

of Business at Southeast Missouri State

University. He holds a PhD from Virginia

Commonwealth University. He has pub-

lished over 100 papers in academic refereed

journals, book chapters, encyclopedias and

national conference proceedings. Dr. Wen

has received over 6 million dollars research

grants from various State and Federal funding sources. His areas of

expertise are Internet research, electronic commerce (EC), transpor-

tation information systems, and software development.