Hybrid Routing for Scalable IP/MPLS Traffic Engineering

Huan Pham, Bill Lavery

School of Information Technology

James Cook University

Qld 4811, Australia

{huan, bill}@cs.jcu.edu.au

Abstract—This paper presents performance data for a new

Hybrid Routing Method (HRM) for IP network traffic

engineering. In this method, the major traffic streams between

some source/destination pairs is routed via MPLS constraint-

based routing, while the remaining traffic is routed via

conventional IGP routing. When Internet traffic is heavy-tailed

distributed, consisting of a few “elephant” and many “mice”

flows, our traffic engineering results indicate that our HRM

normally needs just a small number of tunnels to achieve a

network performance that is comparable to that of fully meshed

MPLS network. The method has been used to implement a HRM

traffic engineering tool. The tool enables network operators to

visualize and manage traffic to avoid congestion, as well as to

decide where to place MPLS routers and tunnels.

Keywords—Hybrid Routing, MPLS, IP, Traffic Engineering

I.	INTRODUCTION

Traffic engineering for IP networks is the process of

mapping traffic demand into the network topology, and

realizing such mapping via routing protocols, so that a

predefined performance objective is achieved. From the

network point of view, the objective is an evenly loaded

network with minimum congestion, i.e. there should not be

unnecessarily over-utilized links while others are under-

utilized.

Current IGP (Interior Gateway Protocol) routing protocols

provide little capability for solving the above traffic

engineering problem. These routing protocols (such as

Intermediate System to Intermediate System - IS-IS, Open

Shortest Path First - OSPF) are all destination-based Shortest

Path First (SPF) protocols. At each router, routes are

calculated in a distributed way, based on the link metric and

the network topology, but not based on the network load

status. The only way to change routes is by changing the

metric associated with each link. Explicit routes and constraint

based routing are therefore impossible. A number of

researchers [1, 2] have reported optimizing the link metrics for

the purpose of traffic engineering, but this approach only

provides limited gain. In many cases, without constraint based

routing and explicit routes capability, congestion caused by

unbalanced load is unavoidable [3, 4].

This traffic engineering problem can be effectively

addressed by the evolving Multi Protocol Label Switching

(MPLS) technology [4, 5, 6, 7]. With MPLS, routes are

calculated at source routers, called Ingress Routers, which take

into account not only the network topology but also traffic

oriented constraint (such as bandwidth, delay, hop count) and

administrative constraints (i.e. some links or nodes are

preferred for certain traffic demands). The network operator

therefore has a greater control over how traffic is routed and

traffic engineering can be more effective.

However, MPLS fully meshed networks have a scalability

limitation. For a network of N routers, the number of Label

Switch Paths (LSPs) that have to be set up and distributed (to

routers along the LSPs) for all sources/destinations is in the

order of N2. This does not scale well for a large network. On

the other hand, IGP has a much better scalability, because

routes are calculated in a distributed fashion at each router,

and the routing table size is in the order of N only. This

suggests that a hybrid of IP and MPLS routing may provide a

reasonable compromise, providing better traffic engineering

than does plain IP routing, but better scalability than does

fully meshed MPLS.

In the next section, we discuss the problems associated

with existing hybrid routing schemes, and introduce our new

Hybrid Routing Method. Section III presents the architecture

of a HRM traffic engineering tool, and section IV explains the

principle of the tunnel placement module, which is the core of

our traffic engineering tool. Traffic engineering results for the

AT&T Internet backbone are discussed in Section V. Finally

Section VI summarizes the main contributions of the paper as

well as our future research.

II. HYBRID ROUTING METHOD

In this new Hybrid Routing Method (HRM), some traffic

is routed via MPLS constraint-based routing while the

remaining traffic is routed via plain IP routing. This hybrid

approach enables network operators to gradually upgrade their

facilities to be MPLS enabled, rather than having to upgrade

all at once; the later is the MPLS fully meshed model. The

traffic engineering problem for this hybrid method reduces to

determining where and how to set up LSPs, and then allowing

the remaining traffic to be routed via plain IP routing.

Commercial tools such as WANDL IP/MPLS View [8]

and OPNET Service Provider Guru [9] address similar

problems. However, these tools are not accessible for public,

and their algorithms for optimizing MPLS tunnels can not be

evaluated.

The published proposal by Wang and Zhang [10, 11] is a

hybrid approach originating from the finding that, it would be

possible to achieve an optimal solution for traffic engineering

purposes using an IGP routing protocol if it supported

arbitrarily traffic splitting. The authors then proposed to solve

the optimal routing problem. The outcome is the set of

0-7803-7802-4/03/$17.00 © 2003 IEEE

332

R1

R2

R3

R6

R7

R4

R8

R9

R5

R10

Therefore, our HRM has been designed so as to not rely on

changing IGP metrics for traffic engineering. Rather, we

actively use MPLS tunnels to avoid the potential congestion

links that would otherwise be caused by plain IP routing.

Depending on where one places the MPLS tunnels, the hybrid

traffic engineering can be further classified as "local" or "end-

to-end" tunnel approaches.

The local tunnel approach places MPLS tunnels, or Label

Switch Paths (LSPs), locally around any overly congested

links. In an example, given in Figure 1, traffic from R1 to R5

would normally overload the links R2-R3 and R3-R4 if only

IGP routing was supported. A local tunnel is therefore set up

between R2 and R4 to steer some of the traffic to go via a

longer but wider path R2-R6-R7-R8-R9-R4.

The other option is to create an end-to-end tunnel between

Figure 1: Local and End-to-End tunnels

R7

the major sources and destinations that contribute to the

heavily loaded links, i.e. the tunnel R1-R2-R6-R7-R8-R9-R4-

R5 between R1 and R5.

The two options both have advantages and disadvantages.

The local tunnel approach has a quicker response to network

failures, as it takes less time to inform the ingress router of

R1

R2

R3

R6

R4

R8

R9

R5

R10

intermediate node or link problems, and to switch packets to

back-up tunnels. However, the local tunnels may result in a

sub-optimal "back flow" problem as shown in Figure 2.

Assume that there are two demands between R1-R5 and R1-

R10, plain IP routing would normally cause links R2-R3 and

R3-R4 to be overloaded. The local approach creates a tunnel

to get around the most heavily loaded links, i.e. between R2

and R4. The traffic from R1 to R10 then has to take a round

tour to R4 and back to R10 via R9.

The "end to end" tunnel approach overcomes this "back

flow" problem because each tunnel is created to serve only

one traffic demand causing no side effect on the IGP routing

for other source/destinations. This gives more flexibility for

Figure 2: "Back flow" problem

weights and the traffic splitting ratio when there are multiple

paths between any source/destination pair. Traffic is normally

routed via shortest paths, and MPLS is only used for arbitrary

splitting when it is needed.

However, this approach depends heavily on adjusting IGP

metrics for traffic engineering, and so is problematic in that,

when the traffic requirements change, the optimal set of

weights and the ratio for traffic splitting change accordingly.

Note that changing the whole set of weights in plain IP routing

is generally undesirable as it often results in large convergence

time, during which routing loops and packet losses may occur.

In addition, this approach does not guarantee that the outcome

set of weights are all integer and does require an arbitrary

traffic splitting ratio among parallel paths. In practice, traffic

splitting is usually done by mapping the paths with an integer

number of "hash buckets", and the maximum number of

buckets is often limited (16 for Cisco routers) [12, 13]. That

means a truly arbitrary splitting ratio is not realizable in

practice.

333

the tunnel placement process. For this reason, we chose to

implement this end-to-end approach for our HRM, even

though this may imply additional MPLS enabled routers.

One may ague that in the given example, local approach

can still create two separate tunnels between R2-R4 and R2-

R9 to overcome the “back flow” problems. This is a valid

solution, that can be also viewed as an “end to end” solution if

we only apply traffic engineering for a subset {R2.. R9} of the

network. In other words, our “end to end” notion can be

relaxed, which may apply to a subset of routers and

aggregated traffic between that subset.

For large networks, where response time to network failure

along long tunnels may be of concern, it may be better that

this "relaxed end-to-end" HRM approach is used for its sub-

areas, such as the backbone network of the Internet where the

traffic engineering is often most needed.

III. ARCHITECTURE FOR HRM TRAFFIC ENGINEERING TOOL

For the best performance of the HRM implementation, it is

necessary to have a traffic engineering tool that is responsible

for modeling OSPF traffic routing as well as the tunnel

placement optimization. The architecture of our proposal is

described in Figure 3.

From the bottom of the diagram upward, the traffic

demand module is regularly updated via traffic projection

and/or measurement. Given the topology database and traffic

demands as inputs, a plain IP traffic load on each link is

calculated. Based on the plain IP traffic load assignment,

potential congested links are then identified. This, together

with the topology and traffic database, will then serve as the

inputs for the MPLS tunnel optimization module.

In the next part, we will discuss the tunnel optimization

module, as this is the most important part of the traffic

engineering tool.

IV.	MPLS TUNNEL OPTIMIZATION

In order to decide where and how to place MPLS tunnels, an

optimization objective for overall network performance needs

HRM Load

Plain IP Load

Topology Database	Traffic Demand

Measurement &

Projection

Tunnel

Optimization

to be defined. Without losing generality our tool uses the

objective function of [14], which is based on the average end-

to-end packet delay (or total end-to-end packet delay), taking

into account the self-similar characteristics of the Internet

traffic. This objective function also allows for the link

utilization to be greater than 100%, at a cost of a very high

penalty. The main reason is to discourage any traffic

engineering solution that ends up with overloaded links. Of

course, different objective functions, which might better

represent network performance, can also be utilized for our

HRM approach.

For any given network, the tunnel placement problem then

reduces to setting up a limited number of tunnels between

some source/destination routers such that the network

performance objective function improves the most. For our

objective function, this equates [14] to optimizing the total

packet delay, Φ, summed over all links. That is, the aim is to

minimise:

∑	

Φ	Φa

Figure 3: Traffic Engineering Tool Architecture

In addition, previous research [16, 17] has shown that the

Internet traffic consists of a few “elephant” flows and many

“mice” flows. Moving those “mice” flows from a heavily

loaded links by MPLS tunnels would not significantly

improve the overall network performance, while costing

tunnel set-up and maintenance. Therefore, our algorithm only

searches among the largest flows contributing to heavily

loaded links, as the candidates for potential tunnel placement.

The greedy algorithm is described as follows. We first

calculate traffic flows and the resulting link loads routed by

the SPF routing protocol, using the Dijkstra algorithm. We

now have available the SPF routed flows on each link, as well

as the sources and destinations of the flows f(s,t,la).

Then, the links are considered in the order of decreasing

link utilization. For each link candidate, flows from different

source/destination routers are sorted according to their sizes.

We again consider only a limited number of largest size flow



	ρ



∈

a	A

ρ3



ρ



candidates, as they are the dominant contributors to the link

congestion; typically we use only the largest ten flows per

link.

For each candidate flow, the effectiveness of constraint-

Φa



a

1−

a

ρ

a

3

, for

a

0.9



based routing of that flow is then evaluated. The traffic

demand from the corresponding source/destination pair is



ρ

−

for ρ

≥

24301a21141,

a

0.9

removed from the set of SPF traffic loads on all links, which

will then be routed via a new LSP or parallel LSPs. The

where A is the set of arcs (or unidirectional links), ρa = fa/Ca

is the link utilization, Caand fa are the link capacity and traffic

load on the link a.

The tunnel placement problem without arbitrary traffic

splitting is known to be a NP-hard problem [15]. There is no

algorithm that can guarantee the solution to such problems

within a reasonable time limit.

In this paper, we propose a greedy algorithm, which

attempts to identify the best traffic demand candidates one at a

time, to be routed via a LSP tunnel, and then implements the

new LSP tunnel that improves the objective function the most.

It is observed that, the most influential terms in the objective

function (1) are the links with highest utilizations. Therefore,

the best traffic demand candidates for constraint-based routing

should be found among those traffic flows that contribute to

the load on the heaviest utilized links.

334

process of routing a traffic demand via single path or parallel

paths constraint-based routing is described in the sections

IV.A, and IV.B below.

If the best constraint-based routing solution for the 10

largest flows on the link results in improvement then it is

chosen as the next actual MPLS tunnel implementation, and

we repeat the process for the new heaviest link. If no

improvement results, no tunnel is implemented and the search

is done for the largest size flows on the next heaviest link. The

whole process is repeated until some maximum number of

LSPs, L, is reached, or no further improvement is made.

The algorithm is formally described as follows:

1.  Model HRM traffic routing, initially with no tunnels, to

determine plain IP flows on the links f(s,t,la).

2. Sort the links according to the link utilization, in

decreasing order. Select the highest utilization link as

the candidate link.

3. Sort the plain IP traffic flows on the candidate link

according to the flow size in decreasing order.

4. For each of the ten largest plain IP flows, try constraint-

based routing the candidate traffic demand, using single

LSP or multiple LSPs (as described in the sections

IV.A, and IV.B).

5.  If at least one of the solutions in step 4 results in

improvement then the solution with the best objective

function improvement per number of required LSPs, N,

i.e.	Φold−Φnew, is chosen as the next MPLS tunnel(s) to

L

be implemented. Update HRM traffic. Next candidate

link is the new highest utilization link.

Else (i.e. all solutions in 4 result in no improvement)

the next candidate link is the next highest utilization

link.

6. If the maximum allowed number of LSPs has been

reached, or the next candidate link is the lowest

utilization link (i.e. no improvement can be made), then

stop; otherwise go back to step 3.

Although this greedy algorithm does not guarantee an

optimal set of LSPs (subject to a limited number of LSPs and

non arbitrary splitting ratios), the effectiveness of this

algorithm can be evaluated. Here effectiveness means the

difference between the performance objective function found

by our algorithm, and that found by the "optimal" solution; the

optimal solution allows for unlimited LSPs and arbitrary

splitting and is solved using Flow Deviation (FD) algorithm

described in [18].

A. Single LSP constraint-based routing

In this case, only a single LSP is permitted to route a

candidate traffic demand d. We first remove this demand from

set of plain IP routing demands. Then our objective is to find

an LSP for this demand, to minimise the overall delay increase

on every link on the path. Therefore the new LSP is actually

the shortest path from the source to the destination; given the

link "weight" of each link is the increase in delay on that link

after loading the new traffic demand d:

single traffic demand could be solved optimally, again using

the FD algorithm.

However, in practice traffic splitting among multiple LSPs

can not be arbitrary. The splitting ratio is proportional to the

number of hash buckets associated with each LSP in the set of

parallel LSPs. This can be thought of as equivalent to a

number of evenly load balancing tunnels, some of which

follow the same route and are merged to become a bigger

pipe. This problem can be solved by constraint based routing a

number of equal size LSPs (the number of them is less than or

equal to 16 for Cisco routers); this is similar to the single LSP

scenario presented before, plus merging of the LSPs if they

follow the same route.

Taking into account the fact that different parallel path

constraint-based routing solutions may use different number

of LSPs we use the average delay improvement rate per LSP

as the evaluation criterion. This means the solution that

maximizes 	Φold−Φnew will be chosen for tunnel placement,

L

where	Φold and Φnew are the total delay before and after

constraint-based routing this demand candidate, and L is the

number of parallel LSPs used for constraint based routing this

demand.

C.  Application of End-to-End approach to a subset of

routers

As discussed earlier, "end-to-end" tunnels respond less

quickly to network failure compared to the "local" approach.

Therefore, for a large network, we may want to relax the "end

to end" definition to only a small subset of routers, for

example backbone or core routers. These routers are

identified, together with the accumulated traffic requirement

between each source/destination router pair, including transit

traffic from other "exterior" routers, entering at the source

router and departing at the destination router. This is achieved

via measurement, or by extracting data from the global traffic

demand database. Solving the tunnel placement for the subset

of routers is then one case of the end to end tunnel placement

problem, and implementation is straight.

V.	PERFORMANCE

We have implemented the proposed HRM traffic

engineering tool. To evaluate its effectiveness, the traffic

engineering tool is applied to the AT&T Internet backbone











and all routers are assumed to be MPLS capable. The topology

Φ

Ca,	f	d

	−Φ

,

C	f

w

a

a

a

d

a	a	a



database is obtained from the Rocket Fuel program [19, 20].

For simplicity, we only consider backbone routers. In

addition, routers belonging to the same cities are merged as

This problem is solved using the Dijkstra algorithm.

B. Parallel LSPs constraint-based routing

If truly arbitrarily splitting was supported, the optimal

parallel LSPs placement for one traffic demand candidate

would be just a special case of optimal routing, where there is

only one traffic demand between the single source and

destination under the consideration. The traffic demands from

other sources/destinations can be considered as fixed residual

loads. Therefore this problem of finding optimal LSPs for a

335

one. The simplified topology consists of 107 routers and 140

links. The real link capacities and traffic matrix are not

available. Without losing generality, we assign each link a

capacity of 1000 Kbps. The traffic demand between any

source/destination pair is generated randomly, as follows:

,a eb

Dij

where a is a random number between [0,1] and b is a

random number between [0, ln(MaxDemand)]. We then adjust

the	MaxDemand parameter to get about 150% maximum

utilization to simulate an overloaded network. For this traffic

model, demand between any source/destination pair is random

between [0,	MaxDemand], and mostly small, but can be very

high for a small number of source/destination pairs. This is to

reflect traffic demand characteristics observed in practice, ie

the Internet traffic from a source router mostly heads to few

destination routers, while demands for the other destinations

are very low.

The performance of our HRM is shown in the Figures 5

and 6. Figure 5 shows the network objective function versus

the number of LSPs established. The horizontal line is the

theoretical optimal average packet delay, calculated using the

FD algorithm; the other line is our actual average packet delay

objective function. Initially, with zero LSP, ie plain IP routing,

the average packet delay is 2.2 sec. which is about 100 times

larger than the 22.6ms of the optimally routed networks.

The figure indicates that, about 60 LSPs are needed to

bring average packet delay down to about 23 ms, which is

within 2% to the optimal value of 22.6ms.

Another measure of network performance would be

maximum utilization across all links. Although not

specifically addressed as the objective function, our algorithm

also effectively reduces the maximum utilization, since the

search for new tunnel implementation always first tries the

heaviest utilized link. Figure 6 presents the maximum link

utilization as a function of the number of LSPs. The maximum

link utilization is reduced from 167.5% for the network using

plain IP routing to 84% for the network using 60 LSPs; note

that this utilization is the same as that obtained with optimal

routing.

These results indicate that only 60 LSPs is needed for the

network of 107 routers and 140 links to get to a performance

that is very close to the optimal solution. In contrast, for a

fully meshed MPLS routing, the number of LSPs required is

of the order of 1072 or about 10 000. An earlier paper [21]

reported comparable results for arbitrary networks that are not

2.5

2

1.5

1

0.5

0

0

Figure 4: Network Diagram

40	80	120

Number of LSPs

160

200

as asymetric as the AT&T Internet backbone.

VI.	CONCLUSION

In this paper, we have presented performance data for a

new Hybrid Routing Method, HRM, for traffic engineering

using both IGP protocols (such as OSPF or IS-IS) and MPLS

routing. The HRM routes a limited number, L, of traffic flows

using MLPS, and routes the remainder using the conventional

IGP routing protocol. Simulations suggest that keeping L

small (~60 for a network of 107 routers) yields significant

improvement in network performance, and that network

performance using HRM can approach that of a fully meshed

MPLS network. Importantly, our approach is scalable to large

networks, because it requires much less LSPs (in the order of

O(N) roughly) compared to O(N2) for fully meshed MPLS

networks.

Our method identifies the traffic flows that most impact on

network performance, and determines for those flows the

LSPs that will most improve network performance. This

approach allows for progressive implementation of MPLS into

an existing plain IP network. The network operator can

336

2

1.5

1

0.5

0

0

Figure 5: Objective Function vs Number of LSPs

20	40	60	80

Number of LSPs

Figure 6: Maximum Utilisation vs Number of LSPs

100

M

axim

um

Ut

ilisa

tio

n

Objec

tiv

e F

unc

tion (s

ec

)

identify the routers that might be first upgraded to MPLS

operations as those which will have most impact on network

performance.

Our results also indicate that it may often not be necessary

to ever upgrade all routers in a network to MPLS capability. In

highly asymmetric networks, a small number of MPLS routed

traffic flows implemented with a small number of "core"

routers can produce network performance approaching a fully

MPLS enabled network.

Note that the previous published approach by Wang and

Zhang [10, 11] relies heavily on adjusting the link weight

metric and on MPLS arbitrary traffic splitting ratio

assumption, both of which are not realistic. Our method, on

the other hand, is not based on changing IGP metric, and does

not assume arbitrary traffic splitting ratio, and therefore is

more practical.

For further work, we will investigate a range of

representative network topologies and traffic models, as well

as other algorithms for optimizing the tunnel placement, to

minimize the number of MPLS routers and to allow for the

placement of backup tunnels in case of network failures. The

tunnel placement module will also consider the re-

optimization of the existing tunnels when the traffic demands

vary dramatically from the existing values.

ACKNOWLEDGMENT

We would like to thank A/Prof. Greg Allen and Dr. Bruce

Litow for their helpful discussions and comments. We also

would like to thank the reviewers for their insightful

comments and suggestions. This work has been supported by

James Cook University through an International Post-

Graduate Research Scholarship.

REFERENCES

[1]  B. Fortz and M. Thorup,  "Internet traffic engineering by optimizing

OSPF weights", in Proc. IEEE INFOCOM 2000, March 2000.

[2]  B. Fortz and M. Thorup, "Optimizing OSPF/IS-IS weights in a changing

world", IEEE Journal on Selected Areas in Communications, Spring,

2002.

337

[3]  B. Davie and Y. Rekhter, "MPLS technology and applications", Morgan

Kaufmann Publishers, 2000.

[4]  D. Awduche et al, "Requirements for traffic engineering over MPLS",

IETF RFC2702.

[5]   E. Rosen et al, " Multiprotocol label switching architecture", IETF

RFC3031, http://ietf.org/rfc/rfc3031.txt, 2001.

[6]   X. Xiao, A. Hannan, B. Bailey, and L. M. Ni, "Traffic engineering with

MPLS in the Internet", IEEE Network magazine, March/April 2000.

[7]  G. Swallow, "MPLS advantages for traffic engineering", IEEE

Communication magazine, December 1999.

[8]   WANDL IP/MPLS View software, http://www.wandl.com

[9]  Opnet Service Provider Guru software, http:// www.opnet.com

[10] Y. Wang and L. Zhang, "A scalable and hybrid IP network traffic

engineering approach", IETF Internet Draft, http://www.ietf.org /

proceedings/ 02mar/ I-D/ draft-wang-te-hybrid-approach-00.txt, June

2001.

[11]  Y. Wang, Z Wang and L. Zhang, "Internet traffic engineering without

full mesh overlaying", Proceeding of INFOCOM, Alaska, April, 2001.

[12] Cisco, "MPLS	traffic engineering	and enhancements"

http://www.cisco.com/ univercd/cc/ td/doc/product/ software/ios121/

121newft/  121t/ 121t3/traffeng.htm

[13]  Cisco, "Troubleshooting load balancing over parallel links using Cisco

express forwarding (CEF)" http://www.cisco.com/ warp/ public/ 105/

loadbal_cef.html

[14] H. Pham, B. Lavery, “On performance objective functions for

optimising routed networks for best QoS”, Proceedings of QoSIP, Italy,

February, 2003.

[15] Y. Wang and Z. Wang, "Explicit routing algorithms for Internet traffic

engineering", Proceedings of ICCCN'99, Boston, September 1999.

[16]  W. Fang, L. Peterson,  “Inter-AS traffic partterns and their implecation”,

proceedings of IEEE Globecom, Brazil, December 1999.

[17] K. Papagiannakietall, “On the feasibility of identifying elephants in

Internet backbone traffic”, Technical report, Sprint labs, 2001,

http://ipmon.sprintlabs.com/pubs-trs/trs/TR01_ATL_110918.pdf

[18] A. Kershenbaum, "Telecommunications network design algorithms",

McGraw-Hill, 1993

[19] N. Spring, R Mahajan, D. Wetherall, "Measuring ISP Topologies with

Rocketfuel", proceedings of Sigcomm, Pittsburgh, August, 2002.

[20] Rocketfuel website, http://www.cs.washington.edu/research/networking/

rocketfel/

[21]  H. Pham, B. Lavery "A New Scalable, Hybrid Approach for IP Traffic

Engineering without Full Mesh Overlaying", proceedings of ICT2003,

Tahiti, February, 2003.