Computer Communications 27 (2004) 751–761

Modeling mobility and workload

www.elsevier.com/locate/comcom

for wireless metropolitan area networksq

Matthias Hollicka,*, Tronje Kropa, Jens Schmitta, Hans-Peter Huthb, Ralf Steinmetza

aMultimedia Communications Lab (KOM), Faculty of Electrical Engineering and Information Technology,

Darmstadt University of Technology, Darmstadt, Germany

bDepartment of Information and Communications, Siemens AG, Corporate Technology, Munich, Germany

Received 23 October 2003; accepted 24 October 2003

Abstract

Research on large-scale wireless metropolitan area networks, which offer broadband capacity while supporting user and terminal mobility

suffers from the lack of realistic mobility and workload models. There is a strong need for such models to be able to perform sound

simulations supporting important yet difﬁcult tasks like network planning and trafﬁc engineering. In this paper, a novel approach towards

realistic modeling of user mobility is proposed and studied. We formulate an analytical model, which is a hybrid of an empirical mobility

model and a synthetic trafﬁc model. The model clearly separates the inﬂuence of mobility and trafﬁc to allow for greater ﬂexibility.

The mobility part is based on the combination of statistical zoning information with ﬁeld data of movement patterns. This allows us to predict

the density of users—classiﬁed into different groups—for a given area at a given time. We are able to integrate different trafﬁc characteristics

on top of our mobility model elegantly. The combination of user density with the predicted—synthetic—trafﬁc of the modeled user groups

gives the trafﬁc and ﬂuctuations of trafﬁc throughout the network, thus describing the workload for the envisioned scenario. We present the

instantiation of our model for the example of a real city. Analysis and simulations are provided which show that the proposed scheme is quite

prospective. Our ﬁndings are, that our model is able to cover the macroscopic effects of real-world behavior more precisely than currently

available mobility/workload models.

q 2003 Elsevier B.V. All rights reserved.

Keywords: Mobile communications; Wireless communication networks; Mobility modeling; Workload modeling; Metropolitan area radio access networks

1. Introduction

With the advent of the Internet, the quality of personal

communications has changed signiﬁcantly. The avail-

ability of a nearly ubiquitous communication platform and

simple yet useful applications like e-mail, news or more

recently instant messaging has attracted millions of users.

Moreover, in a broader sense, the last century was

signiﬁcantly inﬂuenced by a trend towards personal

mobility for the masses. The intersection of these

trends gave birth to the area of mobile and wireless

communication networks.

q

This work is partially funded by Siemens AG, CT IC 2 within the

MobQoS project. More information may be obtained at http://www.kom.

tu-darmstadt.de/Research/MobQoS.

* Corresponding author. Tel.:	ю 49-6151-166158; fax:	ю49-6151-

166152.

E-mail address: matthias.hollick@kom.tu-darmstadt.de (M. Hollick).

0140-3664/$ - see front matter q 2003 Elsevier B.V. All rights reserved.

doi:10.1016/j.comcom.2003.10.015

Today multiple technologies and frameworks compete

for the market to support mobile communications: after the

success of second generation cellular networks, the third

generation speciﬁed within the IMT2000 framework is

currently being deployed. On the other hand, there is a

tremendous effort to build large-scale wireless networks

based on wireless local area network technology—

especially in the context of high-capacity usage. We think

both technologies will proliferate—each one complement-

ing the other for special applications—to fulﬁll both

demands: personal mobility combined with ubiquitous

communication capabilities at optimized trafﬁc rates.

These concepts are the foundation of fourth generation

networks and the wireless Internet, embracing a hetero-

geneous set of radio access technologies, which gives rise to

several interesting research challenges.

Induced	by	user	and	device	mobility—while

tightly coupled to usage and application-speciﬁc trafﬁc

752

M. Hollick et al. / Computer Communications 27 (2004) 751–761

patterns—future wireless metropolitan area networks will

experience heavily varying loads on different timescales.

Our goal is the investigation of resource management

issues to deal with the predicted trafﬁc demand of these

networks. Here, we concentrate on solutions based on QoS

routing mechanisms (see, for example, Refs.	[1 – 3]).

The system boundaries for our analysis are deﬁned by

the MobQoS scenario	[4], which is derived from

various concepts of the Internet communities and tele-

communication industries [5 – 9].

We assume a three-tier architecture as depicted in

Fig. 1(a). The mobile nodes (MN) or terminals are

associated to wireless base stations, the so-called radio

access points (RAP) representing the last hop of the

provision network. The function of the ﬁrst tier thus can

be described as radio access. The second tier, the radio

access network (RAN), comprises radio access servers

(RAS), which are used to attach multiple RAPs. The RAS

are meshed with neighboring systems and thus allow the

start of resource management at this level. The core of the

second tier is built by radio access routers (RAR).

Between selected RAS and RAR there are uplinks.

The transition to the third tier of the architecture, the

core provider network, respectively, the Internet, is

performed by one or more edge gateways (EGW).

This scenario reﬂects the paradigm change from strictly

hierarchical topologies (see	Fig. 1(c)), which are nearly

homogeneous with respect to cell sizes and capacity in

today’s personal communication (PCS) networks, towards

highly variable and meshed topologies including high

bandwidth micro and pico cells. We assume our network

to be a routing network beginning at RAS level as well

(see Fig. 1(b)). Additionally, the set of applications used

within such networks will yield different network trafﬁc as

in today’s cellular networks—from constant bitrate voice

communication towards a more Internet-like heterogeneous

mix of trafﬁc. Hence, we believe that today’s mainly static

resource management approaches need reconsideration.

Our work in the area of QoS routing protocols to allow

for improved resource management concentrates on the

effects induced on the data plane by user mobility.

From these investigations, we derive the need for a more

ﬂexible and realistic mobility model than available in

literature. As we will show, current mobility models suffer

from various limitations: they are mostly restricted by the

call- and connection-oriented paradigm of traditional

cellular networks and lack the necessary degree of freedom

and ﬂexibility to reﬂect the future networks and services we

want to investigate.

Our contribution lies in a novel mobility/workload model

for wireless metropolitan area networks. We present in

detail the fundamentals of our model, which is ﬂexible with

respect to parameterization of the user behavior while

maintaining reasonable complexity. The model introduces a

generic classiﬁcation of locations and user behaviors, which

allow the derivation of the aggregated effects of user

mobility. We present the coupling of our mobility model

with trafﬁc predictions to form a workload model, as well.

We provide the analytical description of the model.

The formalization deﬁnes a toolset, which allows for easy

implementation of our model in real metropolitan

environments. The instantiation of our model is performed

for an area within central Darmstadt, a German city of

approximately 145,000 inhabitants.

The paper is organized as follows. In Section 2, previous

and related work is surveyed. Section 3 describes the

fundamentals of our model. This includes the modeling of

locations and user behavior. Section 4 comprises the

analytical description of our model. We give deﬁnitions as

well as equations to exactly represent the model

assumptions. In Section 5, we present the instantiation of

our model and possible applications for networking

Fig. 1. (a) Scenario for a future wireless metropolitan area network. Conceptional model of: (b) the envisioned topology vs. (c) Traditional second and third

generation topologies. Legend: CN, core network; EGW, edge gateway; GPRS, general packet radio service; MN, mobile node; RAN, radio access network;

RAP, radio access point; RAR, radio access router; RAS, radio access server; RNC, radio network controller; SGSN, serving GPRS support node; GGSN,

gateway GPRS support node.

M. Hollick et al. / Computer Communications 27 (2004) 751–761

753

research. The paper is concluded by summarizing the

main results and discussing aspects of future research in

Section 6.

2. Related work

There are different approaches towards realistic models

for mobile communications. A fairly comprehensive

survey on mobility modeling in wireless networks can

be found in Ref.	[10], a more detailed description of

some random models in Ref. [11]. There is, however, no

common scheme to categorize all dimensions of these

models because they are so manifold. Within our work,

we use a broad categorization into two classes:

microscopic	vs.	macroscopic	mobility models, since

these two concepts cover the essential distinction with

respect to our investigation.

†	Microscopic mobility models describe the mobility

behavior of individuals. They are often based on

analytical	descriptions	of	user	movements.

Most synthetic or random models fall into this class.

†	Macroscopic mobility models describe the aggregated

effects of mobility. They are often obtained using

statistical data collection and thus inherently aggregate

the behavior of individuals.

Within our context, we are interested to realistically

model mobility at the scale of a metropolitan area. Given this

viewpoint, we reviewed the following related work.

2.1. Microscopic mobility models

The class of random models is the most prominent

example for microscopic models. Random models describe

the movements of individuals. There are numerous

models including the random walk model, the random

gauss – markov model, the random mobility model, the

markovian model, and the random waypoint model	[11].

These models try to provide an exact analytical description

of the behavior of individuals. Random models have been

optimized for accuracy,	[10]	being one example.

The characteristics of a generalized random model for

cellular networks with respect to mobility patterns are

studied in Ref. [12]. However, these models are not able to

reﬂect macroscopic behavior and often exhibit undesirable

properties if applied improperly [13].

Random models may operate on other levels to reﬂect

group behavior as well [14]. However, the results, neither of

individual mobility nor of group mobility do account for a

city at large. This is mainly due to the mostly homogeneous

nature of random models if aggregated thus leading to fairly

equal distributions for large areas. If we, however, regard a

real city, we observe numerous attractions and hot spots.

These are likely to be crowded during short periods of time

or over day while being empty during the night. It is,

from our perspective, extremely unlikely, that pure random

behavior ﬁts such a scenario. There are some random

models, which can be widely parameterized to match

correct macroscopic behavior as well	[15]. The correct

instantiation for these depends on macroscopic models,

nonetheless. Since the microscopic models violate our

assumption of realism at metropolitan scale, we do not

further investigate this class of models.

2.2. Macroscopic mobility models

Macroscopic mobility models try to match the

aggregated user mobility within large areas. Models from

transportation planning are often used as a basis for

large-scale mobility models. TRANSIMS	[16]	is an

example of a complex synthetical model. It creates a virtual

region with complete representation of individuals and their

activities. The transportation is modeled and trips are

planned to satisfy the individuals’ activity patterns.

The behavior of individuals and the geometrical distribution

of households is derived from census data. The focus of

TRANSIMS is transportation planning. It would be possible

to extend the framework to generate data that ﬁts the needs

of teletrafﬁc modeling for wireless networks. However, this

comes at the expense of a very complex overall framework

which for example tracks the movements of all individuals

and requires very hard calculations and extensive

parameterization.

The comprehensive work of Lam et al. [17] presents and

evaluates teletrafﬁc models for metropolitan, national and

worldwide scale. The work is based on results from

transportation planning. The authors have been able to

show that the model predictions can be validated using real

world data. The focus lies—like in most work in the area of

wireless teletrafﬁc modeling—on the investigation of

control plane issues like location management trafﬁc.

Therefore, the model does not include a distinction into

multiple user types, effectively rendering it unusable for

most parts of our intended model usage, which is coupled to

data trafﬁc.

A general work in the area of teletrafﬁc models for

urban environments can be found in Ref. [18]. The author

investigates various types of cell structures and especially

the inﬂuence of variation in the size of cells. Findings of

Nanda [18] include the relation between handoff rate and

cell size as well as between handoff rate and cell shapes.

Despite the excellent results in the areas investigated, the

work cannot be applied to our scenario, since it does not

provide the necessary detail and ﬂexibility of the user

model. Ref.	[19]	limits its scope on the location

management aspects of user mobility. However, the

underlying mobility model presented makes uses of more

detailed information of user behavior and activities,

which are not further exploited. Moreover, the work

borrows the concept of trips to describe the intention

754

M. Hollick et al. / Computer Communications 27 (2004) 751–761

and whereabouts of users from work in the area of

transportation modeling.

The work of Rocha et al. [20] uses a mobility model

similar to the one in Ref. [19] and implements a graphical

tool to support analysis of the results. The instantiation of

areas and user behaviors is, however, very coarse and does

not reﬂect realistic environments.

Ref. [21] describes a trafﬁc model for “third generation

cellular mobile telecommunication systems”. Part of this

model is a user mobility model described in Ref.	[22],

which is based on the average distance and average

velocity of users. These parameters are estimated for

certain environments including multiple classes of out-

door environments and different vehicles. The work is

limited to the estimation of cell border crosses to predict

handoff rates and call durations. While in Ref.	[21]	the

geographical instantiation models a manhattan grid model

of a city center using relatively coarse characteristics for

zoning information, Ref.	[22]	presents background

information on mobility modeling and a comparison to

classic area zone models for teletrafﬁc modeling including

Ref.	[23]. Other work in this area includes Refs.	[24,25],

which address additional aspects of area zone models and

Ref.	[26], which describes a novel stop-or-move mobility

model to address some shortcomings of purely random

models.

Another group of models for large-scale networks relies

on the analysis of existing infrastructures and network

traces. In particular, work which investigates large-scale

wireless radio access topologies for data trafﬁc with respect

to user mobility has been proposed only recently. The work

of Tang and Baker [27] is able to provide deep insights on

user behavior for a metropolitan area wireless network.

The work of Kotz and Essien [28] claims to be the largest

and most comprehensive real world trace of a production

Wireless LAN. The results, however, do account for a

special campus style network and mainly focus on trafﬁc

analysis—the mobility aspect is restricted by the campus

setup and thus cannot be transferred to public networks.

Balachandran et al.	[29]	concentrate on network

performance of small-scale networks, which are not

representative for the metropolitan scale.

There are two critical points about trace-based

methodologies. First, they only account for services already

deployed and in use and particularly only count ‘early

adopters’. This is especially true if we regard wireless RANs

for data communication at the time of writing. Second, the

results often cannot be separated into mobility and trafﬁc

related parts, thus prohibiting the parametrization of

individual factors for simulation.

2.3. Conclusions

While current approaches including Refs. [17,18] focus

on handoff rates and other parameters coupled to the number

of handovers and user numbers within given cells, they do

not differentiate between different classes of users and trafﬁc

demands. The activity-based mobility models described in

Refs.	[19,20,22]	provide for some basic mechanisms,

which may be used for our purpose. The formulation of

these models and their instantiation is, however, not

optimized for data trafﬁc analysis but for classical teletrafﬁc

applications.

In summary, random models do not account for mobility

within large areas and the existing macroscopic models are

either too restricted for our purpose or too complex for the

intended application. The empirical studies of wireless

LANs are limited with respect to the strict decoupling of

mobility and trafﬁc. Thus, we decided to develop a novel

model to ﬁt the needs of the described scenario. We borrow

concepts from transportation planning to realistically

cover macroscopic areas, such as, for example, a center of

a large city.

3. Fundamentals

Our model is developed in the context of transportation

and land use modeling	[30,31]. We use the well-known

travel demand modeling approach described in Ref.	[30]

and also used in previous work including Refs. [19,20,22].

The basic elements of activity based transport modeling

are the trip, which deﬁnes the movement of a user from an

origin to a destination, and the zone, which deﬁnes areas

with a certain attraction level. Trips are based on the

intended behavior of users while zones represent

homogeneous areas with respect to socio-economic charac-

teristics. The size of zones can range from a few hundred

square meters to several square kilometers in size.

This basic model has been used since the 1960s to

forecast travel demand and various techniques and

submodels have been developed for determining the

variables for each of the elements of travel demand

modeling. It is important to keep in mind that the developed

model represents aggregated information of users and zones

only. Thus the model is well-suited for investigations

covering macroscopic effects.

3.1. Trip generation

The trip generation estimates the total number of trips

that depart and arrive in a speciﬁc zone. In particular,

trips are classiﬁed by trip purpose, such as work or

shopping, for example. The purpose is derived from the

intended user behavior. The corresponding trip is scheduled

to a location where this intention can be fulﬁlled, like, for

example, a workplace or shopping mall. The generated

trips need to be distributed to the destination zones

according to the attraction of these zones, which is

determined by socio-economic characteristics. Moreover,

a modal split accounts for different transportation modes

such as car, bike or subway. According to a trip assignment

M. Hollick et al. / Computer Communications 27 (2004) 751–761

the individual trips are mapped to the transportation	3.3. Classiﬁcation of users

infrastructure and the best route is calculated.

755

We are primarily interested in the user densities for given

zones. This allows us to omit the determination of modal

splits, trip assignment, and route calculation, which reduces

the model complexity signiﬁcantly. To allow for the

remaining trip generation and distribution, our model

needs	to	classify	locations	and	user	behavior.

Upon instantiation, real world data needs to be ﬁtted to

match this classiﬁcation scheme. Finally, we are able to

calculate the user densities for given zones.

3.2. Classiﬁcation of locations

The classiﬁcation of locations has to reﬂect their

different attraction levels over time. A reasonable

granularity can be reached by assigning a base attraction

level to a zone. The base attraction level is set depending

on basic characteristics of the location. Residential areas

and	workplaces	(we distinguish into industrial and ofﬁce

workplaces) account for the major ﬂuctuation of users.

Moreover, the attraction of	commercial,	recreation,

and	education	facilities	is	an	important	factor.

These types of locations are further reﬁned as follows.

Commercial locations include shops and shopping malls;

recreation includes theatres and opera, museums, cinema,

sport events, bars, and pubs. Education places can be

further classiﬁed into schools and universities. A special

location called	transport	accounts for users on the move.

We assume zones of reasonable size, like those given in

most zoning plans for city development. Moreover, the

zones should be homogeneous with respect to the

attraction of the different locations within each zone,

which is usually also true for public zoning information.

See	Fig. 2(a)	for a detailed classiﬁcation of locations.

Please note that the classiﬁcation is non-exhaustive to

keep the level of complexity reasonable.

The accuracy of the approach may be increased if extra

information about special places such as shopping malls,

schools, universities, and sights possibly located in the zone

is combined with the base attraction. This reﬁnement comes

at the expense of insider knowledge being necessary,

though.

The most important criterion for the user classiﬁcation is

to adequately characterize the behavior of types of

individuals. The intentions of the resulting groups need to

match the attracting locations as well. The availability of

information about the constitution and behavior of the

derived groups to allow for proper instantiation is another

crucial factor. This includes information to allow for case

discrimination, in particular to predict state transition or

inactivity of users. Resulting from this information, we are

able to predict the aggregated impact of user mobility.

We differentiate the following types of users (roles):

residents,	workers,	consumers,	trainees, and	travelers.

Residents are the main group of interest during evening

and at night. We distinguish between inactive residents

(sleeping at home) and active ones (being at home). The role

of a traveler accounts for the state transition between other

states. For example, if a resident departs for a shopping

district, he becomes to a traveler before ﬁnally taking the

role of a consumer.

We discriminate consumers into buyers and visitors in

order to separate shopping activity from leisure. The latter

ones have to be distinguished into daily and nightly

visitors (idlers) to model the different locations of

recreation facilities as for example museums, swim-hall,

and shopping during day or cinemas, theatres, and pubs in

the evening. The numbers for daytime visitors includes

tourists as well.

The number of trainees, namely pupils and students,

usually cannot be neglected, nor can the number of workers.

The class of workers is divided into different sub-classes

which account for different zones in which they perform

their job (commercial zone, industrial zone, etc.) and for

expected differing professional communication behavior

(e.g. mobile personnel use the communication network to

carry out their job while an ofﬁce or industrial worker

communicates in breaks or on his way to/from work).

Besides their function as intermediate state, the travelers

represent the number of people just crossing the investigated

area by car, commuting to external work places or from

external housings to internal workplaces/universities.

See Fig. 2(b) for the classiﬁcation of user behavior.

Fig. 2. (a) Classiﬁcation of locations; (b) classiﬁcation of user behaviors.

756

4. Model

M. Hollick et al. / Computer Communications 27 (2004) 751–761

We use a zone- and behavior-dependent split factor fbaрzЮ to

account for this property. With fbaрzЮ we are able to calculate

Our model distinguishes between user and trafﬁc related

issues. It is based on the classiﬁcation of zones, cells and

locations (see Section 3.2) and the classiﬁcation of users and

the number of users uz;bрtЮ within each zone.

uz;bрtЮ ј UbрtЮfbaрzЮ

р4Ю

behaviors (see Section 3.3). Using these prerequisites user

mobility can be modeled as follows. Let

Ar ј {residential,	workplace,	commercial,	recreation,

education, transport} denote the set of types of locations.

B ј {resident,	worker,	consumer,	trainee,	traveller,

inactive} denotes the set of behaviors.

Z ј {z1; z2; …; zn} denotes the set of zones. Zones do not

overlap in space. The union of all zones zi;	i ј 1; …; n

covers the whole area of interest.

To allow for a mapping of locations to a cell-based

network infrastructure let

To convert the number of users into the corresponding

activity, we introduce the notion of intensity	ib;	which

models the fraction of time a user dedicates to communi-

cation purposes. Activity thus denotes the sustained number

of communicating users. For the sake of simplicity, ibmay

be combined with the trafﬁc-part of a workload model and

neglected in the mobility part. For maximum ﬂexibility of

the mobility model, it would be possible to introduce a time

and zone dependent version of ib; ibрz; tЮ: The activity is

az;bрtЮ ј uz;bрtЮibј UbрtЮfbaрzЮibј UрtЮfbuрtЮfbaрzЮibр5Ю

withPz[ZfbaрzЮ ј 1:

We obtain

C ј {c1; c2; …; cm} denote a set of cells:

The model should predict the number of users with

behavior b; which can be expected in a zone/cell at given

AbрtЮ ј

X

z[Z

az;bрtЮ ј

X

z[Z

uz;bрtЮib:

р6Ю

time t: Let

UbрtЮ ј the total number of users with behavior b at time

t: uz;bрtЮ denotes the time-dependent number of users with

Moreover,

X

AрtЮ ј

b[B

AbрtЮ:

р7Ю

behavior b within zone z:

The next step is the transformation of the results from

UbрtЮ ј

X

z[Z

uz;bрtЮ and UрtЮ ј

X

b[B

UbрtЮ

р1Ю

zone to cell level. We use a network model similar to

Ref. [24]. The shape of cells is hexagonal. We assume that p

cells are arranged in р2r 2 1Ю columns. Fig. 3(a) shows that

UрtЮ might vary over time. During night, most users will

be inactive, for example. They change their role to active

residents after getting up and eventually to a worker,

consumer, or trainee. In between they are modeled as

travelers. To account for the fraction of users being active in

a role b, we deﬁne a time-dependent split factor per behavior

fbuрtЮ: In reality, the maximum number of users in the system

is variable, because commuters and travelers may enter and

leave the system over day. To simplify calculations, we

assume a ﬁxed maximum number of users U	including all

we obtain	r	columns of	k	cells and	рr 2 1Ю	columns of

рk 2 1Ю cells. The total number of cells can be calculated as

p ј rk ю рr 2 1Юрk 2 1Ю: This cellular structure is overlaid

on the zone-based area of investigation.

To be able to transform the results from zone to cell

granularity, let szbe the size of zone z and scbe the size of

cell	c:	Let	rc;z	denote the fraction of zone	z;	which is

covered by cell	c:	Assuming an equal distribution of

attraction levels within zones, we are now able to transform

the results using

X	X

users possibly entering the system. If not present in the

system, these users are modeled as inactive residents, which

do not further contribute to the performed calculations.

ac;bрtЮ ј

z[Z

az;bрtЮ

sz

scrz;c;with

z[Z

rz;cј1:

р8Ю

Please note, that fbuрtЮ is independent from the location.

UbрtЮ

Using Eqs. (5) and (8) we can calculate the individual

ac;bрtЮ; which forms the user activity matrix Ac;bрtЮ:

fbuрtЮ ј

U

; withXfbuрtЮ ј 1 for all t

b[B

р2Ю

2

66

a1;residentрtЮ	a1;workerрtЮ	· · · a1;inactiveрtЮ

a2;residentрtЮ	· · ·	· · ·	· · ·

3

77

Using fbuрtЮ	we calculate

UbрtЮ ј UfbuрtЮ:

р3Ю

66

Ac;bрtЮ ј 66

64

...

...

...

...

77

77

75

р9Ю

AbрtЮ	denotes the activity of users (that is, they emit

trafﬁc) with behavior b at time t: az;bрtЮ denotes the time-

ap;residentрtЮ

· · ·

· · · ap;inactiveрtЮ

dependent activity of users with behavior b in zone z:

The user population	UрtЮ	needs to be assigned

corresponding to the	attraction	of locations within zones.

Please note, that the model described above can easily be

extended to reﬂect more details of user behavior or location

attributes. For example, it would be possible to model

M. Hollick et al. / Computer Communications 27 (2004) 751–761

757

Fig. 3. (a) Network system model; (b) excerpt of the instantiation for Darmstadt, including the modeled cell vs. zone size and comparison to the coverage of the

base stations of an actually deployed GSM network.

the time varying aspects of user activity by introducing

fbaрz; tЮ: Another extension we do not describe within this

paper is the adaptation of	fbuрtЮ	to account for additional

inﬂuences as for example workers being on sick leave, etc.

The user activity matrix Ac;bрtЮ	can be separated into time

and location dependent factors. We are particularly

interested in what we call the user activity and user density

charts, which can be combined with a time-dependent trafﬁc

rate to form the workload for a wireless network. The user

activity chart represents the number of active users per

time-interval while the user density chart denotes the

number of users for a given cell.

Since we use discrete cells and time-intervals during

instantiation, we obtain the user activity chart as

X

5. Instantiation and application of the model

In this section, we give a brief introduction of the

instantiation of the model. A detailed description of

the instantiation for the city of Darmstadt, Germany and

the resulting mobility and workload matrices can be found

in Ref. [32].

5.1. Instantiation of the model

Modeling of locations is based on zoning information

usually found in zoning plans for city development.

The important property of zoning information is that the

zones describe nearly homogeneous areas with respect to

our location criterion. Most importantly, public data as

AbрtЮ ј

c[C

ac;bрtЮ	for all time-intervals:

р10Ю

well as census data usually applies to the level of

granularity of zones. This gives exact information

The user density chart can be represented using

of numbers of work places, residents, etc. The granularity

of data available suited our model nicely in most parts.

Uc;bрtЮ ј UbрtЮfca;b	with	fca;bј

X

z[Z

fza;b

sz

scrz;c:

р11Ю

In particular, public information included the number of

residents with main and second address. Residents are

additionally indexed by age (which can be used to

Let M ј {m1; m2; …; mi} denote a set of trafﬁc classes

and rbmрtЮ	be the time-dependent trafﬁc rate of users with

behavior b in trafﬁc class m: We are able to augment trafﬁc

estimates for different classes of users to form a workload

matrix WcmрtЮ for each trafﬁc class m:

X

WcmрtЮ ј	ac;bрtЮrbm	р12Ю

b[B

Using Eq. (12) we are able to calculate the daily

workload as well as the time varying loads for each trafﬁc

class m: The resulting trafﬁc accounts for the trafﬁc initiated

from speciﬁc cells. The distribution of the destinations of

this trafﬁc can be performed as appropriate.

classify pupils and students) and social state (working/not

working). The detailed information about workplaces was

available for all zones, too.

For the classiﬁcation of user behavior, we needed

more precise information than available solely using

census data. While the distribution of users into the

proposed classes can be achieved using public census

records, we needed further information to characterize

the time-dependent nature of trips. Fortunately, these

shortcomings can be addressed using public time budget

studies. The ones available for Germany provided by the

German Federal Statistic Ofﬁce are of extremely

ﬁne-grained level of detail, though. Therefore, we used

758

M. Hollick et al. / Computer Communications 27 (2004) 751–761

Fig. 4. (a) Summarized number of active users over time; (b) summarized workload over day (in kB/s).

secondary sources, which already interpreted these

studies. In addition, we have been able to access

trajectories of personal activities for various German

cities. These have been available upon request at the

municipality of Darmstadt for nonproﬁt use.

Using the above-mentioned sources, we have been able

to instantiate our model for Darmstadt, a German city of

around 145,000 inhabitants. We needed to perform

signiﬁcant post-processing. The zones available in the

zoning plan are for example of arbitrary shape and size

and needed to be manually processed to ﬁt the proposed

cell structure. Moreover, special locations have been

modeled using our intimate knowledge of the city.

Gathering data for these special places proved to be very

labor intensive.

Some graphical results of the instantiation are given

below.	Fig. 3(b) shows the zoning plan of the modeled

area—the city center of Darmstadt. The arbitrary shape of

some zones (black) from census data is drawn alongside

with the cells (black hexagons) of our model and the base

stations of a currently deployed GSM network (white circles;

we use an approximated cell radius of 1.2 km to depict the

cell size).

A user activity chart for all users and user groups is

depicted in Fig. 4(a). It shows the summarized number of

active users over the day being modeled. We see low user

activity in the early morning. Starting from 4:00 h in the

morning, the number of active users increases as people get

ready for work or are on their way to work. From 8:00 h on

most people are up and active. We have observed that the

geographical distribution of user density changes

signiﬁcantly over day, due to the different activities pursued

by the different users. This shift in density can be seen

especially	from	residential	areas	to	workplaces.

The decrease in the evening is not as abrupt as the increase

seen in the morning. Nevertheless, the focus is changed

from work to residential areas.

The combination of the user activity with the prediction

of the trafﬁc caused by this results in the summarized

workload which is illustrated in Fig. 4(b). Because users in

different classes produce different load, the peak trafﬁc

value does not correspond directly with the peak value of

user activity.

As we noted earlier, our trafﬁc distribution is dependent

on the geographical location.	Fig. 5	presents the

corresponding density chart of user activity at cell level.

The depicted values account for the projected aggregated

user activity for all active users from 12:00 to 13:00 h.

The activity is given in user-hours. The city center of

Darmstadt consists of ofﬁce and shopping facilities and

large parts of the university while the region in the east is

mainly covered by industrial areas. It is clearly visible, that

the city center and the nearby university attract most of the

users.	Fig. 6	shows the density of active residents for

the time period from 20:00 to 21:00 h. At that time, the main

residential areas are clearly visible in a belt surrounding

Fig. 5. Density of active users, 12:00 – 13:00 h (in user hours).

M. Hollick et al. / Computer Communications 27 (2004) 751–761

Fig. 6. Density of active residents, 20:00 – 21:00 h (in user hours).	Fig. 8. Density of ofﬁce workers over 24 h (in user hours).

the center of the city and in the west of the city. The center	5.2. Application of the model

itself is only populated with few residents at the time of the

759

presented snapshot.

Investigation of the busy hour of the network for all

individual cells reveals, that within approximately 20% of

the area we expect roughly 50% of the active users

causing 44% of the total trafﬁc. Within approximately

50% of the area, we expect 78% of the active users

causing 76% of the total trafﬁc. This clearly backs our

initial assumption that the heterogeneity within fourth

generation networks demands for adequate access

technology like, for example, hot spots to provide for

high capacity within selected areas. An animation of the

time ﬂuctuation of the user, respectively, trafﬁc density

over day can be found at Ref. [4].

Figs. 7 – 9 depict the activity values of some selected

user groups for the complete day. While the consumers

(see Fig. 7) are concentrated in the shopping areas of the

city center, the ofﬁce workers (see Fig. 8) are spread over

the colocated ofﬁce buildings in the same area and the

district south east of the center. The image for the

trainees (see	Fig. 9) shows a different distribution:

Darmstadt University of Technology, which is located

in the city center and Darmstadt University of Applied

Sciences located south east account for most activity

while the pupils are nearly uniformly distributed through-

out the city.

Fig. 7. Density of consumers over 24 h (in user hours).

We used the workload model to perform an experimental

analysis of a set of QoS routing protocols within a

prototypical metropolitan wireless RAN [33]. The goal was

to investigate the load-balancing capability of the routing

strategies studied. The investigation included combinations

out of the sets {static, dynamic}, {single-path, multi-path}

and {distance, delay} algorithms. We used ﬁve different

trafﬁc distributions (external vs. internal trafﬁc) with the four

trafﬁc classes {conversational, streaming, transactional,

background} from Ref. [9]. We built a proprietary simulation

environment which is based on ns-2 [34].

We now present some results of our study. The entire set

of results can be found in Ref. [33]. We compare the delay

of a measurement stream from a residential area to the city

center.	Fig. 10	depicts the scatter-plot of the delay of

individual packets over 24 h for (a) a single-path algorithm

and (b) a multi-path algorithm. Packets, which are dropped,

are marked in gray. The single path algorithm ‘fails’ if the

backbone is under heavy load. The loss increases and the

delay is as high as 21 ms. The rise is relatively abrupt and

the decline is smoother. This is due to the trafﬁc load, which

is similarly shaped. The delay-constraint multi-path algor-

ithm has only short periods of time where the increase in

delay and loss is considerable, albeit a few packets reach a

delay of 33 ms due to the choice of prolonged paths.

Fig. 9. Density of trainees over 24 h (in user hours).

760

M. Hollick et al. / Computer Communications 27 (2004) 751–761

Fig. 10. Delay (in ms) of individual packets in a measurement ﬂow from a residential area to the city center: (a) shortest path algorithm; (b) delay constraint

multi-path algorithm.

6. Conclusions

We presented a novel approach towards a mobility model

based on statistical data collection covering the macroscopic

effects observed within metropolitan areas. After surveying

the current state of the art approaches towards mobility

modeling, we formulated a novel model to ﬁt the special

needs of metropolitan areas in the context of heterogeneous

wireless RANs.

Our model borrows from models used for transportation

planning while emphasizing the aspects needed for

investigations on trafﬁc related issues of wireless

communication networks. The fundamentals of this

model, the classiﬁcation of the locations as well as the

user behavior, have been described in detail.

Based on the above mentioned concepts, we derived and

explained the analytical description of our model. We gave

detailed deﬁnitions and equations to capture the model

prerequisites and to allow for easy instantiation.

Moreover, we gave information necessary to augment the

results of the mobility model with trafﬁc predictions to form

a workload model for wireless metropolitan area networks.

A transformation of the results to ﬁt a cell-based networking

infrastructure was presented, too.

In related work, we applied our model to serve as

workload for a comparative analysis of quality of service

routing strategies in wireless metropolitan area networks.

The results of the investigation are based on the instantiation

of our model for Darmstadt	[32]	and can be found in

Ref. [33]. Lessons learned with respect to the instantiation

of our model include that data which was not intended to

serve as a basis for network trafﬁc analysis needs signiﬁcant

post-processing, depending on the granularity of data

available.

We perceive the combination of our approach with

microscopic synthetic models as a further interesting

research direction. One possible application would be the

modeling of large-scale metropolitan ad hoc networks.

The major advantage of such a model being the realism for

both, microscopic and macroscopic analysis.

Acknowledgements

We acknowledge the help of the statistics department at

the municipality of Darmstadt, City of Science. Many thanks

go to Nicole Karsten-Berier and Joachim Sokol for initial

stimulating discussion.

References

[1] Z. Wang, J. Crowcroft, Quality-of-service routing for supporting

multimedia applications, IEEE Journal of Selected Areas in

Communications 14 (7) (1996) 1228 – 1234.

[2] S. Chen, K. Nahrstedt, An overview of quality-of-service routing for

the next generation high-speed networks: problems and solutions,

IEEE Network Magazine, Special Issue on Transmission and

Distribution of Digital Video 12 (6) (1998) 64 – 79.

[3] J.L. Sobrinho, Algebra and algorithms for QoS path computation and

hop-by-hop routing in the Internet, In Proceedings of the 20th Annual

Joint Conference of the IEEE Computer and Communications

Societies (INFOCOM)’01, Anchorage, AK, USA, vol. 2, April

2001, pp. 727 – 735.

[4] M. Hollick, J. Schmitt, H.-P. Huth, J. Sokol, MobQoS-Mobility

Aware QoS Routing, August 2003, Available at http://www.kom.

tu-darmstadt.de/Research/MobQoS/

M. Hollick et al. / Computer Communications 27 (2004) 751–761

761

[5] C. Perkins, IP Mobility Support for IPv4, Proposed Standard RFC

3344, August 2002.

[6] D.B. Johnson, C.E. Perkins, J. Arkko, IP Mobility Support in IPv6,

Internet Draft draft-ietf-mobileip-ipv6-19.txt, October 2002.

[7] G. Huston, Next Steps for the IP QoS Architecture, Informational

RFC 2290, November 2000.

[8] D.O. Awduche, A. Chiu, A. Elwalid, I. Widjaja, X. Xiao, Overview

and Principles of Internet Trafﬁc Engineering, Informational RFC

3272, May 2002.

[9] 3GPP, QoS Concept and Architecture TS 23.107, Release 5, version

5.9.0, June 2003.

[10] C. Bettstetter, Mobility modeling in wireless networks: categoriz-

ation, smooth movement, and border effects, ACM SIGMOBILE

Mobile Computing and Communications Review 5 (3) (2001)

55 – 66.

[11] T. Camp, J. Boleng, V. Davies, A survey of mobility models for ad

hoc network research, Wireless Communications and Mobile

Computing (WCMC): Special issue on Mobile Ad Hoc Networking:

Research, Trends and Applications 2 (5) (2002) 483 – 502.

[12] M.M. Zonoozi, P. Dassanayake, User mobility modeling and

characterization of mobility patterns, IEEE Journal on Selected

Areas in Communications 15 (7) (1997) 1239 – 1252.

[13] J. Yoon, M. Liu, B. Noble, Random waypoint considered harmful, In

Proceedings of the 22nd Annual Joint Conference of the IEEE

Computer and Communications Societies IEEE INFOCOM, 2003,

San Francisco, CA, USA, April 2003, pp. 1312 – 1321.

[14] X. Hong, M. Gerla, G. Pei, C.-C. Chiang, A group mobility model for

ad hoc wireless networks, In Proceedings of ACM International

Workshop on Modeling Analysis and Simulation of Wireless and

Mobile Systems, MSWiM’99, Seattle, WA, USA, August 1999,

pp. 53 – 60.

[15] C. Bettstetter, H. Hartenstein, X. Pe`rez-Costa, Stochastic properties of

the random waypoint mobility model: epoch length, direction

distribution, and cell change rate, In Proceedings of the ﬁfth ACM

International Workshop on Modeling Analysis and Simulation of

Wireless and Mobile Systems 2002, Atlanta, Georgia, USA, ACM

Press, New York, 2002, pp. 7 – 14.

[16] Los Alamos National Laboratory, TRansportation ANalysis SIMu-

lation System (2003), Available at http://transims.tsasa.lanl.gov/

[17] D. Lam, D.C. Cox, J. Wilson, Teletrafﬁc modeling for personal

communication services, IEEE Communications Magazine 35 (2)

(1997) 79 – 87.

[18] S. Nanda, Teletrafﬁc models for urban and suburban microcells: cell

sizes and handoff rates, IEEE Transactions on Vehicular Technology

42 (4) (1993) 673 – 682.

[19] J. Scourias, T. Kunz, Activity-based mobility modeling: realistic

evaluation of location management schemes for cellular networks, In

Proceedings of Wireless Communications and Networking Con-

ference, WCNC 1999, New Orleans, LA, USA, IEEE, September

1999, pp. 296 – 300.

[20] M.N. Rocha, G.R. Mateus, S.L. da Silva, QoS and simulation models

in mobile communication networks, In Proceedings of ACM

International Workshop on Modeling Analysis and Simulation of

Wireless and Mobile Systems MSWiM 2000, Boston, MA, USA,

August 2000, pp. 119 – 122.

[21] J. Markoulidakis, G. Lyberopoulos, M. Anagnostou, Trafﬁc model for

third generation cellular mobile telecommunication systems, Wireless

Networks 4 (1998) 389 – 400.

[22] J. Markoulidakis, G. Lyberopoulos, D. Tsirkas, E. Sykas, Mobility

modeling in third-generation mobile telecommunications systems,

IEEE Personal Communications 4 (4) (1997) 41 – 56.

[23] D. Hong, S.S. Rappaport, Trafﬁc model and performance analysis for

cellular mobile radio telephone systems with prioritized and non-

prioritized handoff procedures, IEEE Transactions on Vehicular

Technology 35 (3) (1986) 77 – 92.

[24] P. Camarda, G. Schiraldi, F. Talucci, Mobility modeling in cellular

communication networks, In Proceedings of 21st IEEE Conference on

Local Computer Networks, 1996 Minneapolis, MN, USA, IEEE,

October 1996, pp. 518 – 525.

[25] E. Jugl, H. Boche, A new mobility model for performance evaluation

of future mobile communication systems, In Proceedings of IEEE

International Conference on Communications. ICC’99, Vancouver,

BC, Canada, June 1999, pp. 1751 – 1755.

[26] Y.-C. Tseng, L.-W. Chen, M.-H. Yang, J.-J. Wu, A stop-or-move

mobility model for pcs networks and its location-tracking strategies,

Computer Communications 26 (12) (2003) 1288 – 1301.

[27] D. Tang, M. Baker, Analysis of a metropolitan-area wireless network,

Wireless Networks 8 (2002) 107 – 120.

[28] D. Kotz, K. Essien, Analysis of a campus-wide wireless network, In

Proceedings of Eighth Annual International Conference on Mobile

Computing and Networking (ACM MOBICOM)’02, Atlanta, GA,

USA, September 2002, pp. 107 – 118.

[29] A. Balachandran, G. Voelker, P. Bahl, P.V. Rangan, Characterizing

user behavior and network performance in a public wireless LAN,

ACM SIGMETRICS Performance Evaluation Review 30 (1) (2002)

195 – 205.

[30] E. Beimborn, R. Kennedy, W. Schaefer, Inside the Blackbox: Making

Transportation Models Work for Livable Communities, 1996,

Available at http://www.environmentaldefense.org/documents/

1859_InsideBlackBox.pdf

[31] J.-P. Rodrigue, Transport geography on the web, 1998, Available at

http://people.hofstra.edu/geotrans/

[32] M. Hollick, T. Krop, J. Schmitt, H.-P. Huth, R. Steinmetz, A hybrid

workload model for wireless metropolitan area networks, In

Proceedings of 58th IEEE Vehicular Technology Conference,

VTC’03-Fall, Orlando, FL, USA, October 2003.

[33] M. Hollick, T. Krop, J. Schmitt, H.-P. Huth, R. Steinmetz,

Comparative analysis of quality of service routing in wireless

metropolitan area networks, In Proceedings of 28th Annual IEEE

International Conference on Local Computer Networks, LCN’03,

Bonn, Germany, October 2003, pp. 470 – 479.

[34] K. Fall, K. Varadhan. The ns Manual, 2002, Software and manual

available at http://www.isi.edu/nsnam/ns/.