BERDE LAYOUT  6/20/05  10:05 AM  Page 44

GMPLS: THE PROMISE OF THE

NEXT-GENERATION	OPTICAL CONTROL PLANE

Multilayer Traffic Engineering for

GMPLS-Enabled Networks

Martin Vigoureux and Bela Berde, Alcatel Research & Innovation

Loa Andersson, Acreo (Sweden), and Tibor Cinkler, BUTE (Hungary)

Lieven Levrau, Marconi Ondata, and Didier Colle, IMEC

Juan Fdez-Palacios, TID

Monika Jдger, T-Systems

44

ABSTRACT

In recent years, significant work has been com-

pleted on traffic engineering enhancements to

the generalized multiprotocol label switching

protocol suite [1–3]. As a next step, reproducing

the current trend of switching layers’ integration

happening in the data plane, network control is

foreseen to go beyond the traditional per layer

approach and tend toward an integrated model

[4, 5]. In these multilayer environments, a single

GMPLS control plane drives various distinct

switching layers at the same time and as a coher-

ent whole, taking benefit from the “common”

property of GMPLS. Beyond this application of

supporting network control across different tech-

nologies, in this article we catalog the unified

traffic engineering paradigms, discuss their

applicability, and present their enforcement

techniques. Furthermore, we show that the com-

mon GMPLS concept has the advantage of low

operational complexity, and enables unified TE

capabilities such as efficient network resource

usage and rapid service provisioning.

INTRODUCTION

Generalized multiprotocol label switching

(GMPLS) will enable unified traffic engineering

(TE) for multilayer networks; that is, the same

set of tools and the same paradigms are used for

TE on all network layers. Current practice

among incumbent operators is mostly to perform

TE and network engineering (NE) centralized

and offline. The activities are performed per net-

work technology, independent of similar activi-

ties on other layers. The initial network resource

provisioning and service configuration is derived

from a sophisticated network planning and opti-

mization process.

Network configuration, topology, and

resource status are available in a database that

is continuously updated and used for service

provisioning. As services are being set up and

0163-6804/05/$20.00 © 2005 IEEE

torn down, there is a risk of resources being

fragmented. To alleviate this, activities to free

up resources are necessary and need to be

undertaken carefully in order to avoid service

disruption.

One drawback of the centralized approach

is that it is much slower than actions taken by

a control plane. The management system must

access each node that participates in a path,

and reserve resources and configure connectiv-

ity. Distributed control plane functions allow

t h e   a u t o m a t i o n   o f   t h e s e   p r o c e s s e s   w i t h

reduced delay and increased scalability. The

dynamically switched network is an important

part of the carriers’ vision of future network-

ing since it gives them a way to decrease costs

and improve revenue.

The first stage of deployment has been under

way for some time in networks with packet

switch capable (PSC) devices only. We also see

two other steps being taken, but in different

directions. One step involves deploying networks

with multiple switching capabilities, for example,

PSC and lambda switch capable (LSC), each

controlled by a control plane of its own. The

other is deploying networks with multiple switch-

ing capabilities and one common control plane.

Multiple switching capabilities could be present

in a single node, or there could be just one

switching capability per node.

Packet switching networks and circuit switch-

ing networks are today operated over the same

physical network infrastructure (e.g., wave-

lengths). Frequently the transmission protocol

stacks are disparate; say, synchronous optical

network/synchronous digital hierarchy (SONET/

SDH) or Ethernet. This creates unique opportu-

nities and challenges for novel control plane and

TE enhancements in network architectures with

switching on multiple layers. One objective is

migration from per layer to an integrated mode

of operation. A common control plane instance

replaces the separate control plane instances per

data plane switching layer. The information

IEEE Communications Magazine • July 2005

BERDE LAYOUT  6/20/05  10:05 AM  Page 45

Layer n + 1

LSR

Layer n

LSR	LSR

LSR

LSR

LSR

GMPLS control

plane

Data planes

TE has become an

indispensable

function in many

large networks

because of the high

cost of network

resources and the

n Figure 1. The general case of a network with switching capabilities on two layers.

commercial and

competitive nature

databases created by the common control plane

are also used for unified TE.

GMPLS is a way to integrate the network

control plane and central network management

tasks. The goal is to introduce more dynamic TE

functions based on efficient heuristics and man-

aged TE policies so that resource management

and policies to be applied can be harmonized in

a scalable way for all network layers. One area

where the benefits of this approach are obvious

is in coordination of recovery actions between

different layers; this is hardly possible in non-

GMPLS networks, where the network layers per-

form recovery actions separately. This article is

limited in scope and only covers the area con-

trolled by one GMPLS control plane instance,

for example, one autonomous system (AS). The

global view of the status in the whole network

area and its related TE information from differ-

ent switching layers leads to more efficient

resource utilization and network operation.

Different routes, depending on starting

points, may be used to achieve this goal. Incum-

bent operators with several networking technolo-

gies might start by introducing separate control

planes per layer. Green field operators with a

simpler protocol may choose to go for the uni-

fied model directly.

The rest of the article is organized as follows.

In the next section we present a brief outline of

fundamental TE paradigms. Next, we describe

the strategy we use to improve the efficiency and

robustness of the unified TE features of a

GMPLS control plane for multilayer network

architectures. Last, we present TE enforcement

techniques by means of policy-based manage-

ment (PBM) mechanisms.

TRAFFIC	ENGINEERING CONCEPTS

This section gives an overview of multilayer TE

terms and concepts relevant for the discussion in

this article.

MULTILAYER ENVIRONMENTS

Networks are often partitioned into different

domains (e.g., an AS). In a multilayer environ-

ment such a network partition will have equip-

ment with switching capabilities on multiple

layers. Label switching routers (LSRs) in multi-

layer networks may have different sets of inter-

face switching capabilities (ISC). The example

below and Fig. 1 show a two-layer network, with

switching capabilities on the layers called layer-

(n) and Layer-(n + 1). An LSR could be:

• Layer-(n) switch capable

IEEE Communications Magazine • July 2005

• Layer-(n + 1) switch capable

• Both layer-(n) and layer-(n + 1) switch

capable

In general, there is a mixture of the three

cases, as shown in Fig. 1.

GMPLS control planes in multilayer net-

works controlled by a common GMPLS control

plane are “unified” if the LSRs are either layer-

(n) or layer-(n + 1) switch capable and “inte-

grated” if all LSRs are both layer-(n) and

layer-(n + 1) switch capable. [1].

TRAFFIC ENGINEERING AND

NETWORK ENGINEERING

Network engineering deals, for example, with

design, modeling, and planning of network

resources and capacity (i.e., defining the

resources needed). TE deals, for example, with

efficient and reliable network operations while

simultaneously optimizing network resource uti-

lization and traffic performance in operational

networks (i.e., allocating resources where they

are needed).

There are several TE approaches that deal

with traffic demand variations, traffic perfor-

mance, resource optimization, and failure sce-

narios in a network. TE has become an

indispensable function in many large networks

because of the high cost of network resources,

and the commercial and competitive nature of

modern networking. To achieve the TE goals

these operations should be performed without

any loss of the quality of service (QoS) connec-

tion requirements. The optimization aspects of

TE can be achieved through capacity manage-

ment and traffic management, while traffic per-

formance can be improved through performance

monitoring and a combination of analytical and

empirical methods. Thus, the control dimension

of TE responds at multiple levels of temporal

resolution to network events.

One way to categorize TE is in time-depen-

dent, state-dependent, and event-dependent meth-

ods. Time-dependent TE algorithms are applicable

to predictable traffic variations, such as daytime

peak hours or a shift from business to residential

traffic in the evening. In those cases the QoS

requirements and traffic matrix may be roughly

estimated in advance. State-dependent TE meth-

ods use the current state of the network to adapt

to variations in actual traffic. Constraint-based

routing is one example of state-dependent TE

operations. Event-dependent TE methods are

adaptive and distributed in nature. One example is

the responses to link and node failures.

of modern

networking. To

achieve the TE goals

these operations

should be performed

without any loss of

the QoS connection

requirements.

45

BERDE LAYOUT  6/20/05  10:05 AM  Page 46

To coordinate the TE

capabilities of each

layer in a multilayer

network a key factor

is that the NE is

aware of the

multilayer

characteristics.

Working on longer

time scales, using

knowledge of traffic

trends and variations,

NE is capable of

computing target

(a)

(b)

(c)

network states.

n Figure 2. Virtual topologies: a) one-hop transport LSPs; b) full mess of transport LSPs; c) optimized

mesh of transport LSPs.

TE and NE can be both proactive and reac-	for grooming purposes serves as an abstract or

tive. In the first case the network tries to main-	virtual topology (VT) for the upper layer. This

tain an efficient state. In the latter case actions	type of grooming we consider adaptive grooming.

are only taken when certain inefficiency condi-	Adaptive grooming refers to the case where the

tions are detected (e.g., exceeding a threshold).	upper and lower layers are controlled by a single

In TE these actions may be performed on	control plane instance; a new demand at any

existing connections or apply to new connection	layer has influence on other layers as well (e.g.,

requests. In the former case, connections may be	the routing decision has an effect on multiple

temporarily interrupted or QoS degraded. In the	layers).

latter case, it may take a long time before an	Grooming addresses the TE issue of finding a

undesirable network state is resolved and the	trade-off between the lower cost of switching on

connection is established. In an operational net-	a lower layer and the better resource utilization

work with strong TE requirements both methods	in switching on higher layers. In an integrated

need to be used.	layer 2 over layer 1 network, with network ele-

To coordinate the TE capabilities of each	ments that provide layer 2 and layer 1 switching,

layer in a multilayer network, a key factor is that	there are two extreme cases, and a trade-off

the NE is aware of the multilayer characteristics.	needs to be found:

Working on longer time scales, using knowledge	• The VT matches the physical topology (Fig.

of traffic trends and variations, NE is capable of	2a).

computing target network states. TE, working on	• The VT interconnects the layer 2 switches

shorter time scales, may use theses pre-comput-	in a full mesh (Fig. 2b).

ed network states to achieve a flexible and scal-	The drawback in the first case is a relatively

able resource and traffic optimization.	high number of layer 2 hops, because fiber con-

nectivity generally is a scarce resource. The

46

GROOMING TECHNIQUES

Deployment of multilayer networks brings new

challenges. Switching capabilities on multiple

layers makes it possible to control, operate, and

traffic engineer a network in new ways. TE is no

longer limited to tackling just the traditional

path computation process and setting up the

path the traffic flow will follow. Routing deci-

sions must be made considering all dynamic lay-

ers involved in the transport of the flow,

including at what points cross-layer operations

should be performed. This goal is greatly facili-

tated by the visibility of all layers and the TE

information related to these layers.

Directly linked to the transport of traffic

flows across multiple layers is the grooming

paradigm. To aggregate multiple traffic streams,

sharing a common path, into one or more label

switched paths (LSPs), and later separate them

is referred to as traffic grooming. The set of con-

nections on a given layer dynamically established

drawback in the second case is that layer 1

resources will be wasted.

RESILIENCE CONSIDERATIONS

While TE optimizes the use of resources under

normal operation, resilience mechanisms such as

protection, restoration, reroute, and recovery are

TE mechanisms that are invoked in case of sin-

gle or multiple failures. These mechanisms are

also used to switch traffic back to resources

when they come back up after being repaired.

The resilience techniques defined for single-

layer networks can be used for multilayer net-

works as well [6, 7]. Resilience for multilayer

networks raises issues regarding coordination

between layers during recovery operations. One

of the most critical issues is to decide which

recovery action needs to be taken on which

layer. Following is a nonexhaustive list of the

parameters that play a role in recovery decisions:

• Mode (protection or restoration)

IEEE Communications Magazine • July 2005

BERDE LAYOUT  6/20/05  10:05 AM  Page 47

• Scope (end-to-end or segment-based)

• Scheme (1+1, shared mesh, etc.)

• Risk	(Shared Risk Link Group, SRLG,

diverse)

• Loss of connectivity (from loss of light to

loss of routing adjacency)

Each layer has the capability to detect fail-

ures within that layer and sometimes also trigger

resilience actions. One critical aspect of

resilience actions to be taken into account in a

multilayer network is that they have to be care-

fully coordinated between layers.

APPLICABILITY TO

GMPLS ENVIRONMENTS

GMPLS KEY TE BUILDING BLOCKS

In this section we introduce the key GMPLS TE

building blocks.

TE Link — A TE link is a link between two adja-

cent LSRs that has a set of associated TE capa-

bilities. A TE link can consists of a number of

bundled component TE Links, which can them-

selves recursively comprise bundled TE Links.

Free and allocated link capacities and TE

attributes of the link components are represent-

ed in an aggregated form.

Link Bundling — This is a construct that makes

it possible to group individual TE links together

and shows the available resources in an aggre-

gated way. Link bundling improves the routing

scalability by reducing the amount of informa-

tion to be processed by the control plane. With

link bundling a one-to-one association between a

regular routing adjacency and a TE link is no

longer required. Hence, the number of routing

adjacencies in the network can be kept propor-

tional to the number of control plane adjacen-

cies and is not related to the actual number of

data plane links.

Forwarding Adjacency — TE links have been

extended to nonadjacent devices by introducing

the forwarding adjacency (FA) concept. A

GMPLS-capable node can advertise an LSP as a

TE link into a single routing domain. Such a link

is referred to as an FA, and the corresponding

LSP as an FA-LSP. The routing protocol (Open

Shortest Path first with TE, OSPF-TE, or Inter-

mediate System to Intermediate System with TE,

ISIS-TE) floods the link state information about

FAs just as it floods the information about any

other TE link. This allows other nodes to use

FAs as any other TE link for path computation

purposes. The use of FAs provides a mechanism

for improving bandwidth utilization and enables

aggregating forwarding states. FAs allow the cre-

ation of an LSP hierarchy and thus improve the

scalability of GMPLS-capable control planes.

The TE extensions of the routing protocol

such as OSPF-TE use so-called sub-type length

values (TLVs) for specifying and distributing

information of the TE links. In a multilayer envi-

ronment important TE properties of a TE link

are defined by one or more switching capabilities

associated with it. GMPLS defines control plane

capabilities for packet (PSC), layer 2 (L2SC),

IEEE Communications Magazine • July 2005

time-division multiplexing (TDM-SC), lambda

(LSC), and fiber (FSC) switching capabilities.

The related TE attributes of the switching capa-

bilities are defined, for example, in [3].

Routing and signaling protocols may use the

TE attributes to perform constraint-based path

computation, optimizing the path on other con-

straints than least cost (e.g., resource consump-

tion or available bandwidth).

MULTILAYER	ROUTING AND SIGNALING

This section discusses signaling and routing

issues and two methods of route computation in

a multilayer network.

There are several possible methods for route

computation in multilayer environments, and the

methods could also be combined.

A full view of the state of all layers in the

network and the TE information makes coordi-

nation between layers possible. The coordination

leads to optimal routing, connection establish-

ment, and signaling decisions. It also implies that

the control plane needs to deal with an amount

of information substantially larger than that for

a comparable IP network. One consequence is

that it will be more resource- and time-consum-

ing to run routing algorithms. There are basical-

ly two ways to cope with this, a layered routing

approach and a multilayer approach.

In the first mode, path computation is per-

formed on a per layer basis, and the lower-layer

information is abstracted in a VT the computing

layer can use. In the second mode the knowl-

edge of all layers is used to compute routes.

In the first mode a VT is created by the com-

mon control plane and consists of lower-layer

FA-LSPs between the upper-layer nodes. The

control plane uses the FA-LSPs to establish

upper-layer routes.

As an example, assume that FAs instantiated

by LSC LSPs provide a virtual network topology

to a PSC layer. This topology of FAs provides all

the TE information and enables TE routing at

the PSC layer. An established PSC LSP is com-

posed of links connected to equipment capable

of switching this type of LSP only; that is, the

LSP is routed on a single layer.

TE also plays an important role in multilayer

signaling. GMPLS signaling (and routing) proto-

cols adapt the VT to traffic demands (i.e., FA-

LSPs could be set up or taken down as needed).

When an LSP crosses the boundary from an

upper to a lower layer, it may be nested in or

stitched to a lower-layer FA-LSP. The decisions

to establish or take down an FA-LSP could

make use of thresholds enforced via PBM (see

below) to avoid link state instability.

The second method takes advantage of hav-

ing a complete view of all layers from a topologi-

cal and TE resource standpoint. The route

resulting from path computation will be multi-

layer in nature (i.e., it traverses links of different

switching capabilities). This enables a path to be

set up end to end across the different layers,

where each successive LSP participates in the

transport of the traffic flows.

Consider a PSC and LSC network transport-

ing IP flows; an edge node could decide the

route to traverse [A, B, C] at the PSC layer, then

[C, D, E] at the LSC layer, and finally, [E, F, G]

Each layer has the

capability to detect

failures within that

layer and sometimes

also trigger resilience

actions. One critical

aspect of resilience

actions to be taken

in a multilayer

network is that they

have to be carefully

coordinated

between layers.

47

BERDE LAYOUT  6/20/05  10:05 AM  Page 48

at the PSC layer. For RSVP-TE signaling the

but also dynamic triggering of resources at vari-

Without network

wide policies, there is

the risk that TE

decisions, for

example, on load

balancing, taken

locally by one LSR

may be inconsistent

with decisions taken

by other LSRs.

The PBM empowers

the operator to

coordinate

centralized and

distributed TE

processes.

48

identifiers of the links (used in the example) are

more relevant than the nodes themselves. More

sophisticated schemes may be envisaged (given

more advanced node capabilities), such as impos-

ing a cross-layer point between two routes in the

LSC layer. This example shows the tight rela-

tionship between multilayer routing and multi-

layer signaling. By computing an explicit route

across the different layers, the edge node may

implicitly reques the triggering of LSP establish-

ment in several layers in addition to specifying

their explicit routes.

In the context of establishing multilayer

LSPs between nodes with multiple ISCs or

operating according to the integrated model

raises a specific issue. To avoid unnecessary

resource partitioning, nodes with multiple ISCs

may advertise a single TE link with these multi-

ple ISCs [3]. From that perspective the selec-

tion of a link in the route computation process

needs to explicitly select a specific ISC among

those advertised. Existing signaling protocols

could be extended to disambiguate the indica-

tion of the ISC on multi-ISC links by defining a

new subobject to RSVP-TE.

Taking advantage of both the FA paradigm

and the global multilayer vision, a combination

of the above two methods is expected to be the

practical choice with regards to multilayer rout-

ing and signaling. Routing is dependent on the

network topology and associated link states.

Routing stability may be impaired by frequent

changes in the VT or the status of links in the

VT. Creation and deletion of LSPs may be trig-

gered by adjacent layers or through operational

actions to meet changes in traffic demand.

TE ATTRIBUTES AND INHERITANCE

The different TE attributes introduced above

are expected to play a role in the path compu-

tation process. Path computation typically

relies on running a shortest path algorithm

over a weighted graph. Certain of these TE

attributes could be used to construct a graph

satisfying the requirements of the connection

to establish. For example, TE links not having

enough bandwidth could be pruned from the

TE database and not appear in the graph.

Other TE attributes could be used to assign a

cost to the links of the graph. The TE metric,

or a combination of TE metric and other

attributes, could serve that purpose.

Currently the TE metric is defined on a TE

link basis. In multilayer networks some TE links

may have several switching capabilities. Howev-

er, different switching technologies may have dif-

ferent constraints. While for circuit switching

technologies the objective is most often to groom

traffic as much as possible to optimize resource

usage, in packet technologies load balancing is

sometimes preferred to keep congestion and

blocking probabilities low. When a single TE

link contains several switching capabilities, a sin-

gle TE metric cannot simultaneously tackle both

packet and circuit specifics. This suggests a more

flexible TE metric, allowing a TE metric per

ISC, facilitating layer-specific path computation.

As mentioned previously, FAs enable opti-

mization of preprovisioned or precomputed VTs

ous ISCs. However, challenges arise from rout-

ing an advertisement of FA-LSPs as TE links.

FAs consist of an abstraction of the underlying

layer in which the FA-LSP has been established;

thus, an FA will summarize the TE attributes of

the spanned TE links.

The selection of a given FA link by the path

computation process has implications for both

the transported LSP and the layer in which the

FA-LSP is established. The summarization or

inheritance process should thus appropriately

reflect the lower-layer information of interest

to the upper layer. For example, from a con-

nectivity point of view two FAs are identical

(i.e., they interconnect two upper layer points).

However, one might be two hops and the other

six hops long in the underlying layer. The selec-

tion of one of these FAs and the potential

dynamic triggering of additional bandwidth will

have a clearly different impact on the transport

layer. In addition, different FAs might differ in

terms of delay and/or jitter for a packet layer.

The TE metric could be used to differentiate

these cases.

In multilayer networks the inheritance of pro-

tection and restoration related TE link attributes

must also be considered. However, applying sim-

ple attribute inheritance might not be sufficient to

distinguish different recovery schemes. It is impos-

sible to differentiate a 1:1 end-to-end protected

LSP advertised as a single FA together with its

backup LSP from a single FA-LSP routed over

links providing 1:1 protection. A link protection

type of 1:1 would be used for the two correspond-

ing TE link advertisements. An upper layer would

thus have no means to differentiate the two cases.

However, these two recovery schemes (end-to-

end and span) have major differences in terms of

recovery delay and robustness.

ENFORCING TE THROUGH

POLICY-BASED MANAGEMENT

A policy is a set of rules guiding the LSRs in

making their local TE decisions. The TE policies

are set by the operator and scope the degrees of

freedom of the LSRs, and are intended to tune

the overall network behavior according to the

operational goals. Without network-wide poli-

cies, there is the risk that TE decisions, for

example, on load balancing made locally by one

LSR may be inconsistent with decisions made by

other LSRs. The PBM empowers the operator

to coordinate centralized and distributed TE

processes.

TE policies guide both signaling and routing

in a GMPLS network. In Fig. 3 the management

abstraction and hardware abstraction layers hide

the internal implementation details of the

respective interfaces toward the network man-

agement and hardware. To name only a few,

examples of TE policies for routing are:

• Whether or not to advertise a given LSP as

a TE link, enabling the creation of VTs

• Criteria for bundling taking into account,

for example, SRLG information for facili-

tating risk disjoint path computation over

abstract topologies

Examples of TE policies related to signaling are:

IEEE Communications Magazine • July 2005

BERDE LAYOUT  6/20/05  10:05 AM  Page 49

• Parameters involved in the setup of the

working and protection LSPs

• Stitching (dedicated path segments) and

nesting operations (shared path segments)

in case of loose hop

• Strategies for triggering LSP setup

• Parameters involved in the recovery pro-

cess, such as failure notification and revert-

ing recovery operations

Policy enforcement point

Management abstraction layer

The PBM enforces conditions on an inherent-

ly distributed and completely autonomous

GMPLS TE environment [8].

CONCLUSION

Signaling

controller

TE controller

Routing

controller

This article combines concepts from three areas of

research. First, we show that the unified TE

paradigm improves the accuracy of TE decisions in

the multilayer network. It widens the applicability

of well-known TE techniques to include different

switching technologies. It enables network resource

optimization across multiple network layers. Sec-

(TE database and label/resource

management)

Hardware abstraction layer

Path computation

based on CSPF

ond, the unified TE is based on an extensible

GMPLS protocol framework, and third, the PBM

can be used to improve the accuracy of the TE

operations through the use of operator policies.

Unified TE provides an alternative means for

operators who need to reduce complexity and

improve the operational and cost efficiency of

their multilayer networks.

ACKNOWLEDGMENT

This work was carried out in collaboration

between partners of the IST project NOBEL

funded by the European Commission.

REFERENCES

[1] E. Mannie, Ed., “Generalized Multi-Protocol Label

Switching (GMPLS) Architecture,” RFC 3945, Oct. 2004.

[2] D. Katz et al., “Traffic Engineering (TE) Extensions to

OSPF Version 2,” RFC 3630, Sept. 2003.

[3] K. Kompella et al., “OSPF Extensions in Support of General-

ized Multi-Protocol Label Switching,” draft-ietf-ccamp-ospf-

gmpls-extensions-12.txt, Oct. 2003, work in progress.

[4] K. Shiomoto et al., “Requirements for GMPLS-Based Multi-

Region and Multilayer Networks,” draft-shiomoto-ccamp-

gmpls-mrn-reqs-00.txt, Oct. 2004, work in progress.

[5] E. Dotaro et al., “Multi-Region Networks: Generalized

Multi-Protocol Label Switching (GMPLS) as Enabler for

Vertical Integration,” Wksp. Proc. IEEE GLOBECOM, Dal-

las, TX, Dec. 2004, pp. 374–79.

[6] D. Papadimitriou and E. Mannie (Eds.), “Recovery (Pro-

tection and Restoration) Terminology for Generalized

Multi-Protocol Label Switching (GMPLS),” draft-ietf-

ccamp-gmpls-recovery-terminology-05.txt, Oct. 2004,

work in progress.

[7] J.P. Lang and B. Rajagopalan, Eds., “Generalized Multi-

Protocol Label Switching (GMPLS) Recovery Functional

Specification,” draft-ietf-ccamp-gmpls-recovery-func-

tional-03.txt, Oct. 2004, work in progress.

[8] B. Berde, et al., “Improving Network Performance

through Policy-Based Management Applied to General-

ized Multi-Protocol Label Switching,” 10th IEEE ISCC

2005 Symp., June 2005.

ADDITIONAL READING

[1] K. Kompella and Y. Rekhter, “LSP Hierarchy with Gener-

alized MPLS TE,” draft-ietf-mpls-lsp-hierarchy-08.txt,

Feb. 2001, work in progress.

[2] D. Papadimitriou et al., “Generalized Multi-Protocol Label

Switching (GMPLS) Protocol Extensions for Multi-Region

Networks (MRN),” draft-papadimitriou-ccamp-gmpls-mrn-

extensions-00.txt, Oct. 2004, work in progress.

[3] A. Banerjee et al., “Generalized Multi Protocol Label

Switching: An Overview of Routing and Management

Enhancements,” IEEE Commun. Mag., Jan. 2001.

IEEE Communications Magazine • July 2005

n Figure 3. Internal control plane elements connected to the PBM system.

BIOGRAPHIES

LOA	ANDERSSON	works as principal networking architect at

Acreo AB in internetworking architecture and systems

design, primarily with the Acreo National Broadband

Testbed, but also in supporting Nordic and international

research institutions and networking companies. This

includes working with companies’ design and marketing

organizations to develop product and market strategies. He

has provided leadership in architectural development and

product specification at several networking companies. He

is currently leading an architectural requirement and test

activity for multiservice metro networks in the Nordic

region. He is a member of the technical program commit-

tee of the MPLS World conference, Paris, France, and the

Annual MPLS Conference, Washington, DC. He has been a

speaker and/or chair at IP-dagarna, Stockholm, Sweden,

1999–2003; Telekomdagarna, Sweden, 2000, 2002, and

2003; MPLS World, Paris, France, 2001–2004; and MPLS

2001–200-4, Washington, DC. He has long experience

working in standards organizations, and is also an active

participant in the IETF and several industrial fora. He is cur-

rently co-chair of the MPLS and L2VPN working groups. He

is a co-author of key MPLS, L2VPN, L3VPN, and GMPLS

specifications. He has filed several patents in the area of

networking. He is since March 2005 a member of the Inter-

net Architecture Board. He graduated in chemistry in 1970

and, apart from his engineering degree, has also studied

social anthropology and ethnology at Stockholm University.

BELA	BERDE	[M] (bela.berde@alcatel.fr) works, since 2000,

for the Packet Transport Networking (PTI) group of Alcatel

Research & Innovation (R&I), Marcoussis, France. Previously,

he was a scientific (research) staff member at the French

Aerospace Research Center (ONERA). He received his Ph.D.

in applied mathematics from Universite Pierre et Marie

Curie, Paris, France. He is currently involved in the design

and development of new technologies for GMPLS-enabled

multilayer networks. His current research interests are the

design of network control and management solutions for

communication systems. He is involved in different internal

research programs and external development activities in

liaison with academia, national operators, and industrials.

He is also serving as reviewer and coordinator for the Euro-

pean CELTIC initiative.

DIDIER	COLLE	received an M. Sc. degree in electrotechnical

engineering (option, communications) from Ghent Universi-

ty in 1997. Since then, he has been working at the same

university as a researcher in the Department of Information

Technology (INTEC). He is part of the research group INTEC

Broadband Communication Networks (IBCN) headed by

Prof. Piet Demeester. His research led to a Ph.D. degree in

February 2002. He was granted a postdoctoral scholarship

from the Institute for the Promotion of Innovation through

Science and Technology in Flanders (IWT-Vlaanderen) for

49

BERDE LAYOUT  6/20/05  10:05 AM  Page 50

the period 2003–2004. His research deals with design and

planning of communication networks. This work focuses

on optical transport networks, to support the next-genera-

tion Internet. Until now, he has actively been involved in

several IST projects (LION, OPTIMIST, DAVID, STOLAS,

NOBEL and LASAGNE), COST actions 266 and 291, and the

ITEA/IWT TBONES project. His work has been published in

more than 100 scientific publications in international con-

ferences and journals.

TIBOR CINKLER	[M’95] (cinkler@tmit.bme.hu) received M.Sc.

(1994) and Ph.D. (1999) degrees from Budapest University

of Technology and Economics, Hungary, where he is cur-

rently an associate professor in the Department of Telecom-

munications and Media Informatics. His research interests

focus on routing, design, configuration, dimensioning, TE,

and resilience of IP, Ethernet, MPLS, ATM, ngSDH/SONET,

and particularly WR-DWDM-based multilayer (GMPLS/ASTN)

networks.  He has been involved in a few related European

and Hungarian projects (IP NOBEL, NoE e-Photon/ONe,

CELTIC PROMISE, COST266, COST291, COST293, etc.) and is

a member of ONDM, DRCN, ICC, Opticomm/BroadNets,

EUNICE, and ICOCN Program Committees. He is the author

of over 80 scientific publications and three patents.

JUAN	FERNANDEZ-PALACIOS received his Telecommunications

Engineer degree from Polytechnic University of Valencia,

Spain, in June 2000, where he conduced his final project

on the design of wavelength converters. He joined Telefon-

ica I+D in September 2000, where he has been working on

the evaluation of new transport technologies for access

and core networks. He has participated in several European

research projects such as Eurescom P1014 TWIN, IST

DAVID, MUSE, NOBEL, and ePhoton/ONE as well as other

Telefonica internal projects related to the development of

optical networks in Spain and Latin America.

MONIKA	JДGER	[M] (monika.jaeger@t-systems.com) has

been a senior scientist and project leader in the Network

Architecture Department of T-Systems International GmbH,

Technologiezentrum, Berlin, Germany, since 1998. She

held previous positions with the Fraunhofer Institute for

Open Communication Systems (FOKUS) and DeTeWe in

Berlin. In 1992, she graduated in electrical engineering

from Technische Universitдt Mьnchen, Germany. Her cur-

rent research interests are in the area of optical transport

network design. She is involved in the European IST pro-

ject NOBEL.

LIEVEN	LEVRAU [M] (llevrau@riverstonenet.com) received an

M.Sc. degree in applied physics engineering (focus on pho-

tonics) from the Free University of Brussels in 1994, and he

has studied avionics at the University of Ghent. He has par-

ticipated in several European and national projects, such as

NOBEL, Dynamo, and GSN. His current research interests

are focused on design issues related to multilayer network

control architectures for core and metro networks. He is

currently employed by Riverstone Networks as a technology

consultant engineer.

MA R T I N	VI G O U R E U X	[M] (martin.vigoureux@alcatel.fr)

received an M.Sc. degree in high frequency electronics in

2000. He has been working since at Alcatel Research and

Innovation. His current areas of interest are focused on

GMPLS and its applicability to integrated systems architec-

tures and advanced networks environments. He has co-

authored publications on multiregion networks, including

submissions to the CCAMP Working Group of the IETF, as

well as others on policy-based management. He is also

responsible for development of a GMPLS-enabled and

flow-based network simulator as well as for a multilayer

traffic engineering software.

IEEE COMMUNICATIONS	MAGAZINE	— CALL FOR	PAPERS

NETWORK	& SERVICE	MANAGEMENT	SERIES

IEEE Communications Magazine announces the creation of a new series on Network and Service Management. The series will be

published twice a year, in April and October, with the first issue planned for October 2005. It intends to provide articles on the lat-

est developments in this well-established and thriving discipline. Published articles are expected to highlight recent research achieve-

ments in this field and provide insight into theoretical and practical issues related to the evolution of network and service

management from different perspectives. The series will provide a forum for the publication of both academic and industrial

research, addressing the state of the art, theory and practice in network and service management. Both original research and review

papers are welcome, in the style expected for IEEE Communications Magazine. Articles should be of tutorial nature, written in a style

comprehensible to readers outside the speciality of Network and Service Management. This series therefore complements the newly

established IEEE Electronic Transactions on Network & Service Management (eTNSM). General areas include but are not limited to:

* Management models, architectures and frameworks

* Service provisioning, reliability and quality assurance

* Management functions

* Management standards, technologies and platforms

* Management policies

* Applications, case studies and experiences

The above list is not exhaustive, with submissions related to interesting ideas broadly related to network and service management

encouraged.

IEEE Communications Magazine is read by tens of thousands of readers from both academia and industry. The magazine has also

been ranked the number one telecommunications journal according to the ISI citation database for year 2000, and the number

three for year 2001. The published papers will also be available on-line through Communications Magazine Interactive, the WWW

edition of the magazine. Details about IEEE Communications Magazine can be found at http://www.comsoc.org/ci/.

SCHEDULE FOR THE FIRST ISSUE

Manuscripts due: March 30, 2005

Acceptance notification: June 30, 2005

Manuscripts to publisher: July 30, 2005

Publication date: October 2005

SERIES EDITORS

Prof. George Pavlou

Center for Communication Systems Research

Dept. of Electronic Engineering

University of Surrey

Guilford, Surrey GU2 7XH, UK.

e-mail: G.Pavlou@eim.surrey.ac.uk

50

Dr. Aiko Pras

Center for Telematics and Information Technology

Dept. of Electrical Engineering, Mathematics and Computer Science

University of Twente

P.O. Box 217, 7500 AE Enschede, The Netherlands.

e-mail: a.pras@utwente.nl

IEEE Communications Magazine • July 2005