Information Processing Letters 90 (2004) 73–80

www.elsevier.com/locate/ipl

Packet loss probability for DiffServ over IP and MPLS reliable

homogeneous multicast networks ✩

Abdullah AlWehaibia,∗, Michael Kadochb, Anjali Agarwala, Ahmed ElHakeema

a Department of Electrical and Computer Engineering, Concordia University, Montreal, Canada

b Department de Genie Electrique, Ecole de Technologie Supйrieure, Universitй du Quebec, Montreal, Canada

Received 9 May 2003

Communicated by F. Dehne

Abstract

Multicasting has become increasingly important with the emergence of Internet-based applications such as IP telephony,

audio/video conferencing, distributed databases and software upgrading. IP multicasting is an efﬁcient way to distribute

information from a single source to multiple destinations at different locations. In practice IP is considered as a layer 3 protocol.

Multiprotocol Label Switching (MPLS) replaces the IP forwarding by a simple label lookup. MPLS combines the ﬂexibility of

layer 3 routing and layer 2 switching.

In order to provide QoS in group communications for real time applications such as video conferencing, reliable multicasting

is used. Miscellaneous efforts have been undertaken to provide reliability on top of IP multicast. Two error control strategies

have been popular in practice. These are the FEC (Forward Error Correction) strategy, which uses error correction alone, and

the ARQ (Automatic Repeat Request) strategy, which uses error detection, combined with retransmission of data.

In this paper, we present a new fair share policy (FSP) that utilizes Differentiated Services to solve the problems of QoS and

congestion control when reliable ARQ multicast is used. The results should provide insight into the comparisons of the residual

packet loss probability between IP multicast in MPLS networks using FSP and plain IP multicasting using the same policy

when DiffServ are adopted and when reliable ARQ multicast is considered.

2004 Elsevier B.V. All rights reserved.

Keywords: Interconnection networks; Packet loss; Multicast; IP; MPLS

1. Introduction

Multicasting has been at the center of interest in

the area of Internet activities and has already attained

✩	Work supported by NSERC/BELL Canada, Joint CRD grant

program.

*	Corresponding author.

E-mail addresses: aa_alweh@ece.concordia.ca

(A. AlWehaibi), ahmed@ece.concordia.ca (A. ElHakeem).

0020-0190/$ – see front matter	2004 Elsevier B.V. All rights reserved.

doi:10.1016/j.ipl.2004.01.008

major successes. IP multicast supports group commu-

nications by enabling sources to send a single copy

of a message to multiple recipients at different loca-

tions who explicitly want to receive the information

[1]. With the huge increase demand for bandwidth, one

of the challenges the Internet is facing today is to boost

the packet forwarding performance.

Recent developments in Multiprotocol Label

Switching (MPLS) open new possibilities to address

some of the limitations of IP systems. MPLS is an In-

74

A. AlWehaibi et al. / Information Processing Letters 90 (2004) 73–80

ternet Engineering Task Force (IETF) standard [2]. It

replaces the IP forwarding by a simple label lookup

mechanism. MPLS combines the ﬂexibility of layer 3

(L3) routing and layer 2 (L2) switching, which en-

hances network performance in terms of scalability,

computational complexity, latency and control mes-

sage overhead. Besides this, MPLS offers a vehicle

for enhanced network services such as Quality of Ser-

vices (QoS)/Class of Service (CoS), Trafﬁc Engineer-

ing and Virtual Private Networks (VPNs). IP multicast

in MPLS networks is still an open issue [2–4].

On the other hand, the IETF DiffServ working

group is looking at a more scalable model and more

likely to be easier to implement than IntServ/RSVP

model [5]. In the DiffServ architecture, trafﬁc that re-

quires the same Per-Hop-Behavior (PHB) is aggre-

gated into a single queue. The DiffServ architecture

[6] focuses on the use of DiffServ (DS) byte, which

is the redeﬁned 8-bit Type of Service (TOS) ﬁeld in

the IPv4 header or the IPv6 Trafﬁc Class octet as a

QoS mechanism. Packets are classiﬁed into the cor-

responding queues using their DiffServ Code Points

(DSCP). Packets use DSCP bits in order to receive

a particular PHB, or forwarding treatment. Marking,

classiﬁcation, trafﬁc conditioning or policing are done

at network boundaries (ﬁrst router for example) and

packet treatment and handling is carried on each net-

work node [6].

Reliable multicasting is used to provide QoS in

group communications for real time multimedia ap-

plications such as video conferencing. Two main error

control strategies are well known. These are the FEC

(Forward Error Correction) strategy, which uses error

correction alone, and the ARQ (Automatic Repeat Re-

quest) strategy, which uses error detection, combined

with retransmission of repair data [7–9].

In ARQ strategy, when an error is detected at the

receiver, a request (NAK) is transmitted to the sender

to repeat the incorrect message, and this continues

until the message is received correctly. ARQ can

be divided into two types: stop-and-wait ARQ and

Continuous ARQ which can be further divided into

two subtypes: go-back-N ARQ and selective-repeat

ARQ. In our work, we will use selective repeat

ARQ. When reliable multicasting is used, there is a

scalability problem to accommodate arbitrarily large

groups of receivers where each receiver would be

sending an acknowledgment to the sender, which in

case of large group could easily result in	feedback

implosion	problem. This problem can be solved by

allowing the receivers to send NAKs only in case of

errors or lost data. There are two methods to send the

repair packets from the sender to the receiver or group

of receivers:

(1)	Multicast repairs: In case of receiving a NAK

from one or more receivers the sender multicast

again the repair packet to all receivers.

(2)	Unicast repairs: With unicast repairs, if the sender

received a NAK from one or more receivers, it

resends the repair packet to only the receivers who

did not receive the packet correctly in a unicast

manner.

The multicast repairs method is simpler than the uni-

cast repairs method and requires less overhead; how-

ever the multicast repairs methods consumes much

more bandwidth. In our work we will evaluate the per-

formance of the ARQ with multicast repairs only.

In this paper, we compare QoS performance of IP

and MPLS multicasting, given their particular con-

straints [10]. In regular IP multicasting only over-

head pertaining to IP multicast tree should be estab-

lished, while in MPLS multicasting we have to add

also the corresponding MPLS multicast tree establish-

ment times and control packets. We present a new fair

share policy and by taking the above constraints into

consideration, we evaluate the QoS performance in

terms of residual packet loss probability for a typical

binary tree in the two cases of IP and MPLS multi-

casting. We also consider Differentiated Services; i.e.,

trafﬁcs with different priority classes when reliable

ARQ multicast is used. Analysis tools will be used to

evaluate our fair share policy (FSP) for different ho-

mogeneous network scenarios.

2. The analytical model underlying Fair Share

Policy (FSP)

FSP is not a call admission rather it is a trafﬁc

policing mechanism. In FSP, packets are discarded in

case of congestion differently at each queue accord-

ing to source priority and the maximum number in the

queue; i.e., the source with higher priority will expe-

rience less packet discarding than sources with lower

A. AlWehaibi et al. / Information Processing Letters 90 (2004) 73–80

Fig. 2. The coupled state diagrams.

75

Fig. 1. The analytical model.

where maxp, p = 1, 2, 3, is calculated as:

Pr

priorities. Moreover, FSP guarantees fairness among

ﬂows having the same priority (i.e., required QoS) in

maxp=∑p

pPrp

∗ B,

buffer space allocated to lower priority trafﬁc is larger;

thus leading to less packet discard [10]. Our analytical

model is shown in Fig. 1. In this model, a typical IP

or MPLS router and our FSP trafﬁc policing mecha-

nism process three independent sources corresponding

to different input trafﬁc classes. Source 1 is assigned

the highest priority, then source 2 and ﬁnally source 3.

For this model, the enforcement is assumed to occur at

the router (node) according to Fair Share Policy.

The following assumptions are used:

(1) Assume a Bernoulli arrival for all sources; in

order to be short and discrete interarrivals.

(2) FSP uses non pre-emptive priority queuing.

(3) The arrival probabilities are α1, α2and α3for

each source respectively. Note that α represents

the probability of receiving a packet while one

packet is served on the channel.

(4) Service disciplines for different queues are β1,

β2and β3for each source respectively.

(5) Average queue sizes are E1(n), E2(n) and E3(n)

for each source respectively.

(6) Maximum buffer sizes are max1, max2 and max3

for each source respectively.

(7) Total system buffer size:

B = max1+max2+max3,

where Prpis source p priority.

(8) All of MPLS or IP routers on the subject Inter-

net are homogeneous in providing resource and

trafﬁc conditions.

(9) All packets are of the same length.

(10) Steady state conditions prevail such that the

distribution of the number of packets in the

queue will not change with time and hence E1(n)

for source 1 for example will be taken as a

representative ﬁgure of the actual number in the

queue n1.

The coupled state diagrams for the analytical model

in Fig. 1 are shown in Fig. 2.

This diagram represents a typical router with 3

priority classes. The solution of the number in every

class depends on the solutions of the other classes;

where β1= 1 always in order to give source 1 with

highest priority the best service probability, β2= P01;

i.e., packets from source 2 will be served only when

the buffer corresponding to source 1 (which has higher

priority) is empty and ﬁnally β3=P01P02; i.e., packets

from source 3 will be served only when the buffers

corresponding to source 1 and source 2 (which have

higher priority) are all empty. Pcpis the probability of

successful delivery to next router for priority p trafﬁc

(p = 1, 2, 3).

76

A. AlWehaibi et al. / Information Processing Letters 90 (2004) 73–80

1 − σ	λ

Packet loss probability for each source can be

obtained by calculating the probability to be in last

Pn=

µ

Pn−1 −

µ

Pn−2

stage in the state diagram Pmax1,Pmax2andPmax3

respectively.

For IP based networks, the source arrival probabil-

for n = 3, 4, . . . , maxp.

Eq. (8) can be rewritten as:

1 − σ	λ

(8)

ity α is actually a composite one; for instance α1(for

source 1) can be written as:

Pn+2=

µ

Pn+1 −

µ

Pn

α1= τ α11+α12,τ =

∆1+ ∆2

∆1

,

(1)

for n = 3, 4, . . . , maxp.

Deﬁne m = (1 − σ )/µ and q = λ/µ.

(9)

where ∆1is the processing time at lower layers (for

example MAC layer),	∆2is the processing time at

Eq. (9) is the 2nd order homogeneous difference

equation [13], which has the general form:

IP layer and τ	is the IP processing time factor (or

processing factor).

Pn+2+2aPn+1+bPn= 0,

(10)

α12

1	is the intrinsic arrival probability, α1is the extra

arrival probability due to IP control overhead used to

where a = −m/2 and b = q , the general solution of

Eq. (10) is of the form [13]:

establish the IP multicast tree. The above equation can

be rewritten in terms of α11as:

Pn=Ar1n+Br2n,

(11)

α1= τ α11 + ξ1α11,ξ1=

α2

1

α1

,

(2)

where r1	and r2	are the distinct roots of the Eq. (10);

A and B are constants. The characteristic equation of

Eq. (10) is:

1

where	ξ1is the IP control overhead factor (or IP

factor).

r2− mr + q = 0

which has the solution:

(12)

Similarly for MPLS based networks,	α1can be

written as:

α1= α11+α12+α13,(3)

r1=

m +

Ⲛ

m2 − 4q

2

,

r2=

m −

Ⲛ

m2 − 4q

2

.

where α1

2

The initial conditions for the set of equations are P1

and P2. Using Eq. (11), we write:

1andα1are the same as in the case of IP

networks; α13 is the extra arrival probability due MPLS

control overhead used to establish MPLS multicast

paths or tree. α1can be rewritten in terms of α11as:

α3

α1=(1 + ξ1+ξ2)α11,ξ2=1,(4)

P1= Ar1+ Br2= kP0,

P2= Ar12 + Br22 = ωP0,

where

(

(13)

(14)

α1

ω =

(1 − σ )α

− α

and	k =

α

.

1

where	ξ2is the MPLS control overhead factor (or

µ2

µ

µ

MPLS factor).

By writing the balance equations for the state

diagrams in Fig. 2 [11,12], and solving these equations

Substituting for	r1and	r2and solving Eqs. (13)

and (14) together to ﬁnd A and B, we obtain:

(	(

to ﬁnd the probabilities. In order to write the equations

B =

ω − kr1

r2

2	− r1r2

P0and	A =

ωP0− Br22

r2

.

in simpler forms we deﬁne:

λ = (1 − Pcβ)α,	µ = (1 − α)Pcβ,

1

In order to ﬁnd	nth probability Pn, our solution for

Eq. (11) can be written as:

σ = αPcβ + (1 − α)(1 − Pcβ),

α

P1=	P0,

(5)

(6)

Pn=

(

ωP0− Br22

r2

(

m +

Ⲛ

m2 − 4q

2

n

P2=

µ

1 − σ

µ

P1−

α

µ

P0=

[

(1 − 6)α − α

µ2	µ

]

P0,	(7)

+

(

1

ω − kr1

r2

2 − r1r2

P

(

0

m −

Ⲛ

m2 − 4q

2

n

.	(15)

A. AlWehaibi et al. / Information Processing Letters 90 (2004) 73–80

∑maxp

77

Taking into account that

n=0Pn=1,	P0can be

are the routers with receivers underneath them. As

found using the following equation:

1

P0=	∑maxp

shown in the ﬁgure the depth of this tree is 4 and the

total number of routers is 15.

=

1 +

n=1Pn

1

max

max

3. Reliable ARQ multicast repairs

1 + k + ω + Ar13+Br23 + · · · + Ar

p

p

=

1 + k + ω + Ar1

1

1 rmaxp−2 1

1−r1+Br2

1+Br2

.	(16)

-261 rmaxp−2 2

1−r2

Pcp	which is the probability of successful delivery

to next router for certain priority trafﬁc, would be

given as:

Therefore, the solution of probability of steady state

of the number of packets in the buffer is now given by

Pcp=(1 − Pop − Pep )L,	p = 1, 2, 3.

(19)

Eq. (15). The expected number of packets in the buffer

for a speciﬁc source p can be found as:

max∑p

Ep (n) =	n ∗ Pn

n=0

= 1 ∗ k ∗ P0+ 2 ∗ ω ∗ P0

max∑p(	n

+	n ∗	Arn(17)

1+Br2.

n=3

Notice that the packet loss probability for source p	is

equal to the probability to be in last stage of the state

diagram:

Popis the byte overﬂow for a certain priority trafﬁc

which can be obtained by dividing the packet overﬂow

probability in Eq. (18) by packet length (L). Pepis the

byte error probability for a certain priority trafﬁc p and

it is assumed to be equal to Pop.

In Eq. (19), two assumptions are made:

(1) Packet loss of source packet is caused by consec-

utive byte losses at the intermediate routers.

(2) Interleaving is used in order to break byte burst

losses and efﬁciently turn them independent ran-

dom byte losses at the source and destination [9].

PLp=Pmaxp(p = 1, 2, 3).

(18)

Probability of no packet loss for certain priority trafﬁc

is given by:

The same solution above applies to state diagrams in

Figs. 2(middle) and 2(bottom) as well except that in

Fig. 2(middle) α = α2, β = β2	and max = max2	and

that in Fig. 2(bottom)	α = α3,	β	= β3	and max =

max3.

One more assumption is added for reliable multi-

cast that we use a complete binary tree, where each

parent router has two children routers until we reach

leafs. Fig. 3 shows an example of a complete binary

tree with the root, which is the nearest router to the

sender or the rendezvous point, and the leafs, which

Pno packet lossp= (1 − Pop)L

∼1− LPop

for small values of Pop, p = 1, 2, 3.

Therefore, probability of packet loss for small values

of Popand for certain priority trafﬁc	p	can be

expressed as:

Ppacket lossp= 1 − Pno packet lossp= LPop,

Ppacket lossp

Pop=

L

,	p = 1, 2, 3.

Fig. 3. A complete homogeneous binary multicast tree.

In this case upon the receipt of a NAK from

one or more receivers, the sender multicast again the

repair packet to all receivers. Due to the use of ARQ

multicast repairs, the intrinsic arrival probability αp1

for certain priority trafﬁc p would increase according

to:

α1p=αp1(1 + Fp ),	p = 1, 2, 3.	(20)

78

A. AlWehaibi et al. / Information Processing Letters 90 (2004) 73–80

Fp	is the number of failures for certain priority trafﬁc

p and N	is the total number of routers in the multicast

tree. This increase in the intrinsic arrival probability is

due to that every router in the whole network receives a

copy of each repair packet. The Probability of success

for worst case scenario for certain priority trafﬁc p	is

given as:

The total number of ARQ trials	Tp	for speciﬁc

priority trafﬁc p can be expressed as:

Tp=Psp+2Psp(1 − Psp) + 3Psp(1 − Psp)2+ · · · .

(23)

Therefore, the number of failures (or retransmissions

only) for certain priority trafﬁc p can be given as:

Pspw = Probability of success

Fp=Tp − 1,

(24)

= PcNp	worst case.

(21)

where Pspis the average probability of packet success

Eq. (21) represents an upper bound for worst case sce-

nario of probability of success when ARQ multicast

repairs method is used. However, using ARQ multi-

cast repairs have a better chance of success with each

trial since the number of receivers who did not receive

for priority p trafﬁc corresponding to one ARQ trial.

Deﬁning Pspas the ﬁnal probability of success for

priority p trafﬁc:

Psp= Psp+(1 − Psp)Psp + · · · + Psp(1 − Psp)Tp−1

the packet correctly decreases with each trial. There-

fore, the average probability of success for a certain

= 1 − (1 − Psp)Tp.

(25)

priority p	packet in a typical transmission multicast

trial from sender can be calculated as:

PcNp+ Pc(N/p2)+1 + · · · + Pc(N/p2D)+D−1

Eq. (25) is for (Tp) trials of a typical packet to the

multicast tree; Where we note that for one trial Psp=

Psp	and for inﬁnite retransmission trials Psp=1 as it

should be. Therefore, the residual loss (after all ARQ

Psp avg =

D

,

(22)

trials) is given by:

Plossp=1 − Psp.

(26)

where D is the network depth. If the packet does not

suffer loss or error on any of the	N	routers of the

multicast tree, with probability PcNp	no further repair

is needed, this explains the ﬁrst term of Eq. (22).

However, if there has been an error or loss which

located at level 1 (see Fig. 3), then the repair packet,

then the repair packet would be sent from sender to

the router at level 1, and then the repair packet will

ﬂow to N/2 routers under level 1. All such (N/2) +

1 transmissions of repair packet have to be correct,

otherwise further repair is needed and so on. The

probability of these (N/2) + 1 correct transmissions

of subject repair packet is given Pc(N/p2)+1	and so on

for the remaining terms in Eq. (22).

We divide by	D	(network depth) because we

assume that errors are equally likely to occur on

different levels of the tree giving rise to the addition

of different terms (Eq. (22)) and the division by the

depth D where: D = log2(N + 1).

A more accurate expression for Psp	was evaluated

in [14]. However, results of [14] shows that of Eq. (22)

is very close to the exact value over a wide range of

Pcp	and N .

4. Analysis results

Figs. 4 to 6 show the performance comparisons be-

tween IP sources and MPLS sources in the multicast

tree when ARQ multicast repairs mechanism is ap-

plied. Fig. 4 shows the residual packet loss probability

for all sources for both IP and MPLS versus IP fac-

tor (ξ1) for small processing factor (τ ). It shows that

IP and MPLS sources have very same residual packet

loss probability (almost zero).

However, Fig. 5 shows that when the processing

factor (τ ) increases MPLS will have superiority over

IP in terms of the residual packet loss probability. As

shown in Fig. 5 the residual packet loss probability

in case of MPLS (which is zero) is less than IP for

all sources and this difference is clear for low priority

sources 2 and 3.

In Figs. 4 and 5 MPLS factor was constant and

relatively small; explaining why MPLS performance

was better or very similar to IP performance. However,

in the following ﬁgure we will study the effect of

MPLS factor (ξ2) on MPLS performance. Fig. 6 shows

2

3

A. AlWehaibi et al. / Information Processing Letters 90 (2004) 73–80

3

2

79

α11=0.2, α1= 0.15, α1= 0.1,	β1=1,	Tp=2, D = 4,

B = 30, ξ2=0.1, τ = 1.2, L = 500

Fig. 4. Residual loss probability versus IP factor (small τ ).

3

2

α11=0.25, α1= 0.2, α1= 0.15, β1=1, Tp=2, D = 4,

B = 30, ξ2=0.1, τ = 1.8, L = 500

Fig. 5. Residual loss probability versus IP factor (large τ ).

that IP will be superior over MPLS when MPLS factor

increases. As shown in Fig. 6 the residual packet loss

probability in the case of IP (which is zero) is less

than MPLS for all sources and this difference is clear

for low priority sources 2 and 3. This means when

α1

1=0.2, α1= 0.15, α1= 0.10, β1=1, Tp=2, D = 4,

B = 30, ξ1=0.2, τ = 1.2, L = 500

Fig. 6. Residual loss probability versus MPLS factor.

the extra arrival rate due MPLS control overhead used

to establish MPLS multicast paths or tree increases,

IP will be perform better especially for low priority

trafﬁcs and when the intrinsic trafﬁcs increase.

5. Conclusions and future work

In this paper, a performance comparison between

IP multicast trees and MPLS multicast trees is car-

ried using analysis tools. In addition to that a new

Fair Share Policy (FSP), which is a trafﬁc policing

mechanism, is proposed to ensure proper QoS. Also,

Differentiated Services and ARQ reliable multicasting

are used in this comparison. In this paper, we found

that when the difference in packet processing time (τ )

between IP and MPLS is high and when MPLS fac-

tor is small, IP multicast will perform less efﬁciently

than MPLS in terms of residual packet loss probabil-

ity. However, when this difference in packet process-

ing time is small IP performs very similar to MPLS.

In addition to that when MPLS has higher arrival rate

due to MPLS trees establishment control overhead and

when the processing factor is small, IP would perform

better than MPLS.

80

A. AlWehaibi et al. / Information Processing Letters 90 (2004) 73–80

Analysis results revealed that there is a notice-	[2] E. Rosen, A. Viswanathan, R. Gallon, Multiprotocol label

able improvement in QoS deﬁned as the residual

packet loss probability for a higher priority trafﬁc

when MPLS multicasting replaces IP multicasting es-

pecially if MPLS factor is small and processing factor

is large.

In addition to that, the study ﬁnds that when

applying the ARQ multicast repairs mechanism, there

would be a noticeable improvement in terms of the

residual packet loss probability which enhances the

reliability of multicasting for both IP and MPLS trees.

The routers in the network could be identical in

their capabilities (homogeneous network) or different

(heterogeneous network).

Each router may have different capabilities; for ex-

ample one router could have the ability to correct er-

rors (FEC) and use ARQ, one may use only ARQ but

cannot correct errors, a third one may not have MPLS

capability. In this paper, the study carried only homo-

geneous networks. In the near future, heterogeneous

networks would be considered.

References

[1] B. Quinn, et al., IP multicast applications: challenges and so-

lutions, RFC 3170, 2001, http://www.ietf.org/rfc/rfc3170.txt.

switching architecture, RFC 3031, 2001, http://www.ietf.org/

rfc/rfc3031.txt.

[3] D. Ooms, et al., Framework for IP multicast in MPLS, IETF

Draft, draft-ietf-mpls-multicast-07.txt, 2002.

[4] D. Ooms, W. Livens, IP multicast in MPLS networks, in:

Proceedings of the IEEE Conference on High Performance

Switching and Routing, 2000, pp. 301–305.

[5] J. Wrocklawski, The use of RSVP with IETF integrated

services, RFC 2210, 1997, http://www.ietf.org/rfc/rfc2210.txt.

[6] M. Carlson, et al., An architecture for differentiated services,

RFC 2475, 1998, http://www.ietf.org/rfc/rfc2475.txt.

[7] S. Lin, D. Costello, Error Control Coding: Fundamentals and

Applications, Prentice-Hall, Englewood Cliffs, NJ, 1983.

[8] B. Li, Reliable multicast transmissions using forward error

correction and automatic retransmission requests, in: Canadian

Conference on Electrical and Computer Engineering, 2001,

vol. 2, 2001, pp. 1145–1150.

[9] S. Wicker, Error Control Strategies—for Digital Communica-

tion and Storage, Prentice-Hall, Englewood Cliffs, NJ, 1995.

[10] A. AlWehaibi, A. Agarwal, M. Kadoch, A. ElHakeem, Perfor-

mance comparison between MPLS multicast and IP multicast

using fair share policy and priority buffers, in: IASTED CSN

2002, Spain, 2002.

[11] T. Saadawi, M. Ammar, A. Elhakeem, Fundamentals of

Telecommunication Networks, Wiley, New York, 1994.

[12] L. Kleinrock, Queuing Systems, vols. I and II, Wiley, New

York, 1975.

[13] D. Jordan, P. Smith, Mathematical Techniques, Oxford Univer-

sity Press, Oxford, 1994.

[14] A. AlWehaibi, M. Kadoch, A. ElHakeem, Computation of

the residual packet loss probability in a binary multicast tree,

in: IEEE Canadian Conference on Electrical and Computer

Engineering, CCECE 03, Montreal, Canada, 2003.