ARTICLE IN PRESS

Computer Communications xxx (2007) xxx–xxx

www.elsevier.com/locate/comcom

QoS provisioning by EFuNNs-based handoﬀ planning in cellular

MPLS networks

Behrouz Shahgholi Ghahfarokhi *, Naser Movahhedinia

Department of Computer Engineering, University of Isfahan, Hezar Jarib Avenue, 81744 Isfahan, Iran

Received 30 May 2006; received in revised form 27 May 2007; accepted 8 June 2007

Abstract

In recent years, Multi Protocol Label Switching (MPLS) has been considered as the preeminent technology to incur Quality of Service

(QoS) for integrated services. However, in wireless networks the remotes mobility endangers resource management procedure and QoS

provisioning. In this paper we propose a new location prediction method based on Evolving Fuzzy Neural Networks (EFuNNs), to man-

age Label Switched Paths (LSPs) in an MPLS domain. The proposed predictor employs geographical characteristics of underlying area

and the movement history of a remote, to produce a set of conﬁdence ratios as the output. That set is considered as a criterion for estab-

lishing and managing LSPs so that QoS preserved. The simulation results have shown superior performance in terms of prediction accu-

racy and utilization improvement for the proposed methods.

2007 Elsevier B.V. All rights reserved.

Keywords:	Cellular MPLS networks; LSP management; Evolving Fuzzy Neural Networks; Quality of Service; Mobility prediction

1. Introduction

The third generation of mobile systems provides diﬀer-

ent services to end users, extending the scope of second

generation mobile from simple voice telephony to complex

data applications such as voice over IP, web browsing and

ﬁle transfer as discussed in [7]. As the result, QoS support

for diﬀerent traﬃc requirements will be necessary in next

generation wireless networks. However, nodes mobility

jeopardizes the resource allocation process, and decreases

the quality of service provided to delay sensitive traﬃc.

Limited bandwidth and mobility of users in wireless net-

works, accentuates that traditional QoS engineering meth-

ods are insuﬃcient.

In the other hand, increasing number of subscribers and

bandwidth demand have pushed the mobile networks to be

designed with smaller area cells to attain higher frequency

*	Corresponding author. Tel.: +98 311 7932683.

E-mail addresses:	shahgholi@eng.ui.ac.ir (B.S. Ghahfarokhi),	naser-

m@eng.ui.ac.ir (N. Movahhedinia).

0140-3664/$ - see front matter	2007 Elsevier B.V. All rights reserved.

doi:10.1016/j.comcom.2007.06.001

reuse. It is widely accepted that as long as the area cell

shrinks, an increment in the number of handovers is

unavoidable	[12]. To have smooth and robust handover

procedure, necessary measures such as minimizing errone-

ous handover decisions, protecting connections from dis-

ruption and preserving QoS during handovers should be

considered. In performing such procedures, location predic-

tion plays an essential role in allocating enough resources

before handing over a moving remote.

To incur QoS for diﬀerent services, MPLS	[8] has been

considered as the preeminent technology in recent years.

In the core networks, MPLS is emerging as the technology

of facilitating traﬃc engineering and internetworking.

Label Switched packet transferring is an extension to

packet forwarding whereby short ﬁxed length labels are

attached to packets at entry nodes. The labels are assigned

according to packets FECs1which are determined based on

traﬃc engineering metrics and QoS policies. Packets would

be delivered to their destinations (or mediating nodes)

1	Forwarding Equivalence Classes.

Please cite this article in press as: B.S. Ghahfarokhi, N. Movahhedinia, QoS provisioning by EFuNNs-based handoﬀ planning in cel-

lular ..., Comput. Commun. (2007), doi:10.1016/j.comcom.2007.06.001

2

ARTICLE IN PRESS

B.S. Ghahfarokhi, N. Movahhedinia / Computer Communications xxx (2007) xxx–xxx

through LSRs2that swap labels according to traversing

paths. These paths are established between egress and inte-

rior nodes in MPLS domain according to FEC of traﬃc

and minimum expenditure. MPLS technology can provide

QoS by employing CR-LDP3[10] or RSVP-TE4protocols.

These protocols utilize traﬃc attributes for setting up and

holding LSPs and providing resources for them. MPLS

can provide both COS5and virtual path objectives. So, it

has advantages of both IP Diﬀserv and ATM in providing

QoS for diﬀerent types of traﬃc. Owing to MPLS advanta-

ges, there are a lot of eﬀorts to exert it to wireless networks

for QoS improvement.

In this paper, we consider service quality provisioning in

an MPLS cellular network. A method is proposed to main-

tain QoS requirements for oﬀered services by properly

managing	handoﬀ	procedure.	To	provide	adequate

resources to the service given to a remote, the LSPs are

established and managed based on the predicted next cell

of presence of that remote. The location prediction is done

by a new method based on EFuNNs. Each output of the

predictor indicates the degree of conﬁdence for the corre-

sponding neighboring cell, showing that how likely the

remote may move to that cell.

In the next section of this paper, we will review related

works on location prediction, handoﬀ management and

MPLS and its employment in wireless cellular networks

for QoS implementation. In Section	3	EFuNNs is

explained. The proposed location predictor is presented

in Section 4. In Section 5, our proposed method for QoS-

based resource management in an MPLS domain using

location prediction is described. Simulation results and dis-

cussions are presented in Section 6, and ﬁnally, the paper is

concluded in Section 7.

2. Related works

This paper contributes in three issues: location predic-

tion, handoﬀ management and cellular MPLS. In this sec-

tion, we review some related works on each of these areas.

2.1. Location prediction methods

Predicting the next cell of presence may be employed in

various cellular network procedures such as resource man-

agement, admission control [16,39] and handoﬀ procedure.

Location prediction methods may perform some of the fol-

lowing techniques as mentioned in [38]:

– Predicting next cell of presence for a remote,

– Predicting time of presence in a cell for a remote,

– Historical-based prediction,

– Topography-aware prediction,

2	Label Switched Routers.

3	ConstRaint-based Label Distribution Protocol.

4	ReSerVation Protocol & Traﬃc Engineering.

5	Class Of Service.

– Per-user prediction,

– Per-cell prediction.

In	[13], authors have proposed a simple prediction

scheme using pattern matching. The predictor stores the

historical movement pattern of each user in a database

and compares the recent states with previous tracks to pre-

dict future movement. Authors in	[14]	use a statistical

method to predict the number of channels needed in each

cell to be reserved for mobile nodes in neighboring cells.

This predictor uses speed, direction and node arrival

elapsed time as parameters. To improve call connectivity,

authors of [17] have proposed the concept of shadow cluster.

A shadow cluster deﬁnes the area of inﬂuence of a mobile

node and indicates a set of BS6s to which the MN would

likely be connected in future. Each BS in the shadow cluster

anticipates the MN arrivals and reserve resources for them.

Two neural network-based predictors have proposed in

[11,12]. Both of these predictors are per user. In	[11],

authors have employed a feed-forward 3-layer neural net-

work. Inputs to this predictor are a set of BSs taken from

the movement history of MN. The outputs are 8 possible

directions and the distance (number of hops) of the future

movement. The simulations has showed 93% average accu-

racy for uniform movement, 40–70% average accuracy for

regular movement and 2–30% average accuracy for ran-

dom movement patterns. In [12], feedback neural network

structures (Joredan-, Elman- and Hierarchical Elman-Net-

works) are used for the same purpose as [11].

As the mentioned predictors are per-user and movement-

dependent, their operation is broadly based on movement

patterns and movement history of mobile nodes. Although

noticing to the topography of underlying area, such as a

high-way in a cell, may facilitate the next presence cell pre-

diction, these methods do not beneﬁt from topography.

Authors of [18] have proposed a neural network-based

topography aware predictor for handoﬀ management. This

predictor combines the mobile nodes movement history,

current state and topography of the cells as inputs. To take

the properties of underlying area into account, RSSIs7of

surrounding BSs is measured by MNs as movement his-

tory. To utilize the movement history for prediction, this

model employs a predictor per each BS. This predictor is

a feed-forward 3-layer neural network which anticipates

the next RSSIs for a remote. Simulations by authors have

shown that the proposed method is more accurate in pre-

dicting mobility patterns when topographical features have

impact on users’ movements.

2.2. Handoﬀ management methods

Handoﬀ management methods may be classiﬁed accord-

ing to the metrics used to decide about the necessity of

6	Base Station.

7	Received Signal Strength Indicators.

Please cite this article in press as: B.S. Ghahfarokhi, N. Movahhedinia, QoS provisioning by EFuNNs-based handoﬀ planning in cel-

lular ..., Comput. Commun. (2007), doi:10.1016/j.comcom.2007.06.001

ARTICLE IN PRESS

B.S. Ghahfarokhi, N. Movahhedinia / Computer Communications xxx (2007) xxx–xxx

3

handover. These metrics include signal strength [18,21], dis-

tance [22] and bit error rate [23]. Due to the fuzzy nature of

parameters such as distance or RSS, the handoﬀ planning

is a complex procedure. There are several algorithms that

employ advanced techniques such as Neural Networks

[15,24–28], fuzzy logic	[24–27]	and pattern recognition

algorithms [29–31].

Authors of [32] have proposed a fuzzy inference system

for handoﬀ decision. The input metrics of this system are

RSS8from the current and candidate neighboring access

points, ratio of used capacity to the total capacity of those

access points and relative direction and speed of MNs. The

outputs of this system (which are numbers between 1 and

9) are the membership value of a mobile node for currently

serving and candidate access points. The MN does not

handover if its membership value for its current access

point is above a predetermined threshold. When handoﬀ

is necessary, the access point with better membership hys-

teresis value than the current access point will be selected.

The results of simulations have been shown on VCL9-based

tactical communication systems.

In	[19], a fuzzy technique has been represented for

microcellular handoﬀ. In this method, RSSI and distance

have been selected as metrics for planning. The output of

this algorithm is called	hand-over factor	(HO-Factor). A

value of HO-Factor close to 1.0 is interpreted as strong case

of handoﬀ and a value close to 0.0 is the opposite. When

the HO-Factor	value for current BS exceeds the value for

another target BS (neighboring BS), the handover is

necessary.

Authors of [41] have proposed an adaptive fuzzy logic-

based algorithm that can adapt itself with the dynamic con-

ditions of hybrid networks.10It uses the mobile terminal

speed estimation and the traﬃc volume in wireless LAN

as additional input parameters.

In	[42], a fuzzy logic-based scheme has been presented

for selection of the best base station at the time of handoﬀ.

This scheme considers three criteria, namely, received

power level, user population and utilized bandwidth of

each base station.

The approach proposed in [43] is based on Fuzzy Logic

to evaluate the average and variation in signal strength

received by a mobile station. To lessen the handoﬀ latency,

the mobile station performs an active scan process only

once to obtain the complete handoﬀ parameters.

A serious issue in handoﬀ planning is frequent unneces-

sary handovers. For example an RSS-based algorithm may

encounter a problem which is called ‘‘ping-pong’’ eﬀect.

This problem occurs when the remote locates around the

mid-point between two access points so that the received

signal strength oscillates and the remote may be handed

over several times. To remedy this problem, in	[33]	a

threshold is introduced to avoid the MN from handover

8	Received Signal Strength.

9	Virtual Cell Layout.

10	Internetworking between wireless LANs and mobile networks.

until the RSS from the currently serving BS degrades below

predetermined threshold. Another technique is to intro-

duce a hysteresis to the RSS-based algorithm	[33].

Although both of the above methods reduce the ping pong

eﬀect, but introduce delay to handoﬀ procedure and are not

able to respond fast enough in microcellular environments

[32]. Such a problem indicates that the handover algorithm

requires making crisp decision in a region of uncertainty

with metrics that are fuzzy in nature as implied in [40].

2.3. MPLS in cellular networks

In many existing models for MPLS employment in wire-

less networks, Mobile Nodes need not to know any about

MPLS technology. MPLS domain starts from Base Sta-

tions or interconnecting Routers. In fact, base stations or

routers are responsible for connecting wireless domain to

MPLS domain	[6]. In	[4] a wireless network architecture

which is based on MPLS has been proposed. In this archi-

tecture, MPLS domain does not include base stations, so

the LER11s are the routers which are connected to base sta-

tions (MLSN12s).

In mobile internet connectivity, MIP13has been chosen

as the core of mobility management mechanism for recent

cellular networks. However, basic MIP inherits its bases

from IP, so is incapable of providing QoS. The interest in

MPLS, as an underlying forwarding scheme for MIP, is

justiﬁed by its capability to accommodate scale and QoS

guarantees [35]. In [2,3], authors have proposed a method

which is based on integration of MPLS and MIP.

2.4. Handoﬀ management in cellular MPLS

In previous research, generally, handoﬀ management is

performed by MPLS nodes (LSRs) in backbone. In	[1]

two methods for handoﬀ management have been proposed

in which there are special LSPs from the nearest router to

the base station, for every connection between an MN14

and its destination. In the ﬁrst method which is named

‘‘Dynamic Rerouting’’, during handover, the new BS

detects the nearest LSR in the previous LSP (between the

previous BS and destination) and establishes a new LSP

between itself and that LSR. Actually a new LSP is pro-

vided to transfer data from MN to its destination, consid-

ering Make-Before-Break policy. This scenario is shown in

Fig. 1 where LSR2 is the nearest common LSR.

In the second method called ‘‘pre-established paths’’, a

set of pre-established LSPs is used. There is a pre-estab-

lished LSP from each BS to the destination node of a con-

nection. While an MN is moving from a cell to another,

packets sent from this remote traverse through a new

LSP in the corresponding cell. The communicating node

11	Label Edge Router.

12	Mobile Label Switched Node.

13	Mobile IP.

14	Mobile Node.

Please cite this article in press as: B.S. Ghahfarokhi, N. Movahhedinia, QoS provisioning by EFuNNs-based handoﬀ planning in cel-

lular ..., Comput. Commun. (2007), doi:10.1016/j.comcom.2007.06.001

4

ARTICLE IN PRESS

B.S. Ghahfarokhi, N. Movahhedinia / Computer Communications xxx (2007) xxx–xxx

Backbone

LSR2

LSR1

CN

BS1

BS2

E-LSP

L-LSP

MPLS Core

CN

LSN3

BS1

BS2

LSN1

LSN2

BS3

Fig. 3. QoS support in WMPLS networks.

Another proposed way for QoS support in MPLS archi-

tecture is using CR-LSP.15CR-LDP16deﬁnes two proper-

ties for each CR-LSP called setup priority and hold priority.

CR-LDP reserves resources for each LSP according to its

QoS requirements and establishes a new LSP if there is

Fig. 1. Dynamic rerouting process.

recognizes the handover from LSP change, and uses the

new LSP for sending packets to that remote.

An important problem in handoﬀ management is han-

dling handoﬀ in a diversity area. In a diversity area a

remote can receive and transmit packets from and to two

or more base stations. MPLS has the inherent capability

to support multiple parallel LSP tunnels between multiple

nodes and merge these LSPs using Label Merging capabil-

ity. As considered in	Fig. 2, in ‘‘Dynamic Rerouting’’ sce-

nario, when the mobile is transmitting packets to two or

more base stations, these base stations forward the packets

with diﬀerent labels to the same LSN (the nearest LSR to

the new BS in old LSP)	[1]. In this LSN labels will be

merged and packets will be forwarded with the same label.

2.5. QoS support in wireless MPLS

There are two approaches to provide QoS in an MPLS-

based wireless network [5]. The ﬁrst one is to use a single

LSP to connect each BS to backbone. So, multiple classes

of traﬃc from a base station will be carried within one

LSP which is named ‘‘E-LSP’’. QoS diﬀerentiation, in this

method, is provided by packet marking and per-hop-

behavior using COS ﬁeld of headers.

The second way is to use multiple LSPs named ‘‘L-LSP’’

to connect each BS to the backbone. Each LSP carries one

class of traﬃc. These approaches are shown in Fig. 3.

CN

LSR2

LSR1

Label Merge Point

enough resources for it or there is a previously established

LSP with smaller hold priority than setup priority of emerg-

ing LSP.

Our proposed location predictor is based on EFuNN.

This network will be discussed in the next section.

3. Evolving Fuzzy Neural Network

EFuNN is based on ECOS17framework for building

online, adaptive intelligent systems that have both their

structure and functionality evolving in time. The model is

called evolving because of the nature of the structural

growth and structural adaptation of the whole evolving

connectionist system which it is a part of that. EFuNN sug-

gests a neuro-fuzzy systemic approach that employs grow-

ing supervised/unsupervised, knowledge-based learning

methods	[34]. While new connections and new neurons

are created during the operation of the system, EFuNNs

can accommodate new input data, including new features

and new classes through local elements tuning. These char-

acteristics are useful in cases where the number of features

or classes is not determined. The reports in [36] exhibit that

the EFuNN model not only gives the analogous perfor-

mance compared to other complex neuro-fuzzy systems,

but also provides the feature of the expeditious one pass

parameter training which makes it highly suitable for the

low power requirement. Authors of	[44]	have used

EFuNNs for zone radius estimation which is used for zone

routing protocol in Bluetooth networks.

The EFuNN employs a feed forward neural network to

process fuzziﬁed data and defuzziﬁes the fuzzy data as the

output. Generally, an EFuNN includes ﬁve processing

stages, which are network initiation, inputs feed forward,

parameters tuning, node aggregation and pruning, and rule

extraction, respectively. The fuzzy input layer carries out

fuzzy quantization of the inputs. The rule layer contains

rule nodes that can evolve through learning. As shown in

Fig. 4, after each input vector is fed into the EFuNN, the

BS1

LSN

BS2

network updates its parameters, evolves connections,

aggregates and prunes nodes based on the output error

15	ConstRaint-based LSPs.

16	ConstRaint-based Label Distribution Protocol.

Fig. 2. Handling handoﬀ in the diversity areas.

17	Evolving Connectionist Systems.

Please cite this article in press as: B.S. Ghahfarokhi, N. Movahhedinia, QoS provisioning by EFuNNs-based handoﬀ planning in cel-

lular ..., Comput. Commun. (2007), doi:10.1016/j.comcom.2007.06.001

ARTICLE IN PRESS

B.S. Ghahfarokhi, N. Movahhedinia / Computer Communications xxx (2007) xxx–xxx

5

Input Layer

Inputs

Fuzzy Input Layer	Rule Layer	Fuzzy Output

Layer

Output layer

W3

W2

OutN

A1A2

If the activation of the selected rule node is smaller than a

predetermined sensitivity threshold, a new rule node is cre-

ated and the new connection weights are established for the

new fuzzy input and fuzzy output pair Xfand Yf. So, the

new network has two new weight vectors	W1,(N + 1)and

W2,(N + 1)	where

W1;рN ю 1ЮјXf

W2;рN ю 1ЮјYfр3Ю

On the other hand, when the activation of the selected rule

node is larger than the sensitivity threshold, it will be

passed forward to the next layer to compute the output

of Fuzzy Output Layer A2as follows:

W1

Fig. 4. Sample EFuNN structure.

A2ј satlinрW2A1Ю

р4Ю

during the last epoch if necessary. Then the EFuNN prop-

agates the signals forward, and computes the output error

again. Then the defuzziﬁcation for the fuzzy output layer

gets done in the output layer [34].

where W2= [W2,1W2,2. . . W2,N] and satlin(.) represents the

saturating linear transfer function. Similarly, a new rule

node will be created if the following fuzzy output error is

larger than a predeﬁned threshold value,

р5Ю

FEoutј kA2Yfk

At last, crisp output value Yccan be derived by Eq. (6):

3.1. Network initiation

Ycј W3A2

р6Ю

At the network initialization phase, the connection

weight matrixes	W1and	W2(as shown in	Fig. 4) are set

to some predeﬁned values based on the past experience of

the network. W1and W2represent the coordinates of the

sphere center in the fuzzy input space and in the fuzzy out-

put space respectively. The membership functions, the

number of the rule nodes and the connection patterns of

W1and	W2are decided based on speciﬁcations of the

problem.

3.2. Input feed-forward layer

When a new sample is fed as input, ﬁrst it is fuzziﬁed at

the fuzzy input layer, then the Fuzzy Distance (FD)

between the output of the fuzzy input layer and the connec-

where W3denotes the connection weight matrix between

the fuzzy output layer and the output layer.

3.3. Parameter tuning

The training process of the network includes the updates

of the connection weights W1and W2, the learning rate and

the sensitivity thresholds for each rule node. W1is adjusted

using unsupervised learning based on the similarity

between the fuzzy input vector	Xfand the stored proto-

types W1,j	for the jth rule node as follows:

W1;jрt ю 1Ю ј W1;jрtЮ ю gjрW1;jрtЮ  XfЮ	р7Ю

and	W2is updated according to the Widrow–Hoﬀ least

mean square (LMS) algorithm [37] that minimizes the fuz-

zy output error,

tion weights W1are calculated to determine if the input

falls into the receptive distance of some speciﬁc rule node.

W2;jрt ю 1Ю ј W2;jрtЮ ю gjрA2YfЮ:A1;j

р8Ю

The fuzzy distance between the two fuzzy membership vec-

tors of input Xfand the connection weights of the jth rule

node, W1,j	is deﬁned as follows:

kXfW1;jk

In both equations, gjstands for the learning rate of the jth

rule node. Note that gjcan be expressed as gj= 1/ACCj,

where ACCjis the accumulated number of accommodated

examples for the jth rule node.

The sensitivity threshold for the rule node, which has

FDрXf; W1;jЮ јkXfю W1;jk;	j ј 1;	2; . . . ; N

р1Ю

been referred in Eqs. (5), (6) or (7), is given by

where N is the number of rule nodes, W1,j is the jth column

vector of matrix W1, iXfW1,jidenotes the sum of all the

absolute values of the vector obtained by subtraction of

vectors	Xfand	W1,j. Likewise,	iXf+ W1,jiis the one ob-

tained by summation of vectors Xfand W1,j.

Sjрt ю 1Ю ј SjрtЮ ю FDрW1;jрt ю 1Ю; W1;jрtЮЮ

3.4. Rule node aggregation

р9Ю

We then select the rule node with the highest activation

which is the lowest FD value. The activation set for the rule

layer is deﬁned as matrix A1with the row vectors given by:

A1;jј1	FDрXf; W1;jЮр2Ю

After certain number of training samples has been pre-

sented, some neurons and connections may be pruned or

aggregated. If the fuzzy distance as given in Eq.	(1)	for

every two out of K nodes is less than a predeﬁned threshold

for both connections W1and W2, the K nodes can be aggre-

Please cite this article in press as: B.S. Ghahfarokhi, N. Movahhedinia, QoS provisioning by EFuNNs-based handoﬀ planning in cel-

lular ..., Comput. Commun. (2007), doi:10.1016/j.comcom.2007.06.001

6

ARTICLE IN PRESS

B.S. Ghahfarokhi, N. Movahhedinia / Computer Communications xxx (2007) xxx–xxx

gated into one single rule node with the following connec-

tion weights and sensitivity threshold:

PK

Rj : IF In1 is ‘‘low’’ with a degree of 0.45, and is ‘‘medium’’

with a degree of 0.65,

AND In2 is ‘‘Low’’  with a degree of 0.67, and is ‘‘High’’

Wr;aggј

iј1Wr;i

K

; r ј 1; 2

р10Ю

with a degree of 0.39,

AND In3 is ‘‘Low’’  with a degree of 0.44, and is ‘‘High’’

Saggј 1	MaxрFDрW1;agg;W1;iЮЮ

i21::K

where	W1,agg,W2,aggandSaggrepresent the connections

and	sensitivity	threshold	for	the	aggregated	node,

respectively.

3.5. Rule extraction and insertion

Every rule node in the network can generate a fuzzy rule

from W1and W2connections. We assume that there exists

a fuzzy rule in the network as in Fig. 5.

In Fig. 5 the number after each fuzzy label represents the

degree to which the centers of the input and the output

hyper-sphere belong to the respective membership func-

tion. The degrees associated with the premise and the con-

sequent parts of the rule are the connection weights of W1

and W2, respectively.

For manual insertion of new fuzzy rules, a new rule

node, ri, will be inserted in the rule layer such that the con-

nection weights W1(rj) and W2(rj) of the rule node represent

this rule. It means that only for corresponding linguistic

terms in rule, values of W1(rj) and W2(rj) are one and for

others are zero.

4. Proposed predictor

In this section, we propose an EFuNN-based predictor

which considers uncertainty characteristics of handover

process. The proposed model is a motion predictor with

the following characteristics:

– It considers the topographical characteristics of underly-

ing area, in addition to motion history of users.

– There is a predictor per each cell which predicts the next

cell of presence for all of the remotes in that area cell.

– The proposed predictor is velocity adaptive.

with a degree of 0.68,

, …

THEN Out1 is ‘‘low’’  with a degree of 0.35, and is

‘‘medium’’ with a degree of 0.88

Fig. 5. The fuzzy rule extracted from EFuNN network.

patt ј hhSN; SN1; . . . ; S2; S1i; hS0ii	р11Ю

where Siis the motion history in ith previous time step and

S0is the current motion status which is recorded by MN.

The ﬁrst part of patt vector will be used as the input com-

ponent and the second part, ЖS0ж is the corresponding tar-

get output. Siis a vector of the following form:

Siј hRSSрBScurрiЮЮ; RSSрBS1рiЮЮ; . . . ; RSSрBSMрiЮЮ; Di; Vii

р12Ю

where M is the number of neighboring cells. The received

signal strength and the velocity (V) values are normalized

to lie in the range of [0, 1] and direction (D) is a value with-

in [0, 360] interval. The RSS(BSJ(i)) value in	Siis the re-

ceived signal strength value from Jth Base Station where

BSs are numbered in a clockwise direction around the cur-

rent cell in ith history element.

The training patterns are preprocessed by an adaptive

weighted averaging operation with variable weight, c and

constant window length (N) using the following formula:

XN

Sinputјci1Si;c 2 р0; 1	р13Ю

iј1

The parameter c is the forgetting factor which diminishes

the eﬀect of long term history. The variable weight is up-

dated based on the previously predicted MN velocity as

shown in Fig. 5. For higher velocities, the long history is

less correlated with the current status. As such, the value

of c should be smaller to diminish the eﬀect of long history

ASSI(BSCUR)

SNRSSICUR

The predictor is illustrated in Fig. 6. The inputs to this

predictor are ASSI18of currently serving BS and neighbor-

ing BSs, AV19and AD20of an MN. The outputs are the

predicted RSSIs of that MN, velocity and direction which

may be used in handoﬀ decision.

To collect the training data set, each BS uses the move-

ment history of alive MNs in its area cell. Each movement

history element includes a set of RSSs from the neighbor-

ing BSs, current velocity (V) and direction (D) of that

MN. An example of a training data set is in the form of

below:

18	Averaged Signal Strength Indication.

19	Average Velocity.

SN-1

.

.

.

.

.

S1

ASSI(BS1)

ASSI(BSM)

AD

AV

.

.

.

.

EFNN-

Based

Predictor

RSSI1

.

.

.

.

RSSIM

PD

PV

20	Average Direction.

Fig. 6. Proposed mobility predictor.

Please cite this article in press as: B.S. Ghahfarokhi, N. Movahhedinia, QoS provisioning by EFuNNs-based handoﬀ planning in cel-

lular ..., Comput. Commun. (2007), doi:10.1016/j.comcom.2007.06.001

ARTICLE IN PRESS

B.S. Ghahfarokhi, N. Movahhedinia / Computer Communications xxx (2007) xxx–xxx

7

1

0.5

0

0

Low            Medium             High

0.2	0.4	0.6	0.8	1

Degree

1

0.5

0

0

FirstQ          SecondQ     ThirdQ     FourthQ

90	180	270	360

Degree

Fig. 7. ASSI, AV, PV and RSSI membership functions.

further. Regarding that,	c can be taken proportional to

1	Vpwhere Vpis the predicted velocity. The Sinput	vector

which is the input vector to EFuNN-based predictor can be

interpreted as below:

Sinputј hASSIрBScurЮ; ASSIрBS1Ю; . . . ; ASSIрBSMЮ; AV; ADi

р14Ю

where each parameter is the averaged value of the corre-

sponding parameters in the history vectors (Si).

The training samples which are applied to EFuNN dur-

ing training phase are in the form of ЖSinput,S0ж	pairs of

vectors.

The membership functions of fuzzy variables, ASSI,

AV, PV and RSSI, are shown in Fig. 7.

The membership functions of averaged direction (AD)

and predicted direction (PD) are represented in Fig. 8.

The predicted values of RSSI and direction PD can be

used for handover decision using the adaptive fuzzy handoﬀ

algorithm with direction biasing which has been proposed in

[24]. Also, as the other purpose of this paper, the RSSI part

of the output is used in two diﬀerent proposed policies for

LSP management. These policies will be discussed in the

next section separately. In the ﬁrst policy, we select the

maximum output as the most probable next cell of presence.

In the second policy, outputs will be compared with a

threshold T to ﬁnd a set of neighboring cells which are cho-

sen as the most probable cells of presence with conﬁdence

degrees equal to the respective output values.

5. LSP management using predicted RSSIs

This research is continued by proposing and comparing

two policies for LSP management in MPLS domain using

location prediction results. We assume that MPLS domain

begins in base stations. Therefore, BSs operate as LERs

that label packets of subordinate MNs based on their des-

tinations. In the ﬁrst policy, a temporary LSP will be estab-

lished in the most probable next cell of presence for each

existing LSP. In the second policy, a set of temporary LSPs

will be established in the set of most probable cells of pres-

ence for each existing LSP. These LSPs are CR-LSP21s [10],

21	ConstRaint-based LSP.

Fig. 8. The membership functions for AD and PD fuzzy variables. There

are four fuzzy sets per each quarter of trigonometric circle.

which are utilized for QoS provisioning. These two policies

are explained in the following subsections in more detail.

5.1. Pre-established temporary LSPs

As mentioned earlier, the ﬁrst policy considers simple

LSPs in the most probable next cell of presence. Hence,

the process of LSP establishment and management is pre-

sumed in three situations as follows:

(a) When the handover occurs and the MN moves to a

new cell. If there are no pre-established temporary LSPs

for its traﬃc, these LSPs will be established between BS

and destinations for existing FECs, using LDP protocol

[9]. Then, the pre-established or currently established LSP

will be used by BS to deliver the traﬃc of that MN.

(b) At every time step. Each BS predicts next location of

its MNs and informs neighboring BSs to establish new tem-

porary LSPs before MNs handover. Moreover, if previous

prediction for each MN is diﬀerent from current predic-

tion, preceding temporary LSPs for that MN should be

omitted after a predetermined number of time steps.

(c) When an MN is leaving a cell. Current BS keeps

respective LSPs for that MN until it leaves the diversity

area. So, that MN can transfer its traﬃc using both LSPs.

5.2. Pre-established temporary CR-LSPs

In the second policy, CR-LSPs are used as temporary

pre-established paths. The outputs of proposed predictor

which are	conﬁdence degrees	are employed for setup and

hold priorities. These priorities are deﬁned between 1 and

7 in each CR-LDP and the highest priority holder is used

for permanent LSPs for each FEC. A hold and setup prior-

ity between 1 and 6 is assigned to each temporary CR-LSP

according to its	conﬁdence degree. The process of LSP

establishment and management is further described in three

situations as below:

(a) When the handover occurs and the MN moves to a

new cell. If there are no pre-established temporary CR-

LSPs for its traﬃc, these LSPs will be established with max-

imum hold and setup priority, 7. On the other hand, if

there are previously established CR-LSPs, their hold prior-

Please cite this article in press as: B.S. Ghahfarokhi, N. Movahhedinia, QoS provisioning by EFuNNs-based handoﬀ planning in cel-

lular ..., Comput. Commun. (2007), doi:10.1016/j.comcom.2007.06.001

8

ARTICLE IN PRESS

B.S. Ghahfarokhi, N. Movahhedinia / Computer Communications xxx (2007) xxx–xxx

ities will be modiﬁed to 7. Then the BS uses these CR-LSPs

for that MN.

(b) At every time step. Each BS predicts next locations

of its MNs and obtains the	conﬁdence degrees	for each

neighboring cell. Then, the BS chooses a set of next cells

of presence for each MN if their conﬁdence degrees are lar-

ger than a preset threshold T. Afterwards, BS informs cho-

sen neighboring BSs about establishment of the new

temporary CR-LSPs with setup and hold priorities accord-

ing to respective conﬁdence degrees (according to heir order

in the above set). Those BSs must establish new CR-LSPs

for each MN using CR-LDP before handover. Moreover,

if previous prediction for each MN is diﬀerent from current

prediction, precedent temporary CR-LSPs for that MN

should be omitted or their	hold priorities	should be

modiﬁed.

(c) When an MN is leaving a cell. Those CR-LSPs in

neighboring cells which are not necessary will be removed.

Moreover, currently used CR-LSPs for this MN will

remain until MN leaves the diversity area.

In the next section, the simulation results for proposed

predictor and handoﬀ management in a wireless MPLS

domain are presented.

6. Simulations results

The simulations are performed in two phases. First, the

accuracy of proposed predictor is evaluated and then, the

performance of proposed LSP management method is

examined.

Simulation environment has been implemented in NS

[20], and includes of 25 circular cells and one highway.

A population of 20 MNs is considered where half of

MNs have random mobility model while others move

with diﬀerent speeds in highways. The value of N and

the initial value of	c are chosen to be 10 and 0.9, respec-

tively. The time step has been considered to be 0.5 s, sim-

ilar to	[44] (as the measurement sampling period is 0.5 s

in GSM	[24]). We have used the propagation model

which has been proposed in [45] to compute the RSS val-

ues from the distances. Although the value of velocity is

determined by NS, authors of	[44]	have proposed several

methods to estimate the velocity. After online training of

EFuNN-based predictor using our proposed method, the

prediction error for random and regular movement pat-

terns have been computed and illustrated in	Figs. 9

and 10.

In the second phase of simulation, we consider data

delivery criterion in a packet-based network to investigate

the performance of the proposed method for LSP manage-

ment. We assume that 1 Mbps links connect each BS to

backbone, and there are four MNs which have four estab-

lished connections with four corresponding nodes (CN1,

CN2, CN3 and CN4) in backbone. Each MN transmits

1 Mbps of CBR traﬃc over its connection. The data deliv-

ery ratio is observed using four loss monitors in the corre-

sponding nodes (CNs). The normalized value of delivered

Prediction Error (in Cell)

5

4

3

2

1

0

0                      900                     1800              2700                         3600

Time steps

Fig. 9. Prediction error for random movement pattern.

Prediction Error (in Cell)

5

4

3

2

1

0

0                      900                  1800             2700                         3600

Time steps

Fig. 10. Prediction error for regular movement pattern.

data in corresponding nodes is evaluated for three diﬀerent

handoﬀ management methods. The ﬁrst method is a basic

scheme without prediction. The other two methods are

the proposed policies for LSP management using predictor.

The simulation results are shown in Figs. 11–13 for 3600

time steps using the same traﬃc pattern for those three

methods.

Utilization

1

0.8

0.6

0.4

0.2

0

0                      900                    1800                  2700                     3600

Time steps

Fig. 11. Utilization for the classic handover method.

Please cite this article in press as: B.S. Ghahfarokhi, N. Movahhedinia, QoS provisioning by EFuNNs-based handoﬀ planning in cel-

lular ..., Comput. Commun. (2007), doi:10.1016/j.comcom.2007.06.001

ARTICLE IN PRESS

B.S. Ghahfarokhi, N. Movahhedinia / Computer Communications xxx (2007) xxx–xxx

The results of simulations show that in the tradi-	Utilization

tional handoﬀ management method, there are quality

degradations during handoﬀs. Furthermore, the band-	1

width from a BS to the corresponding node is shared

9

between a numbers of traﬃc ﬂows in the current cell.

For our ﬁrst proposed method, the process of handoﬀ

is faster. Thus, the traﬃc degradation during handover

interval is lower, although there is shared links between

diﬀerent MNs.

Since the bandwidth is reserved for each traﬃc ﬂow in

the second proposed method, the traﬃc degradation is

minimized for admitted ﬂows and providing quality of ser-

vice for CBR traﬃc is more convenient. As the BS-to-core

links are band-limited, the utilization decrease occurs dur-

ing some short time intervals due to incorrect prediction

and late LSP establishments in a number of traversed cells.

This reduction is near 100% because of bandwidth reserva-

tion for other previously established LSPs.

To compare the average utilization for the three dis-

cussed methods, the utilization is moving averaged over a

500 steps averaging window. The results are shown in

Fig. 14. This ﬁgure illustrates that the CR-LSP presents

better performance, in terms of delivered data, over the

two other methods.

7. Conclusions

In this paper we considered QoS provisioning in MPLS

wireless networks by means of mobility prediction. To oﬀer

adequate resources for a ﬂow before handoﬀ, we proposed

two methods for LSP management based on location pre-

diction. A new mobility predictor using evolving fuzzy neu-

ral networks is proposed. The predictor provides enough

accuracy to reduce the number of unnecessary handovers.

Furthermore, two methods for LSP management by the

proposed predictor are proposed. Using prediction results

in resource management reduces the ping-pong eﬀect

because of existing pre-established LSPs. Moreover, using

CR-LSPs to pre-reservation of resources for diﬀerent traf-

ﬁcs has signiﬁcant eﬀects on data delivery. Simulations

results show that pre-established CR-LSPs based on the

proposed method presents superior performance in terms

Utilization

1

0.8

0.6

0.4

0.2

0

0                      900                  1800                  2700                     3600

Time steps

Fig. 12. Utilization for the pre-established LSPs method.

0.8

0.6

0.4

0.2

0

0                      900                  1800                 2700                    3600

Time steps

Fig. 13. Utilization for the pre-established CR-LSPs method.

Average Utilization

1

0.8

0.6

0.4

0.2

0

0                     900                    1800                   2700                    3600

Time steps

Without Predictor

Using pre-established LSP

Using pre-established CR-LSP

Fig. 14. Averaged utilization for the three methods.

of utilization and data delivery. The VBR traﬃc and inves-

tigating the eﬀects of proposed schemes on its quality of

service will be considered in the future research.

References

[1] B. Jabbari, R. papneja, E. Dinan, Label switched packet transfer for

wireless cellular networks, IEEE Wireless Communications and

Networking Conference (2000) 33–40.

[2] M.C. Chuan, K. Medepalli, S. Jajoo, S. Park, MPLS-Mux: an

eﬃcient protocol for providing QoS in IP-based wireless networks,

IEEE Fifty-eighth Vehicular Technology Conference (2003) 3512–

3516.

[3] K. Sethom, H. Aﬁﬁ, G. Pujolle, Wireless MPLS: a new layer 2.5

micro-mobility scheme, Mobiwac’04 (2004) 64–71.

[4] S. Vijayarangam, S. Ganesan, QoS implementation for MPLS based

wireless networks, ASEE Conference (2002) 1–9.

[5] Y. Guo, Z. Antoniou, S. Dixit, IP transport in 3G radio access

networks: an MPLS-based approach, Wireless Communications and

Networking Conference (2002) 11–17.

[6] F. Palmieri, An MPLS-based architecture for scalable QoS and traﬃc

engineering in converged multiservice mobile IP networks, Journal of

Computer Networks 47 (2004) 257–269.

[7] N. Baghaei, R. Hunt, Review of quality of service performance in

wireless LANs and 3G multimedia application services, Journal of

Computer Communication 27 (2004) 1684–1692.

[8] E. Rosen et al., MPLS architecture, IETF RFC 3031 (2001).

[9] L. Andersson et al., LDP speciﬁcation, IETF RFC 3036 (2001).

Please cite this article in press as: B.S. Ghahfarokhi, N. Movahhedinia, QoS provisioning by EFuNNs-based handoﬀ planning in cel-

lular ..., Comput. Commun. (2007), doi:10.1016/j.comcom.2007.06.001

10

ARTICLE IN PRESS

B.S. Ghahfarokhi, N. Movahhedinia / Computer Communications xxx (2007) xxx–xxx

[10] E. Jamouss et al., Constraint based LSP setup using LDP, IETF

RFC 3212 (2002).

[11] B.P. Kumar, P. Venkataram, Prediction-based location management

using multilayer neural networks, Journal of Indian Institute of

Science 82 (2002) 7–21.

[12] J. Biesterfeld, E. Ennigrou, K. Jobmann, Neural networks for

location prediction in mobile networks, Proceedings of the Interna-

tional Workshop on Applications of Neural Networks to Telecom-

munications 3 (1997) 207–217.

[13] G.Y. Liu, G.Q. Manguire, Eﬃcient mobility management support for

wireless data, VTC’95 (1995) 902–906.

[14] J. Hou, Y. Fang, Mobility-based call admission control schemes for

wireless mobile networks, Wireless Communications and Mobile

Computing (2001) 269–282.

[15] J.P. Makela, K. Pahlavan, Performance of neural network handoﬀ

algorithms under varying mobile velocities, FINSIG’05 (2005) 42–45.

[16] M.M. Islam, M. Murshed, L.S. Dooley, A direction-based bandwidth

reservation scheme for call admission control, ICCIT ’02 (2002) 345–

349.

[17] D.A. Levin, L.F. Akyildiz, M. Naghshineh, The shadow cluster

concept for resource allocation and call admission in ATM-based

wireless networks, MOBICOM’95 (1995) 142–150.

[18] A. Sleem, A. Kumar, Handoﬀ management in wireless data networks

using topography-aware mobility prediction, Journal of Parallel and

Distributed Computing 65 (2004) 963–982.

[19] G. Edward, R. Sankar, Microcellular handoﬀ using fuzzy techniques,

Wireless Networks 4 (1998) 401–409.

[20] K. Fall, K. Varadhan, The ns Manual, <http://www.isi.edu/nsnam/

ns/ns-documentation>, visited on may 2006.

[21] M. Holtzman, A. Sampath, Adaptive averaging methodology for

handoﬀs in cellular systems, IEEE Transactions on Vehicular

Technology 44 (1994) 59–66.

[22] W.R. Mande, Evaluation of a proposed handover algorithm for the

GSM cellular systems, IEEE 40th Vehicular Technology Conference

(1991) 264–269.

[23] K.G. Cornett, S.B. wicker, Bit error rate estimation techniques for

digital land mobile radios, IEEE Vehicular Technology Conference

(1991) 543–548.

[24] N.D. Tripathi, Generic Adaptive Handoﬀ Algorithms Using Fuzzy

Logic and Neural Networks, Ph.D. Thesis, The Virginia Polytechnic

Institute and State University (1997).

[25] H.M. Lozoya et al., Handoﬀ algorithms based on fuzzy classiﬁers,

IEEE Vehichular Technology 49 (2000) 2286–2294.

[26] K. Vikorn, B. Homman, W. Benjapolakul, Comparative evalu-

ation of ﬁxed and adaptive soft handoﬀ parameters using fuzzy

logic inference systems in CDMA mobile communication sys-

tems, IEEE Vehichular Technology Conference 2 (2001) 1017–

1021.

[27] H. Bongkarn, W. Benjapolakul, Trunk-resource-eﬃciency-controlling

soft handoﬀ based on fuzzy logic and gradient decent method, IEEE

Vehichular Technology Conference (2001).

[28] H. Pahlavan et al., Handoﬀ in hybrid mobile data networks, IEEE

Personal Communication 7 (2000) 34–47.

[29] K.D. Wong, D.C. Cox, A pattern recognition system for handoﬀ

algorithms, IEEE Journal on Selected Areas in Communication 18

(2000) 1301–1312.

[30] S. Mangold, S. Kyriazakos, Applying pattern recognition techniques

based on hidden Markov models for vehicular poition location in

cellular networks, IEEE Vehichular Technology Conference (1999)

780–784.

[31] R. Narasimhan, D.C. Cox, A handoﬀ algorithm for wireless systems

using pattern recognition, PIMRC’98 (1998).

[32] T. Onel et al., A multicriteria handoﬀ decision scheme for the next

generation tactical communications system, Computer Networks 46

(2004) 695–708.

[33] M. Hata, T. Nagastu, Mobile location using signal strength

measurements in a cellular system, IEEE Vehichular Technology

Conference (1980) 245–252.

[34] N. Kasabov, Evolving fuzzy neural networks for supervised/ unsu-

pervised on-line knowledge-based learning, IEEE Transactions on

Systems, Man and Cybernetics, Part B: Cybernetics 31 (6) (2001) 902–

918.

[35] A.M. Taha, H. Hassanein, H.T. Mouftah, Integrated solutions for

wireless MPLS and Mobile IP: current status and future directions,

CCECE’ 04 (2004) 1463–1466.

[36] A. Abraham, Neural fuzzy systems: state-of-the-art modeling tech-

niques, Connectionist Models of Neurons, Learning Processes, and

Artiﬁcial Intelligence, in: Jose Mira, Alberto Prieto Eds.), Springer-

Verlag, Berlin, 2001, pp. 269–276.

[37] J. Principe, N. Euliano, W. Lefebvre, Neural and Adaptive Systems:

Fundamentals Through Simulations, John Wiley & Sons, Inc., New

York, 2000.

[38] N. Samman, A. Karmouch, A mobility prediction architecture based

on contextual knowledge and spatial conceptual maps, IEEE Trans-

actions on Mobile Computing 4 (6) (2005) 537–551.

[39] N. Tripathi et al., Handoﬀ in cellular systems, IEEE Personal

Communications (1998) 26–37.

[40] C.G. Gaytan, et al., Uncertainty in cellular fuzzy handover, expert

systems with applications, 15 (1998) 391–397.

[41] A. Majlessi, B.H. Khalaj, An adaptive fuzzy logic based handoﬀ

algorithm for internetworking between WLANS and mobile net-

works, PIMRC’02 (2002).

[42] M.S. Dang et al., Fuzzy logic based handoﬀ in wireless networks,

VTC2000 (2000) 2375–2379.

[43] C. Chang, et al., Using fuzzy logic to mitigate IEEE 802.11 handoﬀ

latency, Fourteenth Conference on Fuzzy Systems (2005) 37–42.

[44] M.D. Austin, G.L. Stuber, Velocity adaptive handoﬀ algorithms for

microcellular systems, IEEE Transactions on Vehicular Technology

43 (3) (1994) 549–561.

[45] P. Dassanayake, Eﬀects of measurement sample averaging on

performance of GSM handover algorithm, Electronic Letters 29

(1993) 1127–1128.

Behrouz Shahgholi Ghahfarokhi received his B.Sc.

in Computer Engineering from University of

Isfahan, Isfahan, Iran in 2004, and his M.Sc. in

Artiﬁcial Intelligence from University of Isfahan

in 2006. Currently, he is the Ph.D. student in

Computer Architecture at University of Isfahan.

His research interests are computer networks,

wireless technology and artiﬁcial intelligence.

Naser Movahhedinia	received his B.Sc. from

Tehran University, Tehran, Iran in 1967, and his

M.S.E from Isfahan University of Technology,

Isfahan, Iran in 1990 in Electrical and Commu-

nication Engineering. He got his Ph.D. degree

from Ottawa-Carleton Graduate Institute, Otta-

wa, Canada in 1997, where he was a research

associate at System and Computer Engineering

Department, Carleton University for a short

period after graduation. Currently he is an

assistant professor at Computer Department,

The University of Isfahan. His research interests are wireless networks,

personal communications systems and Internet Technology.

Please cite this article in press as: B.S. Ghahfarokhi, N. Movahhedinia, QoS provisioning by EFuNNs-based handoﬀ planning in cel-

lular ..., Comput. Commun. (2007), doi:10.1016/j.comcom.2007.06.001