Computer Communications 26 (2003) 845–860

www.elsevier.com/locate/comcom

Quality of service provisioning through trafﬁc engineering with

applicability to IP-based production networks

Panos Trimintziosa,*, Timothy Baugeґb, George Pavloua, Paris Flegkasa, Richard Eganb

aCentre for Communication Systems Research, University of Surrey, Guildford, Surrey, GU2 7XH, UK

bThales Research Ltd., Worton Drive, Reading, RG2 0SB, UK

Received 7 August 2002; accepted 7 August 2002

Abstract

Production networks require the transport of high-quality multimedia trafﬁc between outside broadcast vans and the main studio. This is

typically done through dedicated terrestrial or satellite links, with bandwidth purchased from third party network providers, which is

expensive and lacks ﬂexibility. Given the emergence of IP networks and the Internet as the multi-service network of choice, it is plausible to

consider their use for transporting production network trafﬁc with high bandwidth and low delay and packet loss requirements. Emerging

technologies for quality of service such as Differentiated Services and MPLS can be used for premium quality trafﬁc. In this paper we try to

use the emerging IP technologies to support services like production network trafﬁc. We present a Trafﬁc Engineering and Control System

that starts from agreed services with customers and provisions the network according to the expected trafﬁc demand so as to meet the

requirements of contracted services while optimising the use of network resources. We devise a non-linear programming formulation of the

problem and show through extensive simulations that we can achieve the objectives and meet the requirements of demanding production

network trafﬁc. Our solution is generic enough and not only tuned to production networks, so it can be used in other contexts for supporting

services with stringent quality of service requirements.

q 2002 Elsevier Science B.V. All rights reserved.

Keywords: Quality of service; Trafﬁc engineering; Differentiated services; Production networks

1. Introduction

Differentiated Services	[1]	is seen as the emerging

technology to support Quality of Service (QoS) in IP

backbone networks in a scalable fashion. Multi-Protocol

Label Switching (MPLS) [2] can be used as the underlying

technology to support Trafﬁc Engineering (TE). It is

possible to use these technologies to support production

network or other multimedia trafﬁc with stringent real-time

requirements. This can be done through careful trafﬁc

forecast based on contracted premium services with

customers and subsequent Network Provisioning (NP) in

terms of routing and resource management strategies.

Production network trafﬁc has very stringent require-

ments in terms of bandwidth, delay and loss. Today,

production networks use dedicated high-capacity network

infrastructures. These are inﬂexible and very expensive. In

* Corresponding author. Tel.:	ю 44-1483-686005; fax:	ю44-1483-

686001.

E-mail address: p.trimintzios@eim.surrey.ac.uk (P. Trimintzios).

this paper we show that it is feasible to support such

demanding trafﬁc requirements in a generic IP-based

network environment. This can be achieved by taking

advantage of the emerging IP technologies, Differentiated

Services (DiffServ) and MPLS, and through careful

provisioning and service management and control

decisions. Although the key motivation behind our work

is the support of production network trafﬁc, our ﬁndings are

generic enough and not only tuned to production networks,

so that they can be used in other contexts for supporting

services with stringent quality of service requirements.

This rest of this paper is organised as follows. Section 2

introduces production networks and their connectivity

requirements and Section 3 describes the related work. In

Section 4 we describe the QoS provisioning system for

supporting production network trafﬁc and describe its

operational cycle. Section 5 describes the NP algorithm

that is based on a non-linear programming formulation of

the problem. In Section 6 we provide the experimental

evaluation of our TE system through simulations. Finally, in

Section 7 we present our conclusions.

0140-3664/03/$ - see front matter q 2002 Elsevier Science B.V. All rights reserved.

PII: S 0 1 4 0 - 3 6 6 4 ( 0 2 ) 0 0 2 1 8 - 9

846

P. Trimintzios et al. / Computer Communications 26 (2003) 845–860

2. Packet-based production networks

Two types of studios are identiﬁed in production

networks: the Main Studio and Outside Broadcast (OB)

Vans. The main studio is where the ﬁlming takes place,

where the cameras, microphones, television monitors and

speakers are found. It accommodates the producer who

manages the personnel through talkback headset and

controls how the programme is put together. A storage

area is the part of the studio where audio, video and data

need to be stored, either short term for deferred broadcast or

long term for archiving. The second type of studios are the

OB Vans, which are mobile studios, taken to ﬁlm events

outside the main studio, such as races, concerts, sport

events, foreign correspondents, etc. They contain limited

production facilities, but enough to produce a broadcast

quality program from the range of cameras and microphones

deployed at the event. If the event is to be covered live, OB

vans can send the programme to the main studio in real time

via dedicated networks (satellite or terrestrial networks).

The production network audio and video trafﬁc is the

industry standards. For video the dominant standard is the

Serial Digital Interface (SDI). It produces 270 Mbps of

video with ancillary data for audio, metadata and time-

codes. The main standard for production audio (as opposed

to talkback which uses low quality audio channels) is AES3

(from the Audio Engineering Society). It can be transmitted

separately or embedded in SDI. AES3 contains two

channels, which can accommodate a stereo pair. The

transmission rate is 1536 Kbps.

2.1. Studio connectivity

The production network consists of all the video and

audio devices. They are divided into three separate

networks: the video network, the audio network and the

talkback network for the intercommunication trafﬁc.

Additionally, control and synchronisation signals must be

distributed across the site. For each network there is a

central studio router to which devices connect using

dedicated point-to-point links. The studio router is exclu-

sively designed to interconnect audio or video devices using

audio or video industry standards. Conﬁguring a studio for a

production session requires a signiﬁcant amount of work.

Audio, video, synchronisation and control connectivity

must be provided via separate cabling. Many devices use

proprietary protocols and cabling which increases the

complexity of interconnection.

Designing an IP network to replace the existing video,

audio, talkback and control networks inside a studio is not a

trivial task. Switched local area network technologies such

as Fast and Gigabit Ethernet can provide the required

capacity. However, the performance expectations placed on

the network require stringent quality of service guarantees.

This paper focuses only on the use of IP between studios

rather than inside them, the various approaches may be used

to provide adequate quality of service within the studio,

including over-provisioning and the use of the IP Integrated

Services framework.

2.2. Inter-studio connectivity requirements

Inter-studio connectivity is either studio to studio or OB

Van to studio. Connectivity may be through dedicated

terrestrial or satellite links, depending on availability and

economic factors, with bandwidth purchased from third

party network providers. Given the cost of such connec-

tivity, only one multimedia stream is sent from the van to

the main studio, possibly with talkback channels and control

signals if required. The latency on these links is orders of

magnitude higher than for intra-studio connections, but this

is acceptable as far as there are few feedback loops that use

the inter-studio link. In some cases such loops are

unavoidable, such as a speaker in the main studio

interviewing a foreign correspondent the other side of the

world. However, no control loops are required as each

studio (or OB Van) is autonomous. Bandwidth requirements

may also be relaxed by using compression.

The increasing availability of general purpose IP net-

works make them attractive candidates for interconnecting

studios and OB Vans. However, the issue of quality of

service over these networks must be successfully addressed

before they are seriously considered for this demanding

application. The current best effort paradigm is not

acceptable for real time video and audio transfer requiring

broadcast studio quality.

In order to provide adequate quality guarantees over an

IP Wide Area Network (WAN) that serves multimedia

production trafﬁc, we propose to use the DiffServ frame-

work together with MPLS for TE.

Fig. 1	shows a high level overview of the proposed

solution. We consider a studio and an OB van requiring to

exchange an SDI video ﬂow, a number of AES3 audio

streams, talkback trafﬁc and control signals. They are

connected over an IP WAN, which supports MPLS-TE and

DiffServ. As the sources and sinks send packets to the

WAN, routing protocols will determine paths across the

networks, usually based on a shortest path algorithm.

Conventional protocols do not take into account the load on

different parts of the network, which may lead to certain

areas of the network being congested while others under-

utilised. DiffServ selectively distributes resources among

the various classes of trafﬁc, which helps maintain quality

for selected trafﬁc aggregates but does not tackle the

congestion problem. In order to provide quality guarantees

while optimising the use of network resources, it may be

best to take a longer but less congested path through the

network [3]. This requires alternative methods of routing to

complement the operation of DiffServ.

This is achieved through TE, by explicitly routing trafﬁc

through the network using a set of calculated paths that

provide better control of the network load. Fig. 1 shows two

P. Trimintzios et al. / Computer Communications 26 (2003) 845–860

Fig. 1. Inter-studio connectivity over IP.

847

different paths between the OB Van and the studio, one

requiring 3 hops and the other 4. In this example, the

shortest path could be used to route the video stream, which

requires minimal delay, whereas the less critical talkback

trafﬁc would be kept separate and made to use the longer

path.

A trafﬁc engineering management system is used to

determine and enforce the trafﬁc engineered paths. Deter-

mining these paths in order to deliver the required quality of

service for the expected trafﬁc matrix is far from trivial. The

remainder of this paper focuses on the functionality, design

and evaluation of the proposed TE and QoS Delivery

Management System in the context of IP WANs that can

serve high-volume high-quality trafﬁc as required by IP-

based inter-studio production networks.

3. Related work

The problem of TE has attracted a lot of attention in recent

years. TE entails the aspect of network engineering that is

concerned with the design, provisioning, and tuning of

operational Internet networks. In order to deal with this

important emerging area, the Internet Engineering Task

Force (IETF) has chartered a Trafﬁc Engineering Working

Group (TE-WG) to deﬁne, develop, specify, and recommend

principles, techniques and mechanisms for TE in the Internet.

The main output of this working group until now is that it has

deﬁned the basic principles for TE [4] and the requirements

to support the interoperation of MPLS and Diffserv for TE

[5]. It is in the plans of this group to look into technical

solutions for meeting the requirements for Diffserv-aware

MPLS TE, the necessary protocol extensions, interoper-

ability proposals and measurement requirements.

Two similar works with the work presented in this paper

are Netscope	[6]	and RATES	[7]. Both of them try to

automate the conﬁguration of the network in order to

maximize network utilisation. The ﬁrst one uses measure-

ments to derive the trafﬁc demands and then by employing

the ofﬂine algorithm described in Ref. [8] it tries to ofﬂoad

overloaded links. The latter uses the semi-online algorithm

described in Ref. [9] to ﬁnd the critical links which if they are

chosen for routing will cause the greatest interference (i.e.

reduce the maximum ﬂow) of the other egress – ingress pairs

of the network. Both of these works do not take into account

any QoS requirements and only try to minimize the

maximum load of certain links.

The work described in this paper can be categorised as

time-dependent ofﬂine trafﬁc engineering [4]. Such problems

can modelled as multi-commodity network ﬂow optimis-

ation problems	[10]. The related works use optimisation

formulations, focusing on the use of linear cost functions,

usually the sum of bandwidth requirements, and in most of

the cases they try to optimise a single criterion, minimize

total network cost, or combine multiple criteria in a linear

formula.

In Mitra et al. [11]	the TE problem is seen as a multi-

priority problem which is formulated as a multi-criterion

optimisation problem on a predeﬁned trafﬁc matrix. This

approach uses the notion of predeﬁned admissible routes,

which are speciﬁc for each QoS class, and each source –

destination pair, where the objective is the maximization of

the carried bandwidth. In Ref. [12], the authors address the

resource allocation and routing problem in the design of

Virtual Private networks (VPNs). The main objective is to

design VPNs which will have allocated bandwidth on the

links of the infrastructure network such that, when the trafﬁc

of a customer is optimally routed, a weighted aggregate

measure over the service provider’s infrastructure is

maximized, subject to the constraint that each VPN carries

a speciﬁed minimum. The weighted measure is the network

revenue, which is a function of the trafﬁc intensity. The

algorithm proposed in that paper solves ﬁrst the optimal

routing problem for each VPN independently. Then

calculates for each VPN the linear capacity costs for all the

links. These quantities are used to modify appropriately the

current capacity allocations so that the network revenue of

the infrastructure network for the new capacities is

maximized. In Ref. [12] it is shown that this is equivalent

to minimizing a linear function of the capacity costs subject

to constraints imposed by the link capacities.

In Ref. [13] a model is proposed for off-line centralized

TE over MPLS. This uses the following objectives: resource-

oriented or trafﬁc-oriented TE	[4]. The resource-oriented

848

P. Trimintzios et al. / Computer Communications 26 (2003) 845–860

problem targets load balancing and minimization of resource

usage. Capacity usage is deﬁned as the total amount of

capacity used and load balancing is deﬁned as one minus the

maximal link utilization. The objective function that has to be

maximized is a linear combination of capacity usage and load

balancing, subject to constraints imposed by the capacity of

the links. The trafﬁc-oriented model suggests an objective

function that is a linear combination of fairness and

throughput, where throughput is deﬁned as the total

bandwidth guaranteed by the network and fairness as the

minimum weighted capacity allocated to a trafﬁc trunk. In

Ref. [14] the authors propose an algorithm which has two

phases, a pre-processing phase and an on-line one. In the pre-

processing phase the algorithm uses the notion of multi-

commodity ﬂows, where commodities correspond to trafﬁc

classes. The goal is to ﬁnd paths in the network to

accommodate as much trafﬁc as possible from the source to

the destination node. The algorithm tries to minimize a linear

cost function of the bandwidth assigned to each link for a

trafﬁc class. The second phase performs the on-line path

selection for LSP requests by using the pre-computed output

of the multi-commodity pre-processing phase.

Works like	[8,15,16]	try to achieve optimal routing

behaviour by appropriately conﬁguring the shortest path

routing metrics, assuming no MPLS is supported. Wang

et al.	[15]	proved theoretically that any routing conﬁgur-

ation, including the optimal one, could be achieved by the

appropriate setting of the shortest path routing metrics.

Finally, regarding online algorithms, they are mainly

based on extensions of the QoS-routing paradigm	[17].

These approaches are heuristics, recently known in the IETF

as Constraint-Shortest Path First (CSPF), which utilise

information kept in TE databases populated through

information obtained from the routing ﬂooding mechanisms

about link capacities, unreserved capacity, colour afﬁnities

etc. Other online TE approaches	[18,19]	mainly focus on

load balancing on multiple equal or non-equal cost paths.

These works are complementary to work of this paper, since

they can be used in conjunction.

4. A provisioning system for QoS delivery and trafﬁc

engineering

This section presents the functional model that will

enable an Internet Service Provider (ISP) to conﬁgure and

provision its autonomous system (AS) in order to satisfy the

requirements of inter-studio production network trafﬁc. We

are focusing on deﬁning a time-dependent IP Trafﬁc

Engineering and Control System that operates at medium

(minutes to days) and long (weeks to months) timescales

[4].

We assume that customers1will have bilateral Service

Level Agreements (SLAs) that contain both an adminis-

1	In this article we assume that a customer is an owner of a production

network as described in Section 2.

trative aspect (terms, conditions, pricing information, etc.)

and a technical part known as a Service Level Speciﬁcation

(SLS)	[20]. Our system focuses to satisfy the SLSs of

quality-critical customers.

In the rest of the section, we will present the overall

proposed system architecture and we will elaborate on the

operational timescales.	Fig. 2	shows the model of the

proposed system, which has evolved from our initial quality

of service framework described in	Ref. [21], aspects of

which have also been developed further in Ref. [22].

The system in Fig. 2 is designed to provide an automated

mechanism to determine, implement and monitor network

conﬁgurations, which satisfy the following criteria:

1. Deliver the services as deﬁned in SLSs (customer

perspective),

2. Optimise the use of network resources (ISP perspec-

tive)

The above system is decomposed into three main sub-

systems: SLS management, TE and Monitoring. While the

main focus of this paper is on the TE part, we will also

brieﬂy describe the functionality of all the sub-systems for

completeness.

SLS management is responsible for SLS negotiation and

admission control. The negotiation and admission control

process by which SLSs are accepted or refused is outside the

scope of this paper. We assume that all existing SLSs, in the

SLS repository, have been admitted by an external process

and that no SLS will be added or deleted. Note though this is

not strictly true since new SLSs are typically added and

deleted in the system during its operation; these are only

taken into account by the TE system in the next provisioning

cycle. The basic production network SLSs we consider here

are described in Section 4.3. Trafﬁc forecast tries to predict

the anticipated trafﬁc demand based on the current

subscribed SLSs, policy-based over-provisioning factors,

and data both from SLS usage measurements and the history

of past predictions.

We monitor network performance in order to assess our

time-dependent periodic provisioning solution. Getting

monitoring feedback is very important for both time and

state-dependent [4] TE approaches, but only the latter uses

monitoring feedback to react to network conditions.

NP performs the main task of TE by periodically (i.e.

every provisioning period) running an ofﬂine algorithm

which computes the network conﬁguration in terms of

explicitly routed paths and capacity requirements for the

queues of each router; these are calculated according to the

forecasted demands. Route and resource managers are

responsible to apply the proper network conﬁguration for

every forecasting period. A route manager exists per ingress

router and sends explicit MPLS Labelled Switched Path

(LSP) set-up conﬁguration requests. LSPs are set-up

through a label distribution protocol, e.g. Constraint-based

routing label distribution protocol (CR-LDP) or the extended

P. Trimintzios et al. / Computer Communications 26 (2003) 845–860

Fig. 2. Time-dependent IP TE and control system.

849

resource reservation protocol for TE (RSVP-TE). A

resource manager exists in every router and is responsible

for applying the conﬁguration of the DiffServ Per-Hop

Behaviours (PHBs) for all the router interfaces.

4.1. Provisioning and forecasting scheduling

In this section we will describe the timing and scheduling

properties of our system. We assume that scheduling in the

proposed model is performed at two levels:

†	The NP Scheduler, which deﬁnes provisioning periods.

†	The Trafﬁc Forecast Scheduler, which deﬁnes forecast-

ing periods.

4.1.1. Network provisioning scheduler

The IP Trafﬁc Engineering and Control System aims to

provision the network in order to provide the required QoS to

contracted SLSs while at the same time optimising the usage

of resources. As the trafﬁc requirements that are derived from

the SLSs change over time, the provisioning guidelines need

to be updated. This is performed periodically through the NP

Scheduler. The system is required to re-calculate a set of

provisioning guidelines for each provisioning period. As the

current period comes to an end, the Provisioning Scheduler

will trigger re-provisioning of the network. Typical frequen-

cies for the scheduler could be once a day or once a week.

The scheduler is part of the TE sub-system, and relies on

the provisioning period schedule; the latter provides the

frequency of the provisioning period.

4.1.2. Trafﬁc forecast scheduler

As the provisioning periods are quite long (days, weeks),

the trafﬁc requirements may vary during a provisioning

period. The granularity of the NP Scheduler is not ﬁne

enough to optimise the conﬁguration of the network. We

therefore use a second scheduler, the Trafﬁc Forecast

Scheduler, which has the following characteristics:

†	It is not necessarily periodic (could schedule one 4 h

period followed by one 10 h period)

†	For all forecasting periods	FP(i ) that constitute a

provisioning period PP, it holds: FP(i ) # PP

The principle behind the double scheduling is that the NP

system is invoked once every provisioning period, and

calculates multiple conﬁgurations, which will be enforced at

each forecasting period. The various conﬁgurations are

successively applied at the appropriate times (triggered by

Trafﬁc Forecast Scheduler) by the Resource and Route

managers.

The Trafﬁc Forecast Scheduler is part of our TE sub-

system, which relies on the forecasting period schedule. The

latter deﬁnes the forecasting periods.

4.1.3. Scheduling example

To clarify the roles of each scheduler, this section

provides some examples.

Consider that provisioning period is a week and that

Trafﬁc Forecast Scheduler deﬁnes 4 – 6 h forecasting

periods (forecasting period being periodic in this case).

This means that once a week, four logical topologies are

calculated: one for the morning, one for the afternoon, one

for the evening and one for the night (6 h periods). During

each day of the week, Trafﬁc Forecast Scheduler triggers at

06:00 to enforce the morning conﬁguration, at 12:00 to

enforce the afternoon conﬁguration, at 18:00 to enforce the

evening conﬁguration and at 24:00 to enforce the night

conﬁguration. This is shown in Fig. 3.

850

P. Trimintzios et al. / Computer Communications 26 (2003) 845–860

Fig. 3. Two level scheduling.

There is no requirement for Trafﬁc Forecast Scheduler to

be periodic. For example, we could have four forecasting

periods per day, morning, afternoon, evening and night with

different lengths as shown in Table 1.

Another more realistic and more complex example could

be to have ﬁve forecasting periods: weekday morning,

afternoon, evening, night, and all weekend. The interesting

feature of this arrangement is that the Trafﬁc Forecast

Scheduler does not trigger the enforcement of the

conﬁgurations in a simple cyclical fashion: the 4 weekday

conﬁgurations must be implemented cyclically for 5 days,

and the weekend conﬁguration is enforced for the remaining

two days of the week.

4.2. Operational cycle: a simple scenario

In order to illustrate the notions described above we will

walk through the sequence of events, which make up a full

operational cycle. Refer to Fig. 2 for the information ﬂow

described in this example. Fig. 4 shows a part of Fig. 3 in

detail, covering one provisioning cycle and the ﬁrst

conﬁguration enforcement.

4.2.1. Initialisation

There are two prerequisites for the operation of the

model. First, a number of SLSs have been agreed between

various customers (on behalf of users such as studios, OB

vans etc.) and the network operator. We assume that the

negotiation phase has taken place and the admitted/per-

mitted SLSs do not exceed the overall network capacity. No

SLS will be added after the initialisation phase. The SLSs

are entered into the SLS repository. Second, the network

topology (including the link capacities) exists in the

request. Then, the Trafﬁc Forecast Scheduler requests the

forecasting period schedule. It is important that the Trafﬁc

Forecast and Trafﬁc Forecast Scheduler have identical

views of the forecasting period schedule.

4.2.2. Operation

This scenario starts just before the beginning of a

provisioning period, at marker 1 in Fig. 4. The NP Scheduler

triggers NP to calculate a set of logical topologies for each

forecasting period of the forthcoming provisioning period.

In order to achieve this, NP requires the trafﬁc forecasts for

the given period, i.e. the forecasts for each forecasting

period in the next provisioning period.

In order to compute these forecasts, Trafﬁc Forecast

requests from the SLS repository a list of SLSs, which are

expected to be active during the next provisioning period.

Based on these SLSs, forecasts are generated, one for each

forecasting period, and are passed to NP (marker 2 in Fig. 4).

Trafﬁc Forecast requests the SLSs only after NP has

requested the forecast.

NP calculates a logical trafﬁc engineered topology for

each received forecast (i.e. forecasting period), so as to

provide the agreed QoS requirements while at the same time

optimising resource utilisation. These conﬁgurations are

stored in the C&C database (marker 3 in	Fig. 4). NP

Scheduler is inactive until the next re-provisioning is due,

i.e. before the end of the next provisioning period.

The enforcement of the conﬁgurations is the responsi-

bility of Trafﬁc Forecast Scheduler. At the appropriate times

(i.e. at each forecasting period boundary), Trafﬁc Forecast

Table 1

Forecasting periods schedule example

Connectivity and Conﬁguration (C&C) database. The

topology will not be changed after the initialisation phase.

The C&C database makes the topology available to

Network Provisioning and Network Monitoring upon

Morning

Afternoon

Evening

Night

08:00 – 12:00

12:00 – 17:00

17:00 – 22:00

22:00 – 08:00

P. Trimintzios et al. / Computer Communications 26 (2003) 845–860

851

Fig. 4. Scheduling detail, re-provisioning and conﬁguration enforcement.

Scheduler triggers the enforcement of the new conﬁguration.

Marker 4 in Fig. 4 shows the Trafﬁc Forecast Scheduler

triggering the enforcement of conﬁguration ‘c5’. Route

Managers and Resource Managers enforce2the conﬁgur-

ation. The Route Manager focuses on provisioning the trafﬁc-

engineered paths, while the Resource Manager conﬁgures

the buffers and PHBs of the interfaces of each network node.

Here we assume that the implementation of the PHBs on the

various interfaces allows conﬁguration, by deﬁning an upper

limit on the buffer, and by either setting the relative weight

(Weighted Fair Queuing-WFQ) and/or the relative priority

(Class-Based Queuing or Priority Queuing).

The Trafﬁc Forecast Scheduler also causes monitoring to

retrieve the new logical topology information in order to

monitor the performance of the current network

conﬁguration.

4.3. From SLSs to classes of service

SLS management in the context of this paper is

responsible for providing trafﬁc forecasts to the NP. The

forecasts are based on the following information:

2	Note that a very important operational consideration has to do with the

transition from one conﬁguration to another with the minimal disruption to

the network and the current trafﬁc. This issue, though very important, is

outside the scope of this article, but a simple way out could be to apply the

new conﬁguration at the same time with the old. Then wait for all the

already established ﬂows of old conﬁguration to drain before tearing it

down, while at the same time all the new trafﬁc demand is directed through

the new conﬁguration.

†	SLSs negotiated between the customers and network

operator.

†	Policy data set by the operator, including the forecasting

period schedule.

†	Measurements and historical data.

The SLS repository is a passive database. In this work we

assume no negotiation or admission control procedure and

the SLSs are entered through a user interface. The SLSs are

indexed by start and end time. We are considering the

following SLS types: video (SDI or compressed), audio

(AES3 or compressed), which both have stringent delay and

loss requirements, and production network control trafﬁc

which has less strict delay and certainly smaller throughput

requirements. Note that for performance assessment pur-

poses we also assume some background trafﬁc SLSs, for

different kinds of trafﬁc than the studio production network

types described above.

4.3.1. Trafﬁc forecast

The role of the Trafﬁc Forecast is to estimate the

anticipated trafﬁc for each forecasting period. The forecast-

ing periods are determined based on the forecasting period

schedule. For each forecasting period, four successive

functions are performed: translation, mapping, aggregation,

and forecasting.

Translation. The Customer SLSs stored in the repository

use a customer-orientated terminology. These SLSs have

the following semantics. They are the either bi-directional

or unidirectional and follow either the pipe or the VPN

model. The ﬁrst function of translation is to decompose

these SLSs into a number of simple unidirectional SLSs,

which follow only the pipe model (one ingress to one

egress). The ingress and egress are speciﬁed in geographical

terms, therefore there is a need to translate those into IP

source – destination addresses and from them to infer the

speciﬁc ingress – egress addresses of our Autonomous

System (AS). Services are given names (Olympic Gold/-

Silver/Bronze Service, Virtual Leased Line). This terminol-

ogy must use networking semantics (throughput, delay,

loss). So, another important aspect of translation is from the

customer SLS to the appropriate network parameters.

Mapping. The simple unidirectional SLSs could poten-

tially request (or be translated to) any value of delay, loss or

throughput, while the network supports only a few discrete

values. The mapping function is therefore to map the SLS

requirements to the services actually supported by the

network, i.e. the Classes of Service (CoS). Another

important function is to map the IP source – destination

addresses. To illustrate the mapping function, consider the

following simple example. A customer SLS may require

‘1 Mbps of Virtual Leased Line Premium service with

50 ms delay between London and Manchester,…’. The

translation function would translate this Customer SLS into

to two simple unidirectional SLSs: ‘1 Mbps, EF, 50 ms

delay, from 122.12.2.143, egress 103.124.32.111,…’ and

852

Table 2

P. Trimintzios et al. / Computer Communications 26 (2003) 845–860

Deﬁnitions of class of service and trafﬁc trunk

Class of service (CoS)	PHB scheduling class (PSC)

Delay range

Loss probability range

Trafﬁc Trunk

Ingress IP address

Egress IP address

CoS

Throughput requirement

another one with the same values but in the opposite

direction. Suppose the network offers two services: a 10 and

100 ms delay premium virtual wire services, therefore we

need to map the SLS to the 10 ms service. The mapping is

static as far as the mapping algorithms are ﬁxed and

deterministic. The result is a list of SLSs that are active

during the next provisioning period and are deﬁned in

network terms.

Aggregation. Up to this point of the forecasting

procedures we had information proportional to the number

of customers and SLSs. For scalability, we base our TE

solution on trafﬁc loads per ingress/egress pair and per class

of service (see Section 2). We use the concept of trafﬁc

trunk [23], which is deﬁned by the ingress/egress node pair,

the class of service and the capacity requirement (Table 2).

The simple unidirectional SLSs are aggregated into trafﬁc

trunks by ‘adding’ their throughput requirements if and only

if they have the same ingress/egress pair and they require a

similar treatment, i.e. they have the same CoS values. We

deﬁne the distinct CoS to be uniquely identiﬁed by the

required PHB Scheduling Class [24], a delay range and loss

probability range (Table 2).3SLSs may be multiplexed. If

two clients require the same service but at different times

within a forecasting period, they are multiplexed in the

trafﬁc forecast. Note that multiplexing may occur within a

class of service, but not between classes.

Forecasting	has two main functions. On one hand, an

over-subscription factor per CoS is included. This factor is

deﬁned as the ratio of the capacity reserved by all the SLSs

in a given CoS to the capacity expected to be actually used.

For expensive SLS types, the over-subscription factor is

likely to be one. For cheaper services, the factor may be

larger. On the other hand, at this stage we have all the

ingress – egress demand (trafﬁc matrix). It is possible to run

an extrapolation algorithm utilising the information on the

history of trafﬁc matrices. Candidate algorithms are the

spline or more generally any polynomial extrapolation

method.

5. A trafﬁc engineering solution

Network Provisioning is the main module of our TE

system (Fig. 2). It provides guidelines for provisioning the

network according to trafﬁc estimates and resource

utilisation policies (see Section 4.3.1).

3	Normally a SLS and consequently a CoS will have a delay variation

(jitter) parameter, but since our current TE solution does not take it into

account (Section 5) we are omitting it from the table.

The objectives are both trafﬁc and resource-oriented. The

former relate to the obligation towards customers, through

the SLSs. These obligations induce a number of restrictions

about the treatment of trafﬁc. The resource-oriented

objectives are related to the network operation, more

speciﬁcally they are results of the high-level business policy

that dictates the network should be used in an optimal

fashion. Congestion is the main cause of network perform-

ance degradation that inﬂuences both these TE objectives.

Two are the main reasons behind congestion: either the

demand exceeds the network capacity, or the trafﬁc is not

spread optimally, causing parts of the network to be over-

utilised while others are under-utilised. The ﬁrst reason can

only be handled by adequate network planning, i.e. adding

more physical resources. The latter can be handled by

adequate bandwidth and routing management, and this is

what NP is aiming for.

NP has as input the network topology (including link

capacities), the estimated trafﬁc matrix (expected trafﬁc

trunks, see Section 4.3.1), and resource-oriented policy

directives4. It will provide as an output a list of explicitly

routed paths for each source destination included in the

trafﬁc matrix, and the capacity requirements for each PHB

scheduling class, and therefore physical queue	[24]	for

every interface of all the network nodes.

5.1. Network and trafﬁc model

The network is modelled as a directed graph G ј рV ; EЮ;

where V is a set of nodes and E a set of links. Each link l [ E

is speciﬁed by the pair l ј рvl;i;vl;eЮwhere vl;i;vl;e	are the

nodes where trafﬁc is entering and exiting the link,

respectively.

With each link	l [ E	we associate the following

parameters: the link physical capacity	Cl, the link

propagation delay dlprop;the set of the physical queues H,

i.e. PHB Scheduling Classes (PSCs), supported by the link.

For each PSC, h [ H we associate a bound dhl	(deterministic

or probabilistic depending on the PSC) on the maximum

delay incurred by trafﬁc entering link l and belonging to the

h [ H, and a loss probability phl	of the trafﬁc entering link l

and belonging to h [ H.

The basic trafﬁc model of NP is the trafﬁc trunk. A trafﬁc

trunk is an aggregation of a set of trafﬁc ﬂows characterised

by similar end-to-end performance requirements [3]. Also,

each trafﬁc trunk is associated with one ingress node and

one egress node, and is unidirectional. Each trunk

4	In this work we do not consider the way policies inﬂuence our system.

For a comprehensive discussion of policies in this context see Ref. [25].

P. Trimintzios et al. / Computer Communications 26 (2003) 845–860

853

corresponds to a CoS as deﬁned in the context of this paper

(see Section 4.3.1). The set of all trafﬁc trunks is denoted by

T. Each trunk t [ T, from ingress node vito egress node ve

(vi; ve[ V ), is associated with a bandwidth requirement, Bt:

The following information about each trafﬁc trunk is

available because of the CoS deﬁnition (see Table 2):

†	The PHB Scheduling Class (PSC) of the trafﬁc carried on

the trunk, thus inferring the PHB h [ H of the trunk. For

certain PSCs (e.g. AF1x) the mapping to PHB is not ‘1 –

1’, providing more ﬂexibility. This means that each

trafﬁc trunk t [ T	is associated (eventually) with one

PHB.

†	The bound Dt(deterministic or probabilistic depending

on the PSC) maximum end-to-end delay expected

between the ingress and egress nodes. This might be

the minimum value of the delay class range of the CoS

deﬁnition.

†	The maximum end-to-end loss probability, Ptrequired

between the ingress and egress nodes. Similarly to the

previous case this might be the minimum value of the

has to be quantiﬁed. Various deﬁnitions are possible. In

general, the load (or cost) on a given link is an

increasing function of the amount of trafﬁc the link

carries. This function may refer to link utilization or may

express an average delay, or loss probability on the link.

In the context of this work, the QoS seen by the trafﬁc

using the different PHBs varies, so the link load induced

by the trafﬁc of each PHB may vary. This leads us to the

following general form of the cost of link	l.

Let xhl	denote the capacity demand (ﬂow) for PHB h [ H

satisﬁed by link l. Then the link cost induced by the load on

PSC h [ H is a convex function, flhрxhlЮ; increasing in xhl:

The total link cost per link is deﬁned as:

X

FlрxlЮ ј	flhрxhlЮр1Ю

h[H

where xIј {xhl}h[H	is the vector of demands for all PHBs

of link l. In order to take into account that link capacities

may be different, the cost may be modiﬁed to reﬂect

equivalent utilization as follows:

X

loss range of the CoS deﬁnition.

†	The bandwidth Btrequirements.

FlрxlЮ ј

h[H

flhрxhlЮ

Cl

р2Ю

5.2. Cost deﬁnition and optimisation objectives

We need to provide a set of routes for each trafﬁc trunk.

For each ingress – egress pair	vi;	vethese routes are

implemented as LSPs at the routers. We also need to

provide the amount of bandwidth each route is expected to

carry, and the amount of bandwidth that is to be allocated

to the PSCs at the various interfaces in the network.

The	primary objective	of such an allocation is to

ensure that the requirements of each trafﬁc trunk are met

as long as the trafﬁc carried by each trunk is at its

speciﬁed minimum bandwidth. This objective ensures

that our SLS requirements are met. The objective is to

provide a feasible solution (i.e. routes and route

bandwidths respecting the link capacities) that satisﬁes

the SLSs delay and loss constraints.

However, with the possible exception of heavily loaded

conditions, there will generally be multiple feasible

solutions. Hence, the design objectives can be further

reﬁned to incorporate other requirements such as:

1. Avoid overloading parts of the network while other parts

are underloaded. This way, spare bandwidth is available

at various parts of the network to accommodate

Provided that appropriate buffers have been provided at

each router and the scheduling policy has been deﬁned,

then	flhрxhlЮmay specify the	equivalent capacity	needed

by PHB	h [ H	on link	l	in order to satisfy the loss

probability associated with that PHB. Hence, the total

cost per link is the total equivalent capacity allocated on

link	l. Note that with this approach the link costs are

very naturally deﬁned. The drawbacks are: (1) the cost

deﬁnition depends on the PHB implementation at the

routers, (2) the cost functions may not be known, or may

be too complex. Hence approximate cost functions must

be used.

In this work we are using the following cost function:

xh

flhрxhlЮ јl	р3Ю

Cl2 xhl

This function combined with Eq. (1), gives as a total cost

function the average number of packets in the system

based on the hypothesis that each queue behaves as a M/

M/1 queue	[26].

Now we can express objectives (a) and (b) in

mathematical terms.

Avoid overloading parts of the network:

unpredictable trafﬁc requests. In addition, in case of

X

.

!

link failures, smaller amounts of trafﬁc will be disrupted

and will need to be rerouted.

minimise max FlрxlЮ ј max

l[E	l[E

h[H

flhрxhlЮCl

р4Ю

2. Provide overall low network load (cost).

The last two requirements do not lead to the same

Minimize overall network utilization:

.

!

optimisation objective. In any case, in order to make the

last two requirements more concrete, the notion of ‘load’

minimise

X	X

FlрxlЮ ј

l[E	l[E

X

h[H

flhрxhlЮCl

р5Ю

854

P. Trimintzios et al. / Computer Communications 26 (2003) 845–860

It is possible to provide an objective that compromises

between the previous two. More speciﬁcally,

!n

as follows:

X

minimise

X

flhрxhlЮ=Cl

!n

р7Ю

minimise

XрFlрxlЮЮnјX

l[E	l[E

X

h[H

flhрxhlЮ=Cl

;	n $ 1	р6Ю

l[E

subject to:

h[H

When n ј 1, this objective Eq. (6) reduces to Eq. (5), while

X

; KP};	;rt[ Rt

р8Ю

when n ! 1 it can be shown that it reduces to Eq. (4). The

exponent n is important for the results presented in Section

l[rt

1 # min{KD

t

t

6. By using Eq. (6), even if we are using the linear cost

function for	flhрxhlЮ;	the problem becomes a non-linear

optimisation problem.

X

rt[Rt

brtјBt;	;t [ T

р9Ю

5.3. Handling the delay and loss constraints

xhlј

X

X

brt;

;l [ E;	;h [ H

р10Ю

Each trafﬁc trunk is associated with an end-to-end delay

Dtand loss probability Pt, constraint of the trafﬁc belonging

to the trunk. Hence, the trunk routes must be designed so

t[T:htјh	rt:l[rt;rt[Rt

brt$0;	;rt[ Rt;	;t [ T

р11Ю

that these two QoS constraints are satisﬁed.

Given the trafﬁc trunks’ route r ј {l1; l2; …; lk}; the end-

to-end delay is bounded by the sum of the propagation

delays and the per node delay over the route. The same

holds for the end-to-end loss probability. Let	dhl	be the

bound on the maximum delay on the link l associated with

PHB	h,	dprop	h

l	is the propagation delay, and	plis the

maximum loss probability of that link.

Both the constraints above are constraints on additive

path costs under speciﬁc link costs (dhlandphl;respectively).

However, the problem of ﬁnding routes satisfying these

constraints is in general NP-complete [27]. Given that this is

only part of the problem we need to address, the problem in

its generality is rather complex. Fortunately, a reasonable

simpliﬁcation can be made.

Usually, loss probabilities and delay for the same PHB on

different nodes (routers) are of similar order. Therefore we

use the maximum delay, dmaxhјmaxl[E{dlh} and maximum

loss probability,	phmaxјmaxl[E{phl } of a particular PHB

over the whole network, in order to translate the delay and

loss constraints to an upper bound on the number of hops

along a route. For each trafﬁc trunk t [ T, let KDdenote the

Eq. (7) provides the optimisation objective, where	n is a

variable that provides the relative merit of low overall cost

and avoid overloading parts of the network. In Eq. (8) we

add the hop-count constraint, which (see Section 5.3) is

deduced from the delay and loss QoS requirements of the

SLSs. Eq. (9) states that the bandwidth of the explicit routes

per trafﬁc trunk should be equal to the trunks’ capacity

requirements, and Eq. (11) ties the optimisation objective

with the optimisation parameters.

In the formulation above (Eqs. (7) – (11)), we have not

included any constraint for the physical link capacity. We

handle this by deﬁning the cost function Eq. (3) to increase

very fast when the total link ﬂow becomes greater than the

physical link capacity, for example exponentially or

polynomially to utilisation	xhl=Cl; which in this case will

be greater than one. This will avoid getting over the link

capacity. Special care is required since the cost function and

its ﬁrst derivative need to be continuous at the point of

change.

This is a network ﬂow problem and considering the non-

linear formulation described above, the optimal solution can

based on the general gradient projection method [28]. This

t

upper bound on the number of hops induced from the delay

constraint, and	KPt	the upper bound because of the loss

probability constraint. It should be noted that since loss

probabilities per PHB are generally small relative to the

end-to-end loss probability, the resulting bound in this case

will be large and hence it will not have a big effect on the

optimisation. In any case, the resulting constraint is now

simpler.

5.4. Optimisation problem formulation

For each trafﬁc trunk	t [ T	we denote as	Rtthe set of

(explicit) routes deﬁned to serve this trunk. For each rt[ Rt

we denote as	brthe capacity we have assigned to this

is an iterative method, where we start from an initial feasible

solution, and at each step we ﬁnd the minimum ﬁrst

derivative of the cost function path and we shift part of the

ﬂow from the other paths to the new path, so that we

improve our objective function (Eq. (7)).

If the path ﬂow becomes negative, the path ﬂow simply

becomes 0 (max {0,x}) in order to handle the non-negativity

constraint Eq. (11). This method is based on the classic

unconstraint non-linear optimisation theory, and the general

point is that we try to decrease the cost function through

incremental changes in the path ﬂows.

The optimisation variables are the capacity variables brt

assigned to each route of each trunk, i.e.	b ј {brt:rt[

Rt; t [ T }: In order to apply the gradient projection method

t

explicit route. The optimisation problem can be deﬁned now

we need to handle all the constraints. Substituting Eq. (10)

P. Trimintzios et al. / Computer Communications 26 (2003) 845–860

in the optimisation function Eq. (7), we get:	6.1. Topologies

0	0	1	1n

855

FрbЮ ј

X

@

X

flh@

X

X

brtA=ClA

р12Ю

When evaluating a system like the one proposed in this

l[E

h[H

t[T:htјh; rt:l[rt;rt[Rt

paper, care is required regarding the simulation conditions,

including the topologies used, the trafﬁc demand generation

The constraint Eq. (9) states that the capacity assigned to

each variable brt	should be equal to the trunks’ capacity

procedure and the intensity of the expected load.

requirements, i.e.Pr

t[Rt

b

rtј Bt; ;t [ T : We can handle

The main topology used is shown on Fig. 5; it has 10

nodes and 34 unidirectional links. This is a rather small

this by substituting at each iteration i and for each of the

trunks	;t [ T, one of the variables brtrt[Rt, say	brt;is

substituted by

X

topology that resembles the early NSFNet backbone. More

complex and bigger topologies (up to 300 nodes) where also

used, according to the widely used models for random

brtјBt2

rt[Rt2{rt}

brt

р13Ю

topology generation presented in Ref. [31]. We used both

random and transit-stub networks, which represent the AS

system hierarchical structure better than pure random. The

The hop-count constraint Eq. (8) can be handled as

follows. At each step of the algorithm we are required to ﬁnd

a minimum weight path for each trunk	t [ T	with the

weights being the ﬁrst derivative of the cost function

topologies used were of 50, 100, 200 and 300 nodes with

average node degree between 3 and 6 (bi-directional), which

are close to the typical values for real networks. In general,

for the results we opted for 90% conﬁdence level with a

›FрbЮ

јX

h

i

conﬁdence interval 8 – 10% of the corresponding values.

dbrtј

›brt

"

l[rt

nрFlрxlЮЮn21рFlрxlЮЮ0

!n21

#

6.2. Initial conditions

јX

l[rt

n

| C | n l |
| --- | --- |

X

h[H

flhрxhlЮ

d

dxhl

рflhрxhlЮЮ

р14Ю

The initial feasible solution (step 0) of the iterative

this dbrt	is the minimum ﬁrst derivative length path, which

should be included in the optimum solution. In order to

consider the hop-count constraint, the minimum weight path

computation algorithm has to check whether the path found

satisﬁes the hop-count constraint as well. If not, then we

need to ﬁnd another path (not necessarily with the minimum

weight but with a total weight less than at least one of the

paths in Rt), which has hop-count, at least the hop-count

constraint. This procedure is done by either using a	k-

shortest path	algorithm	[29], or by re-trying a simple

shortest path, e.g. Dijkstra’s [30], on a reduced graph.

We need to have a way to control the procedure

described above. We will accept a solution if the relative

improvement in a step	i ю 1 from step	i, is less than a

parameter	e. More speciﬁcally the iterative procedure

terminates when the following equation becomes true,

Fрbiю1Ю2 FрbiЮ

, e	р15Ю

procedure described in Section 5.4 is set to be the same as if

the trafﬁc trunks were to be routed with a shortest path ﬁrst

(SPF) algorithm. That corresponds to the case that all the

trafﬁc of a particular class from ingress to an egress will be

routed through the same shortest path according to some

weight (routing metric). If there exist more than one such

shortest paths, the load is distributed equally among them.

The metric we are using for the SPF algorithm is set to be

inversely proportional to the physical link capacity. The

scenario we are using for step 0 described above is the norm

in today’s operational networks.

The experiments shown in this section correspond to only

one forecasting period, i.e. one run of the provisioning

algorithm. It is straightforward to get results for the

provisioning period by concatenating more than one

forecasting period results. The termination parameter	e,

see (Eq. (15)), is set equal to 0.0008.

Fрbiю1Ю

6. Experimental evaluation

In this section we will investigate the performance of the

system described in Section 5 in terms of long to medium

term average link load and total network average delay.

The NP algorithm described in Section 5 was

implemented in ANSI C and a random SLS/Forecast

generator in Java (see later). For the experimental results

presented in the section we used a 2 Ј 450 Mhz ULTRA

SPARC II with 1024 MB memory running Solaris 7.

Fig. 5. Fixed topology used for experimentation.

856

Table 3

Loading proﬁles for each SLS type

SLS type

P. Trimintzios et al. / Computer Communications 26 (2003) 845–860

with this procedure, resulting in one matrix for each SLS ﬁle

for each loading proﬁle.

6.4. Simulation results

Video (%)	Audio (%)	Control trafﬁc (%)

Fig. 6	shows the link loads for the 10-node topology

Medium load 40%	30

High load 70%	50

6.3. Trafﬁc load generation

9

18

1

2

shown in Fig. 5 for the ﬁrst step and after the algorithm has

run. It is clear that at step 0 solution, which corresponds to

the SPF with equal cost multi-path distribution enabled,

parts of the network are over utilised while others have no

trafﬁc at all. The ﬁnal step, which corresponds to the ﬁnal

In order to be able to experiment with a wide variety of

trafﬁc loads we have implemented a random SLS and trafﬁc

trunk generator in Java. This takes as input a topology ﬁle

with a speciﬁed the number of edge nodes. In our

experiments we selected the edge nodes to be in the range

of 30 – 50% of the total network nodes. We have deﬁned a

number of SLS types offered by the ISP to the costumers.

These are production network-oriented SLS types (see

Table 3) deﬁning video (SDI or compressed), audio, and

control trafﬁc.

We deﬁne as total throughput of a network the sum of the

capacities of the ﬁrst-hop links of all the edge nodes. This is

actually an upper bound of the throughput and in reality it is

a much greater than the real total throughput a network can

handle. This happens because, although the sum of the ﬁrst-

hop link capacity imposes this limit, the rest of the backbone

might not be able to handle so much trafﬁc, which in our

case is particularly true due to the random nature of the

topologies used. Therefore in our experiments we used 70%

load of the total throughput of the network, as the highly

loaded condition, and a 40% load as a medium loaded one.

The set of SLS instances that adhere to the high or medium

load proﬁle (70 or 40%) are assigned randomly at the edge

node pairs and are placed into a ﬁle, which corresponds to

the SLS repository of Fig. 2. Because of the random nature

of the assignment to edge nodes, we can provide any

number of such ﬁles for a particular load proﬁle and a

particular network topology. Each SLS ﬁle is then formatted

into a trafﬁc trunk ﬁle (corresponding to the expected trafﬁc

matrix), according to the procedure described in Section

4.3.1. SLSs are aggregated per ingress egress pair into CoS

classes (see Table 2). The set of trafﬁc trunks is generated

output of our provisioning algorithm, balances the trafﬁc

over the whole network. Note that the largest over utilised

link at step 0 is not necessarily the largest at the ﬁnal

solution.

Fig. 7 shows the mean and standard deviation of the link

loads for the 10-node network after each iteration of the

algorithm. As we can see the load becomes more balanced

over the network after each iteration (standard deviation

reduces). We run those experiments with the exponent

n ј 2, see Eq. (6). This value compromises between

minimising the total (sum) of link costs and minimising

the maximum link load. These two objectives generally lead

to different solutions, with the ﬁrst favouring path solutions

with the least number of links while the other does not care

about the number of links but only for the maximum link

load (and therefore the deviation from the mean). We can

observe the effect of these different objectives at the various

ups and downs over the various steps of the algorithm.

The same behaviour observed with the 10-node network

is also observed with larger random and transit-stub

topologies. We experimented with large topologies up to

300 nodes. The maximum, mean, standard deviation and

link utilisation resulted after the ﬁrst step (SPF) and after

algorithm ﬁnished are shown in Fig. 8.

Now we are going to look at the effect of exponent n of

the cost function as we deﬁned it in Eq. (6). This parameter

compromises between the two objectives, Eqs. (4) and (5),

for minimising the maximum link load and minimising the

overall cost. Fig. 9 shows the results of the experiment with

varying the exponent of the cost function for the 10 and 50

node networks. We can see that the maximum link load

reduces as	n	increases, while the mean link load slightly

Fig. 6. Link load distribution of after the ﬁrst and the last step.

P. Trimintzios et al. / Computer Communications 26 (2003) 845–860

Fig. 7. Average and standard deviation of link load per iteration.

Fig. 8. Maximum, average and standard deviation of link utilisation for various network sizes.

857

858

P. Trimintzios et al. / Computer Communications 26 (2003) 845–860

Fig. 9. The effect of the exponent for 10 and 50 node network.

increases since minimising the maximum link utilisation

results in solutions with paths having more links. The

solution with	n ј 1 means that we only optimise for the

total cost and we do not take into account the maximum link

load objective. We can observe that the reduction of the

maximum link load, and the corresponding increase of the

average link load, is important at the ﬁrst increase of n from

1 to 2, but further increments give only a small difference.

This is a consequence of the fact that we have the hop-count

constraint (see Section 5.3) that limits the number of links

per path and therefore very long paths, which help reducing

the maximum link load, are prohibited. Finally, we can see

that the same behaviour persists for the 10 and the 50 node

networks, as well as all for the larger topologies we used for

experimentation.

We are now going to see the average delay over the

network. We calculated the average delay as a function of

the link utilisation according to the following formula [26]:

1	X	xhl

continuing the work presented in this paper we are planning

to have such results from the simulator and from the PC-

based router testbeds we maintain in our laboratories.

Fig. 10	shows the average total delay in seconds

of the solutions provided for the various network sizes

according to the formula given above. The important

observation is that the algorithm manages to keep the

average total delay for all network sizes at about the

same levels. This is particularly important since at

the ﬁrst step we have many links with utilisation more

than 100%, which yields very high (almost inﬁnite) total

average delay. Another important observation is that the

provisioning algorithm results in average delay for

medium and high loaded networks being very close

together, almost proportionally to their relative load

difference. This is an indication that even for high loads

g

l[E

Cl2 xhl

where g ј Fall=savgis the average of the total rate of the

incoming packets in the network, Fallis the total in expected

rate and savgis the mean packet size (set to 1 K for our

experiments). The above formula is based on the assump-

tion that each queue (PSC) at each link can be modelled as

M/M/1 queue. Although, this is not always true, we can

only use the qualitative nature of the result and not the

exact values. Note that the only way to ﬁnd the exact

impact on delay and loss is by monitoring the network

after applying the provisioning conﬁguration. As part of

Fig. 10. Average overall network delay.

P. Trimintzios et al. / Computer Communications 26 (2003) 845–860

859

Table 4

Average running times in seconds for the various network sizes

into account the expected trafﬁc demand and QoS

constraints. Using IP networks to transport demanding

10

50

100

200

300

Medium load

0.055

9.761

123.989

529.532

981.175

High load

0.061

10.164

302.079

1002.245

1541.937

trafﬁc will result in lower cost and greater ﬂexibility.

Acknowledgements

The work presented in this paper was partially supported

by the EPSRC/LINK Project GR/M84169 ‘Production of

Broadcast Content in and object-oriented IP-based Network

the provisioning algorithm manages to restrain the level

of increase of average delay.

Finally, in Table 4 we provide the average running times

of the various experiments conducted in this paper. We can

see that even for quite large networks the running times are

acceptable. For example for the 300 nodes networks, for

medium load the running time is about 17 min, and for high

load about 25 min. These times are perfectly acceptable

taking into account the timescale of the provisioning system

operation (see Section 4.1).

7. Conclusions

Supporting demanding applications such as production

network trafﬁc requires dedicated networks with high

switching capacity. In this paper we investigate the

possibility of using common IP-based packet network

infrastructure, with Differentiated Services and MPLS as

the key QoS technologies, in order to support such trafﬁc

with the appropriate Service Level Agreements and network

provisioning.

We propose a management and control system for the

provisioning of such networks, targeting to support

demanding production network oriented Service Level

Agreements while at the same time optimising the use of

network resources. Our model is based on the notion of a

provisioning cycle that comprises a number of forecasting

periods; for each of these we need to apply a different

network conﬁguration. Trafﬁc forecasting is based on the

established SLSs, SLS usage and historical measurement

data. We deﬁned a number of CoS to which we assign

the various SLSs. With this model, adequate provisioning

for the CoSs ensures adequate provisioning for SLSs

as well. Our objectives are then to place the CoS

demands to the network in such a way as to avoid

overloading parts of the network while at the same time

minimising the overall network cost. We devised a non-

linear programming formulation and we proved though

extensive simulation that we achieve our objectives.

In conclusion, we believe it is possible to support

production network trafﬁc through IP networks with

DiffServ and MPLS, as long as the appropriate SLSs are

deﬁned and agreed. They can then be used to calculate

anticipated trafﬁc while the provisioning TE algorithm takes

(PRO-NET)’, and the EU IST Project IST-1999-11253

‘Trafﬁc Engineering for Quality of Service in the Internet at

Large (TEQUILA)’. The authors would like to thank their

project partners for the fruitful discussions while forming

the ideas presented in this paper and the anonymous

reviewers for their useful suggestions to improve the quality

of presentation of this work.

References

[1] S. Blake, D. Black, M. Carlson, E. Davies, Z. Wang, W. Weiss, An

architecture for differentiated services, IETF Informational RFC-2475

December (1998).

[2] E. Rosen, A. Viswanathan, R. Callon, Multiprotocol label switch-

ing architecture, IETF Standards Track RFC-3031 January (2001).

[3] D. Awduche, J. Malcolm, J. Agogbua, M. O’Dell, J. McManus,

Requirements for trafﬁc engineering over MPLS, IETF Informational

RFC-2702 September (1999).

[4] D. Awduche, A. Chiu, A. Elwalid, I. Widjaja, X. Xiao, Overview and

principles of Internet trafﬁc engineering, IETF Informational RFC-

3272 May (2002).

[5] F. Le Faucheur, W. Lai, Requirements for support of Diff-Serv-aware

MPLS trafﬁc engineering, IETF Internet draft June (2002) kietf-tewg-

diff-te-reqts-05.txtl, in press.

[6] A. Feldmann, J. Rexford, IP network conﬁguration for intradomain

trafﬁc engineering, IEEE Network Magazine 15 (5) (2001) 46 – 57.

[7] P. Aukia, M. Kodialam, P.V. Koppol, T.V. Lakshman, nH. Sarin, B.

Suter, RATES: a server for MPLS trafﬁc engineering, IEEE Network

Magazine 14 (2) (2000) 34 – 41.

[8] B. Fortz, M. Thorup, Internet trafﬁc engineering by optimising

{OSPF} weights, Proceedings of IEEE INFOCOM 2000,Tel Aviv,

Israel March (2000) 519 – 528.

[9] M. Kodialam, T.V. Lakshman, Minimum interference routing with

applications to trafﬁc engineering, Proceedings of IEEE INFO-

COM2000, Israel March (2000) 884 – 893.

[10] R.K. Ahuja, T.L. Magnanti, J.B. Orlin, Network Flows: Theory,

Algorithms and Applications, Prentice Hall, Englewood Cliffs, NJ,

1993.

[11] D. Mitra, K.G. Ramakrishnan, A case study of multiservice, multi-

priority trafﬁc engineering design for data networks, Proceedings of

IEEE GLOBECOM 99, Rio de Janeiro December (1999) 1087 – 1093.

[12] D. Mitra, J.A. Morrison, K.G. Ramakrishnan, Virtual private

networks: joint resource allocation and routing design, Proceedings

of IEEE INFOCOM 99, New York, USA March (1999).

[13] F. Poppe, S.V. den Bosch, P.L. Valle-Poussin, H.V. Hove, H. De Neve

and G.H. Petit, Choosing the objectives for trafﬁc engineering in IP

backbone networks based on quality-of-service requirements, Pro-

ceedings of the First International workshop on Quality of Future

Internet Services (QofIS’00), Berlin, Germany September (2000)

129 – 140.

[14] S. Suri, M. Waldvogel, P.R. Warkhede, Proﬁle-based routing: a new

framework for MPLS trafﬁc engineering, Proceedings of the Second

860

P. Trimintzios et al. / Computer Communications 26 (2003) 845–860

International Workshop on Quality of future Internet Services	differentiated services in MPLS-based networks, IEEE Communi-

(QofIS’01), Portugal September (2001) 138 – 157.	cations Magazine 39 (5) (2001) 80 – 88.

[15] Z. Wang, Y. Wang, L. Zhang, Internet trafﬁc engineering without full

mesh overlaying, Proceedings of IEEE INFOCOM, Alaska April

(2001).

[16] Y. Breitbart, M. Garofalakis, A. Kumar, R. Rastogi, Optimal

conﬁguration of OSPF aggregates, Proceedings of IEEE INFOCOM,

New York, USA June (2002).

[17] S. Chen, K. Nahrstedt, An overview of quality-of-service routing for

the next generation high-speed networks: problems and solutions,

IEEE Network Magazine, Special Issue on Transmission and

Distribution of Digital Video 12 (6) (1998) 64 – 79.

[18] A. Elwalid, C. Jin, S.H. Low, I. Widjaja, MATE: MPLS adaptive

trafﬁc engineering, Proceedings of IEEE INFOCOM2001, Alaska,

USA April (2001) 1300 – 1309.

[19] Z. Cao, Z. Wang, E. Zegura, Performance of hashing-based schemes

for Internet load balancing, Proceedings of IEEE INFOCOM 2000,

Tel Aviv, Israel March (2000) 332 – 341.

[20] D. Grossman, New terminology and clariﬁcations for Diffserv, IETF

Informational RFC-3260 April (2002).

[21] P. Trimintzios, G. Pavlou, I. Andrikopoulos, Providing trafﬁc

engineering capabilities in IP networks using logical paths, Eighth

IFIP Workshop on Performance Modelling and Evaluation of ATM

and IP Networks (IFPM ATM and IP 2000), Ilkey, UK July (2000).

[22] P. Trimintzios, I. Andrikopoulos, G. Pavlou, P. Flegkas, D. Grifﬁn,

P. Georgatsos, D. Goderis, Y. T’Joens, L. Georgiadis, C. Jacquenet, R.

Egan, A management and control architecture for providing IP

[23] T. Li, Y. Rekhter, Provider architecture for differentiated services and

trafﬁc engineering (PASTE), IETF Informational RFC-2430 October

(1998).

[24] F. Le Faucheur, L. Wu, B. Davie, S. Davari, P. Vaananen, R. Krishnan,

P. Cheval, J. Heinanen, Multi-protocol label switching (MPLS)

support for differentiated services, IETF Standards Track RFC-3270

May (2002).

[25] P. Flegkas, P. Trimintzios, G. Pavlou, A policy-based management

system for differentiated services networks, IEEE Network Magazine,

Special Issue on Policy-Based Networking 28 (2) (2002) 50 – 56.

In press.

[26] D. Bertsekas, R. Gallager, Data Networks, Prentice Hall, Englewood

Cliffs, NJ, 1992.

[27] Z. Wang, J. Crowcroft, Quality of service routing for supporting

multimedia applications, IEEE Journal of Selected Areas in

Communications (JSAC) 14 (7) (1996) 1228 – 1234.

[28] D. Bertsekas, Nonlinear Programming, second ed., Athena Scientiﬁc,

1999.

[29] D. Eppstein, Finding k-shortest paths, SIAM Journal on Computing 28

(2) (1998) 652 – 673.

[30] E.W. Dijkstra, A note on two problems in connection with graphs,

Numerische Mathematik 1 (1959) 269 – 271.

[31] E.W. Zegura, K.L. Calvert, S. Bhattacharjee, How to model an

Internetwork, Proceedings of IEEE INFOCOM 96, San Francisco 2

(1996) 594 – 602.