Computer Communications 27 (2004) 1684–1692

www.elsevier.com/locate/comcom

Abstract

Review of quality of service performance in wireless LANs

and 3G multimedia application services

Nilufar Baghaei, Ray Hunt*

Department of Computer Science, University of Canterbury, Private Bag, Christchurch 4800, New Zealand

Received 25 May 2004; accepted 25 May 2004

Available online 31 July 2004

Although, many architectures (Integrated Services, Differentiated Services, MPLS, Trafﬁc Engineering, etc.) have been proposed to

provide service differentiation in ﬁxed networks, research has shown that what works well in a wired network cannot be directly applied in

the wireless environment where bandwidth is scarce and channel conditions are time varying. Quality of Service (QoS) is a key challenge for

today’s wireless IP networks and implementation of QoS, particularly for supporting voice, video, data and multimedia services in general

incurs a number of difﬁculties that have to be analysed and resolved. A considerable amount of work has been carried out by the various

standards groups in an effort to quantify and specify protocols to support QoS in wireless environments. This paper reports on these efforts

outlining existing limitations, requirements and solutions proposed by organisations such as the IEEE 802.11 Task Group E for wireless

LANs and the UMTS effort for 3G/wireless WANs.

q 2004 Elsevier B.V. All rights reserved.

Keywords: Quality of Service; IEEE 802.11a/b/g WLANs; 3G/UMTS/cdma2000 Networks; Multimedia over wireless networks

1. Introduction

Most current network architectures treat all packets in the

same way—a single level of service. Applications, however,

have diverse requirements and may be sensitive to latency

and packet losses. Examples include interactive and

real-time applications such as IP telephony; streaming

services such as audio, video and bulk data streaming;

and interactive services such as voice, web and transaction

service processing. When the latency or the loss rate

exceeds certain levels, these applications become unusable.

In contrast, both reliable (tcp) and unreliable (udp) services

can tolerate signiﬁcant delay and loss without much

degradation of perceived performance.

The capability to provide resource assurance in a

network is often referred to as Quality of Service (QoS)

which is critical requirement in order that new IP-based

applications can operate within well-deﬁned parameters.

Resource assurance can currently be provided in a

multiservice network by the use of IP service differentation

(DiffServ), although resource assurance can only truly be

* Corresponding author. Tel.: ю 64-3-3642347; fax: ю64-3-3642569.

E-mail address: ray@cosc.canterbury.ac.nz (R. Hunt).

0140-3664/$ - see front matter q 2004 Elsevier B.V. All rights reserved.

doi:10.1016/j.comcom.2004.05.019

guaranteed by the use of the Integrated Services (IntServ)

model. Implementing these QoS capabilities has become

one of the most difﬁcult challenges, particularly as this often

requires changes to its basic network architecture.

The requirements for each type of trafﬁc ﬂow can be

characterised by four primary parameters: reliability,

delay, jitter, and bandwidth. Several common applications

are listed in	Table 1	along with the stringency of their

requirements.	Table 2	(adopted from Ref.	[1]) shows a

similar set of requirements for applications operating in

a wireless WAN (3G) environment.

Most IP-based networks rely on the TCP protocol to

detect congestion in the network and to reduce the

transmission rates accordingly. TCP-based resource

allocation requires all applications to use the same

congestion control scheme. Although such co-operation is

achievable within a small group, in a network as large as the

Internet, it can be easily abused. Furthermore, many

UDP-based applications	do not	support TCP-like

congestion control, and real-time applications typically

cannot cope with large ﬂuctuations in the transmission rate.

The service currently provided by default is often

referred to as best effort. When a link is congested,

packets are simply discarded as the queue overﬂows.

N. Baghaei, R. Hunt / Computer Communications 27 (2004) 1684–1692

1685

Table 1

Common wired-network performance characteristics

have been proposed and experimented with to date. Finally,

it discusses the new IEEE 802.11e QoS enhancements.

Application

E-mail

Reliability	Delay

High	Low

Jitter

Low

Bandwidth

Low

2.1. An Overview of IEEE 802.11 MAC Operation

File transfer

High

Low

Low

Medium

In general, the IEEE 802.11 WLAN standard covers the

Web access

High

Medium	Low

Medium

MAC sub-layer and the physical layer of the OSI network

Remote login

High

Medium	Medium	Low

reference model. The Logical Link Control (LLC) sub-layer

Audio on demand	Low

Video on demand	Low

Low

Low

High

High

Medium

High

is speciﬁed in the IEEE 802.2 standard. This architecture

Telephony

Low

High

High

Low

provides a transparent interface to higher layer users:

Videoconferencing	Low

High

High

High

stations may move, roam through an IEEE 802.11 WLAN

and still appear as stationary to the IEEE 802.2 LLC

sub-layer and above. This allows existing network protocols

Since, the network treats all packets equally, any ﬂows

could be hit by congestion and this particularly impinges on

wireless and mobile connections, commonly as a result of

limited bandwidth. Although best-effort service is

adequate for some applications that can tolerate large

delay variation and packet losses, it clearly does not satisfy

the needs of many new time-sensitive multimedia-based

applications.

Resource assurance is critical for many new wireless

applications. Although, the Integrated Services (IntServ)

and Differentiated Services (DiffServ) paradigms ﬁgure

predominantly as QoS solutions, they focus on the IP layer

and it is necessary for the underlying layers to be able

to respond to and conﬁgure such IP-based service

requirements.	The	following	sections	address

the speciﬁcation and provisioning of these underlying

QoS-based requirements for both wireless LAN and

wireless WAN (3G) architectures.

2. QoS in IEEE 802.11 Wireless LANs

In its current form, the IEEE 802.11 wireless LAN

standard [2] cannot provide QoS support for the increasing

number of applications which demand QoS parameters—

typical of many multimedia applications. A number of IEEE

802.11 QoS enhancement schemes have been proposed, each

focusing on a particular mode of operation. This section ﬁrst

analyses the QoS limitations of the IEEE 802.11 MAC layer

and then summarises the QoS enhancement schemes that

Table 2

Common wireless WAN (3G) network performance characteristics

Application	Reliability	Delay	Jitter	Bandwidth

(such as TCP/IP) to transparently operate over IEEE 802.11

WLANs without any special considerations.

At the PHY layer, the IEEE 802.11 standard provides

three alternatives for operation in the 2.4 GHz band:

†	Infrared (IR) base-band PHY

†	Frequency Hopping Spread Spectrum (FHSS) radio

†	Direct Sequence Spread Spectrum (DSSS) radio.

All three PHY layers support both 1 and 2 Mbps

operation. In 1999, the IEEE deﬁned an 11 Mbps 802.11b

standard designed to operate in the 2.4 GHz free ISM

(Industrial, Science, and Medical) band and a 54 Mbps

802.11a	OFDM	(Orthogonal	Frequency	Division

Multiplexing) standard for operation in the 5 GHz frequency

band.

The IEEE 802.11 MAC sub-layer deﬁnes two related

medium access coordination functions, the Distributed

Coordination Function (DCF) and the optional Point

Coordination Function (PCF) (Fig. 1, adopted from Ref. [3]).

The IEEE 802.11 MAC protocol supports two types of

transmission:	Asynchronous	and	Synchronous	[2].

Asynchronous transmission is provided by the DCF, which

implements the basic access method for the IEEE 802.11

MAC protocol. DCF is based on the Carrier Sense Multiple

Access with Collision Avoidance (CSMA/CA) protocol,

and is the default implementation. The Synchronous service

(also called contention free service) is provided by PCF and

implements a polling-based access method. The PCF uses a

centralised polling approach that requires an Access Point

(AP) to act as a Point Coordinator (PC). The AP cyclically

polls stations to give them the opportunity to transmit

packets. Unlike the DCF, the implementation of the PCF is

not mandatory. Furthermore, the PCF itself relies on the

underlying asynchronous service provided by the DCF.

Although providing different service functions, neither

DCF nor PCF ю DCF have the ability to offer true QoS to

E-mail

Low

High

–

Low

Wireless LAN applications.

File transfer	Low – medium	High

–

High

Web access	Low – medium	Medium	–

Medium

2.2. QoS limitations of IEEE 802.11 MAC

Remote login	Low

Control	Null

Low

Low

–

–

Low

–

In addition to providing channel access (via DCF or

Real time

Low – medium	Low

Low	Medium – high

PCF ю DCF), the wireless LAN MAC layer needs to

1686

provide facilities for:

†	maintaining QoS

†	providing security.

N. Baghaei, R. Hunt / Computer Communications 27 (2004) 1684–1692

Fig. 1. PCF and DCF in IEEE 802.11 MAC layer.

services such as Voice over IP, audio and videoconferencing

require speciﬁed bandwidth, delay and jitter, but can also

tolerate some loss. However, in DCF mode, all the stations

in one BSS or all the ﬂows in one station compete for the

resources and channel with the same priority. There is no

Wireless links have speciﬁc characteristics such as

reliability, bandwidth, packet delay and jitter. Furthermore,

the wireless link characteristics are not constant and may

vary over time and place. Mobility of users may cause the

end-to-end path to change when users roam, and further,

users will expect to receive the same QoS as they change

from one Access Point (AP) to another. This implies that the

new path should also support the existing QoS by service

reservation, and problems may arise when the new path

cannot support such requirements.

There are two ways to characterise QoS in WLANs, viz.:

parameterised or prioritised QoS [4,5]. Parameterised QoS is

a strict QoS requirement, which is expressed in terms of

quantitative values, such as data rate, delay bound, and jitter

bound. In a Trafﬁc Speciﬁcation (TSpec)—such as is used in

the IntServ model, these values are expected to be met by the

MAC data service in support of the transfer of data frames

between peer stations. In a prioritised QoS scheme, the values

of QoS parameters such as data rate, delay bound, and jitter

bound—typically resulting from a DiffServ model, may vary

during the transfer of data frames. In this instance, there is no

need to reserve the required resources by negotiating the

TSpec between the station and the AP as the DiffServ queue

architecture is relied upon to manage the QoS.

2.2.1. QoS limitations of DCF

DCF can only support best-effort services and does not

provide any QoS guarantees. Typically, time-bounded

differentiation mechanism to guarantee bandwidth, packet

delay and jitter for high-priority stations or multimedia

ﬂows [6].

2.2.2. QoS limitations of PCF

Although PCF has been designed by the IEEE Working

Group to support time-bounded multimedia applications,

this mode has some major problems, which leads to poor

QoS performance. In particular, the central polling scheme

is inefﬁcient and complex and causes deterioration of the

performance of PCF high-priority trafﬁc under load.

Additionally, all communications have to pass through the

AP which degrades the bandwidth performance [7].

2.3. QoS enhancement schemes for IEEE 802.11 MAC

QoS issues in wired Ethernet have been neglected due to

the relative ease with which the physical layer bandwidth

has improved Normally, the IP layer assumes that a LAN

scarcely drops or delays packets. However, in WLANs,

the challenges of the wireless channel make physical layer

data rate improvements more difﬁcult to achieve,

particularly as the IEEE 802.11 WLAN was originally

designed for best-effort services. The physical layer’s error

rate can be more than three orders of magnitude larger than

that of wired LAN. Further, high collision rate and frequent

retransmissions cause unpredictable delay and jitter,

which further degrade the quality of real-time voice

N. Baghaei, R. Hunt / Computer Communications 27 (2004) 1684–1692

1687

and video transmission. To address these issues, a number of

proposals have been made which are detailed in the

following sections.

2.3.1. Service differentiation based enhancement schemes

QoS enhancement can be supported by adding service

differentiation into the MAC layer This can be achieved by

modifying the parameters that deﬁne how a station or a ﬂow

should access the wireless medium. Current service

differentiation-based schemes can be classiﬁed with respect

to a multitude of characteristics. For example, a possible

classiﬁcation criterion can be based upon whether the

schemes base the differentiation on per-station or per-queue

(per-priority) parameters. Another classiﬁcation depends on

whether they are DCF (distributed control) or DCF ю PCF

(centralised control) enhancements.	Fig. 2	shows this

classiﬁcation. Previous research work has mainly focused

on the station-based DCF enhancement schemes	[6,8,9],

while other recent work has focused on queue-based hybrid

coordination (combined PCF and DCF) enhancement

schemes	[10 – 12], since queue-based schemes perform

more efﬁciently.

2.3.2. Error control based enhancement schemes

In parallel, QoS enhancement can also be obtained by

error control mechanisms Since, the network may

occasionally drop, corrupt, duplicate or reorder packets,

the transport protocol (e.g. TCP) or the application itself

(e.g. if UDP is being used) must recover from these errors on

an end-to-end basis. Error recovery in the sub-network is

justiﬁed only to the extent that it can enhance overall

performance. However, some sub-networks such as wireless

links require link layer error recovery mechanisms to

enhance the performance, but these enhancements need to be

lightweight	[4]. For example, wireless links normally

require link-layer error recovery (such as IEEE 802.2

LLC) and MAC-level error recovery in the sub-network.

2.3.3. IEEE 802.11e QoS enhancement standards

The focus of the IEEE 802.11 Task Group e (802.11e) is

to enhance the IEEE 802.11 MAC (DCF, PCF) to support

QoS, providing classes of service, enhanced security and

authentication mechanisms. It aims to enhance the ability of

all the physical layers (IEEE 802.11b, 802.11a, 802.11g) to

Fig. 2. Classiﬁcation of service differentiation based schemes.

deliver time-critical multimedia data, in addition to a best

effort data service. There are many new features in the IEEE

802.11e draft 3.0 [13] which enhance the existing DCF and

PCF ю DCF functionality in order to support new QoS

applications [4] and the most important of these are brieﬂy

described.

†	EDCF (Enhanced Distribution Coordination Function)

†	HCF (Hybrid Coordination Function)

†	Direct communication in infrastructure mode

†	AP mobility

†	MAC-level FEC (Forward Error Correction).

2.3.3.1. Enhanced distribution coordination function

(EDCF) and hybrid coordination function (HCF).

Since, DCF and PCF do not differentiate between trafﬁc

types or sources, the IEEE 802.11 Task Group e is

proposing QoS enhancements via two alternative modes

of operation These changes would let critical service

requirements be fulﬁlled while maintaining backward-

compatibility with current IEEE 802.11 standards.

The	proposed	enhancement	to	DCF—Enhanced

Distribution Coordination Function (EDCF)—introduces

the concept of trafﬁc categories. Each station has eight

trafﬁc categories, or priority levels. Using EDCF, stations

try to send data after detecting that the medium is idle and

after waiting a period deﬁned by the corresponding trafﬁc

category called the Arbitration Interframe Space (AIFS).

A higher-priority trafﬁc category will have a shorter AIFS

than a lower-priority trafﬁc category.

To avoid collisions within a trafﬁc category, the station

counts down an additional random number of time slots,

known as a contention window, before attempting to

transmit data. If another station transmits before the

countdown has ended, the station waits for the next idle

period, after which, it continues the countdown where it left

off. No guarantees of service are provided, but EDCF

establishes a probabilistic priority mechanism to allocate

bandwidth based on trafﬁc categories.

Another way IEEE 802.11e aims to extend the polling

mechanism of PCF is with the Hybrid Coordination

Function (HCF). A hybrid controller polls stations during

a contention-free period. The polling grants a station a

speciﬁc start time and a maximum transmit duration. EDCF

appears to be gaining more acceptance than HCF [13,14]. In

addition to the IEEE 802.11e standard, a group of vendors

have proposed Wireless Multimedia Enhancements

(WME), to provide an interim QoS solution for IEEE

802.11 networks.1

2.3.3.2. Additional IEEE 802.11e daft 3.0 features. IEEE

802.11e adds a capability for stations to send trafﬁc directly

1	For more information, refer to	http://grouper.ieee.org/groups/802/11/

Documents/D2T551-600.html.

1688

N. Baghaei, R. Hunt / Computer Communications 27 (2004) 1684–1692

to each other in the infrastructure mode, which signiﬁcantly

improves the bandwidth in certain networks.2

In IEEE 802.11e, AP mobility	[13]	is introduced by a

concept of QAP-Capable and such a station can operate

either as a real QoS AP (QAP) or a wireless station (WSTA).

Table 3

Typical QoS application requirements in 3G

Type of application and example

(Kbps)	Losses

(%)

Delay

(ms)

Therefore, in the IEEE 802.11e draft 3.0 [13], AP mobility

implies the transfer of the AP functionality between different

Data

FTP

Limitless	0

TCP

timer

QAP-Capable stations, i.e. a station can become an AP and

Real time

Audio	Voice

#64

1024#300

Voice over IP	10 – 64	5 – 1022#300

subsequently change back to a station again.

3. QoS in 3G Wireless Networks

2G networks such as Global System for Mobile

Video	MPEG-4

H.320

Non-real time	Audio	CD

Video	MPEG-4

#2000	1022#40

#64	1024#40

150	1024	Buffer

length

Limitless	0	Buffer

length

communications (GSM)/code-division multiple access

(CDMA), have essentially only one QoS option—i.e. speech

at full-rate coding in GSM. Subsequently, a half-rate service

Network

services

Limitless	0

–

was introduced, thus offering a new QoS. In reality,

however, this was done to save network capacity and

therefore serving more users in congested hot spots, rather

than offering a new grade of service to users. The user was not

offered the choice of full/half rate, but, more often, those with

half-rate capable mobile phones were put onto half rate,

without the subscriber knowing that the speech quality has

been deliberately lowered by the network being used.

In 2.5G networks such as general packet radio service

(GPRS), there has been a deliberate attempt to introduce

mechanisms whereby the subscriber can request a different

QoS (average/peak data throughput, packet delay, etc.).

In principle, this QoS requirement can be established at the

beginning of the data transfer session (at Packet Data

Protocol (PDP) context set-up). For example, a user

intending to use an interactive service (such as web surﬁng)

may want to use a service with a faster reaction time/lower

round-trip delay. They can then ask for a smaller packet

delay at PDP context set-up time, and the network can

conﬁrm whether this request is accepted or rejected.

3G is a wireless industry term for a collection of

international standards and technologies aimed at improving

the performance of mobile wireless networks 3G wireless

services offer packet data enhancements to applications and

these include higher speeds, increased capacity for voice

and data services as well as QoS facilities. The two main 3G

technologies for which QoS is being standardised are:

†	UMTS (Universal Mobile Telecommunications System)

†	cdma2000 (Code Division Multiple Access 2000).3

Table 3	(adopted from Ref.	[15]) shows QoS-based

application requirements in terms of bandwidth, delay,

and losses for different categories such as data, real-time

2	This would be the same as ad hoc mode, however in ad hoc mode, there

is no AP deﬁned in the architecture.

3	3GPP is responsible for the UMTS standards speciﬁcation, while

3GPP2 is responsible for the cdma2000 standards speciﬁcation. This has

resulted in two 3G standards being released, i.e. UMTS and cdma2000.

trafﬁc, non-real time trafﬁc, and network services in 3G

networks.

3.1. UMTS/3GPP-deﬁned QoS

3GPP,4Third Generation Partnership Project, has

standardised a common QoS framework for IP-based data

services. They have deﬁned a comprehensive framework for

end-to-end QoS covering all subsystems in a UMTS

network, including core network, wireless and universal

terrestrial radio access networks, etc. UMTS is the ﬁrst

wireless data service, which offers a comprehensive QoS

speciﬁcation across a wireless wide area network

infrastructure. In addition, the speciﬁcation provides for

control signaling, user plane transport and QoS management

functionality.

QoS enables a network to deliver classes of service

(CoS), i.e. different prioritised treatments to different

services or to different groups of users. QoS allocates

network capacity according to the type of trafﬁc required

for a certain type of service, while CoS provides

preferred allocation of the network resources in a similar

manner to which DiffServ does for IP-based services.

CoS is implied in a QoS policy associated with a

subscriber. It is used by the network to provide

differential	QoS	treatments	to	different	services

subscribed by different users.

UMTS deﬁnes QoS classes [16], but cdma2000—at least

in its early formulations—does not. Users of these services

may communicate with both ﬁxed networks and other

mobiles, thus end-to-end performance is also inﬂuenced

by the features of these networks on which other parties may

be situated.

The 3GPP end-to-end QoS speciﬁcation, which includes

the deﬁnition of UMTS QoS architecture, bearer services,

and recommendations for supporting QoS mechanisms, also

4	www.3GPP.org.

N. Baghaei, R. Hunt / Computer Communications 27 (2004) 1684–1692

1689

Fig. 3. UMTS trafﬁc classes.

establishes four overriding UMTS QoS classes or trafﬁc

classes for wireless data, taking into account the restrictions

and limitations of the air interface. The characteristics of

these four QoS classes are described in the following

sections.

3.1.1. UMTS QoS Basic Classes

The basic classes deﬁned by UMTS/3GPP are	[17]:

1. Conversational

2. Streaming

3. Interactive

4. Background.

The main distinguishing factor between these trafﬁc

classes lies with sensitivity to delay (Fig. 3	(adopted from

Ref. [18])).

3.1.1.1. Conversational class. This class applies to any

application that involves real-time person-to-person

communication such as audio voice, videophone etc.

The basic qualities required for speech are low delay,

low jitter, reasonable clarity (common codecs and quality),

and absence of echo. In the case of multimedia applications,

Table 4

End-user performance expectations—conversational/real-time services

such as videoconferencing, it is also necessary to maintain

synchronisation of the different media streams. Failure to

provide low enough transfer delay will result in

unacceptable lack of quality. This class is tolerant of some

errors, e.g. voice packet corruption lasting for up to 20 ms.

However, the degree of error protection required varies with

applications. (Table 4)

3.1.1.2. Streaming class. The streaming class consists of

real-time applications that exchange information between

viewer and listener, without any human response. Examples

of this include video-on-demand, live MPEG4 listening,

Web-radio, news streams, and multicasts. Because of the

absence of interaction, there is no longer a need for low

delay, but the requirements for low jitter and media

synchronisation remain. Error tolerance is a function of

the application. The removal of the low delay criterion

makes it possible to use buffering techniques in the end-user

equipment, so the acceptable level of network jitter is higher

than for the conversational class. (Table 5).

3.1.1.3. Interactive class. This class covers both humans and

machines that interact with another device Examples of this

include some games, network management systems polling

for statistics, and Web-browsing or database retrieval.

Applications in this class are characterised by the request-

response pattern of the end-user. Round-trip delay and

tolerance to packet loss are key QoS characteristics.

(Table 6).

3.1.1.4. Background class. The background class covers all

applications that either receive data passively or actively

request it, but without any immediate need to handle this

data Examples of this include e-mails, short message

service, and ﬁle transfers. The only requirement is for data

integrity, although large ﬁle transfers will also require an

adequate throughput.

Table 7	summarises the characteristics of each of the

above four classes.

Medium	Application

Degree of symmetry

Data rate

(Kbps)

Key performance parameters and target values

End-to-end one	Delay variation	Information loss

way delay	within call

Audio

Conversational voice

Two-way

4 – 25

, 150 ms pref.

, 400 ms limit

,1 ms

,3% Frame error rate

Video

Videophone

Two-way

32 – 384	, 150 ms pref.

, 400 ms limit

Lip-sync ,100 ms

,1% Frame error rate

Data

Telemetry—two way control	Two-way

,28.8

, 250 ms

NA

Zero

Data

Interactive games

Two-way

,1

, 250 ms

NA

Zero

Data

Telnet

Two-way—asymmetric	,1

, 250 ms

NA

Zero

1690

Table 5

N. Baghaei, R. Hunt / Computer Communications 27 (2004) 1684–1692

End-user performance expectations—streaming services

Medium	Application

Degree of Symmetry	Data rate (Kbps)	Key performance parameters and target values

Audio	Speech/music medium/high quality

Primarily one-way	5 – 128

Startup

delay (s)

, 10

Transport delay

variation

,2 s

Packet loss at

session layer

,1% Packet loss ratio

Video	Movie clips surveillance real-time video	Primarily one-way	20 – 384

, 10

,2 s

,2% Packet loss ratio

Data

Data

Table 6

Bulk data transfer/retrieval, layout,

synchronisation information

Interactive games

Primarily one-way	,384

Primarily one-way

, 10

, 10

NA

NA

Zero

Zero

End-user performance expectations—interactive services

Medium	Application

Degree of Symmetry	Data rate	Key performance parameters and target values

One-way delay	Delay variation	Information loss

Audio	Voice messaging

Primarily one-way	4 – 13 Kbps	,1 s for playback	,1 ms

,2 s for record

,3% Frame error rate

Data

Data

Data

Web-browsing—HTML

Transaction services—high priority,

e.g. ATM

E-mail (server access)

Primarily one-way

Two-way

Primarily one-way

,4 s/page

,4 s

,4 s

NA

NA

NA

Zero

Zero

Zero

3.1.2. UMTS QoS targets for mobility categories

UMTS deﬁnes speciﬁc QoS targets to provide an

adequate service to mobile wireless users [17] (Table 8).

In UMTS, the maximum speed envisaged for the high

mobility category is 500 km/h using terrestrial services to

cover all high-speed train services and 1000 km/h using

satellite links for aircraft. The data-rate bands are also

related to the environment and cell size in UMTS with rural

environments and satellite links being restricted to

144 Kbps aggregate for a single mobile and with 2 Mbps

only available as an instantaneous rate rather than a

guaranteed rate.

3.2. cdma2000 QoS

cdma2000 does not set such explicit QoS targets nor

deﬁne its own classes analogous to UMTS. In practice,

however, it supports the same general range of applications

and provides appropriate degrees of support through the use

of radio link level features and the capabilities of Mobile IP.

Table 7

UMTS QoS trafﬁc classes

3.2.1. cmda2000 QoS control plane

The IP Multimedia Subsystem (IMS) framework, end-to-

end QoS support requires signalling, trafﬁc regulation and

resource allocation capabilities [19]. QoS signalling is used

to provision and enforce QoS parameters between endpoints

and is handled in the application layer, network layer and

link layer. The Session Initiation Protocol (SIP) [20] is used

as the application level signalling protocol to establish

sessions while the Session Description Protocol (SDP) [21]

parameters are carried as part of the SIP payload, and

contain session speciﬁc information.

The three sequential steps for QoS signalling and trafﬁc

regulation are as follows:

†	Mobile Station (MS) or user registration

†	Session initiation, including media and QoS negotiation

†	Trafﬁc regulation.

SIP is used for user registration, session management and

media and QoS negotiation. Session management involves

initiation and termination of a SIP session using a three-way

handshake with INVITE, OK and ACK messages. Media

Trafﬁc class

Conversational class

Streaming class

Interactive class

Background class

Fundamental characteristics	Preserve timing of stream

conversational pattern—stringent,

low-delay

Preserve timing of stream	Request response pattern

preserve payload content

Destination does not care

about arrival time Preserve

payload content

Application Example

Voice

Streaming video

Web Browsing

Background —e.g. e-mails

Adopted from 3GPP 23.107.

N. Baghaei, R. Hunt / Computer Communications 27 (2004) 1684–1692

1691

Table 8

IMT-2000 mobility categories

†	Delay jitter. The system shall be able to operate under

delay jitter of three times the RLP (Radio Link Protocol)

Category

Limited mobility

Full mobility

High mobility

Physical speed

Up to 10 km/h

Up to 120 km/h

More than 120 km/h

Data rate

2 Mbps

384 Kbps

144 Kbps

retransmission time in the network with retransmission

activated.

†	Error rate. The service shall operate over channels with

end-to-end BER (Bit Error Rate) in the order of 1023

(for circuit-switched network services) and FER (Frame

Error Rate) in the order of 1022	(for packet-switched

network services).

and QoS negotiation is conducted using a SDP payload of

SIP messages	[22]. SDP negotiation contains parameters

that are related to QoS, such as media stream codec and

bandwidth requirements.

SIP is an application layer control and signalling protocol

used	for	creating,	modifying,	and	terminating

multimedia sessions with one or more participants.

These participants could be two mobile stations or a mobile

station	and	an	application	server.	SIP	messages

(e.g. INVITE) carry session descriptions that allow

participants to agree on a set of compatible media types

and QoS requirements. SIP operates over different transport

protocols such as TCP or UDP.

QoS parameters are negotiated between endpoints

running SIP user agents through the SIP proxy and AAA

server. The Policy Decision Point (PDP) implements the SIP

proxy to determine the allowed QoS parameters based on

SIP negotiation and local policy of the network.

Session speciﬁc QoS parameters are exchanged via SDP

while SIP header ﬁelds containing the QoS parameters are

enforced using the Policy Enforcement Point (PEP) as part

of the Packet Data Serving Node (PDSN) in cdma2000 [19].

3.2.2. cmda2000 QoS data plane

QoS-based data in cdma2000 is handled by the DiffServ

architecture at the IP layer. These may result from a

SIP-based signalled request or be based on user

proﬁle 3GPP2 differentiated services class options

registered with a home RADIUS server. In the case of

Mobile IP, the service class has to be copied by the home

agent from the IP packets to the differentiated services ﬁeld

of the mobile tunnel.

The QoS targets (so far)	[23]	for audio and video

streaming are:

†	Synchronisation. For transmission of combined audio

and video streams, the inter-media skew should be kept

below 20 ms.

†	Minimum bandwidth. Minimum bandwidth for audio and

video streaming applications are deﬁned.

†	Play-out delay. The video streaming service shall be able

to provide service of reasonable end-to-end delay to

accommodate data transfer from the source to the mobile

terminal, and shall support buffering at the terminal to

accommodate transmission path degradations to a

speciﬁc level. The recommended maximum play-out

delay is 30 s.

For videoconferencing, the targets are similar, except the

play-out delay has to be much less so that end-to-end delay

does not exceed 400 ms. The degree of jitter that must be

compensated is up to 200 ms. Throughput must range from

32 Kbps upwards, including the speciﬁc rates of 384 and

128 Kbps for packet and circuit switching, respectively.

4. Conclusions

Providing QoS for modern audio and video-based

multimedia applications is a key challenge for today’s

wireless mobile networks. Limited bandwidth, varying

channel conditions, mobility, as well as QoS interface

requirements between a variety of wireless and wired

network infrastructures is a very complex problem to solve.

The speciﬁcation of current standards focus on Layer 2 for

wireless LANs (typically implying upper layer support of a

differentiated-based IP architecture) while for wireless

WANs the protocol speciﬁcation encompasses multiple

layers of the protocol stack.

This paper has addressed the fundamental concepts of

QoS provisioning in wireless LANs and 3G networks. Much

work has yet to be carried out in order to offer this

same service across a concatenation of ﬁxed/mobile and

wired/wireless networks. Further, addition of security

functionally (deﬁned individually for WLANs via

IEEE802.1x or IPSec and WWANs via IPSec, WAP2.0,

etc) across these various network infrastructures will require

a considerable amount of further research.

Although advances have been made in the bandwidth

capacity of 3G networks (UMTS networks can now offer up

to 2 Mbps with limited mobility), the problems become

much more complex as issues of diverse and multiple

networks are added. Network operators rarely have end-to-

end control over a data path, and the problems of

guaranteeing IP-based QoS service across multiple

networks remains. While mechanisms such as RSVP offer

QoS guarantees, this still relies upon these mechanisms

being implemented by the service providers across multiple

wired/wireless networks and the expectation that the

underlying lower layer infrastructure can respond to these

stringent requirements.

1692

References

N. Baghaei, R. Hunt / Computer Communications 27 (2004) 1684–1692

[13] IEEE	802.11	WG.	Draft	supplement	to	standard	for

telecommunications and	information exchange between sys-

[1] S.V. Vasconcellos, J.F. Rezende, QoS and mobility in 3G Core

Networks, Proceedings of Workshop on QoS and Mobility, Brazil,

Nov. 2002, www.gta.ufrj.br/publicacoes.

[2] IEEE 802.11 WG, Reference number ISO/IEC 8802-11:1999(E)

IEEE Std 802.11, 1999 ed.

[3] G. Held, The ABCs of IEEE 802.11, IT Professional, 3(6), Institute of

Electrical and Electronics Engineering, Inc., Nov. 2001, pp. 49 – 52.

[4] Q. Ni, L. Romdhani, T. Turletti, I. Aad, QoS Issues and Enhancements

for IEEE 802.11Wireless LAN, INRIA, France, 2002.

[5] J.M. Ho, Some comments on 802.11e Draft 2.0, IEEE 802.11e

working document 80211-02/005r0, January 16, 2002.

[6] I. Aad, C. Castelluccia, Differentiation mechanisms for IEEE 802.11,

IEEE Infocom 2001, Anchorage, Alaska, USA, April 2001.

[7] A. Lindgren, A. Almquist, O. Schelen, Evaluation of quality of service

schemes for IEEE 802.11 wireless LANs, Proceedings of the 26th

Annual IEEE Conference on Local Computer Networks, Florida,

USA, Nov. 15 – 16, 2001.

[8] J. Deng, R.S. Chang, A priority scheme for IEEE 802.11 DCF access

method, IEICE Transactions in Communications 82-B (1) (1999).

[9] A. Veres, A.T. Campbell, M. Barry, L.H. Sun, Supporting service

differentiation in wireless packet networks using distributed control,

IEEE Journal of Selected Areas in Communications (JSAC), Special

Issue on Mobility and Resource Management in Next-Generation

Wireless Systems 19 (10) (2001) 2094 – 2104.

[10] I. Aad, C. Castelluccia, Remarks on per-ﬂow differentiation in IEEE

802.11, in: Proceedings of European Wireless (EW2002), Florence,

Italy, Feb. 2002.

[11] S. Mangold, S. Choi, P. May, O. Klein, G. Hietz, L. Stibor, IEEE

802.11e wireless LAN for quality of service, Proceedings of European

Wireless (EW2002), Florence, Italy, Feb. 2002.

[12] L. Romdhani, Q. Ni, T. Turletti, AEDCF: enhanced service

differentiation for IEEE 802.11 wireless ad-hoc networks, INRIA

Research Report No. 4544, Sept. 2002.

tems—LAN/MAN	speciﬁc	requirements.	Part	11:	Wireless

medium access	control (MAC)	and	physical	layer	(PHY)

speciﬁcations: Medium access control (MAC) enhancements

for Quality of Service (QoS), IEEE 802.11e/Draft 3.0, May

2002.

[14] A.	Ganz,	A.	Phonphoem,	Z.	Ganz,	Robust	super-poll

with chaining protocol for IEEE 802.11 wireless LANs in

support of multimedia applications, Wireless Networks 7 (2001)

65 – 73.

[15] F. Fitzek, M. Krishnam, M. Reisslein, Providing application-level

QoS in 3G/4G wireless systems: a comprehensive framework based

on multi-rate CDMA, IEEE Wireless Communications 9 (2) (2002)

42 – 47.

[16] ETSI TS 123 107, QoS Concepts and Architecture, V4.3.0, 2001,

http://www.etsi.org.

[17] ETSI TS 122 105, Services and Service Capabilities, V5.2.0, 2002,

http://www.etsi.org.

[18] Nortel Networks white paper, Beneﬁts of Quality of Service (QoS) in

3G wireless Internet, www.nortelnetworks.com, 2001.

[19] M.A. Siddiqui, K. Guo, S. Rangarajan, S. Paul, End-to-End QoS

Support for SIP Sessions in CDMA2000 Networks, Bell Labs

Technical Journal 9 (3) (2004).

[20] J. Rosenberg, H. Schulzrinne, G. Camarillo, A. Johnston, J. Peterson,

R. Sparks, M. Handley, E. Schooler, SIP: Session Initiation Protocol,

RFC 3261, Jun, 2002.

[21] S. Gaast, A. Hajjaoui, E. Meeuwissen, Quality of Service for SIP

Sessions in 3GPP-based Networks, Bell Labs Technical Journal 9

(2004) 3.

[22] G.Ed. Camarillo, W.Ed. Marshall, J. Rosenberg, Integration of

Resource Management and Session Initiation Protocol (SIP),

RFC-3312 (Oct. 2002).

[23] 3GPP2 S.R0021/R0022, Video Conferencing Services—Stage 1,

2002. http://www.3gpp2.org.