Information Sciences 141 (2002) 61–79

www.elsevier.com/locate/ins

Schedulable region for VBR media

transmission with optimal resource

allocation and utilizationq

Ray-I Chang *, Meng-Chang Chen, Jan-Ming Ho,

Ming-Tat Ko

Department of Information Management, National Central University, Chung-Li, and Academia

Sinica, Institute of Information Science, Nankang, Taipei 115, Taiwan, ROC

Received 17 November 1999; received in revised form 23 December 2000; accepted 8 May 2001

Abstract

Media data are variable-bit-rate (VBR) in nature due to the coding and compression

technologies applied. As VBR streams are complicated for network management, dif-

ferent approaches were proposed to shape the VBR stream as a transmission schedule

with smoothed traﬃc burst. In this paper, instead of giving a ﬁxed schedule result, a

novel traﬃc shaping scheme is proposed to decide a schedulable region for all optimal

transmission schedules that provides the minimal allocation and maximal utilization of

system resources (such as network bandwidth, initial delay and client buﬀer). Experi-

ments have shown that our obtained shaping results show dramatic improvements than

that of conventional approaches in both the client buﬀer size and the network idle rate

achieved. Based on the schedulable region provided, the ready time and deadline for

each media packet can be precisely speciﬁed to support real-time network scheduling

and error control. It allows users to determine their own optimal schedules under

various quality-of-service (QoS) requirements and resource constraints.	2002 Else-

vier Science Inc. All rights reserved.

Keywords:	Traﬃc engineering; Traﬃc shaping/smoothing; Multimedia; Real-time

networking

q This work was partially supported by NSC under grants NSC88-2213-E-001-011 and NSC88-

2213-E-001-012.

* Corresponding author. Tel.: +886-2-2788-3799; fax: +886-2-2782-4814.

E-mail address: william@iis.sinica.edu.tw (R.-I. Chang).

0020-0255/02/$ - see front matter	2002 Elsevier Science Inc. All rights reserved.

PII: S 0 0 2 0 - 0 2 5 5 ( 0 1 ) 0 0 1 9 1 - 8

62

1. Introduction

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

Recently, multimedia applications such as digital library, home shopping,

distance learning, video-on-demand (VOD), and video-conferencing (VC) have

attracted great attentions. In these applications, media data such as audio and

video are transmitted from server to clients via network according to some

transmission schedules. Diﬀerent from the conventional data streams, end-to-

end quality-of-service (QoS) is necessary for media transmission to provide

jitter-free playback. As network resources are allocated exclusively in ﬁxed-size

chunks to serve diﬀerent data streams, it is simple to support constant-bit-rate

(CBR) transmission. Grossglauser and Keshav [12] have investigated the per-

formance of CBR traﬃc in a large-scale network with many connections and

switches. They concluded that the network queuing delay for CBR transmis-

sion is less than one cell time per switch even under heavy loading. However,

media streams are notably variable-bit-rate (VBR) in nature due to the coding

and compression technologies applied [11,12]. For example, in an MPEG-1

movie, the average frame size is usually less than 25% of its maximal frame size.

The conventional network service model that allocates a CBR channel to

transmit the VBR stream by stream’s peak data rate would be a waste of

bandwidth. In past years, diﬀerent traﬃc shaping approaches were proposed to

reduce the traﬃc burst. Instead of giving a ﬁxed transmission schedule, we try

to decide the upper bound and the lower bound for all transmission schedules

that provides the optimal resource allocation and utilization in this paper. It is

called the schedulable region of optimal transmission schedules (or schedulable

region, for short). Based on the schedulable region provided, the ready time and

deadline for each media packet can be precisely speciﬁed to support real-time

network scheduling and error control. It allows users to determine their own

optimal schedules under various QoS requirements and resource constraints.

In a multimedia system, we generally measure the performance of a trans-

mission schedule by the following four indices: peak bandwidth, network uti-

lization, initial delay time, and client buﬀer size.

•	Peak bandwidth	is the maximum network bandwidth allocated for media

transmission. A user request is admitted if the peak bandwidth required is

smaller than the available bandwidth of the current network.

•	Network utilization is the ratio of the total bandwidth consumed to the total

bandwidth allocated. Generally, the higher the network utilization means

more users can be served at the same time.

•	Initial delay is the length of time interval from the time that the client sends

the media request to the time that the client starts playing the received data.

It is an important QoS indicator for users.

•	Client buﬀer acts as a reservoir to regulate the diﬀerence between the trans-

mission rate and the playback rate. It is an important resource for users to

prevent playback jitters, i.e. buﬀer overﬂow and underﬂow.

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

63

While serving a VBR media stream, a good transmission schedule is designed

to minimize the peak bandwidth, initial delay and buﬀer size required to keep

the network utilization as large as possible. Moreover, end-to-end QoS of

media transmission needs to be guaranteed for supporting jitter-free playback

[6,16,17]. Recently, diﬀerent approaches are proposed to shape the traﬃc burst

for high network utilization, smaller buﬀer size, and short initial delay. In [19],

the constant-rate transmission and transport (CRTT) method was presented to

transmit VBR media data by a constant bandwidth. By given the available

transmission bandwidth and initial delay, CRTT minimized the required buﬀer

size by the dynamic programming technique. Although the admission control

and transmission schedule were simple, CRTT had the drawback of requiring

large buﬀer and delay. To reduce the required buﬀer, a piecewise CRTT

(PCRTT) method [19] was introduced to evenly divide the media stream into

sub-streams and applied CRTT to each sub-stream. Based on the similar idea,

the renegotiated CBR (RCBR) method [13] was proposed to use the average

data rates in diﬀerent sub-streams. Given initial delay and client buﬀer, the

minimum changes bandwidth allocation (MCBA) [9] and critical bandwidth

allocation (CBA) [10] methods were proposed to minimize the number of

bandwidth changes and the peak bandwidth required, respectively. In [20], the

minimum variability bandwidth allocation (MVBA) method was proposed to

minimize the bandwidth variation for media transmission by the shortest-path

algorithm [18].

Note that, although previous traﬃc shaping methods had reduced some

problem parameters in media transmission, they did not achieve the optimized

schedule results that minimize the initial delay, the client buﬀer and the

bandwidth utilization at the same time. For example, in [20], the allocated

initial delay and the bandwidth utilization were not optimized as discussed in

[3,4,26]. In this paper, a novel traﬃc shaping approach is presented to optimize

both the resource allocation and utilization for VBR media transmission. In-

stead of giving a ﬁxed result, our approach provides the schedulable region for

all optimal transmission schedules. We have proved that all the schedule results

presented in this given region have the minimal initial delay and client buﬀer

for the network channel applied. The remainder of this paper is illustrated as

follows. We introduce the VBR media transmission problem in Section 2. In

Section 3, the proposed algorithm is proposed to identify the schedulable re-

gion for all optimal transmission schedules. Experiments are shown in Section

4. Section 5 shows the conclusion remarks.

2. VBR media transmission

In this paper, we consider the end-to-end transmission of a pre-stored VBR

media stream. While a user request is presented, media data are ﬁrst retrieved

64

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

from the storage sub-system by following the disk retrieval scheduler [2,7,8,24].

The network transmission scheduler then transmits the retrieved data from

server to client at the proper time. On the client side, incoming data are tem-

porarily stored in the client buﬀer and consumed frame-by-frame periodically

by the playback scheduler. If a frame arrives late or is incomplete at its play-

back time, unpleasant jittery eﬀects would be perceived by the audience. To

avoid jittery playback, the transmission schedule must always be ahead of its

related playback schedule so that the client buﬀer would not be underﬂow. On

the other hand, the transmission schedule must avoid sending more data to the

client buﬀer than the total data that the buﬀer can store. Otherwise, the

overﬂow condition is occurred and the client results in loss of data. It will

require an extra bandwidth for retransmission. While serving a media stream, a

good transmission schedule is designed to minimize resource allocation and

maximize resource utilization without playback jitter. In this paper, to con-

centrate on the formalization of the media transmission problem, the disk

retrieval scheduler is assumed to always retrieve suﬃcient data before they

request by the network transmission scheduler [24,27]. More detail descriptions

for the design and implementation of a multimedia disk retrieval scheduler are

shown in [28].

A media stream V can be represented by a set of frames ff0; f1; . . . ; fn1g

where n is the number of frames and fiis the ith frame. We assume that the

media stream is played at t ј 0 and the time to play the ith frame is i	Tf

where Tfis the playback time interval between adjacent frames. (For example,

Tfј 1=30 s in a MPEG video stream [15].) In this paper, without loss of

generality, we let Tfј 1 (unit time). The	ith accumulative frames size of V is

Fiј Fi1юfiwhere the initial value Fkј 0 for k < 0. The media stream size is

the total frame size Fkј jV j for k > n	2. As the client plays the media stream

frame-by-frame periodically (fiis consumed at the	ith frame time), the play-

back schedule can be denoted by its accumulative playback function F рЮ in the

following:

Xx

F рtЮ ј Fxј

iј0

fi8x 6 t < рx ю 1Ю:

р1Ю

Note that	F рЮ	is a nonnegative stair function with jumps at time	t	for

t ј 0; 1; . . . ; n	1. The low corner and the up corner at time	t	are

F рtЮ	ј F рt	1Ю	and F рtЮюј F рtЮ, respectively.

Based on the same idea, we deﬁne the transmission schedule	GрЮ	as a

function that cumulates the amount of media data received at the client. As-

sume that the media data are transmitted by rate rрtЮ at time t, the transmission

schedule is deﬁned as the integration function of rрtЮ	as follows:

Zt

GрtЮ ј

xј0

rрxЮ dx:

р2Ю

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

65

Note that this function is continuous and monotonically non-decreasing. The

peak bandwidth of the network channel allocated for media transmission is

Peak bandwidth : r ј maxfrрtЮ j 8tg:

р3Ю

Let tsј minft j 8rрtЮ > 0g and teј maxft j 8rрtЮ > 0g be the start time and the

end time of the transmission schedule GрЮ, respectively. The value tcј tetsis

the connection time of the network channel allocated. We can compute the

network utilization of the allocated channel as follows:

Network utilization : u ј jV j=рr	tcЮ100%;

р4Ю

Network idle rate ј 100%	u:

According to the deﬁnition, GрtЮ	is the amount of data sent by the server up to

time	t. Assume that there is no transmission error and the network delay is

zero. GрtЮ also represents the amount of data received by the client up to time t.

If the client starts the playback of the media stream at time 0, the value of the

initial delay is shown as follows:

Initial delay : d ј ts:

р5Ю

As the media data must be transmitted before be received and played, the start

time	ts< 0 [12,25]. Note that, at the client,	GрtЮ	and	F рtЮ	represent the cu-

mulated data received and consumed up to time	t	respectively. The buﬀer

occupancy	bрtЮ ј GрtЮ  F рtЮ	is the amount of transmitted data temporarily

stored in the client buﬀer at time t. The minimal client buﬀer size required and

its utilization for media transmission and playback can be computed as fol-

lows:

Client buffer : b ј maxfbрtЮ j 8tg;

Buffer utilization ј

X1

tјd

,X1

bрtЮ

tјd

!

b	100%:

р6Ю

Obviously, b is no smaller than the maximum frame size, and is no larger than

the stream size. An example to illustrate the cumulative playback function, the

cumulative playback function, the initial delay and the buﬀer size is shown in

Fig. 1.

In this paper, a transmission schedule is said to be feasible if it can provide

the jitter-free playback. By deﬁnition, a jitter-free transmission schedule de-

mands a complete media frame before its playback. The cumulative trans-

mission function	GрtЮ	must not to be larger than the cumulative playback

function	F рtЮ. Besides, the buﬀer occupancy must not be smaller than the

speciﬁed buﬀer size. Deﬁne HрtЮ	as the upper bound of GрtЮ	and, in this paper,

HрtЮ ј minfjV j; F рtЮ	ю bg. A feasible transmission schedule satisﬁes the fol-

lowing conditions:

66

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

Client Buffer Size

G(t)

Initial delay

F(t)

time

t

Fig. 1. An example to illustrate the relations among the cumulative transmission function, the

cumulative playback function, the initial delay and the client buﬀer size.

buffer size

Underflow

(a)

F(t)

t

Overflow

buffer size

(b)

F(t)

t

Fig. 2. (a) The underﬂow (or starvation) condition and (b) the overﬂow condition for the cumu-

lative transmission function with a bounded client buﬀer.

F рtЮ 6 GрtЮ 6 HрtЮ:

р7Ю

Fig. 2(a) and (b) shows the underﬂow condition and the overﬂow condition of

media transmission, respectively, with a bounded buﬀer at the client.

3. Schedulable region of optimal transmission schedules

When designing a transmission schedule, two important resources are

considered: network bandwidth and client buﬀer. In this paper, a transmission

schedule is said to be optimal if it allocates the minimal resources (both net-

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

67

work bandwidth and client buﬀer) and has the maximal resource utilization.

From the deﬁnitions shown in Eqs. (4) and (5), the network utilization is

maximized only if the initial delay is minimal. An optimal transmission

schedule must decide the minimal initial delay for media playback. In this

section, we introduce Algorithm-L to decide the minimal resource allocated for

media transmission. Then, Algorithm-A is proposed to maximize the resource

utilization. Based on these two algorithms, the schedulable region for all the

optimal transmission schedules is given to assign the ready time and the

deadline to each packet. Finally, a smoothed optimal transmission schedule is

presented.

3.1. Minimal resource allocation

Given a media stream	V, we ﬁrst introduce Algorithm-L	to decide the

amounts of resource required for media transmission. Let the available net-

work bandwidth be	r. The transmission schedule	LрЮ	obtained is shown as

follows:

Lрn	1Ю ј jV j ј F рn	1Ю;

р8Ю

LрtЮ ј maxfF рtЮ; Lрt ю 1Ю  rg	80 6 t < рn	1Ю:

An example to illustrate the computation of Algorithm-L is shown in Fig. 3.

Note that the media data are transmitted and stored into the client buﬀer as

late as possible. Therefore, at any time	t, Algorithm-L	decides the minimal

buﬀer occupancy for guaranteeing jitter-free playback. It achieves the minimal

client buﬀer size	b ј maxfLрtЮ  F рtЮ j 8tg	and the minimum initial delay

d ј Lр0Ю=r	for the available network bandwidth	r. Besides, given any trans-

mission schedule GрЮ with the peak transmission rate r, we have LрtЮ 6 GрtЮ	for

Lazy Scheme

Given:

r	Transmission Rate

Minimize:

Buffer Occupancy

Initial delay

t

Fig. 3. An example to illustrate the operations of Algorithm-L that minimizes the resource allo-

cation.

68

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

any time t. The achieved result	LрЮ	is called the minimal	r-bounded trans-

mission schedule.

Lemma 1. LрЮ is the minimal r-bounded transmission schedule.

Proof. Suppose the contrary. Let GрЮ be a r-bounded transmission schedule,

for which there exists a time index x such that LрxЮ > GрxЮ. Let y be the smallest

time index that satisﬁes x < y	and LрyЮ ј F рyЮ ј LрxЮ ю r  рy	xЮ. From the

deﬁnition of LрЮ shown in Eq. (8), LрyЮ ј F рyЮ if Lрy ю 1Ю  r 6 F рyЮ. The value

y is existed (at least, we have the initial value Lрn	1Ю ј F рn	1ЮЮ. As GрЮ is

r-bounded, the relation	GрyЮ 6 GрxЮ ю r  рy	xЮ	< LрxЮ ю r  рy	xЮ	is true

[1]. That implies GрyЮ < F рyЮ. The underﬂow condition of the client buﬀer is

occurred and GрЮ is not a feasible transmission schedule. It is a contradiction

and the lemma is proved.

Lemma 2. LрЮ	has the minimal buffer size and initial delay for all r-bounded

transmission schedules.

Proof. Since LрЮ is the minimal r-bounded transmission schedule, it sends the

minimal amount of data to the client buﬀer for guaranteeing jitter-free play-

back. At any time	t, we have	LрtЮ 6 GрtЮ	where	GрЮ is any other	r-bounded

transmission schedule. As Lр0Ю 6 Gр0Ю, the initial delay is Lр0Ю=r 6 Gр0Ю=r. LрЮ

has the minimal initial delay for all r-bounded transmission schedules. More-

over, their buﬀer occupancies have the relation	LрtЮ  F рtЮ 6 GрtЮ  F рtЮ. It

implies that	LрЮ	has the minimal buﬀer size (the required buﬀer size

maxfLрtЮ  F рtЮ j 8tg 6 maxfGрtЮ  F рtЮ j 8tgЮ.

3.2. Maximal resource utilization

In this paper, Algorithm-L is introduced to decide the minimal amounts of

system resource required for guaranteeing jitter-free playback. Given the sys-

tem resources allocated, Algorithm-A is then introduced to maximize the uti-

lization of bounded resources. The obtained transmission schedule	AрЮ	is

shown as follows:

AрtЮ ј minfH рtЮ; Aрt	1Ю ю rg 8рDЮ < t 6 рn	1Ю:

р9Ю

The initial value is AрDЮ ј 0 where d 6 D. Without loss of generality, we let

the value of D be the minimal initial delay d decided by Algorithm-L. It implies

Aр0Ю ј Lр0Ю. An example to illustrate the computation of Algorithm-A	is

shown in Fig. 4. Note that, as the media data are transmitted to the client as

early as possible, this algorithm can maximize the utilization of given resources

– the network bandwidth r and the client buﬀer b. Furthermore, the obtained

transmission schedule is robust against network errors [21]. For any other

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

Aggressive Scheme

Given

69

r

Transmission Rate

Buffer size

Initial delay

Maximize

Network Utilization

Buffer Utilization

t

Fig. 4. An example to illustrate the operations of Algorithm-A that maximizes the resource uti-

lization.

transmission schedule	GрЮ	that has the same transmission bandwidth	r	and

client buﬀer b, we can prove GрtЮ 6 AрtЮ	for any time t. The obtained result AрЮ

is called the maximal рr; bЮ-bounded transmission schedule.

Lemma 3. AрЮ is the maximal рr; bЮ-bounded transmission schedule.

Proof.	Suppose the contrary. Let	GрЮ	be a	рr; bЮ-bounded transmission

schedule, for which there exists a time index x such that GрxЮ	> AрxЮ. Let y be

the largest time index that satisﬁes y < x and AрyЮ ј HрyЮ ј AрxЮ  r  рx	yЮ.

From the deﬁnition of AрЮ shown in Eq. (9), AрyЮ ј H рyЮ	if HрyЮ 6 Aрy	1Ю ю

r. The value y is not existed only when the peak bandwidth r is fully utilized in

AрЮ. It implies GрЮ 6 AрЮ (they are рr; bЮ-bounded). Assume that y is existed.

As GрЮ is a r-bounded, the relation AрxЮ  r  рx	yЮ < GрxЮ  r  рx	yЮ 6

GрyЮ	is true. That implies	HрyЮ	< GрyЮ. The overﬂow condition of the client

buﬀer is occurred and	GрЮ	is not a feasible transmission schedule. It is a

contradiction and the lemma is proved.

Lemma 4. AрЮ has the maximal utilization in buffer size and network bandwidth

for all рr; bЮ-bounded transmission schedules.

Proof.	Since	AрЮ	is the maximal	рr; bЮ-bounded transmission schedule, the

maximal amount of data is sent to the client buﬀer at any time t. Thus, given

any	рr; bЮ-bounded transmission schedule	GрЮ, we have	GрtЮ 6 AрtЮ	and the

buﬀer occupancy	GрtЮ  F рtЮ 6 AрtЮ  F рtЮ	at any time t. From the deﬁnition

shown in Eq. (6), AрЮ has the maximal utilization in buﬀer size. Based on the

same idea, we have GрteЮ 6 jV j ј AрteЮ	where teis the end time of schedule AрЮ.

70

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

Let t0e	be the end time of schedule GрЮ. We have GрteЮ 6 jV j ј Gрte0Ю. It implies

te6 t0e(teis the minimal value of end time for all рr; bЮ-bounded transmission

schedules). From the deﬁnition shown in Eq. (4), AрЮ has maximized the uti-

lization of network bandwidth.

3.3. Schedulable region for optimal resource allocation and utilization

We have shown that, given any	рr; bЮ-bounded transmission schedule

GрЮ; GрtЮ 6 AрtЮ	for all	t. Let рr; bЮ	be the minimal transmission bandwidth r

and client buﬀer	b	obtained by Algorithm-L. The maximal	рr; bЮ-bounded

transmission schedule AрЮ can determine the upper bound of the transmission

schedules that optimize both the resource allocation and utilization. It has the

minimal end time tefor all рr; bЮ-bounded transmission schedules. In this sec-

tion, by applying Algorithm-L and the minimal end time te, we determine the

minimal рr; bЮ-bounded transmission schedule RрЮ as follows:

RрtЮ ј maxfF рtЮ; Rрt ю 1Ю  rg 80 6 t < рn	1Ю:

р10Ю

The initial value RрteЮ ј jV j ј AрteЮ. Given any transmission schedule GрЮ that

have the optimal resource allocation and utilization, we can prove that

RрtЮ 6 GрtЮ	for all	t.	RрЮ	determines the lower bound of the transmission

schedules that optimize both the resource allocation and utilization.

Lemma 5. RрЮ is the minimal рr; bЮ-bounded transmission schedule.

Proof. It can be proved by a similar method shown in Lemmas 1 and 3.

Fig. 5 shows the upper bound	AрЮ	and the lower bound	RрЮ	for all the

optimal transmission schedules with the same peak bandwidth r, buﬀer size b,

initial delay	d	and network utilization	u. Instead of giving a ﬁxed schedule

result, our approach gives the upper bound and the lower bound of the optimal

transmission schedules. It allows users to determine their own optimal sched-

ules under various QoS requirements and resource constraints to support

diﬀerentiated services. For example, if we want to smooth the variance of

transmission bandwidths applied, we can use the MVBA algorithm [20] to the

upper bound and the lower bound of the optimal transmission schedules. As

shown in Fig. 5, the obtained result not only provides the minimal bandwidth

variance but also has the optimal resource allocation and utilization. It is better

from the original MVBA algorithm that does not guarantee the optimum of

initial delay and network utilization. The same idea can be applied to minimize

the number of bandwidth changes by MCBA [9].

Note that, at any time t, AрtЮ	represent the maximal amount of media data

that could be received by client without buﬀer overﬂow. The minimal amount

of media data that should be received is RрtЮ. Let the media stream V be packed

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

up bound

A(t)

low bound

R(t)

smoothed optimal

transmission schedule

71

ts

start time

te

end time

t

Fig. 5. An example that uses MVBA to the upper bound and the lower bound of the optimal

transmission schedules. The result obtained not only has the minimal bandwidth variance but also

has the optimal resource allocation and utilization.

as packets	fp0; p1; . . .g	for network transmission, and the cumulative size

Pxј p0ю p1ю    ю px. When a transmission schedule GрЮ is speciﬁed, packet

pxis transmitted/received at time	t. Based on the upper bound	AрЮ	and the

lower bound RрЮ, we can specify the schedulable region рsx; exЮ	for each data

packet pxas follows:

sxј maxft j AрtЮ ј Pxg;

р11Ю

exј maxft j RрtЮ ј Pxg:

As shown in Fig. 6, the ready time sxis the earliest time that pxcould start its

transmission. The deadline exis the latest time that pxshould be received. By

packet

px

t

s

ready time

e

deadline

x

x

Fig. 6. We can utilize AрЮ	and RрЮ	to specify the schedulable region (ready time, deadline)	ј

рsx; exЮ for each data packet px.

72

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

pre-specifying the real-time constraints (ready time and deadline) for each

media packet, a better network scheduling and error control algorithm can be

provided to transmit multiple media streams.

4. Experiment results

In this paper, diﬀerent MPEG-encoded VBR media traces [22,23] are ex-

amined to evaluate the eﬀectiveness of the proposed algorithm. The statistics of

the test streams are listed in Table 1. It includes their frame numbers, the frame

rates (number of frames played per second), the frame sizes (maximum, av-

erage, and standard deviation), and the group-of-picture sizes (maximum,

average, and standard deviation). For each media stream, diﬀerent perfor-

mance parameters (such as buﬀer size, initial delay, network bandwidth, and

network idle rate) are considered. The ﬁrst video trace examined is a 2-h long

MPEG-encoded movie	Star War. The cumulative playback function of	Star

War is shown in Fig. 7(a) by its ﬁrst 100 frames. Fig. 7(b) presents how the

required buﬀer size and the obtained network idle rate change with the increase

of allocated network bandwidth. As presented in [5], there is a tradeoﬀ between

the allocated buﬀer size and network bandwidth (called the bandwidth-buﬀer-

tradeoﬀ function). The required buﬀer size is piecewise-linearly and mono-

tonically decreasing when the allocated bandwidth is increased. Note that, our

algorithm is optimal in resource allocation. As shown in Fig. 7(b), it requires

only 8 MB buﬀer to transmit Star War by 0.44 Mbps network bandwidth and

20 ms initial delay. Our algorithm is much better than CRTT [19] that requires

over 10 GB client buﬀer to transmit Star War with jitter-free playback.

The second and the third test examples are two nearly 90 min long video

traces: Princess Bride and CNN News. Both of them are encoded by Futuretel

hardware MPEG coder with the same frame rate 30 fps (frame-per-second) and

average frame size 4.89 KB. As the hardware coder uses variable distortion

coding to maintain its target rate, the average group-of-picture (GoP) size is

the same and the standard deviation of the GoP size is small. We show the

Table 1

Statistics of the media streams used in our experiments

Stream

Frame

Frames

Frame size (KB)

GoP size (KB)

name

number

rate (fps)

Max	Avg	S.D.	Max	Avg	S.D.

Star War

CNN News

174136	24

164748	30

22.62	1.90	2.3

30.11	4.89	3.7

118.1	23.4	9.2

94.0	75.0	2.3

Princess Bride	167766	30

29.73	4.89	4.8

102.0	75.0	2.7

Lecture

16316	30

6.14	1.37	1.6

34.8	21.0	4.2

Advertisements	16316	30

10.08	1.86	1.9

124.1	28.5	13.0

fps: frame-per-second; GoP: group-of-picture.

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

73

Fig. 7. Star War: (a) The cumulative playback function of the ﬁrst 100 frames. (b) The optimal

network bandwidth, buﬀer size and network idle rate obtained by our proposed algorithm.

cumulative playback function of the ﬁrst 100 frames of CNN News in Fig. 8(a).

The optimal resource allocation and utilization obtained are shown in Fig.

8(b). As the variance of frame sizes is small, the bandwidth-buﬀer-tradeoﬀ

74

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

Fig. 8. CNN News: (a) The cumulative playback function of the ﬁrst 100 frames. (b) The optimal

network bandwidth, buﬀer size and network idle rate obtained by our proposed algorithm.

function is almost linear. Besides, the network idle rate is near zero when the

transmission bandwidth is lower than the average stream rate 1.15 Mbps. For

example, by applying the average stream rate as the transmission bandwidth,

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

75

the client can continuously play CNN News with 38 KB memory buﬀer and 150

ms initial delay. On an OC-3 link, we can support over 110 users to watch CNN

News	at the same time by the transmission schedule obtained. Comparing

Princess Bride	and	CNN News, the hardware coded streams have almost

identical statistics. The obtained results, i.e. the required transmission band-

width, network utilization, buﬀer size and initial delay, are almost the same.

Diﬀerent from the hardware coder, a software coder may introduce a high

variance in GoP sizes due to the video contents presented. In this paper, our

next two test examples are video traces Advertisements and Lecture encoded by

the UCB software MPEG coder [14]. Both of them are about 10 min long with

very diﬀerent contents. Lecture shows a speaker and slides with zooming and

Fig. 9. The optimal network bandwidth, buﬀer size and network idle rate obtained by our pro-

posed algorithm for (a) Advertisements and (b) Lecture.

76

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

panning. As the frame contents are similar, the variation of GoP sizes is small.

On the contrary, Advertisements contains a sequence of advertisements. It has

the diﬀerent frame contents from one scene to another scene. Therefore, the

variation of GoP sizes is large. Fig. 9(a) and (b) shows the optimal resource

allocation and utilization obtained for	Advertisements	and	Lecture, respec-

tively. Our experiments conclude that the GoP sizes and their variations may

aﬀect the resources required for media transmission. The network bandwidth

and the network idle rate required for transmitting high-variance Advertise-

ments is larger than that required for transmitting low-variance Lecture under

the same client buﬀer size. The relations between the required buﬀer size and

the obtained network idle rate for diﬀerent streams, high-variance Advertise-

ments and low-variance Lecture, are compared in Fig. 10.

Although our approach has already proved optimal in resource allocation

and utilization, we would like to compare our optimal schedule results to the

schedule results obtained by previous approaches to measure the improve-

ments achieved. In Fig. 11, we compare the proposed algorithm to CRTT by

the client buﬀer required for transmitting Advertisement under diﬀerent initial

delays. Experiments show that, only when the initial delay is over 50 s, CRTT

can provide the similar buﬀer size as our approach obtained. Our improvement

in required buﬀer size is dramatic. Fig. 12 shows the comparisons of our

proposed algorithm and MVBA by network idle rate obtained under diﬀerent

initial delays. Note that, as MVBA requires pre-specifying buﬀer size and

initial delay to decide the related network bandwidth, it does not provide a way

to minimize both buﬀer size and initial delay at the same time. To do the fair

comparisons, the optimal buﬀer size and initial delay obtained by our approach

100

Advertisements

Lecture

80

60

40

20

0

0

50

100

150

200

250

300

Client Buffer Size (KB)

Fig. 10. The relations between the required buﬀer size and the obtained network idle rate for

diﬀerent streams, Advertisements and Lecture.

Netw

o

rk

Idle Rate (%)

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

Advertisements

8

proposed

77

7

6

5

4

3

2

1

0

CRTT

0	10	20	30	40	50	60	70

Initial Delay (sec)

Fig. 11. A comparison of the client buﬀer size required for our proposed algorithm and CRTT with

diﬀerent initial delay.

Advertisements

30

proposed

25

20

15

10

5

0

MVBA

0	10	20	30	40	50	60	70

Initial Delay (sec)

Fig. 12. A comparison of the network idle rate required for our proposed algorithm and MVBA

with diﬀerent initial delay.

are assigned as the given parameters to MVBA. Experiments show that our

approach is better than the MVBA approach in network utilization obtained.

5. Conclusion

In this paper, a traﬃc shaping scheme is introduced to decide the suitable

transmission schedules. Instead of giving a ﬁxed schedule result, our approach

Client Buff

e

r Siz

e

(MB)

Netw

or

k Idle Rate (%)

78

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

decides the schedulable region for all the transmission schedules that have the

optimal allocation and utilization in system resources. Experiments have

shown that our algorithm can achieve a dramatic improvement than the

conventional approaches in both the buﬀer size and the network idle rate.

Based on the schedulable region provided, the ready time and deadline are

precisely speciﬁed to each media packet to support real-time network sched-

uling and error control. The proposed approach is shown to be practical, ef-

ﬁcient, and ﬂexible in supporting continuous media transmission.

References

[1] T.M. Apostol, Math. Anal. (1973).

[2] E. Chang, A. Zakhor, Scalable video data placement on parallel disk arrays, in: IS & T/SPIE

Symposium on Electronic Imaging Science and Technology, Febraury 1994.

[3] R.I. Chang, M.C. Chen, J.M. Ho M.T. Ko, Designing the on-oﬀ CBR transmission schedule

for jitter-free VBR media playback in real-time networks, in: IEEE RTCSA, 1997, pp. 1–9.

[4] R.I. Chang, M.C. Chen, J.M. Ho, M.T. Ko, Optimizations of stored VBR video transmission

on CBR channel, in: SPIE VVDC, 1997, pp. 382–392.

[5] R.I. Chang, M.C. Chen, M.T. Ko, J.M. Ho, Characterize the minimum required resources for

admission control of pre-recorded VBR video transmission by an Oрn log nЮ, in: IEEE IC3N,

1998.

[6] R.I. Chang, M.C. Chen, J.M. Ho, M.T. Ko, An eﬀective and eﬃcient traﬃc smoothing scheme

for delivery of online VBR media streams, in: IEEE INFOCOM, 1999.

[7] R.I. Chang, W.K. Shih, R.C. Chang, A new real-time disk scheduling algorithm and its

application to multimedia systems, in: 5th IEEE IDMS, Lecture Notes on Computer Science,

1998.

[8] M. Chen, D.D. Kandlur, P.S. Yu, Optimization of the grouped sweeping scheduling (GSS)

with heterogeneous multimedia streams, in: ACM Multimedia Conference, 1993, pp. 235–242.

[9] W. Feng, F. Jahanian, S. Sechrest, Optimal buﬀering for the delivery of compressed

prerecorded video, in: IASTED International Conference on Networks, January 1996.

[10] W. Feng, S. Sechrest, Smoothing and buﬀering for delivery of prerecorded compressed video,

in: IS&T/SPIE Multimedia Computing and Networking, February 1995, pp. 234–244.

[11] M. Garrett, W. Willinger, Analysis, modeling and generation of self-similar VBR video traﬃc,

in: ACM SIGCOMM, August 1994, pp. 269–280.

[12] M. Grossglauser, S. Keshav, On CBR service, in: IEEE INFOCOM, March 1996.

[13] M. Grossglauser, S. Keshav, D. Tse, RCBR: a simple and eﬃcient service for multiple time-

scale traﬃc, in: ACM SIGCOMM, August 1995.

[14] E.W. Knightly, D.E. Wrege, J. Liebeherr, H. Zhang, Fundamental limits and tradeoﬀs of

providing deterministic guarantees to VBR video traﬃc, in: ACM SIGMETRICS, May 1995,

pp. 98–107.

[15] M. Krunz, H. Hughes, A traﬃc model for mpeg-coded VBR streams, in: ACM SIGMET-

RICS, May 1995, pp. 47–55.

[16] S.S. Lam, S. Chow, D.K.Y. Yau, An algorithm for lossless smoothing of MPEG video, in:

ACM SIGCOMM, 1994.

[17] T. Ott, T.V. Lakshman, A. Tabatabai, A scheme for smoothing delay-sensitive traﬃc oﬀered

to ATM networks, in: IEEE INFOCOM, 1992.

[18] A.R. Reibman, A.W. Berger, Traﬃc descriptors for VBR video teleconferencing over ATM

networks, in: IEEE/ACM Trans. Networking, June 1995.

R.-I. Chang et al. / Information Sciences 141 (2002) 61–79

79

[19] J.M. McManus, K.W. Ross, Video on demand over ATM: Constant-rate transmission and

transport, in: IEEE INFOCOM, March 1996.

[20] J.D. Salehi, Z.L. Zhang, J.F. Kurose, and D. Towsley, Supporting stored video: reducing rate

variability and end-to-end resource requirements through optimal smoothing, in: Proceedings

ACM SIGMETRICS, 1996.

[21] K. Sohraby, On the theory of general on-oﬀ source with applications in high-speed networks,

in: IEEE INFOCOM, 1993, pp. 401–410.

[22] FTP ftp::/thumper.bellcore.com.

[23] URL http::/www.eeb.ele.tue.nl/mpeg/.

[24] Y.C. Wang, S.L. Tsao, R.I. Chang, M.C. Chen, J.M. Ho, M.T. Ko, A fast data placement

scheme for video server with zoned-disks, in: SPIE VVDC, 1997, pp. 92–102.

[25] H. Zhang, E.W. Knightly, A new approach to support delay-sensitive VBR video in packet-

switched networks, in: IEEE NOSSDAV, 1995.

[26] J. Zhang, J. Hui, Traﬃc characteristics and smoothness criteria in VBR video traﬃc

smoothing, in: IEEE ICMCS, June 1997, pp. 3–11.

[27] R.I. Chang, W.K. Shih, R.C. Chang, Real-time disk scheduling for multimedia applications

with deadline-modiﬁcation-scan scheme, Real-time Syst. 19 (2) (2000) 149–168.

[28] R.I. Chang, C.L. Chen, W.K. Shih, R.C. Chang, Design and implementation of a real-time

disk scheduling algorithm by the multi-segment cascade scheme, in: IEEE RTSS (WIP) 1999,

pp. 1–5.