1

COMCOM 2938

18 January 2006	Disk Used

ARTICLE IN PRESS

Computer Communications xxx (2006) xxx–xxx

No. of Pages 11, Model 5+

Sankar (CE) / PadmaPriya (TE)

www.elsevier.com/locate/comcom

2

3

Traﬃc engineering with OSPF-TE and RSVP-TE: Flooding

reduction techniques and evaluation of processing cost

4	Stefano Salsanoa,*, Alessio Bottab, Paola Iovannac, Marco Intermiteb, Andrea Polidoroa

5

6

7

8

9

10	Abstract

a	DIE – Universita` di Roma ‘‘Tor Vergata,’’ Dip. Ingegneria Elettronica Via di Tor Vergata 110, 00133 Rome, Italy

b	CoRiTeL – Consorzio di Ricerca sulle Telecomunicazioni

c	Ericsson Lab Italy S.p.A.

Received 11 July 2003; received in revised form 6 December 2005; accepted 7 December 2005

11	This paper considers two important aspects related to the control plane of Traﬃc Engineered IP/MPLS networks: the ‘‘ﬂooding

12	reduction’’ mechanisms and the evaluation of processing cost for signaling and routing protocols. The ﬂooding reduction mechanisms

13	are needed to reduce the amount of information exchanged by Traﬃc Engineering enabled routing protocols. The trade-oﬀ between the

14	amount of information exchanged and the network performance (connection blocking probability) is discussed in the light of speciﬁc

15	aspects of OSPF-TE routing protocol and RSVP-TE signaling protocol. Diﬀerent mechanisms are analyzed and a suggestion is given

16	for the best one. The dynamic aspects related to the time needed to distribute the routing and signaling information are considered. Final-

17	ly, the combined processing cost of routing and signaling is analyzed, and the possible bottlenecks of the architecture are discussed. It is

18	worth mentioning that the discussed results have been derived not only with simulation/analysis but also with measurements coming

19	from a testbed implementation.

20	2005 Elsevier B.V. All rights reserved.

21	Keywords:	MPLS traﬃc engineering; OSPF-TE; RSVP-TE

22

23	1. Introduction

24	The so-called ‘‘new generation networks’’ handle a huge

dynamically setup and released by means of a proper signal-	37

ing protocol. Each MPLS-TE enabled node supports both a	38

routing protocol and a label distribution protocol. The pos-	39

25 amount of IP traﬃc, a large portion of this traﬃc demands

sible routing protocols are OSPF-TE [4] and ISIS-TE [5],

40

26 more than ‘‘best eﬀort’’ service (for example QoS and reli-

27 ability). Multi-Protocol Label Switching (MPLS) technolo-

28 gy [1] can be useful to cope with these requirements. MPLS

29 can enable smart Traﬃc Engineering (TE) [2,3]	strategies,

30 which handle in the most ﬂexible way the network resources,

31 and react dynamically to traﬃc changes. In this advanced

32 scenario, paths for traﬃc ﬂows can be chosen according to

33 some optimality criteria by the so-called Constraint Based

which extend OSPF and IS-IS respectively. Speciﬁcally,	41

the traditional routing protocols have been enhanced with	42

the ability to carry information related to link attributes/	43

states, to be used for explicit route calculation (e.g., avail-	44

able/reserved bandwidth). The label distribution protocol	45

(or ‘‘signaling’’ protocol) is used to setup the so called Label	46

Switched Paths (LSPs), supporting both explicit route indi-	47

cation and reservation of resources during dynamic LSP set-	48

34 Routing (CBR) algorithm. The input to the CBR algorithm

up. RSVP-TE [6] and CR-LDP [7] are the two ‘‘TE-capable’’

49

35 is the information about the status of the network that is dis-

36 tributed in real-time by the routing protocol. The paths are

label distribution protocols. In the following we will always	50

consider OSPF-TE as the routing protocol and RSVP-TE as	51

the signaling/label distribution protocol. This is consistent	52

with the decisions in IETF to continue with the standardiza-	53

*	Corresponding author. Tel.: +39 06 7259 7450; fax: +39 06 7259 7435.

tion of RSVP-TE rather than CR-LDP [8]. Fig. 1 provides a

54

E-mail address: stefano.salsano@uniroma2.it (S. Salsano).

0140-3664/$ - see front matter	2005 Elsevier B.V. All rights reserved.

doi:10.1016/j.comcom.2005.12.007

representation of the logical entities involved in the TE pro-	55

UNCORRECTED

PROOF

2

COMCOM 2938

18 January 2006	Disk Used

ARTICLE IN PRESS

S. Salsano et al. / Computer Communications xxx (2006) xxx–xxx

No. of Pages 11, Model 5+

Sankar (CE) / PadmaPriya (TE)

Traffic Demands

TE enhanced

Route Decision

Engine - RDE

TE capable

Routing Protocol

CONTROL PLANE

DATA PLANE

TE capable

Signaling

Protocol

RSVP-TE/CR-LDP

OSPF-TE/ISIS-TE

This can be accomplished either with a static set of thresh-	87

olds or with ‘‘dynamic’’ thresholds, by considering the rela-	88

tive variation with respect to the older information. We	89

compare these two approaches, showing that the dynamic	90

approach performs slightly better that the ﬁxed thresholds	91

approach and it is much easier to manage and tune. We will	92

show that these mechanisms can reduce the amount of ﬂood-	93

ing in a network by a large factor (e.g., by 5 or 10 times).	94

After presenting the network and traﬃc models in Sec-	95

tion 2, in Section	3	we will analyze the performance in	96

terms of call blocking probability covering the trade-oﬀ	97

between signaling load and performance. Our results are	98

consistent to those described in the literature ([9–11]) but	99

we introduce noteworthy contributions:	100

IP / MPLS forwarding engine

• the analysis of why the dynamic thresholds are prefera-

101

IP

routing table

MPLS label

swapping table

IP and MPLS

ble to the static one and the reﬁnements of the static	102

thresholds to reach the performance of the dynamic ones	103

• results coming both from simulation and from a testbed	104

implementation with real measurements.	105

Fig. 1. Architecture of a TE enabled node (LER case).

56 cess and of their relationships (including the ‘‘data plane’’

57 elements).

58	We assume that Edge Nodes (LER – Label Edge Rou-

106

We observe that the traﬃc engineering process described	107

so far is a highly distributed process, which can suﬀer of	108

inconsistent co-ordination between the various elements.	109

There are two possible sources of inconsistency that should	110

59 ters) receive the indication of the ‘‘Traﬃc Demands’’ to

60 be supported, and that this is a dynamic process. Note that

61 in this context a Traﬃc Demand (i.e., a ﬂow) is typically an

be taken into account: the ‘‘Information Propagation Time’’

and the ‘‘Imprecise Information’’.

The	Propagation Time	problem is related to the time

111

112

113

62	aggregate	of several IP micro-ﬂows. Once a request has

63 been presented to an Edge Node, we assume that a logical

64 entity, that will be referred to as ‘‘Route Decision Engine’’

65 (RDE), chooses the proper route within the network.1The

66 RDE gathers the information related to the current topol-

67 ogy and resource usage in the network by continuous inter-

68 action with the TE capable routing protocol (OSPF-TE in

69 our assumption). When the RDE has chosen the route for

70 a Traﬃc Demand, the corresponding LSP will be setup

71 using RSVP-TE protocol, which will take care of perform-

72 ing node-by-node admission control and actual resource

73 allocation. OSPF-TE advertises the change of local

needed to propagate the information in the network via sig-	114

naling and routing protocols. In the mean time when the	115

information is not up-to-date, an Edge Node can take	116

incorrect (or sub-optimal) route selection decision. Anoth-	117

er similar problem is related to the race conditions between	118

allocation requests coming from two diﬀerent Edge Nodes	119

and arriving to an internal node almost in the same time,	120

when resources are not enough to accommodate both.	121

Note that in the design of the control architecture the net-	122

work architect has few chances to solve this kind of prob-	123

lems, which are inherent to the distributed approach.	124

Nevertheless, it is important to evaluate their impact on	125

74 resource allocation status to all other LSRs by sending a

75 Link State Update (LSU) message containing a special

the performance of the network.

The Imprecise Information problem is related to the ‘‘re-

126

127

76 kind of Link State Advertisement (LSA) object called opa-

77	que LSA [8]. The object is called opaque because it is ‘‘hid-

78 den’’ to the basic OSPF routing logic, as it is only used by

79 the TE logic. The LSU message is distributed to all LSRs

80 using the OSPF ‘‘ﬂooding’’ procedure. In order to avoid

81 that the information ﬂooding is executed for each minimal

82 change, some ‘‘ﬂooding reduction’’ mechanisms need to be

83 used, so that the origination rate of OSPF-TE LSU mes-

84 sages can be reduced.

85	The basic method to address the signaling ﬂooding prob-

86 lem is the distribution of a ‘‘coarser’’ link-state information.

1	Note that the Route Decision Engine (RDE) is a logical process, from

the physical standpoint it can either run ‘‘on’’ the LER or it can run on a

duced’’ information that can be distributed using OSPF-	128

TE. Due to the ‘‘ﬂooding reduction’’, the information	129

available in the Edge Nodes to take routing decisions will	130

be an approximation of the actual resource status. The	131

impact of this approximation on network performance	132

(e.g., network utilization, call blocking probability) must	133

be evaluated. Note that the network architect has greater	134

control on these aspects, as there are several ﬂooding	135

reduction techniques that can be chosen (and then tuned).	136

A trade-oﬀ can be envisaged between the signaling load to	137

distribute the information and the performance in terms of	138

network utilization and call blocking probability.	139

Some works in the literature describe the problem of	140

Imprecise Information and analyze the network perfor-	141

separate machine connected to the edge node.

mance. The work in [9] focuses on the trade-oﬀ between

142

UNCORRECTED

PROOF

COMCOM 2938

18 January 2006	Disk Used

ARTICLE IN PRESS

S. Salsano et al. / Computer Communications xxx (2006) xxx–xxx

No. of Pages 11, Model 5+

Sankar (CE) / PadmaPriya (TE)

3

143 the amount of ﬂooding and the network performance in

144 terms of utilization/blocking probability. The aspects of

145 processing cost are not explicitly dealt with. In [10], a sim-

146 ilar evaluation on the trade-oﬀ is given and some process-

147 ing	cost	aspects	are	also	considered	([11]	further

148 investigates on the processing cost aspect). The analysis

Table 1

Network topologies

Topology

7nodes

30nodes

N

7

30

L

44

118

h

1.52

3.96

C (Mb/s)

100

635

149 of processing cost in these works is concentrated on the

averaged among all node pairs h	and the link capacity C

178

150 routing protocol aspects and on the calculation of CBR

151 algorithms. The processing cost related to the signaling

152 protocol for path setup is not considered. We believe that

153 this cost cannot be neglected and an important contribu-

154 tion of our work is the combined evaluation of processing

155 cost for routing and signaling protocols given in section 0.

156 Note that the work in [9–11] was based on generic assump-

157 tions regarding TE-enhanced routing and signaling proto-

(Mb/s). The reason to have two diﬀerent topologies is that	179

the smaller 7nodes topology could be implemented both in	180

the simulation study and in a testbed (see II. C below),	181

allowing to compare simulation results with real measure-	182

ments. The 30nodes topology (the same used in [12]) was	183

used to have simulation results for a network size compa-	184

rable with a real life scenario.	185

158 cols, as the protocols were not yet deﬁned. In this paper

159 we could consider the actual behavior of OSPF-TE,

2.2. Traﬃc model and CBR algorithm

186

160 RSVP-TE and their interaction and even provide results

161 coming from a testbed implementation. To conclude the

162 survey on relevant literature, a very detailed analysis of

163 processing cost for OSPF-TE has been performed in [13],

164 anyway the focus of that work was on the stability issues

165 of OSPF and the results cannot be applied in our context.

166	To the best of our knowledge, the issue of Propagation

167 Time, i.e., the impact of the short-term dynamics of OSPF-

168 TE and RSVP-TE has not been thoroughly analyzed

169 before, and this constitutes a second important novelty of

170 our work, reported in Section 4. The goal is to deﬁne the

171 operational range where there is no impact of this inconsis-

172 tency on the network operations.

173	2. Network and traﬃc models

174	2.1. Network model

175	Two diﬀerent network topologies have been considered

176 for our study (Fig. 2). Table 1 reports the number of nodes

177	N, the number of unidirectional links	L, the hop count

In order to model the oﬀered traﬃc, we considered two	187

diﬀerent traﬃc models, a ‘‘uniform’’ model and a ‘‘non-	188

uniform’’ one.	189

We denote every (source, destination) couple as a Traf-	190

ﬁc Relation, the arrival rate of Traﬃc Demands within	191

each Traﬃc Relation	i	is denoted as ki(s1). Under the	192

uniform model, each node generates traﬃc requests	193

directed to all other nodes of the network, according to	194

a Poisson process, with uniform random selection of des-	195

tination nodes, therefore ki= k	"i. The total arrival rate	196

of Traﬃc Demands originating in each node is denoted	197

as	knode= (N	1) k.	198

In the case of ‘‘non-uniform’’ model, the composition of	199

two request arrival processes is considered. In addiction to	200

a background uniform traﬃc, of rate	kBG(s1) per each	201

traﬃc relation, we have a foreground traﬃc generated by	202

a number of hot-spot pairs, with rate kFG(s1). According	203

to [10], we varied the amount of this foreground traﬃc in	204

respect of total oﬀered load up to 30%.	205

We model connection holding times using a negative	206

exponential distribution where T is the mean holding time.	207

The bandwidth of each Traﬃc Demand is uniformly dis-	208

7nodes

tributed between 0 and 2b of the capacity	C	of a link.

209

Therefore, the mean value of a single Traﬃc Demand is	210

bC. The oﬀered load for each traﬃc relation	i	will be

RioјkiTbC (bit/s). In the simulation scenario used in this

paper we set T = 200 s (a relatively short ﬂow duration in

order to have a quite dynamic scenario).

211

212

213

214

In order to characterize the oﬀered load to the network,	215

we deﬁne a ‘‘normalized’’ oﬀered load assuming that all the	216

traﬃc demands are routed through a shortest path. We	217

denote hithe shortest path length of the traﬃc relation i,

hence the normalized oﬀered load becomes (NTRis the

number of Traﬃc Relations):

218

219

220

qSPј

XTR

iј1

,XL

| R | i Ohi |
| --- | --- |

jј1

Cj.

222

Fig. 2. Network topologies.

In the ‘‘uniform’’ traﬃc model the normalized traﬃc	223

load becomes, as NTR= N(N	1):	224

UNCORRECTED

PROOF

4

COMCOM 2938

18 January 2006	Disk Used

XTR

ARTICLE IN PRESS

S. Salsano et al. / Computer Communications xxx (2006) xxx–xxx

i4

No. of Pages 11, Model 5+

Sankar (CE) / PadmaPriya (TE)

R+

226

qSPј

iј1

kTbh L ј NрN	1ЮkTbh=L.

i3

i2

R+

R

R

R-

R+

227 where h is the mean distance (in number of hops) between

228 nodes, averaged across all traﬃc relations (i.e., all pairs of

229 Edge Nodes).

230	In the non-uniform model we can divide the total oﬀered

i1

R-

R

R-

231 load in the two background and foreground components:

,

Static

thresholds

t=t1t=t2t=t3

Dynamic

thresholds

233

qSPј

XTR

iј1

kBGTbh	L ю

N	XSPOT

iј1

kFGTbhi=L

Fig. 3. Static and dynamic thresholds.

234	We considered a CBR algorithms. that favors an evenly

235 distribution of the traﬃc in the network even if it means

236 considering longer path (‘‘least resistance’’	[14]). The cost

237	Siof each link i is Siј BT=BAi	where BTis the maximum

238 link bandwidth in the network, and	BAi	is the available

239 bandwidth in the link i. Links with not enough bandwidth

240 are pruned as well.

241	2.3. Simulation environment and testbed

242	We implemented a ‘‘custom’’ event-based simulator for

243 the OSPF-TE/RSVP-TE environment. The simulator is

244 developed in C++ under the Linux OS, and is available

Using static thresholds, the link capacity is divided in	275

intervals, limited by upper and lower threshold levels. In	276

order to limit the eﬀect of the inaccuracy introduced by	277

the thresholds, it is sensible to ﬁx just a few threshold levels	278

in the lower part of link bandwidth occupancy and much	279

more levels in the higher part of link bandwidth occupancy	280

(near congestion). There is a large degree of freedom in the	281

choice of the number and of the values of the threshold lev-	282

els. In order to experiment with the diﬀerent choices it is	283

reasonable to deﬁne families of static threshold mecha-	284

nisms that can be characterized by few parameters. The	285

two families of threshold mechanisms (‘‘logarithmic’’ and	286

‘‘3-piece-linear’’) that we have considered are described in	287

245 at [15]. The simulator is able to consider two diﬀerent sce-

246 narios. In the ﬁrst one there is the assumption of ‘‘ideal’’

Appendix A. Additional details about the use of threshold

values are given in Appendix B.

288

289

247 (e.g.,	instantaneous)	propagation	of	RSVP-TE	and

248 OSPF-TE information (see results in section 3). In the sec-

249 ond scenario the real propagation of OSPF-TE and RSVP-

The dynamic threshold approach assigns an initial	290

threshold level on the empty link and calculates next upper	291

and lower levels as functions of currently advertised reser-	292

250 TE information (see results in Section 4) is considered in

vation amount. Let C be the link capacity and R the cur-

293

251 the simulation by taking into account the processing and

rently advertised reserved bandwidth, the upper and	294

252 transmission time of RSVP-TE and OSPF-TE messages.

253	The testbed is composed of 7 PCs with a Linux Operat-

254 ing System (RedHat 7.1), which are interconnected by

lower thresholds are calculated, respectively, as

Rюј R ю F  рC	RЮ;	R	ј R	F  рC	RЮ.

295

297

255 point-to-point Ethernet links (100 Mb/s) according to the

256 topology shown in Fig. 2 (7nodes topology). Each PC rep-

257 resents a network node with a fully functional implementa-

Note that, as desired, the diﬀerence between upper and	298

lower thresholds becomes narrower when the available	299

bandwidth decreases. Note also that a larger value of	300

258 tion of the MPLS-TE control plane (including OSPF-TE

F(0 < F < 1) means more spaced dynamic threshold levels

301

259 and RSVP-TE daemons, Route Decision Engine, Traﬃc

and a coarser vision of network status in the RDEs.	302

260 Request Generator). The software packages installed and

261 active on the test bed are: MPLS provided by Sourceforge

262	[16], RSVP-TE daemon from TEQUILA project	[17]	and

263 OSPF daemon by GNU Zebra software, version 0.92	[18]

Fig. 3 provides a sketch of the two mechanism.

3.1. Results and discussion

303

304

264 patched with TE extensions. It implements OSPF v.2

265 according to [19] with Opaque LSA capabilities [20]. Addi-

266 tional details on the testbed can be found in [21,22].

Let us analyze the trade-oﬀ between the amount of	305

ﬂooding and the network performance in terms of connec-	306

tion blocking. We started with a simulation analysis, in the	307

scenario of ‘‘ideal’’ (e.g., instantaneous) propagation of	308

267	3. ‘‘Resource thresholds’’ mechanisms

RSVP-TE and OSPF-TE information.

The main results are reported in Figs. 3–6.2The leftmost

309

310

268	The idea of resource threshold mechanisms is to adver-

269 tise only signiﬁcant changes of link state information.

270 Therefore, a single advertisement is typically performed

271 after a number of LSP setups and releases, instead of com-

272 municating the change of network status for each setup

273 (release) of an LSP. The threshold mechanisms can be clas-

274 siﬁed in static and dynamic ones.

value of the curves represents the network behavior with no	311

threshold mechanisms (perfect vision). When we have a	312

coarser information (smaller number of thresholds in the	313

2	30nodes	topology,	b = 0.05; for the static thresholds: logarithmic

function a = 104. The ﬁgures are obtained under the uniform traﬃc model,

but no diﬀerence can be noticed under the non-uniform traﬃc model.

UNCORRECTED

PROOF

COMCOM 2938

18 January 2006	Disk Used

ARTICLE IN PRESS

S. Salsano et al. / Computer Communications xxx (2006) xxx–xxx

No. of Pages 11, Model 5+

Sankar (CE) / PadmaPriya (TE)

5

0,20

Looking at Figs. 3–6, we observe that there is a region

329

0,18

0,16

0,14

0,12

0,10

0,08

0,06

0,04

0,02

0,00

no_thresholds	14

ρ

ρ

ρ

10

7

4

2

(starting from the left) where the blocking probability does	330

not increase signiﬁcantly while the OSPF-TE message	331

ﬂooding is greatly reduced. This suggests that the optimal	332

working point is where the blocking probability start to	333

increase: in the given scenarios 7 thresholds for the static	334

thresholds or F = 0.7 for the dynamic ones.	335

We deﬁne as ‘‘merit’’ factor the ratio between the	336

amount of ﬂooding with thresholds and without thresh-	337

olds. For oﬀered load 0.6, this factor is 3.1 for static-	338

Number of Samples

threshold and 10.6 for the dynamic thresholds, respectively	339

at 7 thresholds and at F = 0.7 where the blocking probabil-	340

0,20

0,18

Fig. 4. Static thresholds: ﬂooding.

ity is still under control. In Fig. 8 we compare 3-piece linear

(b = 0.75, c = 0.95) static thresholds with 14 and 7 levels,

logarithmic (a = 104) static thresholds with 14 and 7 levels

and dynamic (F = 0.7) thresholds. The 3-piece linear and

341

342

343

344

0,16

0,14

0,12

0,10

0,08

0,06

0,04

0,02

0,00

ruoting failures

setup failures

ρ

ρ

ρ

the logarithmic thresholds have the same merit factor	345

(1.7) for 14 levels while the 3-piece linear yields a larger	346

reduction (merit factor 3.3) than the logarithmic (2.2) for	347

7 levels. The dynamic thresholds have the larger merit fac-	348

tor (7.3). Note that the connection blocking probability	349

using static mechanisms with 14 thresholds is unchanged	350

with respect to the case without any threshold method,	351

and only minimally increased using static mechanism with	352

no_thresholds	14

10

7

4

2

7 thresholds or dynamic mechanisms with F = 0.7.

353

Number of samples

Fig. 5. Static thresholds: connection blocking probability.

0,20

0,18

0,16

ρ

0,14

0,16

0,14

0,12

0,10

0,08

ρ

ρ

0,12

0,10

0,08

0,06

0,04

ρ

ρ

0,06

0,04

0,02

0,00

ruoting failures

ρ

setup failures

0,02

0,00

no_thresh. 0,1	0,2	0,3	0,4	0,5	0,6	0,7	0,8	0,9

Parameter F

Fig. 6. Dynamic thresholds: ﬂooding.

314 static scenario, larger F in the dynamic one) we can drasti-

315 cally reduce the amount of ﬂooding (the number of LSU

316 messages originated per link per second is shown). On

317 the other hand, blocking probability starts to increase

318 when the information is too coarse. The analysis is report-

319 ed for three diﬀerent values of the ‘‘conventional’’ oﬀered

320 load qSPfrom 0.6 up to 0.8. The typical operating point

321 should be qSP= 0.6 or less, where the blocking probability

322 is around 2%, while qSP= 0.7 and qSP= 0.8 can be already

323 considered overload conditions, considering that the block-

324 ing probability is respectively in the order of 8% and 14%.

0,18

0,16

0,14

0,12

0,10

0,08

0,06

0,04

0,02

0,00

no_thresh. 0,1	0,2	0,3	0,4	0,5	0,6	0,7	0,8	0,9

Parameter F

Fig. 7. Dynamic thresholds: connection blocking prob.

0,165	Flooding rate per link (1/s)

Connection Blocking Probability

0,098	0,097

0,074

0,050

0,023

359

360

361

325 Note that we will not show 95% conﬁdence intervals of

No

Static

Static 3pl

Static

Static 3pl

Dynamic

326 simulation results, however results are averages over long

327 runs and such conﬁdence intervals are always smaller than

Thresholds

Logaritm.

Thresh.

(n=14)

Thresh.

(n=14)

Logarithm.	Thresh. (n=7)	Thresh.

Thresh. (n=7)	(F=0.7)

362

363

328 3% of the value.

Fig. 8. Static vs. dynamic threshold.

UNCORRECTED

PROOF

Connection Blocking Probability

Flooding (1/s)

Connection Blocking Probability

Flooding (1/s)

6

COMCOM 2938

18 January 2006	Disk Used

ARTICLE IN PRESS

S. Salsano et al. / Computer Communications xxx (2006) xxx–xxx

No. of Pages 11, Model 5+

Sankar (CE) / PadmaPriya (TE)

364	Several simulations have been carried out for the two

ing probability for an oﬀered load qSP= 0.7 is split into

392

365 considered network topologies, under diﬀerent load scenar-

366 ios and diﬀerent traﬃc models: using the dynamic thresh-

367 olds with	F = 0.7, we obtained a merit factor ranging

368 from 8 to 15 without aﬀecting in signiﬁcant way the net-

369 work performance (same blocking probability). The results

370 with static thresholds are not equally stable. Comparing

the two components of ‘‘routing’’ failures and ‘‘setup’’	393

failures. The former ones represent the connections reject-	394

ed by the CBR algorithm in the ingress Edge Node, the	395

latter ones the connections which are accepted by the	396

CBR algorithm, but then rejected by the RSVP-TE setup	397

procedure due to the local admission control in one of the	398

371 the static thresholds with the dynamic ones, we think that

crossed nodes. According also to	[9], we note that the

399

372 it is much easier to reduce OSPF-TE protocol message

373 exchange with the dynamic ones. Moreover, we can say

374 that the dynamic threshold mechanism is simpler to be con-

375 ﬁgured because only the value of F needs to be ﬁxed. This

376 means that one does not have to conﬁgure all the threshold

377 values in the routers as in the static thresholds. The use of

coarser the information, the larger the number of connec-	400

tions that are rejected during the setup phase, originating	401

an unneeded signaling in the network. This suggests that a	402

more detailed analysis should be performed to take into	403

account also the signaling load in the deﬁnition of the	404

optimal working point. This analysis will be carried out	405

378 dynamic thresholds could represent an important improve-

379 ment with respect to the currently used static thresholds.

380	In order to validate the simulation analysis, the dynamic

381 threshold mechanism has been implemented in our testbed

in Section 0.

4. Impact of message processing/transmission time

406

407

382 and various experiments have been carried out in parallel

383 with the simulation environment with the 7nodes topology

384 (identical to the testbed topology). The main results are

385 reported in	Figs. 9 and 10. These two ﬁgures represent a

386 comparison between the simulated scenario and the emu-

387 lated one (testbed). An oﬀered load	qSP= 0.7 is used. As

388 can be seen from the ﬁgures we have obtained in the test-

As we have observed in the previous section, there is a	408

good agreement between the results coming from the ‘‘ide-	409

al’’ simulator and from the testbed. We recall that in the	410

simulations analyzed in the previous section, an ideal	411

behavior for both reservation and routing protocol has	412

been assumed. This means that all processing and propaga-	413

tion times of control plane messages were considered to be	414

389 bed the same behavior as in the simulation.

null.

415

390	The ﬁnal consideration in this section concerns the sig-

391 naling load due to RSVP-TE. In Figs. 5 and 7	the block-

45

40

Simulator

35

Testbed

30

25

20

The agreement between simulation and testbed results	416

seems to imply that there is no impact of the RSVP-TE	417

and OSPF-TE delays in propagating signaling messages.	418

In this paragraph, we want to verify under which operating	419

conditions this assumption is valid. To analyze the impact	420

on network performance of RSVP-TE and OSPF-TE	421

delays, as a function of the overall connection requests	422

rate, we introduced the processing delays in the simulator	423

and considered the actual behavior of RSVP-TE and	424

15

OSPF-TE in propagating their messages.

425

10

5

0

0	0,1	0,2	0,3	0,4	0,5	0,6	0,7	0,8	0,9

Value of parameter F

Fig. 9. Flooding reduction comparison.

As a preliminary step, we had to ﬁgure out the charac-	426

teristic delays of RSVP-TE and OSPF-TE messages. The	427

value for the processing/propagation time of an OSPF-	428

TE LSU has been taken from [23]. Our simplifying hypoth-	429

esis is that this delay remains constant from hop to hop and	430

over time. Therefore, the propagation time of an LSU	431

ﬂooding procedure is linear with the number of hops	432

0,08

0,07

0,06

0,05

0,04

0,03

0,02

0,01

0

Simulator

Testbed

0	0,1	0,2	0,3	0,4	0,5	0,6	0,7	0,8	0,9

Value of parameter F

Fig. 10. Blocking probability comparison.

crossed. The value of a single hop processing/propagation	433

time has been set to 34 ms.	434

RSVP-TE messages (Path, Resv, PathTear, and Resv-	435

Tear) processing/propagation times were taken from [24].	436

Again, we made the simplifying assumption that all these	437

times remain constant during the evolution of a simulation,	438

as if they were independent from the number of reservation	439

sessions installed. We considered values of 14, 14, 6, and	440

20 ms respectively for Path, Resv, PathTear, and ResvTear	441

processing/propagation times.	442

These delays add inaccuracy in the RDE vision of net-	443

work status. Each router will have a diﬀerent vision of	444

the status of network occupation, and this vision in general	445

is not aligned with the real one. Similarly to the eﬀect of a	446

UNCORRECTED

PROOF

Network LSU rate (s

-1

)

Connection blocking probability

COMCOM 2938

18 January 2006	Disk Used

ARTICLE IN PRESS

S. Salsano et al. / Computer Communications xxx (2006) xxx–xxx

No. of Pages 11, Model 5+

Sankar (CE) / PadmaPriya (TE)

7

447 threshold mechanism this will cause the RDE not to always

ered value of 1/fnode. The impact on blocking probability

488

448 select the optimal paths for LSPs.

449	By means of simulations, we analyzed the impact of the

starts when the inter-arrival time of calls concerning a node	489

is in the order of the characteristic times of routing and sig-	490

450 inaccuracy on network performance. A scenario with no

451 thresholds is analyzed, in order to consider this phenome-

452 non in isolation, the load qSPis 0.7. Under the typical sce-

453 nario assumed so far, with the total requests arrival rate

naling procedures.

5. Combined routing/signaling processing cost

491

492

454	knode	of 0.07 s1, we noticed no impact of processing/trans-

455 mission delays. Therefore, we started to increase the rate of

456 incoming LSP requests in the network. To have a fair com-

457 parison, we kept the network load constant, therefore we

458 reduced the connection holding time. We were able to

In this section, we evaluate the processing cost of the	493

combined OSPF-TE/RSVP-TE architecture. We will show	494

that threshold mechanisms are eﬀective in decreasing the	495

load component due to OSPF-TE, and that the RSVP-	496

TE processing load must be carefully considered as it con-	497

459 understand when the considered delays start to be inﬂuent

stitutes the system bottleneck.

498

460 on network performance. Fig. 11	reports the connection

461 blocking probability and setup failures versus the total

462 arrival rate for the ‘‘ideal’’ system and the system with pro-

463 cessing/transmission delays. The blocking probability of

464 the ideal system is obviously not dependent on the arrival

465 rate. It can be seen that RSVP-TE and OSPF-TE messages

466 delays start to inﬂuence the connection blocking probabil-

467 ity in the system with processing/transmission delays when

The evaluation is based on the deﬁnition of a theoretical	499

model of processing costs, combined with the simulator	500

environment. Using our simulator, we can evaluate the	501

number (and the rate) of OSPF-TE ﬂooding procedures	502

that are started by a node. We can also count the number	503

of RSVP-TE messages (Path, Resv, PathTear, and Resv-	504

Tear). Then we are able to evaluate the total processing	505

cost by multiplying the processing cost of each message	506

468 the request rate is increased by a factor of 20. The degrada-

wmsgfor its rate rmsg.

507

469 tion of connection blocking is relatively mild, considering

470 that for an increase of request rate by a factor of 100, it

We will also conﬁrm the theoretic/simulation model	508

results with measurements performed in the tested, related	509

471 goes from 8% to 9.5%. On the other hand, the inaccurate

472 vision of network status causes a rapid growth of setup fail-

473 ures, which are almost null in our initial scenario with knode

474 of 0.07 s1. When	knode	is 20 times higher (	1.4 s1), the

to message rates and to the CPU load.

5.1. Message processing cost

510

511

475 setup failures are in the order of 3% of oﬀered calls.

476	In order to understand the previous results, consider

477 that a node is concerned by a connection when it is source,

478 destination or in the path of an LSP. Let fnode be the arrival

479 rate	of	Traﬃc	Demands	that	‘‘concern’’	a	node:

480	fnodeјknode рh ю 1Ю, where h	is the mean length of LSPs

Let us consider the diﬀerent components of processing	512

cost in a TE enabled MPLS network. A component is relat-	513

ed to the OSPF-TE messages due to the ﬂooding of state	514

information. Another component is the processing cost of	515

the LSP setup (and release) messages via RSVP-TE proto-	516

col. Due to the soft state approach, the processing related	517

481 that are setup (the blocking probability is neglected). 1/

to RSVP refresh messages must be also considered.

518

482	fnode	will be the mean inter-arrival time of two connections

483 that concern a node. Approximating	h	with the shortest

484 path, we have that 1/fnode = 3.25 s for	knode = 0.07 s1.

485 According to the assumed values, the characteristic times

486 of RSVP-TE and OSPF-TE procedures are in the order

487 of 50–100 ms, that is 30–60 times smaller than the consid-

0,14

Pb with delays

Pb "ideal"

The processing cost for each message obviously depends	519

on the speciﬁc implementation of OSPF-TE and RSVP-	520

TE. In general it can be dependent on the network topolo-	521

gy (e.g., on the size of the network) and on the network sta-	522

tus (e.g., number of established LSPs). In order to perform	523

our evaluation what we need is actually the relative pro-	524

cessing cost of the messages, rather that their absolute val-	525

ues. For this purpose, we take as reference the processing	526

cost of an OSPF Link Status Update (LSU) message con-	527

taining the ﬁrst copy of a Link State Advertisement (LSA)	528

0,12

0,1

0,08

0,06

0,04

0,02

0

Setup Failures with delays

0	2	4	6	8	10	12	14	16	18

| Request Rate per Node (s | -1 ) |
| --- | --- |

Fig. 11. Network performance vs. total request rate.

received by a router. We assume that one unit of processing	529

cost is needed to check that the LSA is not yet ‘‘installed’’	530

in the database, to install it and to prepare a copy of it to	531

be sent to all other interfaces but the receiving one. We can	532

now in general deﬁne the processing cost of the other mes-	533

sages with reference to this processing unit, using a set of	534

generic parameters as shown in the third column of Table	535

2. For example a1is the relative processing cost of a	536

‘‘Copy-LSA’’ message with respect to the ‘‘First-LSA mes-	537

sage. The processing cost of RSVP-TE messages is actually	538

split into two factors,	Q	and	bii = 1–5 for the diﬀerent	539

RSVP-TE messages.	Q	represents the relative processing	540

UNCORRECTED

PROOF

Connection Blocking Probability

8

COMCOM 2938

18 January 2006	Disk Used

ARTICLE IN PRESS

S. Salsano et al. / Computer Communications xxx (2006) xxx–xxx

No. of Pages 11, Model 5+

Sankar (CE) / PadmaPriya (TE)

Table 2

Control plane messages

Message

‘‘First-LSA’’

‘‘Copy-LSA’’

Path

Resv

PathTear

ResvTear

RefreshPath

Notation

wﬁrstLSA

wcopyLSA

wPath

wResv

wPathTear

wResvTear

wRefrPath

Processing unit

Generic

1

a1

Q

Qb1

Qb2

Qb3

Qb4

Assumed

1

0.5

5

6

3

7

2.5

50

40

30

20

10

0

0,1

OSPF No Thresholds

OSPF Dynamic Threshold (F=0.7)

RSVP No Thresholds

RSVP Dynamic Threshold (F=0.7)

0,2

0,3

0,4

RefreshResv

wRefrResv

Qb5

2.5

Request Rate per Node (s-1)

Fig. 12. Number of messages per second.

541 cost of a Path message with respect to a First LSA message:

542	Q = wPath/wﬁrst LSA. The factor bi, for each RSVP-TE mes-

543 sage represents its relative processing cost with respect to a

544 Path message.

545	The exact parameter values are obviously dependent on

546 the speciﬁc protocol implementations and also on the net-

547 work operating point. For the purpose of this paper, we

548 assumed reasonable values starting from the results avail-

549 able in the literature. In particular, [24] have been used to

550 infer the relative processing costs of RSVP-TE messages.

sponding the refresh rate RR (s1). In the following, we

will denote hLSPthe average number of hops of an LSP,

leaving out the dependence on the speciﬁc LSP x. A failed

setup of an LSP (see Fig. 13) will generate h(y) Path messag-

es, r(y) Resv messages up to the node where the reservation

fails, r(y) ResvError and ResvTear to tear down the part of

the LSP attempted to set up, and h(y)r(y)	PathError to

advertise source node about the setup failure.

583

584

585

586

587

588

589

590

551	[24]	has been compared to [23], where the processing cost

552 of OSPF messages is discussed, in order to estimate the val-

553 ue of Q. The RSVP-TE processing in typical implementa-

554 tions is dependant on the number	nlink	of active sessions

555 per link, that can be evaluated as

Utilizing our testbed implementation we measured the	591

exact number of messages exchanged among the nodes.	592

We studied the behavior of the whole architecture in term	593

of packets exchanged by the two protocols, OSPF-TE	594

and RSVP-TE, comparing a scenario without any thresh-	595

old mechanism with the one utilizing the Dynamic	596

557	nlinkјktotр1	PBЮT	h=L.

Thresholds with parameter	F	set to 0.7.	Fig. 12	reports

597

558	In our scenario we have a relatively low number of

559 active sessions per links (in the order of 20), therefore we

560 assumed a processing cost for RSVP-TE close to the min-

561 imum values reported in [24].

562	5.2. OSPF-TE and RSVP-TE message rates

563	According to the OSPF behavior, each ﬂooding proce-

564 dure results in the exchange of a number of LSU messages

565 that depends on the topology of the network. For a given

566 topology (only point-to-point links are considered) with

567	N	nodes and average degree	D, the number of messages

568 that	are	generated	by	each	ﬂooding	procedure	is

569	NЖ(D	1) + 1 (see Appendix C). These messages may cor-

570 respond to two diﬀerent processing costs in the node. If an

571 (Opaque) LSA is received from a router for the ﬁrst time, it

572 has to store it and to send it to all the interfaces. When fur-

573 ther copies of the same (Opaque) LSA are received, the

574 node simply discards them, resulting in a lower processing

575 cost. In particular in a ﬂooding procedure there will be

576	N	1 ‘‘ﬁrst-LSA’’s and NЖ(D	2) + 2 ‘‘copy-LSA’’s.

577	Each successful LSP setup will generate a number h(x)of

578 Path and Resv messages, where h(x)	is the number of hops

579 of the LSP x. The release of the same LSP will generate a

580 number h(x)	of PathTear and ResvTear messages. During

581 the lifetime of the ﬂow the soft state nature of the LSPs will

the results of these measures representing the message	598

rate for each protocol, in both scenarios, versus the	599

request rate per node knode. We can see that introducing	600

an eﬃcient threshold mechanism, OSPF-TE ﬂooding is	601

enormously reduced, while the number of RSVP messages	602

exchanged are ‘‘lightly’’ increased, by the presence of the	603

Setup Failures.	604

5.3. Deﬁnition of processing cost model and results	605

We started by considering the scenario where no ﬂood-	606

ing reduction techniques are used: a ﬂooding procedure is	607

executed for each state change. We consider the ideal case,	608

where there is no delay in transmission and processing of	609

OSPF-TE and RSVP-TE messages. Under these assump-	610

tions, the Edge Nodes have a perfect vision of the network	611

status and there will be no blocking at the RSVP-TE level.	612

Let ktotbe the total arrival rate of traﬃc demand to the net-	613

work, PCBRB	the blocking rate due to refusals of the CBR	614

algorithm in the originating Edge Node and nLSPthe mean	615

number of active LSP. The processing cost for this scenario	616

is	617

Ptotј 2ktotр1	PCBRBЮhLSP рN	1ЮwfirstLSAю2ktot

р1	PCBRBЮhLSP ЅN рD	2Ю ю 2 wcopyLSAюktot

р1	PCBRBЮhLSP рwPathюwResvюwPath	TearюwResv	TearЮ

582 originate h(x)	Path and Resv messages with a rate corre-

ю nLSP	RR	hLSP рwRefr	PathwRefr	ResvЮ.

619

UNCORRECTED

PROOF

Message rate (s

-1

)

COMCOM 2938

18 January 2006	Disk Used

ARTICLE IN PRESS

S. Salsano et al. / Computer Communications xxx (2006) xxx–xxx

No. of Pages 11, Model 5+

Sankar (CE) / PadmaPriya (TE)

9

620 The ﬁrst two terms represent the processing load for OSPF-

621 TE messages: each call setup that is accepted spans on

622 average hLSPlinks and on each links it triggers one ﬂooding

623 procedure for the setup and one for the release; the ﬂood-

624 ing procedure in turn generates (N	1) ‘‘ﬁrst’’ LSA mes-

625 sages and N(D	2) + 2 ‘‘copy’’ LSA messages. The third

626 term represents the RSVP-TE messages that are exchanged

627 during the successful setup and release of the LSP. The

628 fourth term takes into account the RSVP-TE messages

629 related to the maintenance of RSVP soft state: RR is the

300

250

200

150

100

50

0

OSPF-TE

RSVP-TE

Refresh

RSVP-TE

630 refresh rate (s1).

631	If we consider the scenario with ﬂooding reduction tech-

632 niques and real processing and transmission times of

633 OSPF-TE and RSVP-TE messages, the setup of an LSP

634 may fail with a probability	PRSVPB. The processing cost

635 can be represented by

1

Ptotј 2ktotр1	PCBRBЮhLSP рN	1ЮwfirstLSAю2ktot

0	0,1	0,2	0,3	0,4	0,5	0,6	0,7	0,8	0,9

Value of parameter F

Fig. 14. Total processing cost and its components.

To conﬁrm these theoretic values, we performed some	654

similar measurements in the testbed. We measured process-	655

ing load in each node in terms of percentage of CPU usage	656

M

in the two diﬀerent scenarios: the ﬁrst one without any	657

р1	PCBRBЮhLSP1 ЅN рD	2Ю ю 2 wcopyLSAюktot

threshold mechanism (upper part of Fig. 15) and the sec-

658

M

ond one where the Dynamic Threshold mechanism is	659

р1	PCBRBЮр1	PRSVPBЮhLSP рwPathюwResvюwPathTear

ю wResvTearЮ юktotр1	PCBRBЮрP RSVPBЮh0LSP рwPath

implemented with factor	F	set to 0.7 (bottom part of

Fig. 15). The ﬁgures show the measured CPU processing

660

661

637

ю xwResvюxwResvErrюxwResvTearю р1	xЮwPathErrrЮ

ю nLSPRR	hLSP рwRefr	PathюwRefr	ResvЮ.

loads related to the two protocols (averaged on all the net-	662

work nodes) versus the requests arrival rate. All measure-	663

ments were been taken in the testbed during simulations	664

638	We notice that the ﬁrst two terms are reduced by the

with network load qSP= 0.7. The reduction of OSPF ﬂood-

665

639 merit factor	M of the ﬂooding reduction technique. The

640 term related to the RSVP load has been split into two terms

641 that take into account the LSPs that are successfully setup

642 and the LSPs that are rejected by RSVP. h0LSP	is the mean

643 length of LSPs that experience a setup failure. The param-

644 eter x takes into account the number of hops of the LSP

645 that can be setup before ﬁnding a node that rejects the

646 request (see Fig. 13).

647	Fig. 14	reports the total processing cost versus the

648 parameter	F	of	dynamic	thresholds	(oﬀered	load

649	qSP= 0.7,	b = 0.05, NSFNET topology,). The total pro-

650 cessing cost is split among the routing component, the

ing by means of Dynamic Threshold mechanism signiﬁ-	666

cantly reduces the total processing load while the increase	667

of RSVP-TE load due to the presence of setup failures is	668

negligible.	669

2,5

No thresholds

2

1,5

651 RSVP-TE (setup and release) and the RSVP-TE refresh.

652 The processing cost of each message is as shown in the last

653 column of Table 2.

1

0,5

0

0,1

0,2

0,3

RSVP-TE

OSPF-TE

0,4

Path

Admission

Contol Error

Path Error

Path

Resv

Resv Error

Resv Tear

Path

Resv

Resv Error

Resv Tear

2,5

2

1,5

1

0,5

0

0,1

Request Rate per Node (s-1)

Dynamic  thresholds

No thresholds

RSVP-TE

0,2	0,3

Request Rate per Node (s-1)

OSPF-TE

0,4

Fig. 13. Failed setup procedure.

Fig. 15. Processing load.

UNCORRECTED

PROOF

Processing cost

CPU Load (%)

CPU Load (%)

COMCOM 2938

18 January 2006	Disk Used

10

ARTICLE IN PRESS

S. Salsano et al. / Computer Communications xxx (2006) xxx–xxx

No. of Pages 11, Model 5+

Sankar (CE) / PadmaPriya (TE)

670	The ﬁrst important result is that the use of dynamic

671 thresholds is eﬀective in reducing the overall processing

672 cost: RSVP-TE processing does not increase in a signiﬁcant

673 way due to setup failures when the network vision become

674 coarser. On the other hand, the overall reduction is less

675 than it was expected considering the large reduction of

676 OSPF-TE ﬂooding. The RSVP-TE cost component, which

677 is basically independent of the ﬂooding reduction technique

678 (see Fig. 14), accounts for the most part of the total pro-

679 cessing cost in the region where these ﬂooding reduction

680 techniques are eﬀective. In particular, the RSVP-TE refresh

90

75

60

45

30

15

0

logarithmic

3-piece linear

681 component has a great impact on the total processing (see

682	Fig. 14), suggesting that attention should be paid to reduce

683 it. In particular, aggregate refresh mechanisms, as well as

684 the reduction of refresh rate (we have considered the

685 default refresh rate of 1/30 s1) could be considered. Our

686 analysis suggests that while total OSPF-TE processing cost

0	1	2	3	4	5	6	7	8	9	10	11	12	13	14

Samples

Fig. 16. Static threshold levels.

ing to [25], the threshold levels can be arbitrarily ﬁxed while

723

687 can be controlled with dynamic threshold mechanisms, the

688 total RSVP-TE processing cost represents a potential

689 bottleneck.

the default is set to 14 levels. These 14 default levels actu-	724

ally deﬁne a 3-piece-linear function (see Fig. 16). We gener-	725

alize this function, assuming that each linear piece will	726

cover one third of the deﬁnition interval and considering	727

690	6. Conclusions

691	In this work, we ﬁrst analyzed the eﬀectiveness of the

two parameters	b	and	c	such that	F (1/3) = b	and	F (2/

3) = c (0 < b < c < 1). A speciﬁc threshold setting for this

family is identiﬁed by (M, b, and c). Therefore, there are

728

729

730

692 ﬂooding reduction techniques for OSPF-TE in a MPLS-

693 TE network. The trade-oﬀ between the amount of ﬂooding

694 and the connection blocking probability has been analyzed

695 for diﬀerent mechanisms. The result is the selection of the

696 dynamic threshold mechanism as the most eﬃcient and

697 simplest one.

698	This analysis has been ﬁrst performed assuming an

699 instantaneous propagation of the signaling/routing infor-

700 mation. Then, the transmission and processing delays of

701 OSPF-TE and RSVP-TE have been considered. This sec-

702 ond analysis was able to identify the operating conditions

703 under which these transmission/processing delays do not

704 impact on the network performance.

705	Finally, the aspects of combined processing cost for

706 routing and signaling have been analyzed. It is shown that

707 the signaling processing cost does not increase signiﬁcantly

708 when the ﬂooding reduction mechanism are used, therefore

709 the goal to reduce the overall processing cost is met. On the

710 other hand, the analysis showed that the processing cost of

711 signaling represents the largest part of processing cost and

712 may constitute the system bottleneck.

two degrees of freedom in adjusting the shape of the func-	731

tion to be sampled. The second family we considered is	732

based on a logarithmic function: F (x) = ln(ax)/ln(a), with	733

a	M. The parameter a deﬁnes the shape of the sampled	734

function, with small a	(e.g., a = 103) the function will be	735

more similar to a linear function. For higher	a	(e.g.,	736

a = 106) there will be less detailed information when the	737

link is not loaded and much more precise information	738

when the link is heavily loaded. Using this ‘‘logarithmic’’	739

mechanism, a speciﬁc choice of thresholds is identiﬁed by	740

(M, a), i.e., we have a single parameter to change.	741

Appendix B. Avoiding oscillations with static thresholds	742

The basic approach is to communicate the middle value	743

of	an	interval	when	a	threshold	is	crossed	[10]:	744

L(k) = (F (k/N) + F ((k + 1)/N))/2.	This	may	lead	to	745

unneeded ﬂooding when the bandwidth oscillates around	746

a threshold level. In	[25]	it is suggested to use diﬀerent	747

increase and decrease thresholds to notify the increase	748

and the decrease of bandwidth occupancy, trying to avoid	749

this oscillation. The ‘‘increase’’ threshold F+(k/N) and the	750

713	Appendix A. Families of static threshold mechanisms

714	Each family can be represented by an increasing func-

715 tion F (x) deﬁned in the interval 0 < x < 1, with range from

‘‘decrease’’ threshold F	(k/N) can be deﬁned starting from

F(k/N) as follows:

Fюрk=N Ю ј F рk=N Ю;

F рk=N Ю ю F ррk	1Ю=NЮ

751

752

754

716 0 to 1 and that is sampled at M equally spaced intervals

717 where M is the number of threshold levels. The threshold

F	рk=N Ю ј

2

.

756

718 values are equal to C Ж F(k/M) where 1 < k < M	1 and

On the other hand [25], considers to advertise the actual

757

719	C	is the link capacity. For example a linear function

720	F (x) = x will deﬁne M equally spaced threshold level.

721	The ﬁrst family that we have considered is a generaliza-

722 tion of the default threshold levels assumed in [25]. Accord-

value instead of a conventional value when a threshold is	758

crossed. When oscillating around a threshold value, for	759

example an increase threshold, a diﬀerent status will be	760

communicated each time that the threshold is crossed in	761

UNCORRECTED

PROOF

T

h

resh

o

ld

 L

evel

s (

%

 L

in

k

 Cap

aci

ty

)

COMCOM 2938

18 January 2006	Disk Used

ARTICLE IN PRESS

S. Salsano et al. / Computer Communications xxx (2006) xxx–xxx

No. of Pages 11, Model 5+

Sankar (CE) / PadmaPriya (TE)

11

762 the increase direction. Therefore, we decided to use the dif-

[7] B. Jamoussi et al., Constraint-Based LSP Setup using LDP, IETF	794

763 ferent increase and decrease thresholds and to communi-

RFC 3212, January 2002.

795

764 cate the middle value as follow:

[8] L. Andersson, G. Swallow, The Multiprotocol Label Switching	796

(MPLS) Working Group decision on MPLS signaling protocols,	797

766	LюрkЮ ј рFюррk ю 1Ю=NЮ ю F	рk=N ЮЮ=2;

IETF RFC 3468, February 2003.

798

768	L	рkЮ ј рF	рk=NЮ ю Fюррk	1Ю=N ЮЮ=2;

[9] A. Shainkh, J. Rexford, K.G. Shin, Evaluating the Overheads of	799

Source-Directed Quality-of-Service Routing’’, International Confer-	800

ence on Network Protocols (ICNP), 1998.

801

769 where L+(k) and L	(k) are the advertised level when the in-

770 crease threshold F+(k/N) and the decrease threshold F	(k/

[10] G. Apostolopoulos, R. Guerin, S. Kamat, S.K.Tripathi, Quality of	802

Service Based Routing: A Performance Perspective, SIGCOMM	803

771	N) are crossed.

1998.

804

772	Appendix C. Number of messages for a ﬂooding procedure

[11] G. Apostolopoulos, R. Guerin, S. Kamat, Implementation and	805

Performance Measurements of QoS Routing Extensions to OSPF,	806

Infocom, 1999.

807

773	Let dibe the degree of node i, N be the number of nodes, D

[12] R.R. Irashko, W.D. Grover, M.H. MacGregor, Optimal capacity	808

placement for path restoration in STM or ATM mesh-survivable	809

774 be the average degree of a node; assume that originating

networks, IEEE/ACT Trans. on Networking, June 1998.

810

775 node is n1. The originating node will send d1copies of the

[13] A. Basu, J.G. Riecke, Stability Issues in OSPF Routing’’, SIG-	811

776 message. Each other node i will send di1 copies (the node

COMM, 2001.

812

777 will not send the message on the receiving interface). Then:

XN

[14] G. Conte, P. Iovanna, R. Sabella, M. Settembre, L. Valentini, A	813

Traﬃc Engineering Solution for GMPLS Network: A Hybrid	814

Approach Based on Oﬀ-line and On-line Routing Methods, ONDM	815

NumOfMsg ј d1ю

iј2

рdi1Ю ј 1 юXNрdi1Ю

iј1

2003 Conference, February 4–6, 2003 Budapest, Hungary.

[15] MPLS-TE	control	plane	simulator	<http://www.coritel.it/

download.html>.

816

817

818

ј 1 юXN

iј1

diN ј 1 ю ND	N

[16] Sourceforge MPLS home page, <http://mpls-linux.sourceforge.net/>.

[17] IBCN testlab Home Page <http://dsmpls.atlantis.rug.ac.be/>.

[18] Zebra Home Page, <http://www.zebra.org/>.

819

820

821

779

ј N рD	1Ю ю 1.

[19] J. Moy, OSPF Version 2, IETF RFC 2328, April 1998.

822

[20] R. Coltun, The OSPF Opaque LSA option, IETF RFC 2370, July	823

1998.

824

780	References

781	[1] E. Rosen A. Viswanathan, R. Callon., Multiprotocol Label Switching

[21] A. Bosco, A. Botta, M. Intermite, P. Iovanna, S. Salsano, Distributed	825

Implementation of a Pre-Emption and Re-routing Mechanisms for a	826

Network Control Based on IP/MPLS Paradigm, ONDM 2003	827

782	Architecture, IETF RFC 3031, January 2001.

Conference, February 4–6, 2003 Budapest, Hungary.

828

783	[2] X. Xiao, A. Hannan, B. Bailey, L. M. Ni, Traﬃc Engineering with

784	MPLS in the Internet’’, IEEE Network, March/April 2000.

785	[3] D. Awduche et al., Requirements for Traﬃc Engineering Over MPLS,

[22] A. Bosco, A. Botta, G. Conte, P. Iovanna, R. Sabella, S. Salsano,	829

Internet like control for MPLS based traﬃc engineering: performance	830

evaluation, Performance Evaluation, vol. 59/2–3, February 2005,	831

786	IETF RFC 2702, September 1999.

Elsevier Science, pp 121–136.

832

787	[4] D. Katz, D. Yeung, K. Kompella, Traﬃc Engineering Extensions to

788	OSPF Version 2, IETF RFC 3630, September 2003.

[23] A. Shaikh, A. Greenberg, Experience in Black-box OSPF Measure-	833

ment, ACM SIGCOMM Internet Measurement Workshop (IMW),	834

789	[5] H. Smith, T. Li, IS–IS extensions for Traﬃc Engineering, IETF

November 2001.

835

790	<draft-ietf-isis-traﬃc-04.txt>, IETF RFC 3784, June 2004.

[24] K. Nemeth, G. Feher, I. Cseleny, Benchmarking of Signaling	836

791	[6] D. Awduche, L. Berger, D. Gan, T. Li, V. Srinivasan, G. Swallow,

792	RSVP-TE: Extensions to RSVP for LSP Tunnels, IETF RFC 3209,

793	December 2001.

Based	Resource	Reservation	in	the	Internet’’,	Networking

2000.

[25] CISCO on line documentation: ‘‘MPLS Traﬃc Engineering’’.

837

838

839

840

UNCORRECTED

PROOF